3GPP RAN WG2 Meeting #123	R2- 2308384

Toulouse, France, August 21-25, 2023

Agenda Item:	7.2.2

Source:	InterDigital, Inc.

Title:	Discussion on sidelink positioning

Document for:	Discussion

# 1. Introduction

In RAN2#122[1], there was some progress on SL positioning, but several areas still left open issues and require more clarification. In this contribution, we would like to discuss sidelink positioning open issues.

# 2. Discussion

## 2.1 Discovery messages

In RAN2#121 bis-e meeting, RAN2 made the following agreement [2].

Agreements: RAN2 confirms that discovery messages will be used to carry information for targeted discovery and candidate selection of SL positioning UEs, including at least the indication of anchor UE, target UE. and server UE roles. FFS how much information is indicated about anchor UEs (e.g., knowledge of location)

RAN2 confirmed that discovery messages include at least an indication of UE role (e.g., anchor UE, target UE, server UE). The following discovery models can be considered for discovery of anchor UEs for SL positioning.

Anchor UE initiated: LMF (or server UE) triggers an anchor UE and the anchor UE broadcasts (at least for target UE) its information.

Target UE initiated: Target UE sends a solicit message to discover anchor UEs (message 1). Anchor UE responds to the target UE (message 2).

Based on the above examples, the following proposal is made.

Proposal 1: Study discovery model(s) (e.g., anchor UE initiated, target UE initiated) to discover anchor UEs and contents of discovery messages for each model.

## 2.2 Criteria for Anchor UE selection

In RAN2#122 meeting, RAN2 made the following agreement [1].

Agreements: Anchor UE selection is supported by information about the candidate anchor UEs. At least the following list can be discussed for use in anchor UE selection: 1. UE roles 2. Supported positioning method 3. In coverage or not 4. RSRP 5. LOS/NLOS 6. Location 7. PLMN A normative requirement on which anchor UEs to select (e.g., ranking) will not be specified. RAN2 impact of this information to be determined. FFS which information would be determined statically/dynamically.

In RAN2#122, RAN2 agreed on the list of potential criteria for anchor UE selection. Information in the list seems to be useful for anchor UE selection but we need to specify more essential criteria for the SL positioning purpose. The supported positioning method is important. If the selected anchor UE does not support an SL positioning method for a target UE, the target UE cannot measure its location. The coverage information is also important. Based on the coverage information of anchor UEs, the target UE can determine UE-only operation or network-based operation. Moreover, a measured RSRP (e.g., SD-RSRP or SL-RSRP) is also an important criterion for anchor UE selection because RSRP is related to the quality of SL measurement results. Also, RAN2 considers which layer (e.g., AS layer and/or SLPP layer) performs the anchor UE selection procedure according to the specified criterion.

Proposal 2: At least positioning method, coverage and RSRP should be specified as selection criteria for anchor UE selection.

## 2.3 Anchor UE selection entity

For anchor UE selection, RAN2 may consider various cases and options can be considered. The selection means which entity (e.g., target UE or LMF (or server UE)) may perform anchor UE selection for SL positioning procedure among candidate anchor UEs based on anchor UE selection criterion.

Case 1) Selection with LMF involvement (e.g., IC or PC scenario)

Option 1: The target UE may provide the list of discovered candidate anchor UEs to the LMF and the LMF selects anchor UEs.

Option 2: The LMF may provide a list of candidate anchor UEs to the target UE and the target UE selects the anchor UEs within the list.

Case 2) Selection with server UE involvement (e.g., OoC scenario)

Option 3: The target UE may provide the list of discovered candidate anchor UEs to the server UE and the server UE selects anchor UEs

Option 4: The server UE may provide a list of candidate anchor UEs to the target UE and the target UE selects the anchor UE within the list.

For options 2 and 4, the target UE needs to be configured (or preconfigured in OoC) with some criterion for anchor UE selection by an LMF (or server UE). RAN2 can consider an assumption for the option 4. For instance, once an OoC target UE discovers a server UE (in advance to anchor UE being detected), the server UE may provide a list of anchor UEs for selection. It is beneficial that the target UE does not need to monitor discovery signals from anchor UEs.

Proposal 3: Specify which entity (e.g., target UE or LMF (or server UE)) performs anchor UE selection.

Proposal 4: For OoC, a target UE discovers the server UE and server UE provides a list of anchor UEs.

## 2.4 UE status change

So far, there was no RAN2 discussion on how to support service continuity. It’s not clear what happens if target UE or anchor UE status changed (e.g. switching between IC and PC, OoC and PC, IC and OoC) in the middle of SL positioning procedure.

For instance, an OoC target UE is connected with a server UE, but the target UE may change UE status IC (from OoC). In this case, the IC target UE may consider whether to connect with an LMF or to maintain the connection with the server UE. Given that some impacts are expected due to UE status change. Thus the status change cases should be studied.

Proposal 5: Study the impact of target or anchor UE status change (IC-PC, OoC-PC, IC-OoC) in the middle of the SL positioning procedure.

## 2.5 Partial coverage

In RAN2#120, RAN2 made the following agreement [3].

Agreements: Proposal 2 (modified) RAN2 to confirm either of UEs including target UE and one or multiple anchor UEs may be OOC in partial coverage scenarios, but with at least one UE being in coverage. How to enable the procedures/signaling for supporting SL positioning in partial coverage will be further discussed in normative work.

In WID [4] includes the partial coverage scenario.

Specify reporting signalling and procedures to facilitate support of SL positioning in all coverage scenarios and for PC5-only and joint PC5-Uu scenarios [RAN2, RAN3]:

In SA2 specification [5] includes the partial coverage scenario as a note.

### 5.5.2 Network based SL Positioning for UE with NAS connection A UE with a NAS connection is in RM-REGISTERED state. The UE can enter CM-Connected state by performing UE triggered Service Request for 5GC-MO-LR or performing Network triggered Service Request for 5GC-NI-LR or 5GC-MT-LR. As the Target UE can establish a NAS signalling connection with the AMF, the functionality specified in TS 23.273 [8] for location services can be reused including e.g. 5GC-MO-LR, 5GC-MT-LR and 5GC-NI-LR with the additional functionality captured in this clause. NOTE 1: Target UE can establish a NAS signalling connection directly or indirectly via a ProSe L2 UE-to-Network Relay.

According to the SA2 specification, both UE-only operation (e.g., OoC) and network-based (e.g., IC) operation are specified, but how to support a partial coverage scenario is not specified. Also, in RAN2 perspective, how/when to support partial coverage scenario (e.g., procedure/signaling) is not discussed.

Given that a target UE is OoC and SL-MO-LR is triggered, the target UE may perform an SL positioning discovery. If (at least) one IC anchor UE is detected, the target UE may not perform UE-only operation since one of the condition for selecting UE-only operation is not satisfied (e.g., Neither Target UE nor SL Reference UE is served by NG-RAN)[5].

In this case, the target UE can perform a network-based operation via the detected IC anchor UE with a partial coverage scenario (i.e., at least one UE being in coverage). However, the target UE is not sure whether the detected IC anchor UE supports a relay function (e.g., a ProSe L2 U2N) since the SL positioning discovery procedure is not related to relay UE discovery. The target UE needs to check the relay function of the detected IC anchor UE. For example, the target UE can exchange a relay capability via the RRC layer with the IC anchor UE or ProSe layer or can trigger a relay discovery procedure. After the checking procedure, once detecting the IC anchor UE does not support the realy function, the target UE can perform UE-only operation.

Proposal 6: RAN2 considers how to support the SL positioning in a partial coverage scenario while at least one anchor UE is IC.

## 2.6 SL resource pool selection

In RAN1#112 meeting, RAN1 made the following agreement [6].

Agreement - A UE can be configured to perform either resource allocation Scheme 1 or Scheme 2, applicable to all resource pools (dedicated or shared resource pools). - SL PRS unicast/groupcast/broadcast can occur in either a shared or a dedicated resource pool.

RAN1 agreed that a UE is configured with two kinds of SL resource pools (e.g., dedicated SL resource and shared SL resource pool) for SL-PRS transmission. According to the RAN1 specification [9], when an SL resource selection is triggered, SL resource pool is selected by the upper layer and the selection is informed to the physical layer.

In the same way, when an SL resource for SL-PRS is triggered, either a dedicated SL resource pool or a shared SL resource pool needs to be selected by the upper physical layer based on some conditions. For example, SL positioning QoS, prioritization rule (e.g., prioritized dedicated SL resource pool, if configured). Hence, RAN2 needs to specify when/how to select a dedicated SL resource pool or a shared SL resource pool (e.g., SL resource pool selection procedure on MAC specification).

Proposal 7: RAN2 considers how to select a dedicated SL resource pool or a shared SL resource pool.

## 2.7 Request for SL-PRS resource

In RAN1#113 meeting, RAN1 made the following agreement [7].

Agreement In a shared resource pool, SL-PRS, associated PSCCH and PSSCH scheduled by the PSCCH are included in the same slot: - The PSSCH is used for 2nd SCI and SL-SCH - Note: the UE may not have data available for transmission. Up to RAN2 how to define the specification support for this case.

According to RAN2 specification, when a UE (e.g., mode 1) has SL data, the UE may provide a network with information about SL data volume. To provide the SL data volume, the UE may trigger an SL-BSR (e.g., MAC CE) and transmit it to the network. Upon receiving the SL-BSR, the network may allocate appropriate SL resource (e.g., SL grant) to the UE. The SL-BSR comprises one or more fields. For example, destination index, SL logical channel group LCG ID, and a buffer size [8].

For SL positioning, a UE (e.g., mode 1) may trigger an SL-BSR to request an SL resource to a network when the UE needs to transmit SL-PRS. Considering the SL-PRS, the SL-PRS is not SL data and thus not associated with a specific SL logical channel (or SL logical channel group ID) and a buffer size (e.g., amount of available data from all SL logical channels). Based on this observation, the UE cannot request an SL resource with the current one or more fields of the SL-BSR. To request an SL resource for SL-PRS transmission of the SL positioning, a new MAC CE (e.g., SL-BSR) or UCI (e.g., SR) or new fields within the SL-BSR can be considered.

When a dedicated SL resource pool is selected, the UE (e.g., no available SL data) can send an UCI to request an SL resource for SL-PRS. For example, one or more UCI are (pre-)configured to request a SL resource for SL-PRS transmission. In another way, the UE can send a new MAC CE (e.g., similar SL-BSR). For example, the new MAC CE comprises one or more fields/indications (e.g., one field for destination index, one field for a request of SL resource for SL-PRS transmission).

When a shared SL resource pool is selected, the UE (e.g., having SL data) can send the current SL-BSR including one or more new fields/indications (e.g., one field for destination index, one field for a request of SL resource for SL-PRS transmission).

Proposal 8: RAN2 considers how to request an SL resource (e.g., mode 1) for SL-PRS transmission.

## 2.8 SL-PRS priority

In RAN1#112 meeting, RAN1 made the following agreement [6].

Agreement: From RAN1 perspective, priority value for SL PRS should be provided by higher layers from Tx UE perspective

The SL priority value is needed as an indication of the first stage SCI field as well as mode 2 resource selection procedure. In SL data transmission, an SL priority is associated with an SL logical channel and defined QoS (e.g., mapping between a QoS flow and SL data radio bearer).

For instance, when SL data belong to an SL logical channel, the SL priority is determined based on the associated with the SL logical channel. In contrast, considering an SL-PRS, the SL-PRS is not SL data and thus not associated with a specific SL logical channel as well as defined QoS, so a priority value for SL-PRS cannot be determined based on the SL logical channel.

To determine an SL priority value, RAN2 already discussed how to determine the SL priority value but has not reached an agreement [1]. For example, the SL priority value is determined from SLPP transport QoS or Ranging/SL positioning QoS (e.g., from SLPP layer). Based on the previous discussion, RAN2 can agree that an SL priority value for an SL-PRS can be determined based on QoS of an SL positioning (e.g., accuracy and/or latency).

Proposal 9: RAN2 determines an SL priority value for SL-PRS, given defined SL positioning QoS.

# 3. Conclusion

In this contribution, the following proposals were made:

Proposal 1: Study discovery model(s) (e.g., anchor UE initiated, target UE initiated) to discover anchor UEs and contents of discovery messages for each model.

Proposal 2: At least positioning method, coverage and RSRP should be specified as selection criteria for anchor UE selection.

Proposal 3: Specify which entity (e.g., target UE or LMF (or server UE)) performs anchor UE selection.

Proposal 4: For OoC, a target UE discovers the server UE and server UE provides a list of anchor UEs.

Proposal 5: Study the impact of target or anchor UE status change (IC-PC, OoC-PC, IC-OoC) in the middle of the SL positioning procedure.

Proposal 6: RAN2 considers how to support the SL positioning in a partial coverage scenario while at least one anchor UE is IC.

Proposal 7: RAN2 considers how to select a dedicated SL resource pool or a shared SL resource pool.

Proposal 8: RAN2 considers how to request an SL resource (e.g., mode 1) for SL-PRS transmission.

Proposal 9: RAN2 determines an SL priority value for SL-PRS, given defined SL positioning QoS.

# 4. References

[1] RAN2#122 chairman notes.

[2] RAN2#121 bis-e chairman notes.

[3] RAN2#120 charman notes.

[4] RP-223549, new WID on Expanded and Improved NR Positioning.

[5] 3GPP TS 23.586.

[6] RAN1#112 chairman notes.

[7] RAN1#113 chairman notes.

[8] 3GPP TS 38.321.

[9] 3GPP TS 38.214.