# 3GPP TSG RAN WG2 Meeting #123                       	    R2-2308396

# 21 August – 25 August 2023, Toulouse FR

Agenda item:	7.2.2

Source:	Qualcomm Incorporated

Title:	Sidelink Positioning Protocol (SLPP) Signaling and Procedures

Document for:	Discussion and Decision

# Introduction

This contribution discusses SLPP design issues including reliable transport, session operation, positioning procedures and transaction modes.

# Reliable transport

RAN2 has taken the following agreements for reliable transport [5],

Agreements: SLPP over PC5-U/Uu will support reliable transport for at least unicast. FFS groupcast. Inform SA2 about our agreements on sidelink positioning, with “take into account” action. SLPP carried over NAS is used between UE and LMF. FFS on how to manage the session/transaction.

Sidelink communication may be susceptible to packet loss due to changes in UE separation/proximity, such as in the case of moving UEs, or due to occlusion between UEs. Such behavior is a characteristic of sidelink communication regardless of cast or transport type, with packet loss possible for operation over unicast, broadcast or groupcast. This packet loss may lead to the loss of SLPP messages, thereby degrading sidelink ranging and positioning performance and compromising the ability to conduct and exchange range and/or position measurements and measurement results.

In the case of LPP for Uu operation, loss of LPP messages is also possible when an LPP message is not successfully forwarded by an MME or AMF. Consequently, LPP already contains a reliable transport mechanism, including acknowledgment and retransmission capability at the LPP level. Since PC5-U transport does not natively provide a reliable transport mechanism, the LPP acknowledgment and retransmission capability could also be supported by SLPP, enabling SLPP procedures to be designed with redundancy and tolerance to transport failure. With redundancy a UE can send an SLPP request message to a group of UEs with each UE in the group responding to acknowledge receipt of the message or responding to acknowledge having performed or being able to later perform the requested operation (e.g., transmission and/or measurement of sidelink reference signals). If each response also carries the original information from the SLPP request message, any UE that did not receive the SLPP request message but receives at least one response message will be able to infer and reconstruct the original SLPP request and thereby overcome the original SLPP message loss. With LPP-based reliability applied to SLPP, the sender of an SLPP request message to a group of UEs can determine when the message was not received by target UEs, and subsequently take appropriate action(s), such as resending the request message at a later time, or not expecting the non-responding UEs to conduct the requested actions.

LPP-based reliability implemented as part of SLPP is relevant to session-based operation, where a group of UEs engaged in a sidelink ranging and positioning session would benefit from reliable transport, in particular if the resulting range/position is applied to group operation. A session leader (initiator) could choose to invoke reliability if the leader determines it is appropriate for the session. In contrast, session-less operation seeks to minimize overhead and latency for ranging/positioning between UEs having a temporally brief ranging/positioning interaction, and as such, may not require reliable transport. Hence, the initiator of a session-based or session-less sidelink ranging and positioning transaction should have the flexibility of deciding whether or not to invoke SLPP reliable transport. Furthermore, LPP-based reliability implemented in SLPP provides the additional advantage that SLPP can operate reliably between a UE and an LMF.

Observation 1: Communication over sidelink may incur packet loss due to changes in participating UE relative position and/or occlusion, potentially leading to SLPP message loss. Packet loss and resulting SLPP message loss over sidelink is independent of cast type, occurring for operation over unicast, broadcast and groupcast, and will result in degraded performance and compromise the ability to conduct and exchange range and/or position measurements and measurement results.

Observation 2: Introducing to SLPP LPP-like reliable transport mechanisms of duplicate detection, acknowledgement and retransmission can be applied to SLPP to mitigate the effects of sidelink packet loss.

Observation 3: Invocation of SLPP reliable transport can be at the discretion of the sidelink ranging and positioning session initiator.

Proposal 1:	SLPP supports at least the LPP reliable transport mechanisms for duplicate detection, acknowledgement and retransmission for unicast, broadcast and groupcast.

# SLPP Location Information exchange over Groupcast/Broadcast

For groupcast and broadcast transmission of SLPP, and group positioning, RAN2 has reached the following agreements [3],

Agreement: RAN2 confirm that from RAN2 perspective, it is feasible to send at least the following positioning signaling for groupcast/broadcast (in addition to unicast): - SL positioning capability - SL positioning assistance data Location information is not excluded and can be further considered in normative work.

Agreements: WA: RAN2 understand that group positioning is to acquire location estimates of multiple target UEs (absolute positioning) or multiple UE pairs (Ranging/relative positioning) per LCS request, in line with the guidance already received from SA2. WA: At least part of the group management for group positioning is performed at upper/application layer.

Sidelink positioning will support numerous use cases, including V2X, public safety, commercial, and IIOT [1]. These use cases may involve stationary UEs, moving UEs or a combination/group of stationary and moving UEs. An individual UE in one of these use cases may initiate a sidelink positioning session with one or more UEs in its vicinity (UEs with which it can establish sidelink communication). Those UEs may constitute all the UEs in the initiating UE’s vicinity, a subset of the UEs in the initiating UE’s its vicinity, or only a single UE in the initiating UE’s vicinity.

The importance of group-based use cases for sidelink ranging and positioning has been further reinforced by SA2 in [21], citing as additional examples an RSU and multiple vehicles, multiple vehicles in a platoon, MCX service and Museum Tour.

A possible example of sidelink ranging/positioning for a V2X use cases is illustrated in Figure 1. In this example, UE1 initiates a sidelink positioning session with the sole UE in its vicinity in Figure 1(A). In this case, both UE1 and UE2 may require the location information resulting from the ranging/positioning session in order to facilitate subsequent action. In Figure 1(B), UE1 initiates a sidelink positioning session with a subset of the four UEs in its vicinity. Similar to Figure 1(A), all three UEs engaged in the ranging/positioning session may require location information for all the session participants in order to facilitate subsequent actions/maneuvers.  In Figure 1(C), UE1 initiates a sidelink positioning session with all of the UEs in its vicinity (4 adjacent UEs). As with Figure 1(A) and Figure 1(B), all five UEs engaged in the session may require location information results for all session participants. Note that although this example is provided for a V2X use case, the three scenarios illustrated are equally applicable to the public safety, commercial and IIOT use cases.

Observation 4: A UE may initiate a sidelink positioning session with one of the UEs in its vicinity, or with a group of UEs in its vicinity. When initiating a sidelink positioning session with a group of UEs, the group could constitute all, or a subset of the UEs in the initiating UE’s vicinity.

Observation 5: When a group of UEs participate in a sidelink ranging and positioning session, a subset or all of the UEs in the session may require location information from all of the other UEs in the ranging/positioning session.

Proposal 2: 	RAN2 agrees to convert the following WA to an agreement, “WA: RAN2 understand that group positioning is to acquire location estimates of target UEs (absolute positioning) or multiple UE pairs (Ranging/relative positioning) per LCS request, in line with the guidance already received from SA2.”

Figure 1: Sidelink Positioning/Ranging example scenarios for a group of UEs

RAN2 has agreed that unicast, groupcast or broadcast may be used for transmission of Capability exchange and Assistance Data transfer [3]. RAN2 has received from SA3 confirmation that security for sidelink ranging and positioning signaling over groupcast and broadcast is feasible [23], and that SA3 is actively designing a security solution. Per the description of Figure 1, there are important sidelink ranging and positioning use cases which require distribution of Location Information results to all group participants. Additionally, the following considerations further reinforce that groupcast/broadcast transmission should be supported for Location Information exchange.

- Security: SA3 has agreed that security can be implemented for sidelink ranging and positioning signaling sent over groupcast or broadcast and is actively designing a security solution to do so. An SA3 solution for secure SLPP transmission over groupcast or broadcast removes any impediments or obstacles to secure distribution of Capability, Assistance Data, and Location Information over groupcast or broadcast.

- 3GPP Precedence: Existing application-layer broadcast messages carried as 3GPP sidelink V2X payload defined by SDOs such as ETSI-ITS, SAE and CSAE include UE GNSS position and UE GNSS position accuracy (e.g., CAM, BSM, CPM, SDSM, etc.). Since SLPP is also carried as V2X payload, transmission of location information should not present a risk to UE operation.

- Sender UE Discretion: Distribution of Location Information can be at the discretion or choice of the sending (transmitting) UE. If the sender is subject to policy or other restrictions regarding Location Information exchange, it can elect not to distribute Location Information.

Observation 6: Security for groupcast or broadcast transmission is not an impediment to Location Information exchange over groupcast/broadcast.

Observation 7: Existing 3GPP V2X payload standardized messages support exchange of UE GNSS location and UE GNSS location accuracy over groupcast and broadcast.

Proposal 3:	SLPP supports Location Information exchange over unicast, groupcast and broadcast.

# SLPP Session-based and Session-less Operation

RAN2 has reached the following agreements regarding session-based and session-less operation [3], [4], [5].

Agreement: Sidelink positioning supports a session-based concept in SLPP, in which signalling messages within a session can be associated with one another by the involved UEs. The relationship to upper-layer designs from SA2 can be discussed during normative work. FFS if there is also sessionless operation and what aspects of session-based operation would not be included.

Agreement: At least in the case that positioning methods are supported that do not require a mutual exchange of SLPP messages associated with one another among UEs, SLPP sessionless operation can be supported. FFS if sessionless operation can be operated with security.

Agreements: R2 agree that for session-based SLPP, a SLPP session is used among UEs in PC5-only case in order to obtain location related measurements/location estimates, to transfer assistance data, or to exchange of capabilities. RAN2 agree that for session-based SLPP, a single SLPP session is created to support a single location request at least in case of a single target UE; FFS how sessions work if there are multiple target UEs in a single location request. RAN2 agree that, for session-based SLPP, SLPP transactions are indicated at the SLPP protocol level with a transaction ID in order to associate messages with one another (e.g., request and response) RAN2 agree that for session-based SLPP, messages within a transaction are linked by a common transaction identifier.

Agreements: SLPP can support multiple target UEs in the same session when LCS requests. RAN2 will not specify group management for multiple target UEs. RAN2 assumption is that a group ID will be provided from upper layers. FFS how session IDs are managed between multiple UEs.

For sidelink positioning and ranging scenarios where the participating UEs are aware of each other’s presence (for example if the three participating UEs in Figure 1(B), or the five participating UEs in Figure 1(C) have an established awareness or association), RAN2 has agreed to support SLPP session-based operation [18]. For sidelink positioning and ranging scenarios that do not require mutual exchange of SLPP messages associated with one another, RAN2 has agreed to support SLPP session-less operation (for example if participating UEs do not have an established association).

Determination of session-based or session-less operation could be made by the Application layer based on the specific scenario, use case and determination of participant UEs (for example perform position/ranging for a specific set of UEs or perform position/ranging for a group of proximate UEs). The subsequent session-based or session-less transactions would then be instantiated by SLPP. The following section provides descriptions of possible SLPP session establishment, termination and modification.

## SLPP Session and Service Management

LPP is designed and intended for Uu-based positioning and does not support explicit management of LPP sessions and services. As a result, SLPP messages based solely on LPP may be insufficient for all aspects of sidelink ranging and positioning.

To address this functionality gap in LPP for sidelink ranging and positioning, SLPP can include as part of the Ranging/Sidelink Positioning layer, a session and service management function in addition to a positioning function. These two functions can be considered as a positioning sublayer, and as a session and service management sublayer, respectively. The positioning sublayer would support the SLPP positioning procedures described in section 5, and can be based on LPP. The SLPP session and service management sublayer can support those aspects of SLPP session-based operation that are not natively supported in LPP. This includes identifying participating UEs and accepting, rejecting and modifying SLPP sessions and services, including one-time location, periodic location or triggered location.

Identifying the UEs participating in a positioning session, and disseminating the set of UEs engaged in the session to all the session participants can benefit certain sidelink ranging/positioning use cases. This includes positioning sessions in which the participating UEs may need to act based on range and/or position knowledge of the other UEs. V2X, where vehicle maneuvers may be predicated on range and/or position knowledge of surrounding vehicles, is a key such use case. Equally important is identifying a change in session participants (UEs joining or leaving a positioning session). No such requirements exist for LPP, as LPP involves one UE and one LMF and does not support the concept of a UE joining or leaving an LPP session.

The SL-MO-LR positioning procedure defined by SA2 [24] (Figure 2, Step 4) highlights the importance of SLPP session management. As noted in the SA2 CR, each UE verifies over PC5 that Ranging/SL positioning is permitted. This verification is not native to LPP, and must therefore be supported outside of LPP. As captured in the Editor’s note below Step 5, support for this verification needs to be confirmed by RAN2. Since SLPP is the protocol currently being designed to manage sidelink ranging and positioning by RAN2, and since SA2 has highlighted the benefit of UEs confirming engagement in a sidelink ranging/positioning session, inclusion of this functionality in SLPP is a natural aspect of the protocol, and therefore should be confirmed by RAN2.

Proposal 4: 	RAN2 confirms UEs in a sidelink positioning procedure verify aspects of the sidelink ranging and positioning session over PC5 via SLPP, including that ranging and sidelink positioning is permitted, ranging and sidelink positioning results may be transferred to an LCS client or AF.

Figure 2: S2-2307552, Figure 6.x.1-1: SL-MO-LR Procedure

More generally, LPP sessions may differ from sidelink SLPP sessions in the following aspects:

Session Size and Participants:

- LPP:  An LPP session is always a one-to-one association between one UE and one location server. Multiple LPP sessions can be used between the same endpoints to support multiple different location requests.

- SLPP: A UE may simultaneously participate in multiple SLPP sessions, as a function of the particular ranging/positioning use case (V2X, MCX, public safety, etc.). Each separate SLPP session may be with a different individual UE, with a different group of UEs, or with overlapping groups of UEs. A UE must maintain knowledge and status of each distinct session it is engaged in, including the session participants and session services.

Session Server:

- LPP:  In LPP sessions, the location server (e.g., LMF in the case of NR) is always assumed to control the LPP session (LPP sessions are not controlled by the UE). The location server (e.g., LMF) decides on the positioning methods and measurements based on the UE capabilities. In particular, only after an LMF sends at least one DL LPP message towards the UE, can the UE then send one or more UL LPP messages towards the LMF.

- SLPP: In SLPP sessions, a UE may decide on the positioning methods and measurements based on the UE capabilities. In SLPP sessions a UE may take the role of positioning server. In particular, in SLPP a UE can initiate a SLPP session.

UE Session Participation:

- LPP:  LPP does not support the concept of a UE deciding not to participate in (reject) an LPP session. This functionality required the introduction of another protocol layer (Supplementary Services (SS) Location Notification and Privacy Verification), since (among others) the LMF may select a non-LPP positioning method (e.g., network-based positioning).

- SLPP: A UE should decide whether to participate in an SLPP session based on UE-specific criteria, including capabilities, privacy restrictions, power consumption. When capable of participating in multiple SLPP sessions, a UE should apply these criteria to accept, reject or terminate session participation. Since sidelink positioning and ranging always requires support of SLPP, introducing a new protocol layer for this function (as is the case in Uu positioning) results in unnecessary complexity, overhead and specification effort.

Session characteristics:

- LPP:  In LPP sessions, the UE normally assumes one-time location. Periodic and triggered location is handled at supplementary services layer.

- SLPP: Each SLPP session a UE is simultaneously participating in may have different requirements for periodicity, ranging, positioning, duration, methods used, privacy, etc. A UE must maintain knowledge and status of each distinct session it is engaged in, including the session participants and session services. Since sidelink positioning and ranging always requires support of SLPP, introducing a new protocol layer for this function (as is the case in Uu positioning) results in unnecessary complexity, overhead and specification effort.

Session Modification:

- LPP:  LPP does not support the concept of modifying service requirements or session participants during an LPP session.

- SLPP: In SLPP sessions for group positioning, RAN2 has assumed group identification is provided by the upper layers (an assumption consistent with the group management requirements of TS 23.287 and TS 24.587). The SLPP Capabilities exchange (SLPP Request Capabilities / SLPP Provide Capabilities) initiating an SLPP session may result in only a subset of the UEs in the upper layer defined group having compatible capabilities, and hence the need to modify the set of UEs participating in the SLPP session.  This is further described in section 4.1.1, below.

Observation 8: A UE must be able to support multiple, simultaneous Sidelink ranging and positioning SLPP sessions, each with potentially differing sets of UEs and potentially different sets of positioning requirements. A UE participating in multiple, concurrent sessions should be able to track each individual session.

Proposal 5: 	The SLPP Layer may assign an SLPP Session ID unique to the specific sidelink positioning session. The SLPP Session ID could be an integer value or might comprise a unique indication of the initiator UE plus an initiator UE-assigned local numeric ID.

Observation 9: UE-specific criteria including at least capabilities and privacy requirements, are the basis for UE to determine participation in sidelink ranging and positioning SLPP session.

Proposal 6:	SLPP supports the ability for a UE to explicitly accept, reject or terminate an SLPP session based on UE-specific criteria.

### SLPP Session and Service Management for Group Positioning

For group positioning RAN2 has reached the following agreements and working assumptions [4], [5].

Agreements: SLPP can support multiple target UEs in the same session when LCS requests. RAN2 will not specify group management for multiple target UEs. RAN2 assumption is that a group ID will be provided from upper layers. FFS how session IDs are managed between multiple UEs.

Agreements: WA: RAN2 understand that group positioning is to acquire location estimates of multiple target UEs (absolute positioning) or multiple UE pairs (Ranging/relative positioning) per LCS request, in line with the guidance already received from SA2. WA: At least part of the group management for group positioning is performed at upper/application layer.

RAN2’s agreement regarding an upper layer provided group ID for group positioning is consistent with the assumptions and requirements in TS 23.287 [13] and TS 24.587 [16] for V2X Application Layer managed groups, where the V2X Application Layer may provide to the lower layers a group definition consisting of the set of {Group Identifier, Group Size and Member IDs}. When the Application Layer provides a managed Group Identifier, the UEs in the managed group all determine the same groupcast destination Layer-2 ID from the group identifier using the procedures defined in TS 24.587 [16] (note that group management by the Application Layer is currently out of scope for 3GPP).

An SLPP session for an Application Layer managed group could be initiated by a UE transmitting an SLPP Request Capabilities message to the group members, as in Step 1 of Figure 3. The SLPP Request Capabilities message could be transmitted using the groupcast destination layer-2 ID derived from the Group ID provided by the upper/application layer (as noted, all UEs in the Application layer managed group are cognizant of the Group ID and also derive the same groupcast destination layer-2 ID from the Group ID). The resulting SLPP Provide Capabilities messages sent in response by the other UEs in the Application layer managed group (UE-C, UE-D, UE-A) may determine that only a subset of the UEs in the managed group have compatible sidelink positioning capabilities. This is in contrast to Uu LPP-based positioning, where only a single UE at a time is participating and the LPP session need only match the individual UE’s capabilities.

In cases where not all the UEs in an Application Layer managed group have compatible and sufficient sidelink positioning capabilities, there is no mechanism within the basic three SLPP message types (Capability exchange, Assistance Data exchange, Location Information transfer) to remove the incompatible UEs from the managed group (e.g., by providing a new group definition), or to direct incompatible/less capable UEs not to participate in the SLPP session. This could result in inefficiencies through extraneous transmissions and unnecessary processing, as illustrated in Figure 3 for a UE-assisted sidelink positioning SLPP session for an Application layer managed group. In this example, after receiving the SLPP Request Capabilities message from UE-B, the other UEs in the Application layer managed group (UE-C, UE-D, UE-A) respond by transmitting their respective capabilities in the SLPP Provide Capabilities message (also transmitted via groupcast to the groupcast destination layer-2 ID derived from the group identifier). Upon receipt of the capabilities from other UEs in the Application layer managed group (UE-C, UE-D, UE-A), the initiating UE (UE-B) will determine that only UE-C and UE-D have SLPP capabilities compatible with UE-B, and that UE-A is incompatible. Without a signaling mechanism to inform the other UEs in the Application layer managed group (UE-C, UE-D, UE-A) of a revised SLPP positioning group consisting of the subset of compatible UEs (UE-C, UE-D and UE-B), and a revised group definition (groupcast destination layer-2 ID), the incompatible UE (UE-A) may transmit SL-PRS, perform SL-PRS measurements, receive and process location information results, and the initiating UE (UE-B) may attempt to continue to perform position/range calculations for UE-A.

Similar to UE removal from an SLPP session as a result of incompatible capabilities, for UEs which do not wish to be part of, or cannot participate in an SLPP session (e.g., due to privacy restrictions or restrictions current UE loading), should be able to reject an SLPP session request. This is the case in LPP, where a UE is always free to reject a session initiated by a location server by returning reject or error messages or not responding to the location server.

Figure 3: UE-Assisted Sidelink Positioning for an Upper/Application-Layer Managed Group

- The Application layer in UE-B provides managed group definition to the SLPP layer including group identifier, group size and member IDs. The Application layer in UE-B then requests sidelink ranging and positioning for the managed group from the SLPP layer.

- All UEs in the Application-Layer managed group (UE-A, UE-B, UE-C, UE-D) know the group information (group identifier, group size).

- All UEs in the Application-Layer managed group (UE-A, UE-B, UE-C, UE-D) use the group identifier to derive a Groupcast destination Layer-2 ID for the managed group.

NOTE:	The exact API between Application Layer and SLPP layer can be regarded as implementation dependent.

- UE B transmits an SLPP Request Capabilities message to the other UEs in the Application layer managed group (UE-C, UE-D, UE-A) requesting the SL positioning capabilities from the UEs participating in this session. The SLPP Request Capabilities is transmitted via groupcast to the groupcast destination layer-2 ID derived from the Application-layer provided group identifier.

- The other UEs in the Application layer managed group (UE-C, UE-D, UE-A) respond with a SLPP Provide Capabilities message. The SLPP Provide Capabilities message is transmitted via groupcast to the groupcast destination layer-2 ID derived from the Application-layer provided group identifier.

- Upon receipt of the SLPP Provide Capabilities messages from the other UEs in the Application layer managed group (UE-C, UE-D, UE-A), UE-B determines that UE-C, UE-D and UE-B form a subset of UEs in the Application layer managed group with compatible SLPP capabilities. UE-B also determines that UE-A has incompatible SLPP capabilities.

- UE-B transmits an SLPP Provide Assistance Data message to the other UEs in the Application layer managed group (UE-C, UE-D, UE-A). The SLPP Provide Assistance Data message is transmitted via groupcast to the groupcast destination layer-2 ID derived from the Application-layer provided group identifier.

Note:  Without a signaling mechanism to either inform the other UEs in the Application layer managed group (UE-C, UE-D, UE-A) of a revised SLPP positioning group consisting of the compatible UEs (UE-C, UE-D and UE-B) and a revised group definition removing the incompatible UE (UE-A) from the group, UE-B transmits the SLPP Provide Assistance Data message via groupcast to the groupcast destination layer-2 ID derived from the Application-layer provided group identifier, resulting in all UEs in the Application layer managed group, including the UE (UE-A) with incompatible SLPP capabilities receiving the message.

- UE-B transmits an SLPP Request Location Information message to the other UEs in the Application layer managed group (UE-C, UE-D, UE-A). The SLPP Request Location Information message is transmitted via groupcast to the groupcast destination layer-2 ID derived from the Application-layer provided group identifier.

Note:  Without a signaling mechanism to either inform the other UEs in the Application layer managed group (UE-C, UE-D, UE-A) of a revised SLPP positioning group consisting of the compatible UEs (UE-C, UE-D and UE-B) and a revised group definition removing the incompatible UE (UE-A) from the group, UE-B transmits the SLPP Request Location Information message via groupcast to the groupcast destination layer-2 ID derived from the Application-layer provided group identifier, resulting in all UEs in the Application layer managed group, including the UE (UE-A) with incompatible SLPP capabilities receiving the message.

- SL-PRS transmission and SL-PRS measurements are performed by all UEs in the Application layer managed group, including the UE with incompatible SLPP capabilities (UE-A). This results in extraneous, unnecessary SL-PRS transmissions and extraneous, unnecessary SL-PRS measurements and processing by all UEs in the group.

- The other UEs in the Application layer managed group (UE-C, UE-D, UE-A) each transmit a SLPP Provide Location Information message with the location results requested at Step 5. The SLPP Provide Location Information message is transmitted via groupcast to the groupcast destination layer-2 ID derived from the Application-layer provided group identifier.

Note:  Without a signaling mechanism to either inform the other UEs in the Application layer managed group (UE-C, UE-D, UE-A) of a revised SLPP positioning group consisting of the compatible UEs (UE-C, UE-D and UE-B) and a revised group definition removing the incompatible UE (UE-A) from the group, the incompatible UE (UE-A) will also transmit the SLPP Provide Location Information message via groupcast to the groupcast destination layer-2 ID derived from the Application-layer provided group identifier. This results in further extraneous, unnecessary transmissions and processing by all UEs in the Application layer managed group.

- UE-B performs range/position computations using the location results received at Step 7 and the location results UE B has obtained at Step 6.

Note:  Without a signaling mechanism to either inform the other UEs in the Application layer managed group (UE-C, UE-D, UE-A) of a revised SLPP positioning group consisting of the compatible UEs (UE-C, UE-D and UE-B) and a revised group definition removing the incompatible UE (UE-A) from the group, the initiating UE-B may incorporate the incompatible UE (UE-A) in its position calculations.

- If requested as part of the SLPP Provide Location Information message in Step 7, UE-B may distribute the location results for the other UEs in the Application layer managed group (UE-C, UE-D, UE-A). The SLPP Provide Location Information message is transmitted via groupcast to the groupcast destination layer-2 ID derived from the Application-layer provided group identifier.

Note:  Without a signaling mechanism to either inform the other UEs in the Application layer managed group (UE-C, UE-D, UE-A) of a revised SLPP positioning group consisting of the compatible UEs (UE-C, UE-D and UE-B) and a revised group definition removing the incompatible UE (UE-A) from the group, UE-B transmits the SLPP Provide Location Information message via groupcast to the groupcast destination layer-2 ID derived from the Application-layer provided group identifier, resulting in all UEs in the Application layer managed group, including the UE with incompatible SLPP capabilities (UE-A) receiving the message. This results in further extraneous, unnecessary transmissions and processing by all UEs in the Application layer managed group.

Observation 10: SLPP group positioning can be based on upper/application-layer defined groups, with group information provided to the SLPP layer.

Observation 11: The SLPP Capabilities exchange (SLPP Request Capabilities message / SLPP Provide Capabilities message) between UEs in an Application layer managed group may result in only a subset of the group members (UEs) having compatible and sufficient sidelink positioning (SLPP) capabilities.

Observation 12: Using existing LPP message types there is no mechanism for a UE which cannot join an SLPP session to reject or decline the SLPP session.

Observation 13: The basic three LPP-based SLPP message types (Request/Provide Capabilities, Request/Provide Assistance Data, Request/Provide Location Information) do not provide a mechanism to remove UEs in an Application layer managed group having incompatible SLPP capabilities from the SLPP positioning session.

The preceding three observations, along with the example of Figure 3 illustrate the group management performed by the upper/application layer and the potential for SLPP session participating UE modification by the SLPP layer, confirming the WA that at least part of group management is performed by the upper/application layer.

Proposal 7: 	RAN2 agrees to convert the following WA to an agreement, “WA: At least part of the group management for group positioning is performed at upper/application layer.”

The examples of Figure 2 and Figure 3 demonstrate the benefits of SLPP supporting a session and service management sublayer. For protocol flexibility, SLPP could be designed such that UE support of both the SLPP positioning services (the positioning sublayer) and the SLPP session and service management (the session and service management sublayer) are optional, each denoted as a UE capability by those UEs supporting them. UEs supporting only the SLPP positioning sublayer would support session-less operation (per section 4.4) and rudimentary SLPP session-based operation in a pairwise fashion with one other UE, or with a server UE-managed group of UEs.

Note that defining separate sublayers, or separate message classes, of a protocol is quite common in 3GPP. Some examples include TS 24.501 NAS, which distinguishes CM from MM messages, TS 38.311 RRC, which distinguishes Uu unicast, Uu broadcast and sidelink RRC messages and TS 38.413 NGAP, which distinguishes RAN management messages from broadcast messages and UE related messages. Applying this construct to SLPP provides a design with clean delineation between required protocol functionalities.

Proposal 8: 	SLPP should be designed to support a positioning sublayer and a session and service management sublayer. The positioning sublayer can support LPP-based functions for range and position determination, while the session and service management sublayer can support those aspects of sidelink ranging and positioning not natively supported by LPP, including UE ability to accept/reject/terminate session participation based on UE-specific criteria.

The following descriptions of SLPP session establishment (4.2) and session modification (4.3) assume SLPP support for the positioning sublayer, and the session and service management sublayer. Note that for session-less operation (4.4), a session and service management sublayer is not required.

## SLPP Session Establishment

Figure 4 illustrates an SLPP session establishment procedure aligned with the preceding descriptions (the sidelink positioning and ranging procedures of Step 6 are described in section 5). This can include application-layer managed groups as described in the preceding section, or may include groups determined by a UE’s application layer based on UE discovery procedures. As explained above, some sidelink ranging and positioning use cases require session participants to be aware of the range and/or location of the other session participants. This requires UEs in a session be knowledgeable of the set of UEs that are members of the SLPP session in order to correctly associate range/position information with the appropriate UE. This information may be known as part of an application-layer managed group for which an SLPP session is established, or may be known by disseminating the list of UEs participating in a session to the session participants. Each participant can be identified by an existing UE ID (e.g., an application-layer ID), or as described in Figure 4, by an SLPP session-specific ID assigned by the session initiator.

Figure 4: SLPP Session establishment procedure

Note:	SLPP is carried as payload over the V2X/ProSe layer, with interaction and exchanges between the Application-layer and SLPP layer per procedures defined in TS 23.387 and TS 23.304.

- The Application layer in UE1 and other UEs may perform UE discovery, determine and verify required services and optionally perform group management (e.g. create a group containing UEs 1 to n). The Application layer in UE1 also verifies that all of the UEs 1 to n support SLPP. The Application layer in UE1 then requests sidelink ranging and positioning results for UEs 1 to n from the SLPP layer in UE1, and provides the SLPP layer in UE1 the application layer IDs and layer 2 IDs for each of the UEs 1 to n, and may provide an application layer group ID and a layer 2 group ID for the group of UEs 1 to n.

NOTE:	The exact API between Application Layer and SLPP layer can be regarded as implementation dependent.

- UE1 sends an SLPP Create Session Request message to each of UEs 2 to n. The message includes an SLPP Session ID assigned by UE1 and may include a layer 2 group ID for the SLPP session. The SLPP Create Session Request message also includes an SLPP transaction ID to associate the SLPP Create Session Request message with received SLPP Create Session Accept and SLPP Create Session Reject messages. The SLPP Create Session Request message may be sent via unicast, groupcast or broadcast.

- Each UE (from the UEs 2 to n) which can accept the Session Request sends an SLPP Create Session Accept Message to UE1. Otherwise, the UE sends a SLPP Create Session Reject message to UE1. The SLPP Create Session Accept message and the SLPP Create Session Reject messages include the SLPP transaction ID associated with the SLPP Create Session Request message. The SLPP Create Session Accept message and the SLPP Create Session Reject message may be sent via unicast, groupcast or broadcast.

- UE1 determines the UEs for the SLPP session from the UEs that return an SLPP Create Session Accept at step 2. In this example of Figure 4, all UEs 2 to n are assumed to accept the SLPP session request.

- UE1 transmits a SLPP Session Start Request message to the UEs 2-n determined for the session at step 3 to start the SLPP session. The SLPP Session Start Request message includes the Application Layer ID and Layer-2 ID for each UE in the session and a UE1 assigned SLPP UE ID for each UE in the positioning session. The SLPP UE ID may be an integer (e.g. in the range 1 to n) identifying each UE within the SLPP session. The SLPP Session Start Request message also includes an SLPP transaction ID to associate the SLPP Session Start Request message with received SLPP Session Start Response message. The SLPP Session Start Request message may be sent via unicast, groupcast or broadcast.

- UEs 2-n acknowledge the session start with an SLPP Session Start Response message, subsequent to which the positioning within the session can start. The SLPP Session Start Response message includes the SLPP transaction ID associated with the SLPP Session Start Request message. The SLPP Session Start Response message may be sent via unicast, groupcast or broadcast.

NOTE:	After performing the session establishment at steps 4 and 5, all UEs 1 to n would be aware of all other UEs in the session, including their Application Layer IDs, Layer-2 IDs, SLPP IDs and the layer 2 group ID, etc. Only the SLPP UE ID and SLPP Session ID are then required in the subsequent SLPP transactions (e.g., at Step 6).

- The SL positioning/ranging procedures are carried out as described in section 5.1.

7-8. Once positioning has been completed (e.g., once the application layer request at step 0 is fulfilled), UE1 may send a SLPP Session End Request message to each participant UE 2-n, which each UE acknowledges by sending a SLPP Session End Response message back to UE1. The SLPP Session End Request and the SLPP Session End Response messages include an SLPP transaction ID to associate the Request and Response messages. The SLPP Session End Request message and the SLPP Session End Response message may be sent via unicast, groupcast or broadcast.

In addition to a Session ID, an SLPP session initiator may assign to UEs participating in a session a UE-specific SLPP UE ID (SLPP ID). The SLPP ID can be an integer, enabling participating UEs to track session participants using a simpler, smaller identifier than through the application layer IDs or Layer-2 IDs.

Observation 14: An Initiator-assigned SLPP UE ID may be a more compact, simpler identifier for an SLPP session than a UE Application ID or UE Layer-2 ID.

Proposal 9: 	The SLPP Layer may assign an individual SLPP UE ID to each UE in an SLPP session. The SLPP UE ID may be an integer (e.g. in the range 1 to n).

## SLPP Session Modification

An SLPP session may be modified through the addition or removal of UEs, for example as illustrated in Figure 5. Session modification is particularly relevant to sidelink positioning given the dynamic nature of sidelink use cases introduced by UE mobility. It is quite likely UE accessibility within any initial set of UEs in a sidelink ranging/positioning session will change during the ranging/positioning session. An initial set of UEs in a SLPP session may also change, for example, during Step 2 in Figure 3 after the SLPP session has been established because not all UEs may have the required SLPP positioning capabilities (e.g., SL-PRS, SL-position method capability, capability to act as an anchor UE, etc.).  Hence, UEs may have to be removed from the SLPP session and newly discovered UEs may have to be added to the SLPP session. The Application Layer may also determine UE addition and removal for other reasons, e.g., due to a change of service requirements for some UEs.

Figure 5: SLPP Session modification Procedure

- The Application Layer in UE1 may request the SLPP layer to add and/or remove UEs to a group of UEs for which an SLPP session was previously established as in clause 1.1. Alternatively, or in addition, the SLPP layer in UE1 may detect that some UEs in an SLPP session are no longer available for SLPP positioning (e.g., due to no longer being in PC5 range of other UEs or due to lack of sufficient resources or capabilities to perform SLPP positioning) and may need to remove these UEs from a previously established SLPP session.

- UE1 sends an SLPP Modify Session Request to the modified set of UEs in the SLPP session (UEs currently in the session included in the modified session and any new UEs). The SLPP Modify Session Request message can be sent unicast, groupcast or broadcast to both current UEs and new UEs (e.g. using a new layer 2 group ID provided by the Application layer at step 0). The SLPP Modify Session Request message may include a new layer 2 group ID for the SLPP modified session and a new SLPP session ID. UEs currently included in the SLPP session can be addressed using their SLPP UE IDs and any new UEs can be addressed using their application layer IDs. If it is not possible or not allowed (e.g. for policy reasons) to groupcast or broadcast the message to both current UEs and new UEs, UE1 performs steps 1 and 2 of the SLPP establishment procedure to create the SLPP modified session for the new UEs. For existing UEs that will not be part of the modified session, UE1 may perform steps 7 and 8 of the SLPP establishment procedure to end the SLPP session for these UEs. The SLPP Modify Session Request message includes an SLPP transaction ID to associate the SLPP Modify Session Request message with received SLPP Modify Session Accept/Reject messages.

- Each UE (from the UEs 2 to n) which can accept the Session modification request sends an SLPP Modify Session Accept message to UE1. Otherwise, the UE sends a SLPP Modify Session Reject message to UE1. The SLPP Modify Session Accept message and the SLPP Modify Session Reject message may be sent via unicast, groupcast or broadcast. The SLPP Modify Session Accept message and the SLPP Modify Session Reject messages include the SLPP transaction ID associated with the SLPP Modify Session Request message.

- UE1 determines the UEs for the SLPP modified session from the UEs that return an SLPP Modify Session Accept message at step 2 or an SLPP Create Session Accept message in the case that steps 1 and 2 of clause 1.1 are performed for any new UEs.

- UE1 continues the SLPP modified session as per steps 4-8 for establishment of an SLPP session (1.1). In this case, new SLPP UE IDs are assigned to all UEs by UE1 at step 4.

Proposal 10: 	SLPP shall support at least the following message types for SLPP Session-based operation:

- SLPP Create Session Request/Accept/Reject

- SLPP Session Start Request/Response

- SLPP Session End Request/Response

- SLPP Session Modify Request/Accept/Reject

## SLPP Session-less Operation

In addition to session-based operation, many sidelink positioning/ranging use cases would benefit from session-less operation, and in support of session-less operation, RAN2 reached the following agreement [3].

Agreement At least in the case that positioning methods are supported that do not require a mutual exchange of SLPP messages associated with one another among UEs, SLPP sessionless operation can be supported. FFS if sessionless operation can be operated with security.

Session-less operation use cases include scenarios with dynamic or mobile UEs where it is desirable to minimize the signaling overhead and associated session-establishment latency that may be incurred with session-based operation. Applicable scenarios include one UE needing to determine position/range to UEs in its vicinity for temporary or brief time intervals, such as vehicles traversing an intersection, vehicles entering a freeway from an on-ramp, or public safety individuals moving about a theatre of operation. An example scenario is illustrated for the V2X use case in Figure 6, for the case of vehicles traversing an intersection with an RSU UE conducting sidelink positioning/ranging to the UEs. In such a scenario, the set of UEs proximate and of interest to the RSU for sidelink positioning changes often, lending itself to a more flexible, session-less operation. Note that although this scenario illustrates the benefit of session-less operation for fixed UE (RSU in Figure 6), session-less operation is equally applicable and beneficial for mobile UEs (vehicles) initiating sidelink positioning.

Figure 6: Sidelink Positioning Session-less operation

For session-less operation, signaling overhead is minimized to accommodate the dynamic UE conditions. This is illustrated in Figure 7.

Figure 7: SLPP session-less operation

- The Application layer in UE1 may request the SLPP layer to initiate sidelink positioning for UEs that may be in the vicinity of UE1. The Application layer in UE1 may make this request of the SLPP layer periodically, through configuration, or based on determination of UEs in its vicinity or by other means. UE1 transmits an SLPP Provide Assistance Data message (see section 9) via connection-less groupcast or broadcast per the existing procedures of TS 23.287. The SLPP Provide Assistance Data message includes the SL-PRS configuration for UEs which are able to participate (e.g., based on compatible capabilities). 
Note: UEs receiving an SLPP Provide Assistance Data message absent the mutual exchange of SLPP session establishment messages may determine this is for a session-less sidelink ranging/positioning session from an allocated space of SLPP Session ID, by omission of the SLPP Session ID, or by other means.

- Each of UEs 2-n determines an SL-PRS configuration to be transmitted at step 4. This may be based on SL PRS configuration information received at step 1. Optionally, each of UEs 2-n transmits an SLPP Provide Assistance Data message (see section 9) via connection-less groupcast or broadcast per the existing procedures of TS 23.287. The SLPP Provide Assistance Data message includes the SL-PRS configuration that each UE will transmit at step 4 and an identity (e.g. an application layer ID) of the UE.

- UE1 and optionally each of UEs 2-n transmits an SLPP Request Location Information message (see section 9) via connection-less groupcast or broadcast per the existing procedures of TS 23.287. The SLPP Request Location Information message indicates the measurements to be obtained at step 5 and/or the location results to be sent at step 6.

- Each participating UE transmits its respective SL-PRS according to the SL-PRS configuration indicated at step 1 or determined at step 2. The duration of SL-PRS transmission may be controlled by a timer provided in the SLPP Provide Assistance Data message at step 1 or step 2.

- Each participating UE performs SL-PRS measurements according to the configurations in the SLPP Provide Assistance Data message at step 1 or step 2 or as requested at step 3.

- Upon completion of the SL-PRS transmission and SL-PRS measurements or periodically during this, the participating UEs transmit a SLPP Provide Location Information message with the measurement results obtained at step 5. SL-PRS transmission and SL-PRS measurement may be performed periodically, based on the configuration received in the SLPP Provide Assistance Data message.

Proposal 11: 	Session-less operation is established without mutual exchange of SLPP session establishment signaling via transmission of the SLPP Provide Assistance Data message.

Proposal 12: 	SLPP session-less operation can be denoted by a specific allocation of SLPP Session ID values or by omission of the SLPP Session ID.

# SLPP Positioning Procedures

## PC5-only based positioning

RAN2 has reached the following agreements regarding range and position calculation by UEs participating in sidelink positioning and ranging, including for range/position calculation by one UE on behalf of other UEs (UE-assisted) and range/position calculation by UEs (UE-based).

Agreement RAN2 agrees to support at least unicast SLPP/RSPP "centralized" operation in the sense used in R2-2210911, i.e., operation where one UE performs range and/or position calculations based on measurement/location information relating to itself and/or other UEs. Agreement From RAN2 perspective, if it is determined to support group positioning, it is feasible to perform at least ranging with the estimate calculation at multiple UEs.

Example procedural flows for SLPP UE-assisted (one UE performing range/position calculation on behalf of other UEs) and UE-based (multiple UEs performing range/position calculation) are illustrated in Figure 8. Note that both UE-assisted and UE-based operation follow the same first seven steps (session establishment, Capability Transfer, and Assistance Data exchange).

Figure 8: Sidelink-only Positioning – UE-assisted (centralized) and UE-based (distributed) position calculation

- UEs perform Discovery as described in section 4.

- UE B performs the SLPP Session Establishment procedure described in section 4.

- UE B transmits a SLPP Request Capabilities message to the group of UEs established at Step 1 requesting the SL positioning capabilities from the UEs participating in this session. The SLPP Request Capabilities message may be sent by UE B to UE A, UE C, UE D via broadcast, groupcast or unicast, depending on the specific scenario.

- The UEs in Step 2 addressed by UE B respond with a SLPP Provide Capabilities message. The UE capabilities may refer to particular sidelink position methods (e.g., SL-RTT, SL-AoA, SL-TDOA, etc.) or may be common to multiple position methods. The capabilities may also include the SL-PRS capabilities, an indication of whether the UE can function as an anchor UE and/or location server UE, position/range calculation capabilities, etc., as described in section 8. If the UE can function as an anchor UE, the SLPP Provide Capabilities message may also include the location coordinates. The SLPP Provide Capabilities message may be sent by UE A, UE C, UE D via broadcast, groupcast or unicast, depending on the specific scenario.

- The SLPP Session may be modified as described in section 4 based on the capability information from Step 3 (e.g., removal of UEs with incompatible capability such as SL positioning method, anchor capability, etc.).

- UE B transmits a SLPP Provide Assistance Data message to the group of UEs established at Step 1 or 4. The SLPP Provide Assistance Data message includes the SL-PRS configuration for each of the UEs in the group. The SLPP Provide Assistance Data message and may be sent via broadcast, groupcast or unicast, depending on the specific scenario. 
After receipt of the assistance data (or at an indicated starting time included in the message), each UE in the group of UEs transmits its respective SL-PRS. The duration of SL-PRS transmission may be controlled by a timer provided in the message or may continue until the SLPP session is terminated at Step 11. The SL-PRS may be sent via broadcast, groupcast or unicast, depending on the specific scenario.

- UE B transmits a SLPP Request Location Information message to the group of target UEs.

- The SLPP Request Location Information message indicates the type of location information needed and associated QoS (e.g., desired accuracy, response time, etc.). The message also includes an indication that range/position calculation will be performed by the initiating UE (UE B). The SLPP Request Location Information message may be sent via broadcast, groupcast or unicast, depending on the specific scenario.

- The SLPP Request Location Information message indicates the type of location information needed and associated QoS (e.g., desired accuracy, response time, etc.). The message also includes an indication that range/position calculation should be performed by each of the participating UEs. The SLPP Request Location Information message may be sent via broadcast, groupcast or unicast, depending on the specific scenario.

- Each participating UE performs the requested SL-PRS transmissions and measurements.

- Upon completion of the transmission and measurements or periodically or after the response time received at Step 6 expired,

- The target UEs each transmit a SLPP Provide Location Information message with the location results requested at Step 6. The message may also include an indication of whether other UEs (the initiating UE B in this case) is requested to provide the location result (e.g., calculated range and/or position) back to this UE. SL-PRS transmission and SL-PRS measurement may be performed periodically, based on the configuration received in the SLPP Provide Assistance Data message. The SLPP Provide Location Information message may be sent via broadcast, groupcast or unicast, depending on the specific scenario.

- The initiating UE B and the target UEs each transmit a SLPP Provide Location Information message with the location results requested at Step 6. The message may also include an indication of whether other UEs (the initiating UE B in this case) is requested to provide the location result (e.g., calculated range and/or position) back to this UE. SL-PRS transmission and SL-PRS measurement may be performed periodically, based on the configuration received in the SLPP Provide Assistance Data message. The SLPP Provide Location Information message may be sent via broadcast, groupcast or unicast, depending on the specific scenario.

- The initiating UE B performs the range/position computations using the location results received at Step 8 and the location results UE B has obtained at Step 7.

- Each participating UE performs the range/position computations using the location results received at Step 8 and the own location results obtained at Step 7.

- If requested at Step 8, UE B may distribute the location results for UEs A, C, D to the other UEs A, C, D in this session. The location results may be distributed via broadcast, groupcast or unicast, depending on the specific scenario.

- If requested at Step 8, each UE may distribute its location results to the other UEs in this session. The location results may be distributed via broadcast, groupcast or unicast, depending on the specific scenario.

- The SLPP session may be terminated as described in section 4.

Observation 15:	UE-based (distributed) sidelink positioning and ranging enables UE position calculation without the reliance on a separate UE, and can reduce overall SLPP signaling required for sidelink positioning.

Proposal 13:	SLPP should support UE-based (distributed) sidelink positioning enabling multiple UEs to determine position and range based on exchanged location information.

# Common SLPP Transaction Modes

As described in section 5, an SLPP message may be sent via broadcast, groupcast or unicast, depending on the specific scenario. SLPP should indicate the transaction (communication) mode to be used for each SLPP message, i.e., whether broadcast mode, groupcast mode or unicast mode is to be used (e.g., in a common SLPP message header).

SLPP should support at least the following generic transaction modes:

- Unicast transaction

- Group Transaction with Group Replies

- Group Transaction with Unicast Replies

- Broadcast Transaction

NOTE:	The exact API between SLPP layer and Transport layer can be regarded as implementation dependent.

## SLPP Unicast Transaction

Figure 9: Generic SLPP Unicast Transaction

- UE A sends a SLPP message to UE B. The message includes the SLPP transport Mode set to 'unicast'. The message includes the SLPP Session ID if applicable, a Transaction ID, and the source and destination UE IDs (e.g., Application Layer IDs, Layer 2 IDs, SLPP UE IDs (see also section 4)).

- If the SLPP message from Step 1 requires a response, UE B sends a SLPP message to UE A with the SLPP transport Mode set to 'unicast', the Transaction ID and the destination ID for UE A as received in Step 1.

## SLPP Group Transaction with Group Replies

Figure 10: Generic SLPP Group Transaction with Group Replies

- UE A sends a SLPP message to a group of UEs (UE B, C, D, etc.). The message includes,

- The SLPP transport Mode set to 'groupcast-group reply' indicating that the destination UEs should respond via groupcast, or set to 'groupcast' in the case the SLPP message does not require a response.

- The SLPP Session ID, and if applicable, a Transaction ID, and the Group information (e.g., application layer group ID, layer-2 group ID, or SLPP Group ID).

- If the SLPP message from Step 1 received by destination UEs B, C, D, etc. requires a response (SLPP Mode was set to 'group cast-group reply'), UEs B, C, D, etc. send a SLPP message with the SLPP transport Mode set to 'groupcast' (which are thus also returned to the other UEs in the group). The message includes the Session ID if applicable, the Transaction ID and the Group information received at Step 1.

## SLPP Group Transaction with Unicast Replies

Figure 11: Generic SLPP Group Transaction with Unicast Replies

- UE A sends a SLPP message to a group of UEs (UE B, C, D, etc.). The message includes the SLPP Mode set to 'groupcast - unicast reply' indicating that the destination UEs should respond via unicast. If no response is requested, the SLPP mode may indicate 'groupcast' as in section 7.1.1. The message includes the SLPP Session ID, if applicable, a Transaction ID and the Group ID (e.g., application layer group ID, layer-2 group ID, or SLPP Group ID).

- If the SLPP message from Step 1 received by destination UEs B, C, D, etc. requires a response, UEs B, C, D, etc. each send a SLPP message with the SLPP Mode set to 'unicast'. The message includes the Session ID if applicable, the Transaction ID and source UE ID of the individual UE, and the destination ID for UE A received at Step 1.

## SLPP Broadcast Transaction

Figure 12: Generic SLPP Broadcast Transaction

- UE A sends a SLPP message with the SLPP Mode set to 'broadcast'. The message includes the SLPP Session ID if applicable and a Transaction ID.

- If the SLPP message from Step 1 received by UEs B, C, D, etc. requires a response, UEs B, C, D, etc. send a SLPP message with the SLPP Mode set to 'broadcast'. The message includes SLPP Session ID if received at Step 1 and the Transaction ID.

Proposal 14: 	SLPP should indicate the transaction (communication) mode to be used for each SLPP message, i.e. whether broadcast mode, groupcast mode or unicast mode is to be used (e.g., in a common SLPP message header). At least the following common transaction modes shall be supported:

- Unicast transaction

- Group Transaction with Group Replies

- Group Transaction with Unicast Replies

- Broadcast Transaction.

NOTE: Not all SLPP message types may be allowed for all transaction modes.

# SLPP Group Positioning Examples

This section provides examples of SLPP group management procedures. SLPP group positioning using transport over Groupcast may include upper/application layer managed groupcast and upper/application layer connection-less groupcast.

- Upper/application layer managed groupcast: The upper/application layer provides the lower layers with group information consisting of the set of {group identifier, group size, member ID}. All group members (UEs) use the group identifier (group ID) to derive a groupcast destination layer-2 ID, to which UEs transmit SLPP groupcast messages.

- Upper/application layer connection-less groupcast: All group members use a groupcast destination layer-2 ID based on the configuration of mapping between service type and layer-2 ID. The SLPP messages include a Session ID assigned by the SLPP layer in the UE initiating the SLPP session.

Observation 16:	Sidelink ranging and positioning over groupcast includes both upper/application layer managed groupcast and upper/application layer connectionless groupcast.

Both upper/application layer managed groupcast and upper/application layer connection-less groupcast use the common SLPP transaction illustrated in Figure 10, with the specific differences described in the following.

## Upper/Application Layer Managed Groupcast

### Group Request with Group Replies

Figure 13: SLPP Group Request with Group Replies

- The UE-A Upper/Application layer provides managed group definition to the UE-A SLPP layer including group identifier, group size and member IDs (all UEs in the managed group are aware of the group definition). The Application layer in UE-A then requests sidelink ranging and positioning for the managed group from the SLPP layer. UE A sends a SLPP message to the destination layer-2 ID for the managed group of UEs (the destination layer-2 ID derived from the group information). The message includes,

- The SLPP transport Mode set to 'groupcast-group reply' indicating that the destination UEs should respond via groupcast, or set to 'groupcast' in the case the SLPP message does not require a response.

- The SLPP Session ID, and if applicable, a Transaction ID, and the Group information (e.g., application layer group ID, layer-2 group ID, or SLPP Group ID).

- If the SLPP message from Step 1 received by other UEs in the upper/application layer managed group requires a response (SLPP Mode was set to 'group cast-group reply'), the other UEs send a SLPP message to the destination layer-2 ID for the managed group of UEs (the same destination layer-2 ID derived from the group information). The message includes the Session ID if applicable, the Transaction ID and the Group information received at Step 1.

### Group Request with Subgroup Replies

For upper/application layer managed groups where only a subset of the UEs in the managed group may be required to reply to an SLPP message (e.g., in scenarios where only a subset of UEs in the upper/application layer managed group have compatible SLPP ranging/positioning capabilities), the UE initiating the SLPP session can inform the managed group of the UE subset designated to reply, and direct those UEs designated as not needing to reply to not participate in subsequent SLPP transactions associated with the Session ID, thereby reducing unnecessary SLPP transmissions, SL-PRS transmission, and SLPP message processing.

Figure 14: SLPP Group Request with Subgroup Replies

- The UE-A Upper/Application layer provides managed group definition to the UE-A SLPP layer including group identifier, group size and member IDs (all UEs in the managed group are aware of the group definition). The Application layer in UE-A then requests sidelink ranging and positioning for the managed group from the SLPP layer. UE A sends a SLPP message to the destination layer-2 ID for the managed group of UEs (the destination layer-2 ID derived from the group information). The message includes,

- The SLPP transport Mode set to 'groupcast-group reply' indicating that the destination UEs should respond via groupcast, or set to 'groupcast' in the case the SLPP message does not require a response.

- The SLPP Session ID, and if applicable, a Transaction ID, and the Group information (e.g., application layer group ID, layer-2 group ID, or SLPP Group ID).

- If the SLPP message from Step 1 received by other UEs in the upper/application layer managed group requires a response (SLPP Mode was set to 'group cast-group reply'), the other UEs send a SLPP message to the destination layer-2 ID for the managed group of UEs (the same destination layer-2 ID derived from the group information). The message includes the Session ID if applicable, the Transaction ID and the Group information received at Step 1.

- Based on SLPP messages exchanged, UE-A determines the subset of UEs (the SLPP Session Sub-Group) that should continue SLPP transactions for this SLPP session (this SLPP Session ID).

- UE-A sends a SLPP message to the destination layer-2 ID for the managed group of UEs (the destination layer-2 ID derived from the group information). The message includes,

- The SLPP transport Mode set to 'groupcast-group reply' indicating that the destination UEs should respond via groupcast, or set to 'groupcast' in the case the SLPP message does not require a response.

- The SLPP Session ID, and if applicable, a Transaction ID, and the Group information (e.g., application layer group ID, layer-2 group ID, or SLPP Group ID).

- The UEs comprising the SLPP Session Sub-Group (e.g., upper/application layer member IDs, SLPP UE IDs for the subset of UEs having compatible SLPP capabilities)

- UE-D informed it is excluded from the SLPP Session Sub-Group will cease to participate in the SLPP Session (based on Session ID), including refraining from SL-PRS and SLPP message transmission.

- The UEs that are members of the SLPP Session Sub-Group continue to transmit SLPP messages to the destination layer-2 ID derived from the upper/application layer group information.

Observation 17:	SLPP sessions for upper/application layer managed groupcast may require the session initiator to indicate which UEs in the managed group should participate in the SLPP session (reply to SLPP session messages and/or perform actions based on receipt of SLPP session messages).

### Group Request with Group Modification

For upper/application layer managed groups where only a subset of the UEs in the managed group may be required to reply to an SLPP message (e.g., in scenarios where only a subset of UEs have compatible SLPP ranging/positioning capabilities), the UE initiating the SLPP session may create a new SLPP positioning group by creating a new group identifier composed of the subset of the UEs in the managed group having compatible SLPP ranging/positioning capabilities. By informing the UEs in the original upper/application layer managed group of the new group definition, those UEs with incompatible SLPP capabilities will remove themselves from subsequent SLPP groupcast transactions, thereby reducing unnecessary SLPP transmissions, SL-PRS transmission, and SLPP message processing.

Figure 15: SLPP Group Request with Group Modification

- The UE-A Upper/Application layer provides managed group definition to the UE-A SLPP layer including group identifier, group size and member IDs (all UEs in the managed group are aware of the group definition). The Application layer in UE-A then requests sidelink ranging and positioning for the managed group from the SLPP layer. UE A sends a SLPP message to the destination layer-2 ID for the managed group of UEs (the destination layer-2 ID derived from the group information). The message includes,

- The SLPP transport Mode set to 'groupcast-group reply' indicating that the destination UEs should respond via groupcast, or set to 'groupcast' in the case the SLPP message does not require a response.

- The SLPP Session ID, and if applicable, a Transaction ID, and the Group information (e.g., application layer group ID, layer-2 group ID, or SLPP Group ID).

- If the SLPP message from Step 1 received by other UEs in the upper/application layer managed group requires a response (SLPP Mode was set to 'group cast-group reply'), the other UEs send a SLPP message to the destination layer-2 ID for the managed group of UEs (the same destination layer-2 ID derived from the group information). The message includes the Session ID if applicable, the Transaction ID and the Group information received at Step 1.

- Based on SLPP messages exchanged, UE-A determines:

- The subset of UEs (the SLPP Session Sub-Group) that should continue SLPP transactions for this SLPP session (this SLPP Session ID).

- A new upper/application layer managed groupcast Group Identification (group ID) for the subset of UEs having compatible SLPP capabilities.

- UE-A sends a SLPP message to the destination layer-2 ID for the original managed group of UEs (the destination layer-2 ID derived from the group information used in Step 1, 2, 3). The message includes,

- The SLPP transport Mode set to 'groupcast-group reply' indicating that the destination UEs should respond via groupcast, or set to 'groupcast' in the case the SLPP message does not require a response.

- The SLPP Session ID, and if applicable, a Transaction ID, and the Group information (e.g., application layer group ID, layer-2 group ID, or SLPP Group ID).

- The UEs comprising the SLPP Session Sub-Group (e.g., upper/application layer member IDs, SLPP UE IDs for the subset of UEs having compatible SLPP capabilities)

- The new upper/application layer managed groupcast Group ID for the modified SLPP Session.

- UE-D is informed it is excluded from the SLPP Session modified group definition and will cease to participate in further groupcast transactions for this group.

- The UEs that are members of the SLPP Session Sub-Group continue to transmit SLPP messages to the new destination layer-2 ID derived from the new upper/application layer group information (Group ID).

Observation 18:	SLPP sessions for upper/application layer managed groupcast may require the session initiator to specify the subset of UEs in the managed group should participate in the SLPP session, and the groupcast destination layer-2 ID(s) those UEs should use for session participation.

## Upper/Application Layer Connection-less Groupcast

A UE initiating an SLPP transaction for upper/application layer connection-less groupcast may transmit an SLPP message via groupcast using the groupcast destination layer-2 ID based on the configuration of mapping between service type and layer-2 ID [13]. The SLPP message may provide to UEs receiving the message a reply indication indicating the appropriate cast type for reply SLPP messages.

Figure 16: SLPP upper/application layer connection-less group request

- The UE-A Upper/Application requests sidelink ranging and positioning for connection-less groupcast from the SLPP layer. UE A sends a SLPP message to the destination layer-2 ID from the configuration of mapping between service type and layer-2 ID. The message includes,

- If the SLPP message transmitted by UE-A requires a reply via groupcast, the SLPP transport Mode set to 'groupcast-group reply' indicating that the destination UEs should respond via groupcast, or set to 'groupcast' in the case the SLPP message does not require a response.

  - The SLPP Session ID, and if applicable, a Transaction ID, and the Group information (e.g., application layer group ID, layer-2 group ID, or SLPP Group ID).

- If the SLPP message transmitted by UE-A requires a reply via unicast, the message includes the SLPP Mode set to 'groupcast - unicast reply' indicating that the destination UEs should respond via unicast. If no response is requested, the SLPP mode may indicate 'groupcast'. The message includes the SLPP Session ID, if applicable, a Transaction ID and the Group ID (e.g., application layer group ID, layer-2 group ID, or SLPP Group ID).

- If the SLPP message from Step 1 received by other UEs requires a groupcast response (SLPP Mode was set to 'group cast-group reply'), the other UEs send a SLPP message to the destination layer-2 ID from the configuration of mapping between service type and layer-2 ID. The message includes the Session ID if applicable, the Transaction ID and the Group information received at Step 1. If the SLPP message from Step 1 received by destination UEs B, C, D, etc. requires a unicast response, UEs B, C, D, etc. each send a SLPP message with the SLPP Mode set to 'unicast'. The message includes the Session ID if applicable, the Transaction ID and source UE ID of the individual UE, and the destination ID for UE A received at Step 1.

Observation 19:	SLPP sessions for upper/application layer managed groupcast may require the session initiator to indicate the destination layer-2 ID(s) other UEs in the session should use for participation in the session.

Proposal 15:	For groupcast SLPP sessions, SLPP can indicate the set of UEs which should participate in the ranging/positioning session, and the appropriate groupcast destination layer 2 ID UEs should use for session participation.

# SLPP Group Positioning: Summary

As described in section 4, Group positioning in SLPP departs from LPP in several aspects. This includes the ability for a UE to participate simultaneously in multiple, concurrent SLPP sessions, with those sessions potentially composed of disjoint or overlapping sets of UEs. Assigning an SLPP Session ID is a key aspect of how a UE manages the various session(s) it may be participating in. The SLPP Session ID enables a UE to maintain knowledge and status of each distinct session the UE is engaged in, including the session participants and session requirements. All UEs participating in a session need to maintain this knowledge, regardless of any role (or roles) the UE may be engaged in, be it anchor UE, target UE and/or server UE.

The multiple, concurrent SLPP sessions a UE is engaged in may also include UE group definitions provided by the upper/application layer and submitted to SLPP for invoking a group ranging/positioning session. Since the UEs comprising the upper/application layer group may differ in their respective SLPP capabilities, SLPP session management should include informing group members of the compatible (and incompatible) UEs, and/or modifying the participants in the SLPP session. Enabling this SLPP session modification is a prerequisite to allowing all UEs engaged in the SLPP session to most efficiently utilize UE and over-the-air resources.

SLPP group positioning transmitted over groupcast includes both managed groupcast (session oriented) and connection-less groupcast (session-less), both of which can be applicable to V2X, public safety and IIOT use cases. Managed groupcast, with group definition provided by the upper/application layer, involves SLPP session initiation by groupcast transmission to the destination layer-2 ID derived from the upper/application layer provided group ID. All UEs that are members of the upper/application layer could be cognizant of the group ID and derive the same groupcast destination layer-2 ID for their respective transmissions. It is also possible for a UE initiating a session to send group addressing information to all other UEs in the session without requiring pre-existing group knowledge by the other UEs. If directed as part of the SLPP session establishment, UEs may reply to the session establishment via unicast, or via a revised groupcast destination layer 2 ID. For connection-less groupcast, SLPP ranging/positioning can be initiated by transmission to the groupcast destination layer-2 ID based on the configuration of mapping between service type and layer-2 ID. UEs replying to this session can do so by groupcast transmission to the same groupcast destination layer 2 ID, or via unicast, if directed as part of the SLPP session initiation.

# SLPP Positioning and Session Operation Messages

RAN2 has reached the following agreements regarding SLPP functionality:

Agreement: In order to enable sidelink positioning, SLPP/RSPP shall support at least the following functionalities: 1. SL Positioning Capability Transfer 2. SL Positioning Assistance Data exchange 3. SL Location Information Transfer 4. Error handling 5. Abort This agreement does not imply any specific signalling structure.

This section summarizes the main functionality for the Capability Transfer, Assistance Data exchange and Location Information transfer message types based on the general positioning procedures shown in section 5 of this contribution, and the message types for session-based operation shown in section 4 of this contribution.

## SLPP Capability Transfer

Capabilities in an SLPP context should refer to the ability to support different position methods defined for SLPP (e.g., SL-RTT, SL-TDOA, etc.), different aspects of a particular position method (e.g. different types of measurements or assistance data) and features specific to certain UE roles (e.g. ability to operate as a server or anchor UE, etc.).

The exchange of capabilities between different endpoints may be initiated by a request or sent as unsolicited information. If a request is used, an endpoint sends an SLPP Request Capabilities message using one of the allowed transaction modes in section 6 with a request for capability information. The addressed endpoint(s) then respond with an SLPP Provide Capabilities message.

NOTE: An endpoint may be one or more UEs or an LMF.

Figure 17: SLPP Capability Transfer Procedure

- The transmitting endpoint may send a request for the SLPP related capabilities to one or more receiving endpoints using one of the SLPP Transaction Modes described in section 6.

- The receiving endpoint(s) transfer their SLPP-related capabilities to the transmitting endpoint using one of the SLPP Transaction Modes described in section 6. The capabilities may refer to particular position methods or may be common to multiple position methods.

The SLPP Capability Indication procedure is used for unsolicited capability transfer.

Figure 18: SLPP Capability Indication Procedure

- The transmitting endpoint provides the SLPP-related capabilities to one or more receiving endpoints using one of the SLPP Transaction Modes described in section 6.

The SLPP-related capabilities may include the general information summarized in Table 1 below.

Information

SL Positioning Capability:

- Supported position methods and measurements per method (e.g., SL-RTT, SL-AoA, SL-TDOA)

- Supported SL-PRS details (RAN1)

- Supported position calculation capability

- UE-based: UE can calculate its own position using its own and other UEs measurements

- UE-assisted: UE can only perform measurements; requires a separate server (UE or LMF) for position calculation

Anchor capability (yes/no):

- Location coordinates of UE (if available) NOTE: Anchor UE location could also be obtained/updated via Location Information Transfer procedures.

- Stationary/non-stationary indication and/or velocity

- Timing/synchronization capability (e.g., for TDOA methods); e.g., GNSS-DFN accuracy.

Location Server capability (yes/no):

- Position calculation functionality for other UEs

- Supported methods: e.g., RTT, AoA, TDOA

- Assistance Data generation/distribution functionality

- Supported assistance data types (TBD)

- May also indicate whether the UE can act as a server-UE with the help of an LMF

Table 1: SLPP-related capabilities

Proposal 16:	An SLPP Provide Capabilities message should provide information on the ability of the device to support different position methods defined for SLPP (e.g., SL-RTT, SL-TDOA, etc.), different aspects of a particular position method (e.g. different types of measurements or assistance data) and features specific to certain UE roles (e.g. ability to operate as a server or anchor UE, etc.).

## SLPP Assistance Data Transfer

Assistance data in this Release comprises the SL-PRS configuration information and may be transferred either by request or unsolicited. If a request is used, an endpoint sends an SLPP Request Assistance Data message to a server (e.g., server UE or LMF (via MO-LR)) for SL-PRS assistance data and may indicate the assistance data needed. The addressed endpoint then responds with an SLPP Provide Assistance Data message.

Figure 19: SLPP Assistance Data Transfer Procedure

- The transmitting endpoint may send a request for assistance data to a receiving endpoint (e.g., server UE or LMF) using one of the SLPP Transaction Modes described in section 6. The message may indicate the particular assistance data needed.

- The receiving endpoint transfers assistance data to the transmitting endpoint using one of the SLPP Transaction Modes described in section 6. The transferred assistance data should match any assistance data requested in step 1.

- The receiving endpoint may transfer additional assistance data to the transmitting endpoint in one or more additional SLPP Provide Assistance Data messages.

The SLPP Assistance Data Delivery procedure is used for unsolicited assistance data transfer.

Figure 20: SLPP Assistance Data Delivery Procedure

- The transmitting endpoint transfers assistance data to one or more receiving endpoint(s) using one of the SLPP Transaction Modes described in section 6.

- The transmitting endpoint may transfer additional assistance data to the receiving UE(s) in one or more additional SLPP Provide Assistance Data messages.

The assistance data details depend largely on RAN1 but may comprise the general information summarized in Table 2.

Information

SL-PRS Configuration Information for participant UEs in a session.

Start time and duration for SL-PRS (if the Provide Assistance Data is used to trigger the SL-PRS transmission at the receiver).

Table 2: SLPP-related assistance

Proposal 17:	The SLPP Request Assistance Data message should allow an SLPP-capable endpoint to request positioning assistance data (e.g., SL-PRS configuration information and resources) from a server (e.g., server UE or LMF). The SLPP Provide Assistance Data message should allow a server (e.g., server UE or LMF) to provide positioning assistance data (e.g., SL-PRS configuration information and resources) to participant UEs in a session.

## SLPP Location Information Transfer

The term "location information" applies to both, an actual position estimate and to values used in computing position (SL positioning measurements). It can be delivered either in response to a request or unsolicited.

Figure 21: SLPP Location Information Transfer Procedure

- The transmitting endpoint may send a request for location information to one or more receiving endpoints using one of the SLPP Transaction Modes described in section 6. The message indicates the type of location information needed and associated QoS.

- In response to step 1, the receiving endpoint(s) transfer location information to the transmitting endpoint using one of the SLPP Transaction Modes described in section 6. The location information transferred should match the location information requested in step 1.

- The receiving endpoint(s) may transfer additional location information to the transmitting endpoint in one or more additional SLPP Provide Location Information messages using one of the SLPP Transaction Modes described in section 6.

The SLPP Location Information Delivery procedure is used for unsolicited location information transfer.

Figure 22: SLPP Location Information Delivery Procedure

- The transmitting endpoint transfers location information to one or more receiving endpoints using one of the SLPP Transaction Modes described in section 6.

- The transmitting endpoint may transfer additional location information to one or more receiving endpoints using one of the SLPP Transaction Modes described in section 6.

Information

SLPP Request Location Information:

- Requested location measurements (e.g., RxTx, AoA, RSTD, etc.) (UE-assisted)

- Request for location coordinates and velocity (UE-based)

- Requested positioning QoS (e.g., accuracy, response time)

- Indication whether calculated location estimate is requested (centralized mode) or whether each UE is requested to calculate its own location (distributed mode).

- Optional Trigger Indication for SL-PRS transmission and measurement

SLPP Provide Location Information:

- Location measurements

- Location coordinates and velocity

Table 3: SLPP-related location information

Proposal 18:	The SLPP Request/Provide Location Information message should allow an SLPP-capable endpoint to request/provide location measurements or location estimate from participant UEs in a session.

## SLPP Session Creation

SLPP-capable UEs can initiate a Sidelink Ranging and Positioning session to other SLPP-capable UEs. The initiation of an SLPP session by an SLPP-capable UE can be done in an unsolicited manner based on an application-layer request for sidelink positioning and ranging results from one or more SLPP-capable UEs.

Figure 23: SLPP Create Session Request/Accept/Reject procedure

- The transmitting UE may send a request to create a Sidelink Ranging and Positioning session to one or more receiving UEs. The SLPP Create Session Request message may be sent by the transmitting UE using one of the SLPP Transaction Modes described in section 6.

- Receiving UE(s) electing to participate in the Sidelink Ranging and Positioning session proposed by the transmitting UE respond to the transmitting UE with an SLPP Create Session Accept message. The SLPP Create Session Accept message may be sent by the transmitting UE using one of the SLPP Transaction Modes described in section 6.

- Receiving UE(s) electing not to participate in the Sidelink Ranging and Positioning session proposed by the transmitting UE respond to the transmitting UE with an SLPP Create Session Reject message. The SLPP Create Session Reject message may be sent by the transmitting UE using one of the SLPP Transaction Modes described in section 6.

The SLPP Create Session Request/Accept/Reject messages may include the general information summarized in Table 4 below.

Information

SLPP Create Session Request:

- SLPP session ID (assigned by transmitting entity)

- Layer 2 Group ID

SLPP Create Session Accept:

- SLPP session ID (assigned by transmitting entity)

- Layer 2 Group ID

- Application Layer ID for each participating receiving UE

- Layer-2 ID for each participating receiving UE

SLPP Create Session Reject:

- SLPP session ID (assigned by transmitting entity)

- Layer 2 Group ID

Table 4: SLPP Create Session Request/Response/Reject

Proposal 19:	The SLPP Create Session Request message should allow an SLPP UE to request creation of a sidelink ranging and positioning session and provide information on the sidelink ranging and positioning session proposed, including SLPP and Layer-2 IDs.

Proposal 20:	The SLPP Create Session Accept message should allow an SLPP UE to accept an SLPP session proposed by an SLPP-capable UE.

Proposal 21:	The SLPP Create Session Reject message should allow an SLPP UE to reject an SLPP session proposed by an SLPP-capable UE.

## SLPP Session Start

An SLPP-capable UE that has created a Sidelink Ranging and Positioning session to other SLPP-capable UEs (an SLPP session) can start that SLPP session for the SLPP-capable UEs that have accepted the proposed SLPP session request.

Figure 24: SLPP Session Start Request/Response procedure

- The transmitting UE may start an SLPP session to one or more receiving UEs using SLPP Session Start Request message. The SLPP Session Start Request message may be sent by the transmitting UE using one of the SLPP Transaction Modes described in section 6.

- Receiving UE(s) respond to the transmitting UE with an SLPP Session Start Response message. The SLPP Session Start Response message may be sent by the transmitting UE using one of the SLPP Transaction Modes described in section 6.

The SLPP Session Start Request/Response messages may include the general information summarized in Table 5 below.

Information

SLPP Session Start Request:

- SLPP session ID

- Layer 2 Group ID

- Application Layer ID for each participating receiving UE

- Layer-2 ID for each participating receiving UE

- SLPP ID for each participating receiving UE

SLPP Session Start Response:

- SLPP session ID

- Layer 2 Group ID

- Application Layer ID for responding receiving UE

- Layer-2 ID for responding receiving UE

- SLPP ID for responding receiving UE

Table 5: SLPP Session Start Request/Response

Proposal 22:	The SLPP Session Start Request message should allow an SLPP UE to request the start of a sidelink ranging and positioning session proposed using the SLPP Create Session Request/Response messages and provide information on the session participating SLPP UEs.

Proposal 23:	The SLPP Session Start Response message should allow an SLPP UE to acknowledge the start of an SLPP session.

## SLPP Session Modify

An SLPP-capable UE that has created and started a Sidelink Ranging and Positioning session to other SLPP-capable UEs (an SLPP session) can modify that SLPP session for the SLPP-capable UEs that have accepted the proposed SLPP session.

Figure 25: SLPP Modify Session Request/Accept/Reject procedure

- The transmitting UE may send a request to modify a Sidelink Ranging and Positioning session (an SLPP session) for the SLPP UEs participating in the SLPP session. The SLPP Modify Session Request message may be sent by the transmitting UE using one of the SLPP Transaction Modes described in section 6.

- Receiving UE(s) participating in the SLPP session electing to accept the proposed session modification respond to the transmitting UE with an SLPP Modify Session Accept message. The SLPP Modify Session Accept message may be sent by the transmitting UE using one of the SLPP Transaction Modes described in section 6.

- Receiving UE(s) participating in the SLPP session electing not to participate in the Sidelink Ranging and Positioning session proposed by the transmitting UE respond to the transmitting UE with an SLPP Modify Session Reject message. The SLPP Modify Session Reject message may be sent by the transmitting UE using one of the SLPP Transaction Modes described in section 6.

The SLPP Modify Session Request/Accept/Reject messages may include the general information summarized in Table 6 below.

Information

SLPP Modify Session Request:

- Active SLPP session ID

- Active Layer 2 Group ID

- New SLPP session ID

- New Layer 2 Group ID

- Active Participants

- SLPP ID

- Added Participants

- Application Layer ID

- SLPP ID

SLPP Modify Session Accept:

- SLPP session ID

- Layer 2 Group ID

- Application Layer ID for each participating receiving UE

- Layer-2 ID for each participating receiving UE

SLPP Modify Session Reject:

- SLPP session ID

- Layer 2 Group ID

Table 6: SLPP Create Session Request/Response/Reject

Proposal 24:	The SLPP Modify Session Request message should allow an SLPP UE to request modification of an existing SLPP session and provide information on the modified SLPP session proposed and session participating SLPP UEs, including SLPP and Layer-2 IDs.

Proposal 25:	The SLPP Modify Session Accept message should allow an SLPP UE to accept an SLPP session modification proposed by an SLPP-capable UE.

Proposal 26:	The SLPP Modify Session Reject message should allow an SLPP UE to reject an SLPP session modification proposed by an SLPP-capable UE.

## SLPP Session Termination

An SLPP-capable UE that has created a Sidelink Ranging and Positioning session to other SLPP-capable UEs (an SLPP session) can end that SLPP session for the SLPP-capable UEs participating in the SLPP session.

Figure 26: SLPP Session End Request/Response procedure

- The transmitting UE may end a Sidelink Ranging and Positioning session (an SLPP session) for the SLPP UEs participating in the SLPP session by sending a SLPP Session End Request message. The SLPP Session End Request messages may be sent by the transmitting UE using one of the SLPP Transaction Modes described in section 6.

- Receiving UE(s) respond to the transmitting UE with an SLPP Session End Response message. The SLPP Session End Response message may be sent by the transmitting UE using one of the SLPP Transaction Modes described in section 6.
The receiving UE(s) delete the SLPP Session ID and SLPP ID when the session is terminated.

The SLPP Session End Request/Response messages may include the general information summarized in Table 7 below.

Information

SLPP Session End Request:

- SLPP session ID

- Layer 2 Group ID

SLPP Session End Response:

- SLPP session ID

- Layer 2 Group ID

- SLPP ID for responding receiving UE

Table 7: SLPP Session Start Request/Response

Proposal 27:	The SLPP Session End Request message should allow an SLPP UE to request the termination of a sidelink ranging and positioning session.

Proposal 28:	The SLPP Session End Response message should allow an SLPP UE to acknowledge the termination of an SLPP session.

# SLPP Discovery

RAN2 reached the following agreements regarding discovery for sidelink positioning [4],

Agreements: Discovery procedure is included in the sidelink positioning procedure at least for out of coverage scenario. RAN2 confirms that discovery messages will be used to carry information for targeted discovery and candidate selection of SL positioning UEs, including at least the indication of anchor UE, target UE. and server UE roles. FFS how much information is indicated about anchor UEs (e.g., knowledge of location). The UE role information is indicated in the discovery SLPP metafield. FFS if this applies to both discovery modes and which messages.

RAN2 has agreed that the Discovery metafield will include an indication of UE role (e.g., target, anchor, server). Since a sidelink positioning transaction involves exchanging SLPP Request Capabilities/SLPP Provide Capabilities messages, beyond an indication of UE role, UEs engaged in sidelink positioning discovery would benefit from an indication that the UE supports SLPP.  Other aspects of the positioning session would be conveyed as part of the Capabilities exchange, are unnecessary to include in Discovery, and their inclusion might constrain forward compatibility.

Regarding sidelink positioning discovery for in-coverage scenarios, SA2 has reached agreement on sidelink ranging and positioning procedures to 23.273 for SL-MT-LR [19] and SL-MO-LR [20]. These procedures (SL-MT-LR, SL-MO-LR) delineate steps for an LCS client and a UE, respectively, to obtain sidelink Positioning/Ranging results over PC5 for a group of n UEs (n ≥ 2). For both SL-MT-LR and SL-MO-LR procedures, UEs perform discovery over PC5. As such, RAN2 should generalize the sidelink positioning discovery procedure for in-coverage, in addition to out of coverage.

Proposal 29:	Discovery is included in the sidelink positioning procedure for all PC5 sidelink positioning scenarios (in-coverage and out-of-coverage).

# Considerations for SLPP ASN.1 Design

Structuring SLPP ASN.1 in a manner similar to LPP enables reuse of the existing LPP design and facilitates commonality between the two designs. Specifically, an SLPP message may include common header information (e.g., session ID, UE IDs, transaction IDs, etc.) and a message body realizing the individual SLPP transaction types (capability transfer, assistance data transfer, location information transfer, etc.).

Proposal 30:	An SLPP message contains a common header (including e.g., session ID, UE IDs, transaction IDs, etc.) and a message body realizing the individual SLPP transaction types (capability transfer, assistance data transfer, location information transfer, etc.).

In LPP, each message body IE is a SEQUENCE of individual IEs, applicable to all or individual positioning methods. For example, the LPP RequestCapabilities message body contains relevant IEs for each LPP positioning method (similar for all other LPP message body IEs):

RequestCapabilities-r9-IEs ::= SEQUENCE {

commonIEsRequestCapabilities		CommonIEsRequestCapabilities		OPTIONAL,	-- Need ON

a-gnss-RequestCapabilities			A-GNSS-RequestCapabilities			OPTIONAL,	-- Need ON

otdoa-RequestCapabilities			OTDOA-RequestCapabilities			OPTIONAL,	-- Need ON

ecid-RequestCapabilities			ECID-RequestCapabilities			OPTIONAL,	-- Need ON

epdu-RequestCapabilities			EPDU-Sequence						OPTIONAL,	-- Need ON

...,

[[	sensor-RequestCapabilities-r13	Sensor-RequestCapabilities-r13		OPTIONAL,	-- Need ON

tbs-RequestCapabilities-r13		TBS-RequestCapabilities-r13			OPTIONAL,	-- Need ON

wlan-RequestCapabilities-r13	WLAN-RequestCapabilities-r13		OPTIONAL,	-- Need ON

bt-RequestCapabilities-r13		BT-RequestCapabilities-r13			OPTIONAL	-- Need ON

]],

[[	nr-ECID-RequestCapabilities-r16	NR-ECID-RequestCapabilities-r16		OPTIONAL,	-- Need ON

nr-Multi-RTT-RequestCapabilities-r16

NR-Multi-RTT-RequestCapabilities-r16

OPTIONAL,	-- Need ON

nr-DL-AoD-RequestCapabilities-r16

NR-DL-AoD-RequestCapabilities-r16	OPTIONAL,	-- Need ON

nr-DL-TDOA-RequestCapabilities-r16

NR-DL-TDOA-RequestCapabilities-r16	OPTIONAL,	-- Need ON

nr-UL-RequestCapabilities-r16	NR-UL-RequestCapabilities-r16		OPTIONAL	-- Need ON

]]

}

-- ASN1STOP

An implementation using this approach would have to compile the entire LPP ASN.1 even in the case of just a single positioning method supported. This is because each field in the SEQUENCE has an explicit ASN.1 definition (e.g., IEs CommonIEsRequestCapabilities, A-GNSS-RequestCapabilities, etc.). Therefore, the footprint size of the actual LPP encoder/decoder is the same, regardless of which/how many positioning methods are supported by a device. This can be a potential problem and cost impediment for low-cost devices having limited memory to support only a small subset of positioning methods they require. This issue was already discussed during Rel-14 when NB-IoT support was added to LPP.

As it is expected SLPP will evolve/increase in future Releases, the SLPP ASN.1 design should allow for "selective ASN.1 compilation", where an implementation needs to compile the ASN.1 for only the SLPP functionality supported. This could be achieved by grouping the "SLPP functionality" into separate, standalone modules. Using LPP as an example, each SLPP module/group could be an individual positioning method. Instead of using an explicit ASN.1 definition for each message body IE, it could be defined as just an OCTET STRING. For example, the RequestCapabilities-r9-IEs above could be defined as follows:

RequestCapabilities-r9-IEs ::= SEQUENCE {

commonIEsRequestCapabilities		OCTET STRING  OPTIONAL,	-- CommonIEsRequestCapabilities

a-gnss-RequestCapabilities			OCTET STRING  OPTIONAL,	-- A-GNSS-RequestCapabilities

otdoa-RequestCapabilities			OCTET STRING  OPTIONAL,	-- OTDOA-RequestCapabilities

ecid-RequestCapabilities			OCTET STRING  OPTIONAL,	-- ECID-RequestCapabilities

epdu-RequestCapabilities			OCTET STRING  OPTIONAL,	-- EPDU-Sequence

...,

[[	sensor-RequestCapabilities-r13	OCTET STRING  OPTIONAL,	-- Sensor-RequestCapabilities

tbs-RequestCapabilities-r13		OCTET STRING  OPTIONAL,	-- TBS-RequestCapabilities

wlan-RequestCapabilities-r13	OCTET STRING  OPTIONAL,	-- WLAN-RequestCapabilities

bt-RequestCapabilities-r13		OCTET STRING  OPTIONAL	-- BT-RequestCapabilities

]],

[[	nr-ECID-RequestCapabilities-r16	OCTET STRING  OPTIONAL,	-- NR-ECID-RequestCapabilities

nr-Multi-RTT-RequestCapabilities-r16

OCTET STRING  OPTIONAL,	-- NR-Multi-RTT-RequestCapabilities

nr-DL-AoD-RequestCapabilities-r16

OCTET STRING  OPTIONAL,	-- NR-DL-AoD-RequestCapabilities

nr-DL-TDOA-RequestCapabilities-r16

OCTET STRING  OPTIONAL,	-- NR-DL-TDOA-RequestCapabilities

nr-UL-RequestCapabilities-r16	OCTET STRING  OPTIONAL	-- NR-UL-RequestCapabilities

]]

}

-- ASN1STOP

The positioning method IEs would be defined in separate ASN.1 PDUs which are imported by the main SLPP PDU. In this way, any (sub) PDU containing an unsupported positioning method or functionality can be excluded from the protocol and would not need to be compiled. Therefore, the message encoder/decoder footprint size is reduced, since it does not contain the entire SLPP. Any unsupported received IE/message would still be correctly decoded (as an OCTET STRING), and then ignored, as is the case for an explicit ASN.1 definition. But in the above example, the content of a not supported OCTET STRING does not need to be decoded individually.

Recognizing the implementation benefits of this approach, RAN2 took the following working assumption (WA) in RAN2 #122 [5].

Agreements: WA: The SLPP ASN.1 design should allow "selective ASN.1 compilation", i.e. The overall SLPP functionality is divided into "groups", where each group is defined as a separate ASN.1 module.

From the preceding points it is evident selective compilation significantly benefits UE implementation, reducing overall memory requirements.  As such, RAN2 should adopt the working assumption as an agreement.

Proposal 31:	RAN2 adopts as an agreement the working assumption, “The SLPP ASN.1 design should allow "selective ASN.1 compilation", i.e. The overall SLPP functionality is divided into "groups", where each group is defined as a separate ASN.1 module.”

# Summary

Observation 1: Communication over sidelink may incur packet loss due to changes in participating UE relative position and/or occlusion, potentially leading to SLPP message loss. Packet loss and resulting SLPP message loss over sidelink is independent of cast type, . occurring for operation over unicast, broadcast and groupcast, and will result in degraded performance and compromise the ability to conduct and exchange range and/or position measurements and measurement results

Observation 2: Introducing to SLPP LPP-like reliable transport mechanisms of duplicate detection, acknowledgement and retransmission can be applied to SLPP to mitigate the effects of sidelink packet loss .

Observation 3: Invocation of SLPP reliable transport can be at the discretion of the sidelink ranging and positioning session initiator.

Observation 4: A UE may initiate a sidelink positioning session with one of the UEs in its vicinity, or with a group of UEs in its vicinity. When initiating a sidelink positioning session with a group of UEs, the group could constitute all, or a subset of the UEs in the initiating UE’s vicinity.

Observation 5: When a group of UEs participate in a sidelink ranging and positioning session, a subset or all of the UEs in the session may require location information from all of the other UEs in the ranging/positioning session.

Observation 6: Security for groupcast or broadcast transmission is not an impediment to Location Information exchange over groupcast/broadcast.

Observation 7: Existing 3GPP V2X payload standardized messages support exchange of UE GNSS location and UE GNSS location accuracy over groupcast and broadcast.

Observation 8: A UE must be able to support multiple, simultaneous Sidelink ranging and positioning SLPP sessions, each with potentially differing sets of UEs and potentially different sets of positioning requirements. A UE participating in multiple, concurrent sessions should be able to track each individual session.

Observation 9: UE-specific criteria including at least capabilities and privacy requirements, are the basis for UE to determine participation in sidelink ranging and positioning SLPP session.

Observation 10: SLPP group positioning can be based on upper/application-layer defined groups, with group information provided to the SLPP layer.

Observation 11: The SLPP Capabilities exchange (SLPP Request Capabilities message / SLPP Provide Capabilities message) between UEs in an Application layer managed group may result in only a subset of the group members (UEs) having compatible sidelink positioning (SLPP) capabilities.

Observation 12: Using existing LPP message types there is no mechanism for a UE which cannot join an SLPP session to reject or decline the SLPP session.

Observation 13: The basic three LPP-based SLPP message types (Request/Provide Capabilities, Request/Provide Assistance Data, Request/Provide Location Information) do not provide a mechanism to remove UEs in an Application layer managed group having incompatible SLPP capabilities from the SLPP positioning session.

Observation 14: An Initiator-assigned SLPP UE ID may be a more compact, simpler identifier for an SLPP session than a UE Application ID or UE Layer-2 ID.

Observation 15: UE-based (distributed) sidelink positioning and ranging enables UE position calculation without the reliance on a separate UE, and can reduce overall SLPP signaling required for sidelink positioning.

Observation 16:	Sidelink ranging and positioning over groupcast includes both upper/application layer managed groupcast and upper/application layer connectionless groupcast.

Observation 17:	SLPP sessions for upper/application layer managed groupcast may require the session initiator to indicate which UEs in the managed group should participate in the SLPP session (reply to SLPP session messages and/or perform actions based on receipt of SLPP session messages).

Observation 18:	SLPP sessions for upper/application layer managed groupcast may require the session initiator to specify the subset of UEs in the managed group should participate in the SLPP session, and the groupcast destination layer-2 ID(s) those UEs should use for session participation.

Observation 19: SLPP sessions for upper/application layer managed groupcast may require the session initiator to indicate the destination layer-2 ID(s) other UEs in the session should use for participation in the session.

SLPP Transport:

Proposal 1:	SLPP supports at least the LPP reliable transport mechanisms for duplicate detection, acknowledgement and retransmission for unicast, broadcast and groupcast.

Proposal 2:   RAN2 agrees to convert the following WA to an agreement, “WA: RAN2 understand that group positioning is to acquire location estimates of target UEs (absolute positioning) or multiple UE pairs (Ranging/relative positioning) per LCS request, in line with the guidance already received from SA2.”

Proposal 3:	SLPP supports Location Information exchange over unicast, groupcast and broadcast.

SLPP Session Operation:

Proposal 4:    RAN2 confirms UEs in a sidelink positioning procedure verify aspects of the sidelink ranging and positioning session over PC5 via SLPP, including that ranging and sidelink positioning is permitted, ranging and sidelink positioning results may be transferred to an LCS client or AF.

Proposal 5:	The SLPP Layer may assign an SLPP Session ID unique to the specific sidelink positioning session. The SLPP Session ID could be an integer value or might comprise a unique indication of the initiator UE plus an initiator UE-assigned local numeric ID.

Proposal 6:	SLPP supports the ability for a UE to explicitly accept, reject or terminate an SLPP session based on UE-specific criteria.

Proposal 7: 	RAN2 agrees to convert the following WA to an agreement, “WA: At least part of the group management for group positioning is performed at upper/application layer.”

Proposal 8:	SLPP should be designed to support a positioning sublayer and a session and service management sublayer. The positioning sublayer can support LPP-based functions for range and position determination, while the session and service management sublayer can support those aspects of sidelink ranging and positioning not natively supported by LPP, including UE ability to accept/reject/terminate session participation based on UE-specific criteria.

Proposal 9:	The SLPP Layer may assign an individual SLPP UE ID to each UE in an SLPP session. The SLPP UE ID may be an integer (e.g. in the range 1 to n).

Proposal 10:	SLPP shall support at least the following message types for SLPP Session-based operation:

- SLPP Create Session Request/Accept/Reject

- SLPP Session Start Request/Response

- SLPP Session End Request/Response

- SLPP Session Modify Request/Accept/Reject

Proposal 11:	Session-less operation is established without mutual exchange of SLPP session establishment signaling via transmission of the SLPP Provide Assistance Data message.

Proposal 12:   SLPP session-less operation can be denoted by a specific allocation of SLPP Session ID values or by omission of the SLPP Session ID.

SLPP Positioning Procedures:

Proposal 13: 	SLPP should support UE-based (distributed) sidelink positioning enabling multiple UEs to determine position and range based on exchanged location information.

SLPP Messages:

Proposal 14: 	SLPP should indicate the transaction (communication) mode to be used for each SLPP message, i.e. whether broadcast mode, groupcast mode or unicast mode is to be used (e.g., in a common SLPP message header). At least the following common transaction modes shall be supported:

- Unicast transaction

- Group Transaction with Group Replies

- Group Transaction with Unicast Replies

- Broadcast Transaction.

NOTE: Not all SLPP message types may be allowed for all transaction modes.

Proposal 15: 	For groupcast SLPP sessions, SLPP can indicate the set of UEs which should participate in the ranging/positioning session, and the appropriate groupcast destination layer 2 ID UEs should use for session participation.

Proposal 16: 	An SLPP Provide Capabilities message should provide information on the ability of the device to support different position methods defined for SLPP (e.g., SL-RTT, SL-TDOA, etc.), different aspects of a particular position method (e.g. different types of measurements or assistance data) and features specific to certain UE roles (e.g. ability to operate as a server or anchor UE, etc.).

Proposal 17: 	The SLPP Request Assistance Data message should allow an SLPP-capable endpoint to request positioning assistance data (e.g., SL-PRS configuration information and resources) from a server (e.g., server UE or LMF). The SLPP Provide Assistance Data message should allow a server (e.g., server UE or LMF) to provide positioning assistance data (e.g., SL-PRS configuration information and resources) to participant UEs in a session.

Proposal 18: 	The SLPP Request/Provide Location Information message should allow an SLPP-capable endpoint to request/provide location measurements or location estimate from participant UEs in a session.

Proposal 19:	The SLPP Create Session Request message should allow an SLPP UE to request creation of a sidelink ranging and positioning session and provide information on the sidelink ranging and positioning session proposed, including SLPP and Layer-2 IDs.

Proposal 20:	The SLPP Create Session Accept message should allow an SLPP UE to accept an SLPP session proposed by an SLPP-capable UE.

Proposal 21:	The SLPP Create Session Reject message should allow an SLPP UE to reject an SLPP session proposed by an SLPP-capable UE.

Proposal 22:	The SLPP Session Start Request message should allow an SLPP UE to request the start of a sidelink ranging and positioning session proposed using the SLPP Create Session Request/Response messages and provide information on the session participating SLPP UEs.

Proposal 23:	The SLPP Session Start Response message should allow an SLPP UE to acknowledge the start of an SLPP session.

Proposal 24:	The SLPP Modify Session Request message should allow an SLPP UE to request modification of an existing SLPP session and provide information on the modified SLPP session proposed and session participating SLPP UEs, including SLPP and Layer-2 IDs.

Proposal 25:	The SLPP Modify Session Accept message should allow an SLPP UE to accept an SLPP session modification proposed by an SLPP-capable UE.

Proposal 26:	The SLPP Modify Session Reject message should allow an SLPP UE to reject an SLPP session modification proposed by an SLPP-capable UE.

Proposal 27:	The SLPP Session End Request message should allow an SLPP UE to request the termination of a sidelink ranging and positioning session.

Proposal 28:	The SLPP Session End Response message should allow an SLPP UE to acknowledge the termination of an SLPP session.

Sidelink Positioning Discovery

Proposal 29:	Discovery is included in the sidelink positioning procedure for all PC5 sidelink positioning scenarios (in-coverage and out-of-coverage).

SLPP ASN.1 Design

Proposal 30:	An SLPP message contains a common header (including e.g., session ID, UE IDs, transaction IDs, etc.) and a message body realizing the individual SLPP transaction types (capability transfer, assistance data transfer, location information transfer, etc.).

Proposal 31:	RAN2 adopts as an agreement the working assumption, “The SLPP ASN.1 design should allow "selective ASN.1 compilation", i.e. The overall SLPP functionality is divided into "groups", where each group is defined as a separate ASN.1 module.”

# References

- RP-230328 Revised WID on Expanded and Improved NR Positioning

- R2-2301904 Report from session on positioning and sidelink relay, RAN2 #121

- R2-2213004 Report from session on positioning and sidelink relay, RAN2 #120

- R2-2304204 Report from session on positioning and sidelink relay, RAN2 #121bis-e

- R2-2306544 Report from session on positioning and sidelink relay, RAN2 #122

- TS 23.586

- R2-2300076_S2-2301464 Reply LS on Reply LS on RAN dependency for Ranging/Sidelink Positioning

- R2-2300079_S2-2301786 Reply LS on SL positioning groupcast and broadcast

- TR 23.700-86

- R2-2300084_S3-230430 Reply LS to LS on SL positioning groupcast and broadcast

- TR 38.845

- TR 22.855

- TS 23.287

- TS 38.305

- TS 23.304

- 24.587

- 24.386

- R2-2210804 Report from session on positioning and sidelink relay, RAN2 #119bis-e

- S2-2305748 Support of MT-LR for Ranging and Sidelink Positioning, CR to 23.273

- S2-2305749 Ranging and Sidelink Positioning MO-LR procedure, CR to 23.273

- R2-2300079_S2-2301786, “LS on SL positioning groupcast and broadcast (R2-2213142/ S2-2300048)”

- R2-2301938_S2-2303837, R2-2302448, "LS on support of multiple Target UEs"

- R2-2300084_S3-230430

- S2-2307552 Ranging and Sidelink Positioning MO-LR procedure, CR to 23.273

- R2-2306334 SLPP session management and operation