3GPP TSG-RAN WG2 Meeting #123	R2-2308397

Toulouse, France, August 21 – 25, 2023

Agenda item:	7.2.3

Source: 	Qualcomm Incorporated

Title: 	Integrity of NR Positioning Technologies

Document for:		Discussion and Decision

# 1.	Introduction

The Work Item on "Expanded and Improved NR Positioning" includes the following objective for "RAT-dependent Integrity" [1]:

- Specify the error modelling parameters, signalling, and procedures to support UE-based and LMF-based integrity of RAT-dependent positioning methods [RAN2, RAN3].

During the Study Item Phase, RAN2 made the general agreement to re-use the GNSS 'Integrity Principle of Operation' also for the NR positioning technologies [2]:

RAN2 to confirm the integrity principle of operation defined in the section 8.1.1a of TS38.305, including integrity definition (e.g., Error, Bound, Time to Alert, DNU, Residual Risk, irMinimum, irMaximum and Correlation Times; FFS if all parameters are needed in the RAT-dependent case), Equations for the GNSS integrity are reused for RAT dependent positioning methods. RAN2 may add the mapping between Integrity definition/Fields (Integrity Alerts, error bounds (mean, StdDev), Residual Risks, Integrity correlation times) and Error sources/assistance data for RAT-dependent positioning methods later once RAN1 identifies new error sources. Use DNU flag for RAT-dependent integrity, with the meaning that the concerned assistance data cannot be used for integrity calculation but may be usable for positioning. Signalling details and relation to error sources can be determined in normative work. FFS which positioning methods are affected based on the progress in RAN1.

At RAN2#121, RAN2#121bis-e and RAN2#122, the following agreements were made [3],[4]:

Agreements:

RAN2 anticipate that the error sources are overbounded by a Gaussian distribution.

LS to RAN1 to check this view and ask about the parameters for the overbound distributions.

Agreements:

TRP related error source bounds can be provided to UE via dedicated LPP providing assistance message or posSIB.

Any interaction between the LMF and NG-RAN to support determination of error sources is in RAN3 scope.  Other aspects of determining the TRP error sources are left to deployment and implementation.

For UE-based RAT-dependent integrity, the PL and/or its corresponding TIR are provided to LMF as legacy, using the existing common LPP signalling from Rel-17.

Agreement:

LS to RAN1 to include a request for confirmation that the beam-related information (Beam Bore-Sight Direction and Beam Antenna Information) are error sources for DL-AoD positioning.

Agreement:

LS to RAN1 to include the question of whether RAN1 identify a need for a DNU flag for measurements.

Agreement:

For RAT-dependent integrity, the PL calculation is performed by the entity which also performs the position calculation for a location process.

Agreements:

For UE-based integrity, the integrity parameters of error sources for RAT-dependent integrity are included in assistance data.

LPP Request/Provide Assistance Data are reused for retrieving the integrity parameters to the UE from the LMF.  The request is per positioning method (as in legacy operation) and the provided integrity parameters are as appropriate for the selected positioning method.

Use of posSIBs for integrity parameters is not excluded.

Working assumption:

For LMF-based integrity, no integrity KPI (TTA, TIR, and AL) and integrity results transfer in LPP message.

Working assumption:

It is left to LMF implementation to decide the measurement error source bound distribution based on the measurement results from UE and/or NG-RAN.

Agreement:

Indicate the WA above in the LS to RAN1 to allow them to register any concern.

Agreement:

Capture the stage 2 impact for RAT-dependent integrity in section 7 of 38.305.  Initial running CR to be seen at next meeting, using R2-2302504 and R2-2303682 as baseline.

Agreement:

For stage2 description of RAT-dependent integrity, move the section of “Integrity Principle of Operation” to a generic section that is not specific to positioning methods.

Agreements:

Represent the TRP and ARP location errors by a Gaussian paired over-bounding.

Represent the RTD errors by a Gaussian paired over-bounding.

At RAN2#121bis, an LS was sent to RAN1 with some remaining questions [5]. RAN1 replied in [6] as follows:

RAN1 would like to provide the answers to the following questions: Working assumption: It is left to LMF implementation to decide the measurement error source bound distribution based on the measurement results from UE and/or NG-RAN. Q0: RAN2 would like to kindly request RAN1 to confirm whether they have any concern on the above working assumption. Reply to Question 0: From RAN1’s perspective, no concerns are identified for the RAN2 working assumption “It is left to LMF implementation to decide the measurement error source bound distribution based on the measurement results from UE and/or NG-RAN.” Q1: Are beam-related information (Beam Bore-Sight Direction and Beam Antenna Information) error sources for DL-AOD positioning? Reply to Question 1: Regarding whether beam-related information (Beam Bore-Sight Direction and Beam Antenna Information) are error sources or not, RAN1 made the following conclusion in RAN1#111, “RAN1 could not reach consensus on whether beam information (NR-TRP -BeamAntennaInfo) and boresight direction of DL PRS (NR-DL -PRS -BeamInfo) are error sources or not for DL-AoD for UE -based positioning integrity mode.”, where the definition of “UE-based positioning integrity mode” can be found in Table 9.4.1.1.1 in TR 38.857. Q2: Are DNU flag(s) for TRP/UE positioning measurements needed or not? Reply to Question 2: As agreed in RAN1#110b-e, from RAN1 perspective, study of the application of DNU flag for determination of positioning integrity is within the scope of RAN2 discussion. Specification impact(s) of DNU flag(s) can be discussed in RAN2.

In this contribution, we discuss the remaining issues for the integrity of the NR positioning technologies.

# 2.	Discussion

##### DNU Flags

The GNSS integrity principle of operation is contingent on 'DNU flags'. I.e., the probability that an error exceeds the bound for longer than TTA under the condition 'no DNU' must be less than a specified threshold. One reason for providing these 'DNU flags' is that the UE must fail safe even in the event of a loss of connectivity [9]. This means that the UE must receive an affirmative signal that the bounds are still valid within the TTA, otherwise it must assume that a feared event may have occurred and the notification was not delivered. If the network detects a feared event in the window between regular assistance data updates the DNU flags are set to 'true'. The DNU flags must be provided more frequently (according to the Time-to-Alert (TTA) requirements) so that the assistance data (e.g., obtained from broadcast or pre-configured) can be invalidated quickly.

For GNSS, the granularity of the DNU flags is currently as follows [8]:

-	per GNSS,

-	per SV,

-	per Signal,

- 	per ionospheric correction,

-	per tropospheric correction.

Employing "do not use" flags for GNSS ensure users drop satellites/assistance data that may be unhealthy or performing poorly. For example, orbit and clock data may be computed separately from atmospheric errors such that the UE can fall back to PPP with integrity in the case the PPP-RTK chain is unavailable, etc..  The same argumentation applies to NR positioning technologies: If e.g., beam antenna information is not available (DNU), the UE can fall back to bore-sight direction information; or if RTD information is not available (DNU), the UE may fall back to DL-AoD, etc.

The 'DNU Flags' for GNSS are provided in the IEs GNSS-RealTimeIntegrity and GNSS-Integrity-ServiceAlert. For NR positioning, a new assistance data element is required for providing 'DNU Flags'; e.g., IE NR-Integrity-ServiceAlert.

Proposal 1:	The DNU flags are provided per TRP and per error contribution (e.g., TRP location, RTD, beam information, etc.) in a new IE NR-Integrity-ServiceAlert.

RAN2 also discussed the potential need for DNU flags for TRP/UE positioning measurements, and asked RAN1 for guidance (see section 1). However, RAN1 consider this issue under RAN2 scope [6]:

As agreed in RAN1#110b-e, from RAN1 perspective, study of the application of DNU flag for determination of positioning integrity is within the scope of RAN2 discussion. Specification impact(s) of DNU flag(s) can be discussed in RAN2.

DNU flags for measurements are currently not part of the "integrity principle of operation" [7], [8]. It is unclear how a receiver can judge whether a single measurement is suitable for integrity determination without performing position calculation. I.e., usually, redundant signals are used to produce several fixes to determine whether or not a fault can be associated with any of the signals. Measurements from faulty signals would then be excluded from the position calculation (and therefore, from integrity determination). Hence, "DNU flags for measurements" appear useless/non-feasible when the position calculation function resides in an LMF. Any information about the integrity of the NR positioning signals themselves must be available at the LMF anyhow (e.g., integrity assistance data).

Proposal 2:	DNU flags for TRP/UE positioning measurements are not needed.

##### Rate Error bounds

The GNSS integrity bounds also include the error changes over time as a bound of the errors mean and standard deviation's rate of change [7], [8]. Since the same behaviour is expected for the NR positioning methods – e.g., inter-TRP synchronization error may change over time (drift) – the mean and standard deviation for the rate error bounds should optionally also be provided for the NR positioning error sources.

Proposal 3:	Analogous to GNSS, the mean and standard deviation of rate error bounds can optionally also be provided for the NR positioning error sources.

##### Correlation Times

UEs/LMFs which may make use of  time-based estimation techniques such as Kalman Filtering must also be provided with information about the time correlation of the errors. For GNSS, the assistance data may include the 'correlation times', defined as the minimum time interval beyond which two sets of assistance data parameters for a given error can be considered to be independent from one another.

An implementation typically uses the same integrity algorithm/principles for all supported positioning technologies, in particular if the position calculation is hybrid (e.g., GNSS+Sensors+NR, etc.). Therefore, a UE may use a sequential integrity monitoring algorithm also for the NR positioning technologies.

Proposal 4:	The 'Integrity Correlation Times', defining the minimum time interval beyond which two sets of assistance data parameters for a given error can be considered to be independent from one another, can optionally be provided for the integrity assistance data.

##### LMF-based PL calculation

To determine the PL by an LMF for UE-assisted mode or network-based positioning methods, the LMF requires the overbounding standard deviation of the UE and TRP measurement error (e.g., RSTD, RTOA, Rx-Tx, AoA, DL-PRS RSRP, DL-PRS RSRPP), which provides an upper bound on the standard deviation of the nominal measurement error σx,i.  That is, under fault-free condition, the standard deviation of the measurement error is no greater than σx,i.

The measurement error is not observable and need to be approximated/estimated via e.g., standard error of the mean which estimates the variability across multiple samples of a population:

| σ=(xi-x)2N | (1) |
| --- | --- |

This standard deviation may serve as a measure of uncertainty and requires a number N of samples. Therefore, periodic measurements are required, where either the UE/TRP determines the sample statistics or the LMF uses the periodic reports from the UE/TRP to determine sample statistics.

The sample statistics may be derived or used by an e.g., Kalman Filter whose details depends on LMF implementation/algorithms used. Instead of deriving the measurement statistics at the UE/TRP and reporting to an LMF, it seems more flexible and implementation-independent if an LMF determines any statistics from the (already specified) UE/TRP measurement report directly. I.e., a LMF could request periodic reporting from UEs and TRPs and could derive any required statistics from the  measurement reports.

This would not require specifying a particular method/equation for determining the error statistics/bounds and therefore, could be up to LMF implementation. For example, the LMF could decide which measurement items are used to determine the bounds (e.g., multiple paths, measurement quality, LOS/NLOS Information, etc.), or how many samples are used or needed (e.g., N in equation (1), etc.). Therefore, it could be implementation defined and would be most flexible (i.e., no "hard coding" of the method/definition would be needed).

A corresponding working assumption was made at RAN2#121bis (see section 1):

Working assumption:

It is left to LMF implementation to decide the measurement error source bound distribution based on the measurement results from UE and/or NG-RAN.

Agreement:

Indicate the WA above in the LS to RAN1 to allow them to register any concern.

As summarized in section 1, RAN1 did not raise any concerns on the Working Assumption [6]:

From RAN1’s perspective, no concerns are identified for the RAN2 working assumption “It is left to LMF implementation to decide the measurement error source bound distribution based on the measurement results from UE and/or NG-RAN.”

Hence, it is proposed:

Proposal 5: 	Confirm the Working Assumption: "It is left to LMF implementation to decide the measurement error source bound distribution based on the measurement results from UE and/or NG-RAN."

##### Error sources for DL-AOD positioning

As noted in section 1 above, "RAN1 could not reach consensus on whether beam information (NR-TRP-BeamAntennaInfo) and boresight direction of DL PRS (NR-DL-PRS-BeamInfo) are error sources or not for DL-AoD for UE-based positioning integrity mode" [6]. However, this does not really answer RAN2's question on whether beam-related information (Beam Bore-Sight Direction and Beam Antenna Information) are error sources for DL-AOD positioning or not. The RAN1 response could imply that the beam information are error sources for UE-assisted mode, but not for UE-based, which would be inconsistent.

The Beam Bore-Sight Direction/Beam Antenna Information plays a role in DL-AoD positioning that is very closely analogous to the role played by the RTD in DL-TDOA. DL-TDOA position calculation works by associating each TDOA measurement between two TRPs with the difference in distance (or equivalently, flight times) from the two TRPs to the UE, and then using the geometry of these distances. This association assumes that the signals begin their flight at exactly the same time; if they do not, the measured TDOA will also reflect this lack of time-alignment, which is not associated with the geometry, and must therefore be removed. The RTD Info provides exactly this information, on how to remove the lack of time-alignment and is thus critical for position computation.

Similar to the way DL-TDOA compares TOAs measured from different PRS (from different TRPs) to infer the difference in distances to those TRPs, the DL-AoD method compares RSRPs from different PRS (from the same TRP) to infer the AoD from the TRP. Thus, just as an uncorrected or incorrectly accounted-for offset between the start-of-flight times of the two PRS results in an error in the TDOA and thus the position, similarly an error in the boresight pointing angles of the Tx beams of the two PRS resources results in an error in the estimated DL-AOD.

Proposal 6:	The beam related information (Beam Bore-Sight Direction/Beam Antenna Information) are error sources for DL-AoD positioning.

# 3.	Summary

In this contribution, we discussed some remaining aspects of integrity support for the NR Positioning Technologies. The following proposals are made.

Proposal 1:	The DNU flags are provided per TRP and per error contribution (e.g., TRP location, RTD, beam information, etc.) in a new IE NR-Integrity-ServiceAlert.

Proposal 2:	DNU flags for TRP/UE positioning measurements are not needed.

Proposal 3:	Analogous to GNSS, the mean and standard deviation of rate error bounds can optionally also be provided for the NR positioning error sources.

Proposal 4:	The 'Integrity Correlation Times', defining the minimum time interval beyond which two sets of assistance data parameters for a given error can be considered to be independent from one another, can optionally be provided for the integrity assistance data.

Proposal 5: 	Confirm the Working Assumption: "It is left to LMF implementation to decide the measurement error source bound distribution based on the measurement results from UE and/or NG-RAN."

Proposal 6:	The beam related information (Beam Bore-Sight Direction/Beam Antenna Information) are error sources for DL-AoD positioning.

# References

[1]		RP-231460, "Revised WID on Expanded and Improved NR Positioning".

[2]		RP-222982, "Status Report to TSG", RAN#98e.

[3]		RP-230327, "Status Report to TSG", RAN#99.

[4]		RP-231092, "Status Report to TSG", RAN#100.

[5]		R2-2304563, "LS on the RAT-dependent positioning integrity", RAN2.

[6]		R2-2307004 (R1-2306157), "LS reply on the RAT-dependent positioning integrity", RAN1.

[7]		3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[8]		3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

[9]		R2-2110141, "Discussion on GNSS Integrity Assistance Data", Swift Navigation, Mitsubishi Electric 					Corporation, Intel Corporation, Ericsson.