3GPP TSG-RAN WG2 Meeting #123	R2-2308400

Toulouse, France, August 21 – 25, 2023

Agenda item:	7.25

Source: 	Qualcomm Incorporated

Title: 	On Positioning Reference Unit support in LPP

Document for:		Discussion and Decision

# 1.	Introduction

RAN2 discussed an incoming LS from SA2 [1] at RAN2#121bis and provided the following response to SA2's questions [2]:

SA2 request: Additionally, SA2 also would like to know what location information can be provided by PRU, and whether RAN1 and RAN2 see benefit to allow PRU to provide its location information together with the location measurements reports.. RAN2 response: A PRU should be able to report (a) the location coordinates and associated location quality/uncertainty of the PRU only, (b) the location measurements performed by the PRU only, or (c) the location coordinates and associated location quality/uncertainty of the PRU together with any performed location measurements, where location measurements above include the currently defined location measurements in LPP and potential new location measurements defined in Rel-18; and location coordinates and location quality/uncertainty provide the location and quality of the reported location, respectively, of the PRU.

At RAN2#122, contributions were submitted which propose a modification to the LPP Request Location Information message [3],[4]. It was argued in [3][4] that in order to support the above RAN2 agreement on location information reporting, a LPP change to the location request is required. In particular, a new LocationInformationType was proposed, e.g.:

LocationInformationType ::= ENUMERATED {

locationEstimateRequired,

locationMeasurementsRequired,

locationEstimatePreferred,

locationMeasurementsPreferred,

...,

[[

locationEstimateAndMeasurementsRequired-r18

]]

}

In this contribution, we show that no LPP changes are required to support the reporting of known location coordinates of the PRU together with any performed location measurements.

# 2.	Discussion

As described in TS 38.305 clause 5.4.5 [5], a PRU is essentially a UE with known location. The PRU performs location measurements (e.g., any measurements defined in LPP) at a known location and/or transmits SRS from a known location which can be measured by TRPs.

With the known PRU location and DL and/or UL location measurements, a LMF may improve the positioning performance for target UEs. A PRU could for example be used to determine time synchronization errors between gNBs, or to derive any other "correction information" for UEs in the vicinity of PRUs. For Rel-18, PRUs are essential to support carrier-phase NR positioning based on double-difference measurements. The PRU-principle is the same as GNSS Reference Stations at known locations used to support differential GNSS (e.g., D-GNSS, RTK).

To exploit a PRU for improving positioning performance or to enable carrier-phase based NR positioning, the measurements must be made at a known location. This known location must be derived independently from the PRU reported location measurements. I.e., the PRU location determination and any PRU measurements reported must be independent of each other. Otherwise both (location and measurements) provide the same information which can not be used to improve positioning performance or support carrier-phase based NR positioning. This is not different compared to GNSS Reference Stations and also noted by RAN1 in their response to SA2 [6]:

RAN1 reply: A PRU can report its location and associated uncertainty as is the case for other UEs. It is not necessary to always include the PRU location information with the PRU measurements in the same report. The PRU location information and measurements should be decoupled, where decoupled means that the PRU location information is determined independently of the reported measurements, even if the PRU location information and the PRU measurements would be included in the same report.

Observation 1:	The PRU location information and measurements are decoupled, where decoupled means that the PRU location information is determined independently of the reported measurements.

PRU support was added to TS 23.273 [7]. Procedures applicable to a PRUs were specified in clause 6.17 of TS 23.273 [7]. In particular, the positioning procedures between an LMF and a PRU may make use of LPP as for a normal UE.

LPP can already support the reporting requirements summarized in section 1 above.

(a) the location coordinates and associated location quality/uncertainty of the PRU only:

The known PRU coordinates can be provided in the PRU Association Request message per SA2 procedures [7] or using any PRU/LMF supported positioning method to determine (or verify) the known PRU location. In practice, it is quite likely that the known PRU location is determined with UE-based or standalone HA-GNSS (e.g., PPP or RTK), but UE-assisted mode could equally be used to determine or verify the known PRU location. No changes to LPP are required to support this functionality.

Observation 2:	To report the (known) location coordinates of the PRU, no LPP changes are required.

(b) the location measurements performed by the PRU only:

To report the location measurements performed by the PRU, the "normal" location reporting for UE-assisted mode can be used (e.g., for DL-TDOA, Multi-RTT, etc.). The type of location measurements depends on the use case. For example, to determine gNB synchronization errors, the PRU (at known location) would report DL-TDOA measurements (RSTD). With the RSTD measurements and the known location, the synchronization error (RTD error) could be determined. To support NR carrier-phase based positioning, the PRU (at known location) would report the same carrier-phase measurements as the target UE (probably at the same time epoch, which requires LPP enhancements discussed in RAN1, which however, are the same for UE and PRU). No changes to LPP (beyond those required for Rel-18 anyhow) are required to support this functionality.

Observation 3:	To report location measurements from the PRU at know location, no LPP changes are required.

(c) the location coordinates and associated location quality/uncertainty of the PRU together with any performed location measurements:

This reporting would be required if the (known) location of the PRU changes during the location measurements/reporting, which may be the case if the PRU is moving as discussed in RAN1 [8][9]. Only the PRU knows that it is moving when performing location measurements or changing its location for other reasons. Therefore, an explicit request by an LMF would not help (or is not needed) to achieve the required functionality. The PRU must have some means to detect movement (e.g., sensors, continuous GNSS positioning etc.) and would need to include the location coordinates at which the location measurements were performed. This can already be achieved with existing LPP by including both,  CommonIEsProvideLocationInformation and method-ProvideLocationInformation (with 'method' = DL-TDOA, DL-AoD, etc.) in a ProvideLocationInformation message body in a LPP message.

Observation 4:	To report location measurements from the PRU together with (known) PRU location, no LPP changes are required.

In addition, the introduction of a new LocationInformationType for 'locationEstimateAndMeasurementsRequired' is misleading in this context, since it implies that this is just a combination of UE-based and UE-assisted mode, which can not enable PRU operation (Observation 1).

Therefore,

Proposal 1:	No new LocationInformationType is introduced in LPP for reporting of PRU location coordinates together with location measurements.

However, since only the PRU knows that it is moving when performing location measurements/reporting or when its location has changed for other reasons, some clarification (e.g., in TS 38.305) may be useful. A corresponding Text Proposal is provided in the Annex of this contribution.

Proposal 2:	Agree the text proposal in the Annex of this contribution.

# 3.	Summary

In this contribution we discussed the location information reporting for PRUs. The following Observations and Proposals were made.

Observation 1:	The PRU location information and measurements are decoupled, where decoupled means that the PRU location information is determined independently of the reported measurements.

Observation 2:	To report the (known) location coordinates of the PRU, no LPP changes are required.

Observation 3:	To report location measurements from the PRU at know location, no LPP changes are required.

Observation 4:	To report location measurements from the PRU together with (known) PRU location, no LPP changes are required.

Proposal 1:	No new LocationInformationType is introduced in LPP for reporting of PRU location coordinates together with location measurements.

Proposal 2:	Agree the text proposal in the Annex of this contribution.

# References

[1]	R2-2302449 (S2-2303861), "LS on PRU procedures", SA2 LS in Rel-18 5G_eLCS_Ph3	To:RAN2, RAN1.

[2]	R2-2304459, "Response LS on PRU Procedures", RAN2 LS out Rel-18 NR_pos_enh2 To:SA2.

[3]	R2-2305345, "Draft CR of new location information type for PRU", vivo.

[4]	R2-2306024, "On the Positioning Reference Units aspects", Ericsson.

[5]	3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[6]	R2-2302409 (R1-2302146), "LS Reply on PRU Procedures", RAN1 LS in REL-18 NR_pos_enh2-Core, 	5G_eLCS_Ph3 To: SA2 Cc: RAN2, RAN3.

[7]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[8]	R1-2302047, "Discussion of SA2 LS on PRU Procedures", Moderator.

[9]	R1-2302106, "Discussion (2nd round) of SA2 LS on PRU Procedures", Moderator.

# Annex: Text Proposal for TS 38.305

### 5.4.5	Positioning Reference Unit (PRU)

A Positioning Reference Unit (PRU) at a known location can perform positioning measurements (e.g., RSTD, RSRP, UE Rx-Tx Time Difference measurements, etc.) and report these measurements to a location server. In addition, the PRU can transmit SRS to enable TRPs to measure and report UL positioning measurements (e.g., RTOA, UL-AoA, gNB Rx-Tx Time Difference, etc.) from PRU at a known location. The PRU measurements can be compared by a location server with the measurements expected at the known PRU location to determine correction terms for other nearby target devices. The DL- and/or UL location measurements for other target devices can then be corrected based on the previously determined correction terms.

From a location server perspective, the PRU functionality is realized by a UE with known location.

A PRU may derive its location via various means. A specific method is not specified as this is implementation-defined. A PRU may provide its location to an LMF during the PRU Association Procedure as specified in TS 23.273 [35] and/or via LPP using the procedures specified in clause 8. A PRU which is moving or changes its location for other reasons shall always provide the requested location measurements (e.g., RSTD, RSRP, UE Rx-Tx Time Difference measurements, etc.) together with the location coordinates of the PRU for the time when the location measurements were obtained.