3GPP TSG-RAN WG2 Meeting #123	R2-2308402

Toulouse, France, August 21-25, 2023

Agenda Item:	7.5.3

Souce:	MediaTek Inc.

Title:	Remaining issues for C-DRX in XR

Document for:	Discussion and decision

# 1 Introduction

In previous RAN2 meetings, the following agreements were reached for the XR power savings [1][2]:

- Define DRX cycle based on rational numbers. Inform RAN1/4 about this and ask them to indicate if this causes issues in their specifications. - To address SFN wrap around, it is proposed to adopt option with a counter in DRX formula that increments at every SFN wrap around and an DRX reference SFN signalled by network. FFS if this is based on H-SFN, E-SFN or a generic counter. - Not use broadcast signalling for counter and reference SFN in XR.

In this contribution, we discuss the remaining issues for XR power savings.

# 2 Discussion

## 2.1 Rational number DRX cycles

In the previous RAN2 meeting, we agreed to introduce DRX cycles expressed as rational numbers. New rational number DRX cycles can be easily defined by introducing two new parameters (per DRX cycle) Dividend and Divisor in DRX-Config in the RRC specification TS 38.331. This will make it very easy to define new rational number DRX cycles with Dividend and Divisor parameters and has the added advantage of being future proof. The corresponding rational number DRX cycle can then be computed as:

drx-ShortCycle = drx-ShortCycleDividend / drx-ShortCycleDivisor drx-LongCycle = drx-LongCycleDividend / drx-LongCycleDivisor

Proposal 1: Implement rational number DRX cycles by introducing Dividend and Divisor parameters in DRX-Config in TS 38.331.

Modulo operation in DRX formulae is currently performed with integer base (integer DRX cycles). When non-integer base (rational DRX cycle) is used, the possibility of rounding errors has been raised as a concern in some contributions [3].

To overcome the rounding errors, the modulo operation can be replaced by the equivalent function as below:

A modulo B =A - B ×floor AB

There have also been suggestions that replacing the modulo function with the above equivalent may sometimes still result in rounding errors [3]. Such errors only occur if the equivalent modulo function is implemented incorrectly, for example if a fixed-point division operation is performed before the multiplication operation. It should be noted that the formula does not make any assumptions on fixed point division or on the order of the operation. However, if incorrect implementation is a concern, one option is to define the exact order of the implementation of the equivalent modulo operation by using parentheses. For example, the following formula can be included in the specifications:

A modulo BC =A - B × floor A ×CBC

Additionally, the result of the modulo function should be rounded to the nearest integer by using the floor function in the DRX formulas.

The resulting changes to the TS 38.321 specification can be illustrated as follows:

1> if the Short DRX cycle is used for a DRX group, and floor [[(SFN × 10) + subframe number] modulo (drx-ShortCycle)] = floor [(drx-StartOffset) modulo (drx-ShortCycle)]: 2> start drx-onDurationTimer for this DRX group after drx-SlotOffset from the beginning of the subframe. 1> if the Long DRX cycle is used for a DRX group, and floor [[(SFN × 10) + subframe number] modulo (drx-LongCycle)] = drx-StartOffset: <Legacy text omitted> If the drx-ShortCycle or the drx-LongCycle are configured as rational numbers, the modulo operation is defined as: A modulo BC =A - B × floor A ×CBC

Table 1 TP for supporting rational number DRX cycles in DRX formulas in TS 38.321

Proposal 2: Adopt the TP in Table 1 for the DRX formulas in TS 38.321 to support rational number DRX cycles, by using the floor operation and defining the alternative modulo operation as in Table 1 in DRX formulae.

## 2.2 SFN wrap-around

To resolve the SFN wrap around issue, a new counter (e.g., E-SFN) can be introduced in the C-DRX formulas as in the example TP for TS 38.321 below:

1> if the Short DRX cycle is used for a DRX group, and [(E-SFN × 10240) + (SFN × 10) + subframe number] modulo (drx-ShortCycle) = (drx-StartOffset) modulo (drx-ShortCycle): 2> start drx-onDurationTimer for this DRX group after drx-SlotOffset from the beginning of the subframe. 1> if the Long DRX cycle is used for a DRX group, and [(E-SFN × 10240) + (SFN × 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset: <Legacy text omitted>

Table 2 TP for introducing E-SFN counter in DRX formulas in TS 38.321

This method involves relatively small changes to the MAC spec and is well aligned with the existing DRX mechanism. We therefore propose to adopt the changes above for the DRX formulas.

Proposal 3: Adopt the TP in Table 2 for the DRX formulas in TS 38.321 to introduce the new E-SFN counter, by adding the additional factor with E-SFN in the DRX formulae.

The initial value of the E-SFN at a specific time (for a specific SFN) must be known by the UE and the network. To achieve this, a reference SFN (SFN_ref) could be indicated to the UE via dedicated RRC signalling (following a similar mechanism as in Rel-16 IIoT). When the UE processes the RRC message with the DRX configuration, it sets the initial value of the E-SFN based on SFN_ref and the SFN at the UE when the reference SFN is received (SFN_UE). This helps to avoid ambiguity in initializing the E-SFN value for situations where the RRC message is sent by the network just before the SFN wrap around and is received by the UE after the SFN wrap around because of HARQ/ARQ retransmissions.

For example, when the network sends the RRC message for DRX configuration at SFN between 512-1023, the network can set SFN_ref to 512. When the UE receives the RRC message successfully, it checks the current SFN (SFN_UE). If SFN_UE is between 512-1023 (512 ≤ SFN_UE < 1023), the UE initializes the E-SFN counter to 0 (there were no HARQ/ARQ retransmissions for the RRC message). Otherwise, the UE initializes the E-SFN to 1 (RRC message was received late by the UE, and the E-SFN has been incremented since).

When the network sends the RRC message for DRX configuration at SFN between 0-511, the network can set SFN_ref to 0 (or not present). When the UE receives the RRC message successfully, it initializes the E-SFN counter to 0 regardless of the SFN_UE (because retransmissions for the RRC message are not expected to cross SFN wrap around boundary).

For the reference SFN, the Rel-16 IIoT mechanism can be re-used: Network provides the reference SFN, SFN_ref (i.e., either 0 or 512) via dedicated RRC signalling, in the RRC configuration for DRX. Once initialised, E-SFN is incremented every time SFN wraps around as already agreed.

Proposal 4: To initialize the E-SFN value on the UE:

- Network indicates the reference SFN, SFN_ref (i.e., either 0 or 512), to the UE via dedicated RRC signalling within DRX configuration (e.g., in DRX-Config).

- When the UE successfully receives RRC message with DRX configuration at SFN=SFN_UE:

  - If SFN_ref=0 (or not present):

    - UE initializes the E-SFN counter to 0.

  - If SFN_ref=512:

    - If 0 ≤ SFN_UE < 512, UE initializes the E-SFN counter to 1

    - Otherwise, UE initializes the E-SFN counter to 0.

RAN2 should also discuss whether the E-SFN has a maximum value or not (i.e., counter runs to infinity). Selecting an appropriate maximum value for the E-SFN counter may ensure perfect alignment of DRX cycles at the E-SFN wrap around, as illustrated in Figure 1 below. The maximum value (M) for the new counter can be selected such that, at the wrap-around of the new counter, the DRX cycles will not overlap, i.e., [10240 × (M+1)] / DRX_Cycle yields an integer value. The maximum value could be decided by the network and indicated to the UE via RRC signalling, for example in the DRX configuration.

Figure 1 DRX cycles at E-SFN wrap around (for DRX cycle = 50/3 ms and maximum E-SFN value = 9)

The shared knowledge of the maximum value for E-SFN on the UE and the network can be beneficial, as there would be no ambiguity with respect to the storage size for the E-SFN counter (for example, whether a 16, 32, or 64-bit variable should be used). This would improve the robustness and inter-operability of different implementations of the specification. The maximum value can be selected by the network such that the DRX cycles corresponding to all known XR frame rates can be supported, for example M=999. However, if there is no concern about the ambiguity for the storage size of the counter and about any potential compatibility issues, there may not be a strong motivation to define a maximum value. In that case, an infinite counter can be used.

Proposal 5: RAN2 to discuss and select one of the following options:

- Option A: The maximum value of the E-SFN counter is decided by the network and indicated to the UE via RRC signalling (e.g., in DRX configuration)

- Option B: There is no maximum value specified for the E-SFN counter (the counter runs to infinity)

# 3 Conclusion

In this contribution, we discuss the remaining issues for XR power savings, and have the following proposals:

Proposal 1: Implement rational number DRX cycles by introducing Dividend and Divisor parameters in DRX-Config in TS 38.331.

Proposal 2: Adopt the TP in Table 1 for the DRX formulas in TS 38.321 to support rational number DRX cycles, by using the floor operation and defining the alternative modulo operation as in Table 1 in DRX formulae.

Proposal 3: Adopt the TP in Table 2 for the DRX formulas in TS 38.321 to introduce the new E-SFN counter, by adding the additional factor with E-SFN in the DRX formulae.

Proposal 4: To initialize the E-SFN value on the UE:

- Network indicates the reference SFN, SFN_ref (i.e., either 0 or 512), to the UE via dedicated RRC signalling within DRX configuration (e.g., in DRX-Config).

- When the UE successfully receives RRC message with DRX configuration at SFN=SFN_UE:

  - If SFN_ref=0 (or not present):

    - UE initializes the E-SFN counter to 0.

  - If SFN_ref=512:

    - If 0 ≤ SFN_UE < 512, UE initializes the E-SFN counter to 1

    - Otherwise, UE initializes the E-SFN counter to 0.

Proposal 5: RAN2 to discuss and select one of the following options:

- Option A: The maximum value of the E-SFN counter is decided by the network and indicated to the UE via RRC signalling (e.g., in DRX configuration)

- Option B: There is no maximum value specified for the E-SFN counter (the counter runs to infinity)

# 4 References

- R2-2306553 Report of 3GPP TSG RAN WG2 meeting #121bis-e

- Draft_R2_122_meeting_report_v2

- R2-2305830 Discussion on XR-specific power saving, Ericsson, RAN2#122,