3GPP TSG-RAN2 Meeting #123	R2-2308405

Toulouse, France, 21st - 25th August 2023

Agenda item:	7.18.2

Source:	Qualcomm Incorporated

Title:	Remining issues on control plane aspects of MT-SDT

WID/SID:	NR_MT_SDT-Core – Release 18

Document for:	Discussion and Decision

# Introduction

In the RAN2 #122 meeting, the following agreements were made for the control plane aspects of MT-SDT.

Agreements:

- Allow support of only MT-SDT in a cell.  A separate SIB configuration will be introduced.  FFS what is put in there.

- For paging indication signalling, a new list of paging records for MT-SDT indication is optionally included in paging message using non critical extension. Each record in this list optionally includes 1 bit MT-SDT indication. UE identity and access type are not included in paging record of this list.

- gNB may include MT-SDT indication in paging message only if UE’s I-RNTI is included in the paging message (i.e. MT-SDT is only used by RAN initiated paging).

- UE selects '0' as the Access Category when the resumption of the RRC connection is triggered by response to the MT-SDT triggering in a PAGING message

- MT-SDT is only applicable to the legacy MT-Access use case (i.e. it is not applicable to access identities 1, 2 and 11-15).

- SRB2 can be used for MT-SDT (i.e. similar to MO-SDT)

- No additional enhancement is needed specifically for RedCap UE to monitor paging for MT-SDT

- When RRC resume is triggered due to MT-SDT and in the case the condition for paging triggered MT-SDT is not fulfilled, the UE initiates RRC Resume procedure with Resume cause “mt-Access”.

- A separate sdt-RSRP threshold for MT-SDT can be configured, at least in the case where MO-SDT is not configured in the cell.

In this paper, we would like to discuss the remaining issues of control plane aspects of MT-SDT.

# Discussion

RAN2 has agreed that UE is allowed to initiate either MO-SDT based resume or non-SDT normal resume using separate procedure before initiation RRCResumeRequest for MT-SDT, and RAN2 agreed to specify a RRC procedure for RRC resume for MT-SDT initiation without checking the UL data availability.

Based on RAN2 agreements, UE can be allowed to select either MT-SDT or MO-SDT to handle the subsequent DL/UL data. If UE determines to respond the paging message with MT-SDT indication, (up to UE check availability of UL data), UE selects to perform MT-SDT and set the MT-SDT resume cause in the CCCH message. If UL data arrives at UE and meanwhile UE receives the paging message with MT-SDT indication (though this is a rare case), UE can either trigger a MT-SDT procedure with MT-SDT resume cause or select Rel-17 MO-SDT procedure without mixing with the MT-SDT. Network can send the DL data at the subsequent phase of MO-SDT.

Given the above understanding, from RRC layer perspective, we think almost all Rel-17 SDT trigger conditions can be reused with one exception, i.e., up to UE to check the pending UL data mapped to the radio bearers configured for SDT. In our view, checking UL data and data volume is not the must for triggering MT-SDT. It could be up to UE to check the UL data volume. If UE determines to do so and the data volume is below the threshold, UE may trigger Rel-17 MO-SDT. Otherwise, UE may select MT-SDT w/o checking UL data. From MAC player perspective, the MO-SDT conditions for RA and CG resource can be mostly reused for MT-SDT. Thus, we have following proposals.

Proposal 1: The Rel-17 SDT conditions can be reused for MT-SDT conditions except checking the pending UL data whether is mapped to the radio bearers for SDT.

Proposal 2: It is up to UE to check the UL data volume when UE receives the paging message with MT-SDT indication.

It has been agreed that the paging message is enhanced to carry the MT-SDT indication so that UE can be informed that DL data is arrived at network. It is up to network configuration on what resources for UE to perform SDT. Since network has less information on the UE radio conditions, which resources the UE chooses for MT-SDT could be up to UE evaluation. Considering the consistent UE behavior on MO- and MT-SDT, it is preferable to allow UE to select either RA or CG resource (if configured) for MT-SDT. Thus, we think which resource (RA or CG) is selected for MT-SDT is not indicated in paging message.

Proposal 3: Which resource (RA or CG) is selected for MT-SDT is not indicated in paging message.

Regarding the RA or CG resource, as long as they are configured to the UE, similar to Rel-17, UE is allowed to use either RA or CG resource to perform MT-SDT. The rule of selection between RA and CG resource could be the same as Rel-17 SDT.

Proposal 4: Upon initiation of MT-SDT procedure, the rule of selection between RA and CG resource could be the same as Rel-17 SDT.

In Rel-17 SDT, separate RACH resource can be configured for SDT users. The MO-SDT UE is allowed to use the separate RACH resource to perform SDT and the separate RO or separate preamble RACH resource serves as the implicit SDT indication to network so that network can assign the appropriate resource for UL/DL data transmission.

For the RACH based MT-SDT, since the MT-SDT is triggered by paging message, there are benefits that network directly indicates the dedicated resource for the target UE. The similar design principle can be observed from CG-SDT that the dedicated CG resource is configured via the RRC release message.

Using dedicated resource for RACH based MT-SDT not only reduces the latency of UE performing RRC resume but also improves the reliability of the resume procedure. The dedicated resource improves the success possibility of the procedure. It is also good for UE power saving. Therefore, we have following proposal.

Proposal 5: A dedicated preamble for MT-SDT can be carried in the paging message for the target UE for RACH based MT-SDT.

In Rel-17 MO-SDT, if gNB receives DL non-SDT data or signaling, the last serving gNB (in case of no anchor relocation) will terminate the ongoing MO-SDT procedure and moves UE to RRC_INACTIVE by sending the RRCRelease message.

For an ongoing MT-SDT data transfer, if DL non-SDT data is arrived, how to handle such scenarios needs to be studied. If the UE full context is already transferred to the receiving gNB, when any DL non-SDT data arrives at receiving gNB, it can simply send RRCResume message and transition UE into RRC_CONNECTED so that UE can receive the DL non-SDT data in RRC_CONNECTED state.

If only partial UE context was transferred to the receiving gNB, when any DL non-SDT data arrives at the last serving gNB, the last serving gNB has to send the RRCRelease message to keep UE in the RRC_INACTIVE because of no late UE context transfer procedure defined so far due to the limited RAN2/RAN3 time. However, even the last serving gNB sends the RRCRelease message and UE is kept in the RRC_INACTIVE state, the last serving gNB still need to send additional RAN paging message to inform UE DL data and trigger UE to perform another RRC resume procedure to RRC_CONNECTED state. It may cause additional signaling overhead and unnecessary delay. One possible solution is that when the last serving gNB sends the RRCRelease with suspendConfig message, an additional indication is carried in the RRCRelease message to indicate the DL non-SDT data arrival so that UE can be triggered to perform RRC resume immediately w/o additional singling. The latency of DL non-SDT data reception is also reduced.

Proposal 6: If DL non-SDT data is arrived during an ongoing MT-SDT procedure, the RRCRelease message can be sent to terminate the MT-SDT and is enhanced to carry an indication to inform /trigger UE to initiate RRC resume immediately after MT-SDT ends.

A new UE optional capability is introduced to indicate whether UE supports Rel-18 MT-SDT. Similar to Rel-17 MO-SDT, the new UE optional capability could be defined separately for DRB and SRB2.

Since MT-SDT procedure can be initiated w/o using RA-SDT resource, and the MT-SDT and MO-SDT are separate two SDT procedure. Thus, the MT-SDT capability should be independent of Rel-17 MO-SDT capability. Also considering the different usage of MO and MT-SDT, it is not necessary to require a Rel-18 MT-SDT capable UE to mandatory support UL data initiated MO-SDT procedure.

Proposal 7: A new optional UE capability is introduced to indicate supporting Rel-18 MT-SDT. The new UE capability is defined separately for DRB and SRB2.

Proposal 8: The Rel-18 MT-SDT capability has no dependency on the Rel-17 MO-SDT capabilities.

# Conclusion

We have the following observations, and we’d recommend RAN2 to discuss and adopt the following proposals:

Proposal 1: The Rel-17 SDT conditions can be reused for MT-SDT conditions except checking the pending UL data whether is mapped to the radio bearers for SDT.

Proposal 2: It is up to UE to check the UL data volume when UE receives the paging message with MT-SDT indication.

Proposal 3: Which resource (RA or CG) is selected for MT-SDT is not indicated in paging message.

Proposal 4: Upon initiation of MT-SDT procedure, the rule of selection between RA and CG resource could be the same as Rel-17 SDT.

Proposal 5: A dedicated preamble for MT-SDT can be carried in the paging message for the target UE for RACH based MT-SDT.

Proposal 6: If DL non-SDT data is arrived during an ongoing MT-SDT procedure, the RRCRelease message can be sent to terminate the MT-SDT and is enhanced to carry an indication to inform /trigger UE to initiate RRC resume immediately after MT-SDT ends.

Proposal 7: A new optional UE capability is introduced to indicate supporting Rel-18 MT-SDT. The new UE capability is defined separately for DRB and SRB2.

Proposal 8: The Rel-18 MT-SDT capability has no dependency on the Rel-17 MO-SDT capabilities.

# References

# Appendix