3GPP RAN WG2 Meeting #123 	R2-2308412

# Toulouse, France, August 21-25, 2023

Agenda Item:	7.5.4.1

Source:	InterDigital Inc.

Title:	Buffer status and remaining time reporting for XR

Document for:	Discussion

# 1. Introduction

The issue of BSR for XR was discussed in RAN2-122 and the following was agreed [1]:

Support one static BSR table with 8 bits BS field for Rel-18 XR (for all cases).

We do not support additional piecewise linear BSR table in Rel-18. Can consider piecewise linearity when discussing how the BSR table values are defined.

Delay information reporting was also discussed, and the following was agreed:

1: UE calculates the remaining time based on the PDCP discard timer value. FFS if UE reports one or multiple values. FFS how this is modelled in PDCP specification. FFS which UEs support this.

When/if UE reports remaining time, the reference time for the remaining time is determined from the point of the first transmission of the information. FFS if intra-UE prioritization can impact this.

In this contribution we discuss further aspects of BSR and delay information reporting for XR.

# Discussion

## BSR formats

With the need to enhance BS tables comes the need to introduce enhancements to BSR format to align with the BS table enhancements.

Since it has been agreed to introduce a new XR specific BSR level (and maybe more will be introduced later), there is a need for the UE to indicate in the BSR which BSR table the buffer level indices are referring to.

Proposal 1:	Enhance the BSR format to include the BSR table index.

Legacy short BSR only consists of 8 bits, with 3 bits used to identify the LCG with buffered data, leaving the UE with only 5 bits to indicate the amount of data in its buffer. Since it is already agreed to introduce a new XR specific BSR table with 8 bits BS field, it is desirable to enable the UE to indicate more than 32 indices that is currently allowed with 5 bits. Usage of long BSR while the UE may only have data in one LCG may lead to unnecessary overhead. As such, enhancements to the short BSR format are needed to align with a more granular XR specific BSR table.

Proposal 2:	Enhance the short BSR format to align with enhancements to BS tables (i.e., with 8 bits BS field).

## Remaining time information

RAN2 has agreed that the remaining delay information is calculated based on the PDCP discard timer value (which is assumed to be correlated with the PSDB value). For the brevity of this document, we will use the term PDU discard timer even though, strictly speaking, a discard timer is for a PDCP SDU.

There are several aspects that need clarification. First, since the PDU set contains multiple PDUs, each with its own discard timer, it is not clear what is meant by “based on the PDCP discard timer value”. There are two cases to consider:

- Case 1: all packets of the PDU set arrive at the PDCP at the same time (i.e., the discard timer for all started at the same time)

- Case 2: packets of the PDU set arrive at the PDCP at different times (i.e., the discard timer for the different PDUs may start at different times)

In case 1, all the discard timers will have the same remaining time for all the PDUs of the PDU set (except for the case of the ones that are already acknowledged by the receiver, where the discard timer will be stopped/released). In this case, the remaining time for the PDU set can be assumed to be the remaining discard time of any of the PDUs that are not acknowledged yet.

In case 2, the discard timer values for the different PDUs of the PDU set will have different remaining time based on when they were started. Also, it is not clear if in this case all the PDUs of the PDU set will be assigned the same discard timer value (i.e., will the first PDU of the PDU set and the last PDU of the PDU set that may arrive at PDCP considerably later than the first PDU both have the same discard timer value?)

Observation 1:	If the PDUs of the PDU set arrive at the transmit PDCP at different times, the remaining time for the different PDUs of the PDU set may be different, even if they were all assigned the same discard timer value.

That is, at any given time, there could be a range of remaining time values, corresponding to the different PDUs of the PDU set that are still waiting acknowledgement from the receiver.

Another aspect is whether the remaining time determination/reporting is made at PDCP level or MAC level. Part of the RAN2 agreement seems to imply that this is carried out by PDCP (“FFS how this is modelled in PDCP specifications”), while another part seems to imply MAC level operation (‘the reference time for the remaining time is determined from the point of the first transmission of the information”).

Observation 2:	From the agreement in RAN2-122, it is not clear whether the remaining time determination/reporting is done at the PDCP or MAC level.

Since PDU set level information may not be readily available at the MAC level, it makes more sense to do the remaining time determination at the PDCP level and provide it to the MAC (e.g., like the way PDCP delivers data volume information to the MAC for BSR).

Observation 3:	Discard timer value for a PDU and the remaining time left before it expires are not readily available at the MAC level.

Observation 4:	PDCP already informs MAC about buffered data level (data volume calculation).

Thus, we propose:

Proposal 3:	The determination of the remaining time for a PDU set is done at the PDCP level.

Proposal 4:	The remaining time is calculated as the remaining time of the discard timer corresponding to the first PDU of the PDU set.

Proposal 5:	PDCP informs MAC about the remaining time for the PDU set (e.g., like PDCP data volume).

Regarding on how the remaining time for a PDU set is to be reported, a new MAC CE can be introduced, or the remaining time information can be included in the BSR. Our understanding is that that the latter is a more reasonable approach because the network will have the remaining time information together with the corresponding BS at that time, and thus will be able to perform the optimal scheduling decision among UEs (i.e., considering the size of the data, the priority of the data, and the required urgency for sending the data, which is the remaining time).

Proposal 6:	The remaining time information is included in the BSR.

Another aspect to consider is whether the BSR reporting per LCG as in legacy is sufficient for this purpose. There could be several LCIDs within an LCG, and each transporting XR traffic. At a given time, the remaining time corresponding to each LCID may be different, and thus several remaining time values may need to be included for one LCG. On the other hand, if LCID level granularity is to be supported/introduced, then only one remaining time value is required for the LCID and the corresponding BS value also explicitly corresponds to that LCID only.

Proposal 7:	RAN2 to choose among the following three options for remaining time reporting within a BSR:

a) the LCG granularity is kept as in legacy BSR, and only one remaining time value is included per LCG. FFS on how the remaining time value is chosen among the different remaining times corresponding to each LCID (e.g., the minimum, maximum, average, etc.,)

b) the LCG granularity is kept as in legacy BSR, but multiple remaining time values, corresponding to each LCID of the LCG, can be included

c) LCID level granularity for BSR is reported, where the BS and remaining time corresponding to each LCID can be explicitly included.

## BSR triggering

In the legacy approach, regular BSR is triggered in two instances: firstly, when new data arrives in any of the LCHs associated with an LCG provided that all other LCHs are empty, and secondly when new data arrives in an LCH of higher priority than any LCH that might still have data buffered. In these two instances, the UE is configured to trigger a BSR to indicate the data payload size closest to a value in a configured BSR table. As such, the one instance where the existing BSR triggering mechanism is not sufficient is when a PDU set with lower priority arrives at an LCH buffer right after a PDU set of higher priority such that the higher priority PDU set has not yet cleared its LCH buffer (e.g., some PDUs might have remained in the higher priority LCH buffer). In such a case, the lower priority PDU set runs the risk of remaining in the LCH buffer until the expiry of the PSDB. One way to address this issue would be the addition of another condition for triggering BSR when a PDU set arrives in an LCH buffer, irrespective of the priority of the LCH. This, however, may result in high signalling overhead due to frequent triggering of BSR.

For high priority PDU sets, the BSR can be triggered immediately after the arrival of the first PDU of the PDU set, as per legacy triggering conditions. In the case of lower priority PDU sets, the UE can keep track of the remaining time with respect to the deadline and not send any explicit indication to the gNB unless the deadline is approaching. This approach involves less signalling overhead while allowing the scheduler to allocate resources on a timely basis for meeting the PSDB. Also, in addition to the remaining time, the amount of remaining data to be transmitted for the PDU set can be considered. For example, it may not be desirable to trigger the BSR for a PDU set even if the remaining time is small, if only a handful of PDUs of the PDU set remain to be transmitted, while triggering the BSR even when the remaining time is large may be desirable if the amount of data yet to be sent for the PDU set is large.

When the remaining time report is triggered, the UE can generate a BSR that contains both the BS and the remaining time. It can be discussed if the UE includes the remaining time information whenever a BSR is triggered due to legacy BSR triggering conditions.

Considering the above, we propose:

Proposal 8:	A BSR containing the remaining time information for a PDU set is triggered when the remaining time for the PDU set falls below a certain configured threshold.

Proposal 9:	RAN2 to discuss whether a BSR triggered due to legacy reasons (i.e., regular BSR triggering conditions, periodicity of periodic BSR, or UL resource available for padding BSR) can also contain the remaining time information.

Proposal 10:	RAN2 to discuss if other BSR triggering conditions are required other than the remaining time information (e.g., that consider the PDU set size, the amount of PDU set data yet to be transmitted, etc.,).

# Conclusion

In this contribution, buffer status and remaining time reporting for XR traffic are discussed and the following observations and proposals are made:

Observation 1:	If the PDUs of the PDU set arrive at the transmit PDCP at different times, the remaining time for the different PDUs of the PDU set may be different, even if they were all assigned the same discard timer value.

Observation 2:	From the agreement in RAN2-122, it is not clear whether the remaining time determination/reporting is done at the PDCP or MAC level.

Observation 3:	Discard timer value for a PDU and the remaining time left before it expires are not readily available at the MAC level.

Observation 4:	PDCP already informs MAC about buffered data level (data volume calculation).

Proposal 1:	Enhance the BSR format to include the BSR table index.

Proposal 2:	Enhance the short BSR format to align with enhancements to BS tables (i.e., with 8 bits BS field).

Proposal 3:	The determination of the remaining time for a PDU set is done at the PDCP level.

Proposal 4:	The remaining time is calculated as the remaining time of the discard timer corresponding to the first PDU of the PDU set.

Proposal 5:	PDCP informs MAC about the remaining time for the PDU set (e.g., like PDCP data volume).

Proposal 6:	The remaining time information is included in the BSR.

Proposal 7:	RAN2 to choose among the following three options for remaining time reporting within a BSR:

a) the LCG granularity is kept as in legacy BSR, and only one remaining time value is included per LCG. FFS on how the remaining time value is chosen among the different remaining times corresponding to each LCID (e.g., the minimum, maximum, average, etc.,)

b) the LCG granularity is kept as in legacy BSR, but multiple remaining time values, corresponding to each LCID of the LCG, can be included

c) LCID level granularity for BSR is reported, where the BS and remaining time corresponding to each LCID can be explicitly included.

Proposal 8:	A BSR containing the remaining time information for a PDU set is triggered when the remaining time for the PDU set falls below a certain configured threshold.

Proposal 9:	RAN2 to discuss whether a BSR triggered due to legacy reasons (i.e., regular BSR triggering conditions, periodicity of periodic BSR, or UL resource available for padding BSR) can also contain the remaining time information.

Proposal 10:	RAN2 to discuss if other BSR triggering conditions are required other than the remaining time information (e.g., that consider the PDU set size, the amount of PDU set data yet to be transmitted, etc.,).

# 4. References

[1] 	RAN2 chairman notes from RAN2#122, May 2023