3GPP TSG-RAN2 Meeting #123	R2-2308414

Toulouse, France, 21st - 25th August 2023

Agenda item:	7.20.2

Source:	Qualcomm Incorporated

Title:	Discussion on multi-DCI multi-TRP with two TAs

WID/SID:	NR_MIMO_evo_DL_UL-Core – Release 18

Document for:	Discussion and Decision

# Introduction

In RAN2 #122 meeting, the multi-DCI multi-TRP with two TAs was discussed and the following agreement was made.

Configure one TAT per TAG to support two TAs for a serving cell, i.e., in this case 2 TAGs are configured for the serving cell.

There are several remaining issues to be further discussed. In this paper, we would like to further discuss relevant topics about two TAs in multi-DCI multi-TRP.

# Discussion

In Rel-18 MIMO evo study, RAN1 has agreed to support the enhancement on two TAs for both intra-cell and inter-cell UL multi-DCI multi-TRP operation. There are two TAs for different TRPs in one serving cell. RAN1 has further agreed that multi-DCI multi-TRP operation with two TAs scenario supports to configure two TAGs belonging to a serving cell. However, in current MAC spec, one serving cell should belong to one TAGs. Thus, the concept of TAGs especially of the PTAG and STAG should be updated and enhanced. From RAN2 perspective, if SpCell is configured with two TAGs, how to define PTAG and STAG in one serving cell should be studied.

In addition, the timeAlignmentTimer is configured to associate with TAG to control if the serving cells belongs to the associated TAG to be uplink aligned or not. RAN2 #122 meeting has agreed that one timeAlignmentTimer is configured per TAG in case that two TAGs are configured for the serving cell. In that sense, the similar UE behavior when timeAlignmentTimer expiry for each TAG could be mostly applied (or with small changes) for the two TAs case. Since UE behavior is defined for PTAG and STAG separately when timeAlignmentTimer expiry in current MAC spec, we think only one PTAG to be defined for multi-DCI multi-TRP with two TAs has least specification changes and it could be the simplest solution for two TAs case. Thus, we have following proposal.

Proposal 1: Only one PTAG should be defined for multi-DCI multi-TRP with two TAs case.

In current MAC spec, PTAG, i.e., Primary Timing Advance Group is defined as a timing advance group containing the SpCell of a MAC entity, while STAG, i.e., Secondary Timing Advance Group refers to other TAGs. For multi-DCI multi-TRP in SpCell of a MAC entity, there could be two TAGs belonging to a serving cell. The concept of PTAG has to be revised to refer to a TAG associating with at least one TRP in SpCell instead of TAG containing the SpCell. For the other TAGs, i.e., other than the PTAG, the STAG could be still referred to. Besides the PTAG and STAG, it is not desirable to have other new terms for TAG in the case of multi-DCI multi-TRP with two TAs scenarios.

Proposal 2: Except the concept of PTAG and STAG, RAN2 will not introduce additional new concept of TAG in the case of multi-DCI multi-TRP with two TAs.

In RAN1 #112 meeting, the following agreement was made [1].

Agreement For associating TAGs to target UL channels/signals for multi-DCI based multi-TRP operation, support the following: Associate TAG to TCI-state - Associate TAG ID with UL/joint TCI state for FR1/FR2 and Rel-15/16 spatial relation in FR2 - For UL transmission, the TAG ID associated with the UL/joint TCI state is utilized - A baseline is UE expects that the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESET Pool Index correspond to one TAG - Working Assumption: A UE may report that it supports that the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESETPoolIndex correspond to both TAGs FFS: on how to handle association when Rel-15/16 spatial relation framework is used for FR1 - PUCCH - DG/CG Type 1/Type 2 PUSCH - AP/SP/P SRS

Further, RAN1 #113 meeting made the following agreement.

Agreement For associating TAGs to target UL channels/signals for multi-DCI based multi-TRP operation, the baseline feature is revised as follows: - UE expects that the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESET Pool Index correspond to one TAG - Association of TAG ID with UL/joint TCI state is via RRC configuration - Above does not impact the association of the indicated TCI states and coresetPoolIndex values as agreed in previous meetings in 9.1.1.1.

According to the RAN1 agreements, it can be observed that the UL/joint TCI states associated to one CORESETPoolIndex correspond to one TAG, and TAG ID is associated with UL/joint TCI state. Thus, if only PTAG is defined as legacy for the multi-DCI multi-TRP with two TAs case, the concept of PTAG can be extended to refer to a TAG containing the UL signals/channels associated with the CORESETPoolIndex #0 in the SpCell of a MAC entity, whereas the other TAGs are referred to the STAG.

Proposal 3: In the multi-DCI multi-TRP with two TAs scenarios, a TAG containing the UL signals/channels associated with the CORESETPoolIndex #0 in the SpCell of a MAC entity is referred to as PTAG, whereas the STAG refers to other TAGs.

In legacy, once the timeAlignmentTimer expires, the serving cells belonging to the associated TAG are un-synchronized. The UE behavior is specified to handle the resource release/clear on the serving cell level when the timeAlignmentTimer is expired. For two TA cases, RAN1 response in the LS [2] is copied below.

Q2: When the time-alignment timer associated with one of the TRPs of a serving cell expires, are certain UL or DL operation only impacted towards that TRP while they are not impacted towards the another TRP? If so, which UL or DL operation? Answer: RAN1 confirms that when the TA timer associated to one TRP expires for a TAG associated with a TCI state, UL or DL operation associated to the another TRP is not impacted. This further depends on PTAG/STAG definition, which is up to RAN2 to decide. Which UL or DL operation is impacted have not been discussed in RAN1.

In our understanding, if the timeAlignmentTimer associated with PTAG expires, it implies the UL signals /channels associated with the CORESETPoolIndex #0 in the SpCell is out of sync. In this case, UE behavior can follow the legacy when timeAlignmentTimer is associated with PTAG, i.e., flush all HARQ buffers, release or clear UL/DL resources or grants, maintain NTA value and consider all running timeAlignmentTimer expired.

Proposal 4: In the multi-DCI multi-TRP with two TAs scenarios, when timeAlignmentTimer associated with PTAG expires, UE should follow the legacy behavior of timeAlignmentTimer expired associated to PTAG.

For the timeAlignmentTimer associated with STAG expires, most of legacy behavior of timeAlignmentTimer associated to STAG can be applied except the HARQ buffer flush. Since there are two TRP configured in one serving cell, if one timeAlignmentTimer associated to one TRP of one serving cell is expired, it is possible that the timeAlignmentTimer associated to another TRP is still workable. Then HARQ retransmission could be performed in some cases according to the UE capability. For example, UE may indicate an ‘incapability’ that UE does not support ryoetransmission from the other TRP. Nevertheless, it is not required to flush HARQ buffer like legacy if the timeAlignmentTimer associated with STAG expires.

Proposal 5: In the multi-DCI multi-TRP with two TAs scenarios, HARQ buffers are not flushed if the corresponding timeAlignmentTimer associated with STAG expires.

How to acquire the TA for the second TAG of one serving cell should be studied. In current NR, TA of STAG is updated by the PDCCH ordered CFRA for the SCell in CA case, and network will require UE to initiate RACH by PDCCH order for TA acquisition of the target SCell.

For multi-DCI multi-TRP with two TAs, the second TAG update could be similar to the method of TA update for the SCell in CA case. For the RRC connected UE, if second TRP of one serving cell is configured by network, network schedules the UE by PDCCH order to trigger CFRA and UE acquires the TA for the synchronization with the second TRP.

Proposal 6: The initial TA update for the second TAG is initiated by network using PDCCH order triggered CFRA procedure.

For inter-cell multi-DCI multi-TRP, RAN1 has some studies and has made conclusions.

Agreement For intercell multi-DCI based Multi-TRP operation with two TA enhancement, support indication of which PRACH configuration to be used in the RACH procedure in the PDCCH order. - FFS: Whether additionalPCI or a generic identifier is indicated in PDCCH order - FFS: The detail of the indication in PDCCH order in terms of whether to support PRACH triggered for inactive additionalPCI.

From RAN1 agreements, it can be observed that additional RACH configuration shall be configured for the cell with additional PCI and PDCCH order is enhanced to support indication of which PRACH resource for the inter-cell case. Thus, it is clear for UE to select which RACH resource to perform RACH and update TA for which TAG.

Observation 1: For inter-cell multi-DCI multi-TRP, UE selects the RACH resources and updates TA for corresponding TAG based on whether the RACH configuration is associated with the additional PCI.

For intra-cell multi-DCI multi-TRP in SpCell, if UE is configured with two TAGs and maintains two TAs, the legacy UE-initiated CBRA can be triggered due to certain events, such as when DL or UL data arrival and meanwhile the UL is un-sync, or BFR triggering and so on. In the legacy UE-initiated CBRA, UE evaluates SSB and selects RACH occasions associated with the qualified SSB. Since SSB is not configured to associate with the two TRPs for intra-cell case, UE does not know which TRP the RACH towards. For Msg2/MSGB reception, network should indicate the TA to be applied to which TAG for intra-cell case. Therefore, for intra-cell case, RAN2 should study the enhancement on the TA update to be applied for which TAG.

Proposal 7: RAN2 to study the enhancement on the TA update to be applied for which TAG for intra-cell multi-DCI multi-TRP with two TA case.

In RAN1 #112bis-e meeting, the following conclusion was made.

Conclusion For multi-DCI based Multi-TRP operation with two TA enhancement, how to indicate the TAG ID via absolute TA command MAC CE is left up to RAN2: - One of two TAG IDs configured in the SpCell can be indicated

It should be noted that the absolute TAC MAC CE is not supported in PDCCH order triggered RACH in current spec. So, the intention of this RAN1 agreement should be for the UE-initiated RACH. When UE initiates 2-step RACH, network should indicate the TAG ID via absolute TA command MAC CE to inform UE the TA update in MAC CE applied for which TAG.

From RAN2 perspective, it simply reuses one reserve bit in absolute TA command MAC CE indicating which TAG the absolute TA command is applied.

Observation 2: RAN1 has agreed that for two TA for multi-DCI multi-TRP, how to indicate the TAG ID via absolute TA command MAC CE is left up to RAN2.

Proposal 8: One reserve bit in absolute TA command MAC CE is used to indicate which TAG the absolute TA command is applied.

For 4-step RACH, RAN1 #113 meeting made the following agreement.

Agreement For intra-cell multi-DCI based Multi-TRP operation with two TA enhancement, down-select one of the following alternatives: - Alt 1: indicate TAG ID as part of TA command in RAR - Alt 3: divide SSBs into two groups, one for each TRP. If a SSB associated to a RACH procedure belongs to the nth group (n=1,2), then the TA obtained via the RACH procedure corresponds to the nth TRP.

Based on RAN1 agreement, two alternatives for RAN1 are for down select, i.e., one is to indicate TAG ID as part of TA command in RAR and another is to enable SSB partitioning to associate with RACH resource.

From RAN2 perspective, the simplest solution is to have unified solution for both absolute TAC MAC CE and RAR, i.e., Alt 1 in RAN1, TAG ID indication in the TA command in RAR. Since there could be up to two TAGs configured in one serving cell, the reserve bit in the first bit of RAR could be reused to indicate which TAG the TA command in RAR is applied. The solution could be applied to intra-cell multi-DCI multi-TRP with two TA cases.

Proposal 9: For the intra-cell multi-DCI multi-TRP with two TAs, the reserve bit in RAR is enhanced to indicate which TAG (the first TAG or the second TAG) the TA in RAR is applied.

# Conclusion

We have the following observations, and we’d recommend RAN2 to discuss and adopt the following proposals:

Observation 1: For inter-cell multi-DCI multi-TRP, UE selects the RACH resources and updates TA for corresponding TAG based on whether the RACH configuration is associated with the additional PCI.

Observation 2: RAN1 has agreed that for two TA for multi-DCI multi-TRP, how to indicate the TAG ID via absolute TA command MAC CE is left up to RAN2.

Proposal 1: Only one PTAG should be defined for multi-DCI multi-TRP with two TAs case.

Proposal 2: Except the concept of PTAG and STAG, RAN2 will not introduce additional new concept of TAG in the case of multi-DCI multi-TRP with two TAs.

Proposal 3: In the multi-DCI multi-TRP with two TAs scenarios, a TAG containing the UL signals/channels associated with the CORESETPoolIndex #0 in the SpCell of a MAC entity is referred to as PTAG, whereas the STAG refers to other TAGs.

Proposal 4: In the multi-DCI multi-TRP with two TAs scenarios, when timeAlignmentTimer associated with PTAG expires, UE should follow the legacy behavior of timeAlignmentTimer expired associated to PTAG.

Proposal 5: In the multi-DCI multi-TRP with two TAs scenarios, HARQ buffers are not flushed if the corresponding timeAlignmentTimer associated with STAG expires.

Proposal 6: The initial TA update for the second TAG is initiated by network using PDCCH order triggered CFRA procedure.

Proposal 7: RAN2 to study the enhancement on the TA update to be applied for which TAG for intra-cell multi-DCI multi-TRP with two TA case.

Proposal 8: One reserve bit in absolute TA command MAC CE is used to indicate which TAG the absolute TA command is applied.

Proposal 9: For the intra-cell multi-DCI multi-TRP with two TAs, the reserve bit in RAR is enhanced to indicate which TAG (the first TAG or the second TAG) the TA in RAR is applied.

# References

- R2-2302455, LS to RAN2/4 on Agreements for Rel-18 MIMO, RAN1.

- R2-2307018, Reply on LS 2TA for multi-DCI multi-TRP, Ericsson.

# Annex