3GPP TSG-RAN2 Meeting #123	R2-2308415

Toulouse, France, 21st - 25th August 2023

Agenda item:	7.20.3

Source:	Qualcomm Incorporated

Title:	Discussion on unified TCI framework extension for mTRP operation

WID/SID:	NR_MIMO_evo_DL_UL-Core – Release 18

Document for:	Discussion and Decision

# Introduction

RAN2 #122 meeting confirmed the working assumption made in the RAN2 #121bis-e meeting as an agreement regarding the unified TCI framework extension for multi-DCI multi-TRP operation.

RAN2 confirm the following working assumption as an agreement: - Revise the legacy unified TCI state activation/deactivation MAC CE by adding a “CORESET Pool ID” field to support mDCI based mTRP operation.

RAN2 #122 meeting also discussed to enable the unified TCI framework for single DCI multi-TRP operation, and the following agreement were made.

For sDCI based mTRP operation using unified TCI state framework, introduce the new MAC CE, with the following high level design principles: - If the signaling type of the unified TCI state configuration is configured by RRC (i.e. either joint DL/UL TCI state or separate DL/UL TCI state), it applies to both TRP (i.e., as configured by RRC for both TRPs). - The following information can be indicated by the MAC CE (for joint DL/UL TCI mode): - if the unified TCI state is for one of the TRPs (i.e., 1st or 2nd) or for both TRPs, - if the indicated TCI codepoint consists of one TCI state, whether the indicated TCI state(s) is for the first or second TRP(s) FFS for the separate DL/UL TCI mode.

In this paper, we would like to further discuss the unified TCI framework for single DCI multi-TRP.

# Discussion

The function of the new MAC CE is to update the joint or separate DL/UL TCI state for one or multiple TRPs. That means, for each TCI codepoint in the TCI field in a DCI format, the MAC CE should be able to indicate the TCI state updated for whether one or both TRPs.

Furthermore, RAN1 has agreed that the MAC CE is able to update a full-set or any sub-set of first/second joint/DL/UL TCI states for each TCI codepoint in the TCI field. It requires the new field can indicate not only the TCI states updated for whether one or both TRPs but also indicate the TCI state is updated for which TRP (the first or the second).

Proposal 1: In the new MAC CE, a field should be able to indicate that the TCI state is updated for the first TRP or the second TRP or both TRPs for each TCI codepoint in the TCI field in a DCI format.

Regarding the unified TCI state framework, the type of unified TCI state is indicated by RRC in the serving cell. We believe such RRC signaling configuration could be reused in the context of unified TCI update MAC CE. It implies that for the unified TCI states update in the new MAC CE, the type of unified TCI state is configured by RRC as legacy, and no additional field is needed about the type of unified TCI state in MAC CE.

Proposal 2: For the TCI states updated in the new MAC CE, its type (joint DL/UL TCI state or separate DL/UL TCI state) is configured by RRC. The Rel-17 RRC signaling can be reused.

If the separate DL/UL TCI state type is configured in the serving cell, the new MAC CE should be able to further indicate the TCI state is updated for DL or UL of each TRP for each TCI codepoint in the TCI field in a DCI format.

Proposal 3: If the separate DL/UL TCI state type is configured in the serving cell, the new MAC CE should be able to indicate either the DL or UL for each TRP that the TCI state is updated for each TCI codepoint in the TCI field in a DCI format.

The RAN1 #112bis-e meeting has made agreement regarding the unified TCI framework extension for single-DCI multi-TRP.

Agreement On unified TCI framework extension for S-DCI based MTRP operation, support the followings: - For a serving cell configured with joint DL/UL TCI mode, a full-set or any sub-set of {first joint TCI state, second joint TCI state} can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 by TCI state activation command (MAC-CE) - For a serving cell configured with separate DL/UL TCI mode, a full-set or any sub-set of {first DL TCI state, first UL TCI state, second DL TCI state, second UL TCI state} can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 by TCI state activation command (MAC-CE) - TCI state activation command (MAC-CE) should indicate that each joint/DL/UL TCI state mapped to a TCI codepoint is the first or second joint/DL/UL TCI state (detail on how to indicate above is up to RAN2 design) - The first/second indicated joint/DL/UL TCI state(s) is updated according to the corresponding first/second joint/DL/UL TCI state(s) mapped to the TCI codepoint received by the UE - If the UE receives a TCI codepoint mapped with a sub-set of {first joint TCI state, second joint TCI state} or {first DL TCI state, first UL TCI state, second DL TCI state, second UL TCI state}, the UE shall update the first/second indicated joint/DL/UL TCI state(s) according to the first/second joint/DL/UL TCI state(s) in the subset and keep other indicated first/second joint/DL/UL TCI state(s) that is not updated by the received TCI codepoint

Since RAN1 has agreed that if the UE receives a TCI codepoint mapped with a sub-set of {first joint TCI state, second joint TCI state} or {first DL TCI state, first UL TCI state, second DL TCI state, second UL TCI state}, the UE shall update the first/second indicated joint/DL/UL TCI state(s) according to the first/second joint/DL/UL TCI state(s) in the subset and keep other indicated first/second joint/DL/UL TCI state(s) that is not updated by the received TCI codepoint. From RAN2 perspective, the usage of MAC CE can be specified accordingly.

Proposal 4: UE should update the first/second indicated joint/DL/UL TCI states according to the indication in MAC CE and keep the other indicated first/second joint/DL/UL TCI state(s) that is not updated when the new MAC CE is received.

# Conclusion

We have the following observations, and we’d recommend RAN2 to discuss and adopt the following proposals:

Proposal 1: In the new MAC CE, a field should be able to indicate that the TCI state is updated for the first TRP or the second TRP or both TRPs for each TCI codepoint in the TCI field in a DCI format.

Proposal 2: For the TCI states updated in the new MAC CE, its type (joint DL/UL TCI state or separate DL/UL TCI state) is configured by RRC. The Rel-17 RRC signaling can be reused.

Proposal 3: If the separate DL/UL TCI state type is configured in the serving cell, the new MAC CE should be able to indicate either the DL or UL for each TRP that the TCI state is updated for each TCI codepoint in the TCI field in a DCI format.

Proposal 4: UE should update the first/second indicated joint/DL/UL TCI states according to the indication in MAC CE and keep the other indicated first/second joint/DL/UL TCI state(s) that is not updated when the new MAC CE is received.

# References

# Annex