3GPP TSG-RAN WG2 #123	R2-2308423

Toulouse, France, 21st Aug- 25th Aug 2023

Agenda Item:	7.13.2

Source:	Ericsson

Title:	Discussion on voice fallback HO failure

Document for:	Discussion, Decision

# 1	Introduction

In RAN2#122-Incheon meeting, the following decisions were agreed-

Agreements:

1    Introduce a new indication in the LTE RLF report for the case an RLF occurs shortly after successful HO from NR to E-UTRAN for voice fallback.

2    UE to log the time until reconnection during RRC connection establishment to the acceptable cell and reconnection cell ID in is absent, which will reuse the legacy field.

In this contribution, we discuss some open issued and propose a solution for them.

# 2	Discussion

## 2.1	Voice fallback HO failure due to blind handover:

It is a plausible scenario that a NR IDLE UE is required to perform a RRC state transition to RRC_CONNECTED and be immediately sent to the LTE network for voice fallback purposes (e.g., when the voice service is not supported by the NR). In such scenarios NR gNB would not have the time and possibility to configure the UE with relevant RRM measurements configuration.

- Network may not be able to configure the UE to perform RRM measurements prior to a voice fallback HO.

Since the network is forced to perform a blind handover, there is a chance that it ends in HOF and UE needs to generate a RLF report. However, UE would not have relevant L3 measurement results available to log in the RLF report upon declaring the radio link failure.

- UE may not have relevant L3 measurement results available to log in the RLF report upon declaring the radio link failure during a voice fallback.

Moreover, such UE may have some early measurement results stored and network may not have had the chance to fetch the EM results. In such scenarios, UE may include the un-fetched EM results in the RLF report so that the network is able to optimize decision for future handovers.

- UE logs the available early measurements in the RLF report after voice fallback HOF.

Furthermore, if the UE includes time it stayed in the source cell in the RLF report, network would be able to analyze and understand that it didn’t have enough time to perform a proper handover of the UE to LTE. Such information enables the network to conclude that lack of sufficient measurements concerning the neighboring cell is not due to lack of radio coverage but due to lack of proper RRM configuration at the UE. Needless to mention, that the UE already measure such time for other purposes and including such information in the RLF report upon voice fallback failure requires negligible effort by the UE.

- In case of immediate/blind handovers, logging time spent in the source cell in the RLF report assists the network to distinguish the lack of proper RRM measurement configuration at the UE than the lack of radio coverage.

- UE logs time spent in the source cell of the HO, in the RLF report upon voiceFallback HOF.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Network may not be able to configure the UE to perform RRM measurements prior to a voice fallback HO.

Observation 2	UE may not have relevant L3 measurement results available to log in the RLF report upon declaring the radio link failure during a voice fallback.

Observation 3	In case of immediate/blind handovers, logging time spent in the source cell in the RLF report assists the network to distinguish the lack of proper RRM measurement configuration at the UE than the lack of radio coverage.

Based on the discussion in the previous sections we propose the following:

Proposal 1	UE logs the available early measurements in the RLF report after voice fallback HOF.

Proposal 2	UE logs time spent in the source cell of the HO, in the RLF report upon voiceFallback HOF.