3GPP TSG-RAN WG2 Meeting #123	R2-2308432

# Toulouse, France, 21st – 25th August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0701 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous stage 2 corrections for sidelink |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TS 38.300 there are several inconsistencies for sidelink that would need to be corrected in order to avoid any misleading behaviour by the UE or the network. This CR is to correct such inconsistencies. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Section 7.1 - Clarification that measurements for sidelink are only CBR Section 16.9.2.6 - Clarified that SL RRC decides measurements for sidelink but also the RLC mode. Section 16.9.3.1 - Fix identation on the last bulled of the section Section 16.9.4.2 - Clarified that UE may also request SIB on-demand while in RRC_CONNECTED Section 16.9.4.3 - Clarified that UE may also request SIB on-demand while in RRC_IDLE and RRC_INACTIVE. Impact Analysis Impacted 5G architecture options: NR SA Impacted functionality: Sidelink Inter-operability: 1. If the network is implemented according to the CR and the UE is not, there is no inter-operability issue. 2. If the UE is implemented according to the CR and the network is not, there is no inter-operability issue. 3. If the TX UE implements the CR and the RX UE does not, there is no inter-operability issue. 4. If the RX UE implements the CR and the TX UE does not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the CR is not approve, inconsistencies about sidelink will remain in the specification and this may cause some uncertain behaviour at the UE and/or the network. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1, 16.9.2.6, 16.9.3.1, 16.9.4.2, 16.9.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

## 7.1	Services and Functions

The main services and functions of the RRC sublayer over the Uu interface include:

-	Broadcast of System Information related to AS and NAS;

-	Paging initiated by 5GC or NG-RAN;

-	Establishment, maintenance and release of an RRC connection between the UE and NG-RAN including:

-	Addition, modification and release of carrier aggregation;

-	Addition, modification and release of Dual Connectivity in NR or between E-UTRA and NR.

-	Security functions including key management;

-	Establishment, configuration, maintenance and release of Signalling Radio Bearers (SRBs) and Data Radio Bearers (DRBs);

-	Mobility functions including:

-	Handover and context transfer;

-	UE cell selection and reselection and control of cell selection and reselection;

-	Inter-RAT mobility.

-	QoS management functions;

-	UE measurement reporting and control of the reporting;

-	Detection of and recovery from radio link failure;

-	NAS message transfer to/from NAS from/to UE.

The sidelink specific services and functions of the RRC sublayer over the Uu interface include:

-	Configuration of sidelink resource allocation via system information or dedicated signalling;

-	Reporting of UE sidelink information;

-	Measurement configuration and reporting of CBR related to sidelink;

-	Reporting of UE assistance information for SL traffic pattern(s).

END OF CHANGES

START OF CHANGES

#### 16.9.2.6	RRC

The RRC sublayer provides the following services and functions over the PC5 interface:

-	Transfer of a PC5-RRC message between peer UEs;

-	Maintenance and release of a PC5-RRC connection between two UEs;

-	Detection of sidelink radio link failure for a PC5-RRC connection.

-	Measurement configuration and reporting related to sidelink;

-	RLC mode to be used between two UEs;

A PC5-RRC connection is a logical connection between two UEs for a pair of Source and Destination Layer-2 IDs which is considered to be established after a corresponding PC5 unicast link is established as specified in TS 23.287 [40]. There is one-to-one correspondence between the PC5-RRC connection and the PC5 unicast link. A UE may have multiple PC5-RRC connections with one or more UEs for different pairs of Source and Destination Layer-2 IDs.

Separate PC5-RRC procedures and messages are used for a UE to transfer UE capability and sidelink configuration to the peer UE, as specified in TS 38.331 [3]. Both peer UEs can exchange their own UE capability and sidelink configuration using separate bi-directional procedures in both sidelink directions.

If it is not interested in sidelink transmission, if sidelink RLF on the PC5-RRC connection is declared, or if the Layer-2 link release procedure is completed as specified in TS 23.287 [40], UE releases the PC5-RRC connection.

END OF CHANGES

START OF CHANGES

#### 16.9.3.1	General

For NR sidelink communication, the UE can operate in two modes as specified in 5.7.2 for resource allocation in sidelink:

-	Scheduled resource allocation, characterized by:

-	The UE needs to be RRC_CONNECTED in order to transmit data;

-	NG-RAN schedules transmission resources.

-	UE autonomous resource selection, characterized by:

-	The UE can transmit data when inside NG-RAN coverage, irrespective of which RRC state the UE is in, and when outside NG-RAN coverage;

-	The UE autonomously selects transmission resources from resource pool(s).

For NR sidelink communication, the UE performs sidelink transmissions only on a single carrier.

END OF CHANGES

START OF CHANGES

#### 16.9.4.2	Control of connected UEs

The UE in RRC_CONNECTED performs NR sidelink communication and/or V2X sidelink communication, as configured by the upper layers. The UE sends Sidelink UE Information to NG-RAN in order to request or release sidelink resources and report QoS information for each destination.

NG-RAN provides RRCReconfiguration message to the UE in order to provide the UE with dedicated sidelink configuration. The RRCReconfiguration message may include SL DRB configuration(s) for NR sidelink communication and/or V2X sidelink communication as well as mode 1 resource configuration and/or mode 2 resource configuration. If UE has received SL DRB configuration via system information, UE should continue using the configuration to perform sidelink data transmissions and receptions until a new configuration is received via the RRCReconfiguration message.

If the NG-RAN does not provide provide a configuration for NR sidelink communication and/or V2X sidelink communication via the RRCReconfiguration message, the UE may request SIB12 (for resource pool configuration and SL DRB configuration for NR sidelink communication) and SIB13, SIB14 (for resource pool configuration for V2X sidelink communication) on-demand while in RRC_CONNECTED according to TS 38.331 [12].

NG-RAN may also configure measurement and reporting of CBR for NR sidelink communication and V2X sidelink communication, and reporting of location information for V2X sidelink communication to the UE via RRCReconfiguration message.

During handover, the UE performs sidelink transmission and reception based on configuration of the exceptional transmission resource pool or configured sidelink grant Type 1 and reception resource pool of the target cell as provided in the handover command.

#### 16.9.4.3	Control of idle/inactive UEs

The UE in RRC_IDLE or RRC_INACTIVE performs NR sidelink communication and/or V2X sidelink communication, as configured by the upper layers. NG-RAN may provide common sidelink configuration to the UE in RRC_IDLE or RRC_INACTIVE via system information for NR sidelink communication and/or V2X sidelink communication. UE receives resource pool configuration and SL DRB configuration via SIB12 for NR sidelink communication as specified in TS 38.331 [12], and/or resource pool configuration via SIB13 and SIB14 for V2X sidelink communication as specified in TS 38.331 [12].

The UE may request SIB12 (for resource pool configuration and SL DRB configuration for NR sidelink communication) and SIB13, SIB14 (for resource pool configuration for V2X sidelink communication) on-demand while in RRC_IDLE or RRC_INACTIVE, according to TS 38.331 [12].

When the UE performs cell reselection, the UE interested in V2X service(s) considers at least whether NR sidelink communication and/or V2X sidelink communication are supported by the cell. The UE may consider the following carrier frequency as the highest priority frequency, except for the carrier only providing the anchor carrier:

-	the frequency providing both NR sidelink communication configuration and V2X sidelink communication configuration, if configured to perform both NR sidelink communication and V2X sidelink communication;

-	the frequency providing NR sidelink communication configuration, if configured to perform only NR sidelink communication.

-	the frequency providing V2X sidelink communication configuration, if configured to perform only V2X sidelink communication.

When the UE performs cell reselection, the UE interested in ProSe service(s) considers at least whether NR sidelink communication is supported by the cell. The UE may consider the carrier frequency providing NR sidelink communication configuration as the highest priority frequency, except for the carrier only providing the anchor carrier.

END OF CHANGES