3GPP TSG-RAN WG2 Meeting #123	R2-2308437

Toulouse, France, 21st – 25th August 2023

Agenda Item:	7.4.2.2

Source:	Ericsson

Title:	Discussion of remaining RRC open issues for LTM

Document for:	Discussion, Decision

# 1	Introduction

In the email discussion [2], many RRC open issues was discussed and seem also be easily resolved.

In this contribution we discuss further RRC aspects on LTM that still need to be addressed.

# 2	Discussion

## 2.1	Handling of BWPs in LTM

One issue that was discussed in the RAN2#119-bis-e meeting, was which BWP the UE should consider when receiving the lower layer switch command from the network to execute the LTM cell switch procedure. Based on the discussion, it was not possible to reach an agreement and the following has been captured:

FFS how the UE determine the BWPs (for DL and UL) to be used upon the execution of L1/L2 inter-cell mobility

According to TS 38.300, the switching between the initial BWP and a dedicated BWP happens by means of RRC signaling but for the case of LTM this procedure is triggered by a lower layer signaling (a MAC CE). The text also mentions that a switching between configured BWPs happens by means of Downlink Control Indication (DCI) or inactivity timer however, these are defined only for the same cell (intra-cell BWP switching). Finally, the text also mentions that a switching between configured BWPs happens upon initiation of random access. However, in LTM one of the goals is to reduce the interruption time, by possible accessing the target cell without random access.

On top of this, another big issue is that the serving DU (which is the node that is responsible to send the lower layer signalling to the UE) may not have knowledge about which BWPs have been configured by a Candidate DU in its candidate cell configuration for LTM.

How the UE determine the BWP (for DL and UL) to be used upon the execution of LTM cell switch is not clear.

The serving DU (which is the node that is responsible to send the lower layer signalling to the UE) may not have knowledge about which BWPs have been configured by a Candidate DU in its LTM candidate cell configuration.

In order to overcome the problems described above, a baseline solution is that which BWP to be used upon the LTM cell switch procedure is indicated directly within the LTM candidate cell configuration.

The BWP to be used by the UE upon the execution of the LTM cell switch procedure can be indicated directly within the LTM candidate cell configuration.

Nevertheless, it is worth noticing that based on the LS received from RAN1 in [1], one of the agreements taken in RAN1 is that the UL/DL BWP ID can also be included in the LTM cell switch command (MAC CE).

Agreement From RAN1 point of view, at least the following information can be included in the cell switch command, which is conveyed by MAC CE - Information to identify the target cell(s) - The details including bit number are designed by RAN2 - TA related information (details up to the discussion in A.I. 9.10.2) - 1 joint or 1 pair of UL and DL unified TCI State index for the target Cell - Note: discussion on target SpCell is not precluded - Active DL and UL BWPs for the target cell - FFS: Triggering of aperiodic TRS transmitted from the target cell - FFS: Triggering the CSI acquisition of the target cell and reporting to the target cell - FFS: Triggering of aperiodic SRS transmission to the target cell - FFS: C-RNTI - FFS: the presence of each field (i.e. always present or configurable)

According to this, one issue is what the UE should do if it receives an UL/DL BWP IDs in the LTM cell switch command and, at the same time, the UL/DL BWP IDs are also included within an LTM candidate cell configuration. In such a case, it would be straightforward that the UL/DL BWP IDs received in the LTM cell switch command override the one contained within the LTM candidate cell configuration.

In the email discussion [Post122][058][Mob18] Contents of Cell Switch MAC CE [3] the two options for determine the active BWP ID of the target cell were discussed:

- Option 1: Only based on the legacy firstActiveUplinkBWP-Id and firstActiveDownlinkBWP-Id in RRC (pre)configuration corresponding to the target cell.

- Option 2: Using optional fields of “active UL/DL BWP ID” in LTM cell switch MAC CE. When included in the LTM MAC CE, it overrides the legacy firstActiveUplinkBWP-Id and firstActiveDownlinkBWP-Id in RRC (pre)configuration of the target cell.

By only supporting Option 1, we would rely on reconfiguration of LTM candidate cells each time the UE traffic changes. A main advantage of LTM is that we can avoid frequent RRC reconfiguration caused by mobility. But if we only support Option 1, it is not clear that we have this advantage any more.

Therefore, we propose:

If UL/DL BWP IDs are included in the LTM cell switch command (MAC CE) the UE shall use those BWPs and ignore the ones in the LTM candidate cell configuration.

## 2.2	RLM and RLF handling in LTM

Based on the agreements taken by RAN2, the following is supported when an LTM RLF is experienced on the MCG.

Following behaviors of LTM supervisor timer are agreed: - - 1: The UE starts the LTM supervisor timer, upon reception of the LTM cell switch MAC CE; - - 2: The UE stops the LTM supervisor timer, upon successful completion of LTM cell switch; - 3: If the LTM supervisor timer for MCG expires, as baseline, the UE considers LTM failure and initiates RRC re-establishment. (SCG switch case FFS)

Assuming that the UE is operating in a serving cell and, at the same time, is configured with a set of LTM candidate cell configurations, one open aspect that need to be discussed is how the RLM and subsequent RLF procedure is performed. So far, the existing solutions for RLF detection in 5G NR rely on radio related problems on a RLM process based on the monitoring of the Special Cell (SpCell), such as the PCell in the Master Cell Group (MCG), or the PSCell in the Secondary Cell Group (SCG). Further, there is no RLM on SCell(s)

So far, the existing solutions for RLF detection in NR rely on radio related problems on a RLM process based on the monitoring of the Special Cell (SpCell).

However, in LTM the UE is not only configured with an SpCell, but in addition, with one or more LTM target candidate cells which the UE can move to upon the reception of a lower layer signaling. Hence, the existing framework for RLF detection and RLM may not be suitable, as the UE may actually leave the coverage of the SpCell to which is currently connected (i.e., its serving cell) but it may be still in coverage of one (or more) LTM candidate cell(s). In this case, according to the legacy solution the UE would declare RLF and initiate the RRC re-establishment procedure with a consequent long connectivity interruption.

The UE may leave the coverage of the SpCell to it which is currently connected (i.e., its serving cell) but it may be still in coverage of one (or more) of LTM candidate cell(s).

According to this, several solutions can be adopted in order to avoid the UE to declare RLF and initiate the RRC re-establishment procedure, as far as one of the candidate cells for LTM is still good.

In one solution, it would be enough for the UE to consider a joint RLM process where the UE not only monitor the status of its serving cell, but also the status of the configured candidate cells for LTM. According to this, the RLF would be declared only is the serving cell and none of the configured candidate cells for LTM are available anymore. In a second solution, the UE may have independent RLM processes, one for the serving cell and one for each configured candidate cells for LTM. In this case, the RLF may be declared locally on the serving cell and on one of the candidate cells for LTM.

Bottom line is that in both solution the UE will have the possibility to recover the connection over one of the configured candidate cells for LTM when an RLF is detected on the serving cell.

There are multiple solutions on how the RLM is performed when the UE is configured with LTM candidate cell configurations:

  - The UE has a joint RLM process where it monitors the serving cell and the configured candidate cells for LTM.

  - The UE has a single RLM process for the serving cell and for each of the configured candidate cells for LTM.

  - The UE has a single RLM process only for the serving cell (i.e., RLM is not configured for candidate cell(s)).

As there are only three RAN2 meetings left to complete Rel-18, we propose to start with a baseline solution but be open to further enhancements:

When UE is configured with LTM, the UE has a single RLM process only for the serving cell, as in baseline (i.e., RLM is not configured for LTM candidate cell(s)).

## 2.3	BFD and BFR handling in LTM

Similar to what has been discussed in section 2.4 for the handling of the RLM and RLF processes, the same situation is present for the BFD and BFR handling in case the UE is configured with one or more candidate cells for LTM.

If the legacy procedure is followed, the UE will perform BFD and BFR only on the serving cell but this it may result in an unnecessary connectivity interruption (since the UE would need to trigger a random access procedure to restore the connection) as one or more LTM candidate cell configurations may be still good.

In order to overcome this problem, and also to avoid the UE to perform BFR in case at least one of the candidate cells for LTM is still good, different solution can be considered.

In a solution, the BFD and BFR is done only on the serving cell. However, in case a BFD is detected, the UE should have the freedom to trigger autonomously the execution of a LTM cell switch procedure on one of the candidate cells, if there is one that is good enough. We think that the UE may be perfectly capable of doing this because the UE may need to perform anyway CSI measurement on all the configured candidate cells for LTM.

In another solution, the BFD and BFR is done by considering a set of beams which include beams from the serving cell plus beams from the configured candidate cells for LTM. In this solution, the BFD can be detected on the serving cell but also on one of the candidate cells for LTM. Bottom line is that the BFR should be initiated only if the UE cannot recover on any of the available beams (beams of the serving cell and beams of the candidate cells).

There is also another solution where the BFD and BFR is done independently on the serving cell and on one or more of the configured candidate cells for LTM. Even if one it may say that this solution may not avoid connectivity interruption, one benefit of this solution is that the UE may constantly monitor the status of the candidate cells and eventually inform the serving cell on when some of the candidate cells become unavailable.

Nevertheless, all the three solutions have pros and cons and RAN2 should discuss them before to reach an agreement on how to handle the BFD and BFR for the case when the UE is configured with LTM candidate cell configurations.

There are multiple solutions for how the BFD and BFR are performed when the UE is configured with LTM candidate cell configurations:

  - BFD/BFR is done only on the serving cell.

  - BFD/BFR is done jointly on the serving cell and the configured candidate cell for LTM.

  - BFD/BFR is done independently on the serving cell and on each configured candidate cell for LTM.

As there are only three RAN2 meetings left to complete Rel-18, also here we propose to start with a baseline solution but be open to further enhancements:

When UE is configured with LTM, BFD/BFR is done only on the serving cell, as in baseline (i.e., BFD/BFR is not performed for LTM candidate cell(s)).

## 2.4	Applicability of s-Measure to LTM

In current RRM measurements, the triggering for performing neighbour cell measurements may depend on the quality of the current PCell, and controlled by an s-Measure parameter. When s-Measure is configured (e.g. s-MeasureConfig set to ssb-RSRP), the UE only performs neighbour cell measurements when the NR SpCell RSRP based on SS/PBCH block, after layer 3 filtering, is lower than ssb-RSRP, as shown below:

***************************************************************************************************************************

3>	if the UE does not require measurement gaps to perform the concerned measurements:

4>	if s-MeasureConfig is not configured, or

4>	if s-MeasureConfig is set to ssb-RSRP and the NR SpCell RSRP based on SS/PBCH block, after layer 3 filtering, is lower than ssb-RSRP, or

[…]

5>	if the measObject is associated to NR and the rsType is set to ssb:

6>	if reportQuantityRS-Indexes and maxNrofRS-IndexesToReport for the associated reportConfig are configured:

7>	derive layer 3 beam measurements only based on SS/PBCH block for each measurement quantity indicated in reportQuantityRS-Indexes, as described in 5.5.3.3a;

6>	derive cell measurement results based on SS/PBCH block for the trigger quantity and each measurement quantity indicated in reportQuantityCell using parameters from the associated measObject, as described in 5.5.3.3;

***************************************************************************************************************************

When the UE is configured with LTM, the UE is likely going to be configured for performing lower layer measurements on neighbour cells which are configured as LTM candidate cells. These may be cells in a frequency associated to a measurement object which is configured in the MeasConfig for RRM measurements. Thus, even though the specifications in clause 5.5 of TS 38.331 refers specifically to RRM measurements when it says, “derive layer 3 beam measurements” and “derive cell measurement results”, it is still not obvious that the UE would ignore that for lower layer measurements on the exact same neighbour cells.

It is unclear if the UE would trigger an RRC Measurement reports for A3 events triggered by LTM candidate cells being an offset better than the PCell.

One option to address this ambiguity is to explicitly define that the UE ignores the s-Measure for LTM related measurements, even if these are performed on neighbour cells. The benefit of this solution is the simplicity of one feature not interfering with the other, and perhaps the minimized changes in RRC specifications and UE implementation for LTM; a simple update to field description of s-MeasureConfig could suffice. One consequence is that sometimes the UE would have a good PCell quality, would not be performing RRM measurements on neighbours, but LTM measurements would still be up and running, perhaps unnecessarily. Thus we propose:

UE ignores the s-Measure, if configured, when performing L1 LTM related measurements.

## 2.5	Coexistence of LTM and L3 HO

At RAN2#121bis, we made the following agreement that network handles (and should avoid) any race conditions:

While configured with LTM candidate cells, the UE can also execute any L3 handover command sent by the network. R2 assumes that is could be up to the network to avoid any issue due to the race condition between LTM execution and RRC Reconfiguration (e.g. L3 HO cmd), e.g. avoid sending LTM switch cmd and L3 HO cmd in the same TB.

One remaining thing to discuss is what happens with the LTM configuration during L3 HO. As the L3 HO uses an RRCReconfiguration message, this message may reconfigure LTM (add, modify and/or release LTM related configuration(s)), so LTM can be used directly in the target gNB. Typically, as inter-CU LTM is not supported in Release-18, the target gNodeB would not modify an existing LTM candidate configuration, but possibly add new LTM candidate cell(s), in case it is LTM-capable, and/or remove the configured LTM candidate(s). In addition to it, the target gNodeB may decide to add and/or remove other LTM related configurations, such as LTM related measurement configuration(s), UL pre-sync configurations (for TA acquisition) and DL pre-sync configuration(s).

Also, there could be scenario in which the UE is not configured with LTM, but it is being handed over to a cell which is associated to a CU/DU which is LTM capable, so that the target gNodeB may want to configure that UE with LTM e.g., based on RRM measurements from the source to the target in the XnAP: Handover Request message.

Since there is no support inter-CU LTM, the LTM candidate cells for inter-gNB HO may not be useful, but for intra-gNB HO would be nice to keep them. We propose:

During L3 HO, the UE keeps the LTM configuration.

HO command may include an addMod and/or Release indication for LTM related configurations (e.g. LTM candidate configuration(s), UL pre-sync, DL pre-sync and lower layer measurement configuration(s)).

## 2.6	RRC suspend and resume and LTM

What happens with the LTM configuration when the UE enters RRC_INACTIVE?

- Alt 1 (baseline): The UE clears the LTM configuration.

- Alt 2 (enhancement): The UE stores at least part of the LTM configuration. Upon RRC resume, the UE restores the stored LTM configuration, possibly controlled by the network (and UE may indicate to the network it has stored it).

Storing configurations when entering RRC_INACTIVE so that can be restored when re-entering RRC_CONECTED has its benefits, especially when the UE is static or semi-static when transitioning between RRC_INACTIVE to RRC_CONNECTED, so that signalling is not wasted. In addition, most likely the same LTM candidates would be released and added back in case LTM related configurations are released when the UE enters RRC_INACTIVE and the UE resume in the same cell, so releasing LTM candidate represents unnecessary efforts.

From the UE perspective, it is simple to store LTM related configuration in the UE AS Inactive Context when it enters RRC_INACTIVE.

A similar discussion has happened for MR-DC configuration(s) in Rel-15 and Rel-16. RAN2 recognized the benefits explained above but due to the lack of time decided to postpone the introduction of the feature to Rel-16. In our view, we should strive to introduce that in the first LTM release, to avoid different solutions. And, to simplify the discussions, we could follow a similar approach as in MR-DC Release 16.

A first point is that the UE configured with LTM stores the LTM related configuration in the UE Inactive AS context when it is suspended to RRC_INACTIVE, in the same way in which MR-DC configurations are stored in Rel-16.

The UE stores all LTM related configurations in the UE Inactive AS context when it is suspended to RRC_INACTIVE.

The follow up question is what happens when the UE tries to resume. In the MR-DC case, for the UEs capable of storing MR-DC configuration while in RRC_INACTIVE, a simple parameter called “restoreSCG” has been added to the RRCResume message. The UE would only restore the MR-DC configuration when that is included, as follows:

***************************************************************************************************************************

#### 5.3.13.4	Reception of the RRCResume by the UE

The UE shall:

[…]

1>	if the RRCResume includes the fullConfig:

2>	perform the full configuration procedure as specified in 5.3.5.11;

1>	else:

[…]

2>	if the RRCResume does not include the restoreSCG:

3>	release the MR-DC related configurations (i.e., as specified in 5.3.5.10) from the UE Inactive AS context, if stored;

2>	restore the masterCellGroup, mrdc-SecondaryCellGroup, if stored, and pdcp-Config from the UE Inactive AS context;

[…]

1>	discard the UE Inactive AS context;

[…]

***************************************************************************************************************************

The benefit of this parameter “restoreSCG” is that it addresses the case in which the target gNodeB is not MR-DC capable, but also other simpler cases in which it is capable. A similar approach could be adopted for LTM, i.e., define a parameters in RRCResume called “restoreLTM”: when that is included the UE restores the LTM related configurations, otherwise it deletes it. In that case, a target gNodeB which is not LTM capable does not need to interpret LTM configuration(s) in the UE Inactive AS Context which has been fetched and/or explicitly release LTM related configurations. In case the UE resumes in the same cell as the last serving cell, or in a cell from the last serving CU, or in a cell of a CU/DU which is LTM capable, the target gNodeB may decide to include or not include that parameter, as suggest by the following TP:

***************************************************************************************************************************

#### 5.3.13.4	Reception of the RRCResume by the UE

The UE shall:

[…]

1>	if the RRCResume includes the fullConfig:

2>	perform the full configuration procedure as specified in 5.3.5.11;

1>	else:

[…]

2>	if the RRCResume does not include the restoreSCG:

3>	release the MR-DC related configurations (i.e., as specified in 5.3.5.10) from the UE Inactive AS context, if stored;

2>	if the RRCResume does not include the restoreLTM:

3>	release the LTM related configurations (i.e., as specified in 5.3.5.x) from the UE Inactive AS context, if stored;

Editor’s Note: FFS define the procedure for releasing LTM related configuration 5.3.5.x when IEs and L1 parameters are stable.

2>	restore the masterCellGroup, mrdc-SecondaryCellGroup, if stored, and pdcp-Config from the UE Inactive AS context;

[…]

1>	discard the UE Inactive AS context;

[…]

***************************************************************************************************************************

When the UE with stored LTM related configurations tries to resume, it restores the LTM related configuration when a parameter (e.g. restoreLTM) is included in the RRCResume. Otherwise, the UE releases stored LTM related configurations.

Regardless of configurations being restored or not, considering that RRCResume is an encrypted and integrity protected message, there is no issue to allow that to add and/or remove LTM candidate cell configuration(s) and other LTM related configuration. Thus, the UE should be able to receive in RRCResume a reconfiguration of LTM related parameters.

The RRCResume message may reconfigure LTM related configurations e.g. addMod/ release LTM candidate cell(s) and/or reconfigure UL pre-sync and DL pre-sync (similar IE(s) defined for the RRCReconfiguration including LTM related parameters).

# 3	Conclusion

According to the discussion in Section 2, the following observations are made:

Observation 1	How the UE determine the BWP (for DL and UL) to be used upon the execution of LTM cell switch is not clear.

Observation 2	The serving DU (which is the node that is responsible to send the lower layer signalling to the UE) may not have knowledge about which BWPs have been configured by a Candidate DU in its LTM candidate cell configuration.

Observation 3	So far, the existing solutions for RLF detection in NR rely on radio related problems on a RLM process based on the monitoring of the Special Cell (SpCell).

Observation 4	The UE may leave the coverage of the SpCell to it which is currently connected (i.e., its serving cell) but it may be still in coverage of one (or more) of LTM candidate cell(s).

Observation 5	There are multiple solutions on how the RLM is performed when the UE is configured with LTM candidate cell configurations:

a.	The UE has a joint RLM process where it monitors the serving cell and the configured candidate cells for LTM.

b.	The UE has a single RLM process for the serving cell and for each of the configured candidate cells for LTM.

c.	The UE has a single RLM process only for the serving cell (i.e., RLM is not configured for candidate cell(s)).

Observation 6	There are multiple solutions for how the BFD and BFR are performed when the UE is configured with LTM candidate cell configurations:

a.	BFD/BFR is done only on the serving cell.

b.	BFD/BFR is done jointly on the serving cell and the configured candidate cell for LTM.

c.	BFD/BFR is done independently on the serving cell and on each configured candidate cell for LTM.

Observation 7	It is unclear if the UE would trigger an RRC Measurement reports for A3 events triggered by LTM candidate cells being an offset better than the PCell.

Observation 8	From the UE perspective, it is simple to store LTM related configuration in the UE AS Inactive Context when it enters RRC_INACTIVE.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The BWP to be used by the UE upon the execution of the LTM cell switch procedure can be indicated directly within the LTM candidate cell configuration.

Proposal 2	If UL/DL BWP IDs are included in the LTM cell switch command (MAC CE) the UE shall use those BWPs and ignore the ones in the LTM candidate cell configuration.

Proposal 3	When UE is configured with LTM, the UE has a single RLM process only for the serving cell, as in baseline (i.e., RLM is not configured for LTM candidate cell(s)).

Proposal 4	When UE is configured with LTM, BFD/BFR is done only on the serving cell, as in baseline (i.e., BFD/BFR is not performed for LTM candidate cell(s)).

Proposal 5	UE ignores the s-Measure, if configured, when performing L1 LTM related measurements.

Proposal 6	During L3 HO, the UE keeps the LTM configuration.

Proposal 7	HO command may include an addMod and/or Release indication for LTM related configurations (e.g. LTM candidate configuration(s), UL pre-sync, DL pre-sync and lower layer measurement configuration(s)).

Proposal 8	The UE stores all LTM related configurations in the UE Inactive AS context when it is suspended to RRC_INACTIVE.

Proposal 9	When the UE with stored LTM related configurations tries to resume, it restores the LTM related configuration when a parameter (e.g. restoreLTM) is included in the RRCResume. Otherwise, the UE releases stored LTM related configurations.

Proposal 10	The RRCResume message may reconfigure LTM related configurations e.g. addMod/ release LTM candidate cell(s) and/or reconfigure UL pre-sync and DL pre-sync (similar IE(s) defined for the RRCReconfiguration including LTM related parameters).

# 4	References

R2-2304620, LS on beam indication of target cell(s) and time gap between a PDCCH order and the corresponding PRACH transmission for LTM, RAN1, RAN2#122, Incheon, South Korea, May 2023.

R2-2308434, [Post122][055][Mob18] Discussion on RRC open issues list for LTM, Ericsson, 3GPP TSG-RAN WG2 Meeting #123, Toulouse, France, 21st – 25th August 2023

R2-2307962, Summary of [Post122][058][Mob18] Contents of Cell Switch MAC CE, Huawei, HiSilicon, 3GPP TSG-RAN WG2 Meeting #123, Toulouse, France, August 21 – 25, 2023