3GPP TSG-RAN WG2 Meeting #123	R2-2308439

Toulouse, France, 21st – 25th August 2023

Agenda Item:	7.4.2.1

Source:	Ericsson

Title:	Stage-2 proposal on early sync for LTM

Document for:	Discussion, Decision

# 1	Introduction

In RAN WG1 #112, RAN1 sent an LS in [1] with agreements regarding the time alignment establishment and the CSI Measurement configuration for LTM, asking RAN2 to taken them into account.

With this LS as one of the inputs, at the RAN2#122 meeting we took these in agreements on early TA acquisition for candidate cells:

- For early TA acquisition for candidate Cells For PDCCH ordered early TA acquisition without RAR, there is no need for UE to maintain the TA timer for candidate cell (i.e. it is NW implementation to determine the TA validity), TA is given in the cell switch MAC CE (when available in the network). RAN2 doesn’t see a need for a solution with RAR in for Rel-18. Observation: Without RAR (without UE maintaining TA), the UE will need to do RACH for link recovery and/or conditional (if supported), which is acceptable in Rel-18 The UE determines to trigger RACH-less cell switch in MAC layer, if the LTM cell switch MAC CE provides the TA value (no RAR is assumed). If the TA maintenance etc for candidate cell(s) in the UE is needed, the TA(s) associated with candidate cell(s) can be maintained during LTM (TDB exactly which cells decide stage-3).

In this contribution, we provide a stage 2 proposal for the early sync for LTM.

# 2	Discussion

## 2.1	Stage 2 TP for early sync in LTM

In release 18, two new procedures for the UE to be UL/DL synchronized with a cell has been introduced in the context of LTM. However, since these procedures are new and can be potentially used also for other features, it would be good to clearly capture then in stage-2 (i.e., 38.300) so to make it clear how these works. Hereafter we provide a TP on how to capture these two new procedure in TS 38.300.

================================== TP to 38.300 ======================================

## 6.x	Early synchronization procedure

### 6.x.1	Early TA acquisition (early UL sync)

When configured by the network, it is possible for a UE in RRC_CONNECTED to be UL synchronized with a cell which is different from the current serving cell.

The following figure described the early TA acquisition (early UL sync) procedure started by the UE:

Figure 6.x.1-1: Early TA acquisition procedures

1.	The gNB to which Cell A belongs provides the TA acquisition configuration to the UE within the RRCReconfiguration message. The TA acquisition configuration includes all information requires to sends a Random Access Preamble to a Cell B so that the gNB to which Cell B belongs can calculate a TA value to be used e.g., in case an LTM cell switch procedure is executed to Cell B. The TA acquisition configuration may include information for one or multiple cells to which the TA acquisition procedure may be executed by the UE.

2.	The UE replies with the RRCReconfigurationComplete message.

3.	The gNB to which Cell A belongs sends a PDCCH order message to the UE in order to initiate a TA acquisition procedure with Cell B. The PDCCH order message includes all information required to sends a Random Access Preamble to Cell B.

4.	The UE sends a Random Access Preamble to Cell B so that the gNB to which Cell B belongs can estimate a TA value to be used by the UE if e.g., an LTM cell switch procedure is triggered to Cell B.

5.	The gNB to which Cell A belongs initiates cell switch procedure to Cell B by proving a cell switch command which includes the TA value calculated by the gNB to which Cell B belongs during the TA acquisition procedure. The cell switch command can be e.g., the LTM cell switch MAC CE.

### 6.x.2	Early TCI state activation (early DL sync)

When configured by the network, it is possible for a UE in RRC_CONNECTED to be DL synchronized with a cell which is different from the current serving cell. This is possible by activating in advance a TCI state that belongs to the cell to which the early DL sync is needed.

The following figure described the early TCI state activation (early DL sync) procedure started by the UE:

Figure 6.x.2-1: Early TCI state activation procedures

1.	The gNB to which Cell A belongs provides a list of TCI states of Cell B to the UE within the RRCReconfiguration message. The gNB to which Cell A belong may provide a list of TCI for one or multiple cells to which the early TCI state activation procedure may be executed by the UE.

2.	The UE replies with the RRCReconfigurationComplete message.

3.	The gNB to which Cell A belongs sends an early TCI state activation MAC CE to the UE in order to initiate a early TCI state activation procedure with Cell B.

4.	The UE activates the TCI state of Cell B indicated in the early TCI state activation MAC CE.

5.	The gNB to which Cell A belongs initiates cell switch procedure to Cell B by proving a cell switch command which indicates Cell B as target cell. The cell switch command can be e.g., the LTM cell switch MAC CE.

==================================================================================

RAN2 to agree to include the TP provided in the Annex into TS 38.300.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to agree to include the TP provided in the Annex into TS 38.300.

# 4	References

R1-2302194, “LS on L1 measurement RS configuration and PDCCH ordered RACH for LTM”, source: RAN1

R2-2306957, 38.300 running CR for introduction of NR further mobility enhancements, MediaTek Inc., vivo, 3GPP TSG-RAN WG2 Meeting #122, Incheon, Korea, May 22~ May 26, 2023

# Annex: State-2 Text proposal

Below is a TP to the 38.300 running CR [2].

==================================================================================

## 6.x	Early synchronization procedure

### 6.x.1	Early TA acquisition (early UL sync)

When configured by the network, it is possible for a UE in RRC_CONNECTED to be UL synchronized with a cell which is different from the current serving cell.

The following figure described the early TA acquisition (early UL sync) procedure started by the UE:

Figure 6.x.1-1: Early TA acquisition procedures

1.	The gNB to which Cell A belongs provides the TA acquisition configuration to the UE within the RRCReconfiguration message. The TA acquisition configuration includes all information requires to sends a Random Access Preamble to a Cell B so that the gNB to which Cell B belongs can calculate a TA value to be used e.g., in case an LTM cell switch procedure is executed to Cell B. The TA acquisition configuration may include information for one or multiple cells to which the TA acquisition procedure may be executed by the UE.

2.	The UE replies with the RRCReconfigurationComplete message.

3.	The gNB to which Cell A belongs sends a PDCCH order message to the UE in order to initiate a TA acquisition procedure with Cell B. The PDCCH order message includes all information required to sends a Random Access Preamble to Cell B.

4.	The UE sends a Random Access Preamble to Cell B so that the gNB to which Cell B belongs can estimate a TA value to be used by the UE if e.g., an LTM cell switch procedure is triggered to Cell B.

5.	The gNB to which Cell A belongs initiates cell switch procedure to Cell B by proving a cell switch command which includes the TA value calculated by the gNB to which Cell B belongs during the TA acquisition procedure. The cell switch command can be e.g., the LTM cell switch MAC CE.

### 6.x.2	Early TCI state activation (early DL sync)

When configured by the network, it is possible for a UE in RRC_CONNECTED to be DL synchronized with a cell which is different from the current serving cell. This is possible by activating in advance a TCI state that belongs to the cell to which the early DL sync is needed.

The following figure described the early TCI state activation (early DL sync) procedure started by the UE:

Figure 6.x.2-1: Early TCI state activation procedures

1.	The gNB to which Cell A belongs provides a list of TCI states of Cell B to the UE within the RRCReconfiguration message. The gNB to which Cell A belong may provide a list of TCI for one or multiple cells to which the early TCI state activation procedure may be executed by the UE.

2.	The UE replies with the RRCReconfigurationComplete message.

3.	The gNB to which Cell A belongs sends an early TCI state activation MAC CE to the UE in order to initiate a early TCI state activation procedure with Cell B.

4.	The UE activates the TCI state of Cell B indicated in the early TCI state activation MAC CE.

5.	The gNB to which Cell A belongs initiates cell switch procedure to Cell B by proving a cell switch command which indicates Cell B as target cell. The cell switch command can be e.g., the LTM cell switch MAC CE.