3GPP TSG-RAN WG2 Meeting #123	R2-2308440

Toulouse, France, 21st – 25th August 2023

Agenda Item:	7.4.2.2

Source:	Ericsson

Title:	Handling of SCells and SCG during LTM cell switch

Document for:	Discussion, Decision

# 1	Introduction

The handling of SCG and SCells for LTM was discussed in email discussions [Post122][055][Mob18] Discussion on RRC open issues list for LTM [2] and [Post122][058][Mob18] Contents of Cell Switch MAC CE [3]. This contribution handles some remaining aspects from these discussions. In addition, from current agreements it is unclear whether the TCI-state indication in the LTM MAC CE refers to the set of beams for early synchronization or toe the set of beams in the LTM candidate cell configuration.

# 2	Discussion

## 2.1	TCI-state indication in LTM MAC CE

In RAN1#112bis-e, it was agreed that the LTM MAC CE command also contains 1 joint or 1 pair of UL and DL unified TCI State index for the target Cell. From the agreement alone it is not clear whether the TCI-state information should refer to the set of beams for TCI state activation for candidate cell(s) used to establish early synchronization or the set of beams in LTM candidate cell configuration? In our view, the beam indication in the LTM command MAC CE should point to the beam to apply in the target cell, and therefore it should refer to a TCI-state configured in the LTM candidate cell configuration.

The TCI-state information in the LTM cell switch command MAC CE refers to a TCI-state configured within the LTM candidate cell configuration.

## 2.2	Handling of SCells

In email discussion [Post122][058][Mob18] Contents of Cell Switch MAC CE [3], one discussion point was whether a field should be included in the LTM cell switch MAC CE to indicate the SCell state (activated/deactivated). Based on the input from participating companies the likely RAN2 conclusion is not to include such a field, and instead rely on the sCellState-r16 field in the stored LTM candidate cell configuration of the target cell to indicate the SCell state of configured SCells, e.g. based on available L3 measurements from the UE at the time of LTM configuration.

As commented in the email discussion, RAN1 is currently working on L1 measurement configurations specific for configured SCells, but if there is no support for SCell activation/deactivation in the LTM cell switch command, there is also no use for L1 measurements on configured SCells of LTM candidate cells, since the source DU will anyway not be able to indicate the SCell activation state to the UE in the LTM command. Therefore, we propose:

If RAN2 agrees not to include a MAC CE field for SCell activation/deactivation, RAN2 to send an LS to RAN1 to inform them about the decision and that no SCell specific L1 measurement configurations are needed.

## 2.3	Handling of SCG at MCG LTM

Another remaining issue is the NR-DC support for LTM, e.g. how to handle the SCG of a NR-DC configured UE upon MCG LTM. This topic was discussed in email discussion [Post122][055][Mob18] Discussion on RRC open issues list for LTM [2]. In the discussions, most participating companies agreed that the LTM-Config IE is per cell group and contains either the MCG LTM configuration or the SCG LTM configuration, but not both. This is in line with the previous agreement to support SCG LTM which does not involve MN, meaning that there is no change in the MCG configuration. Similarly, for the MCG LTM, the proposal from email discussion [Post122][055] [2] means that only MCG LTM without SN involvement is supported, i.e. with no change to the SCG configuration.

As discussed in the email discussion, a remaining issue is still how to handle the existing SCG configuration of a UE configured with NR-DC upon MCG LTM. As commented in the email discussion, the following options exist:

Option 1: SCG is released before MCG LTM configuration. This means that the MN signals the SCG release in a separate procedure, before the MCG LTM candidates are configured. This restriction could be captured in stage-2 description, if options 2 and 3 are not agreed to be supported.

Option 2: SCG is released at MCG LTM execution. This means that the SCG release is included in the LTM candidate configuration, i.e. mrdc-SecondaryCellGroupConfig is set to release. This restriction could be captured in field description of mrdc-SecondaryCellGroupConfig , e.g. that it can only be set to release if part of an LTM candidate configuration.

Option 3: SCG is kept during MCG LTM. This means that the MN may decide to keep the SCG during MCG LTM, but only if there is no change in the SCG configuration, i.e no SN involvement. Since the security key is not changed for the MCG during LTM, there is no need to refresh the SCG key and also no need to signal reconfigurationWithSync for the SCG. Since the SCG configuration is not changed, there is also no need for any complete message towards the SCG during the MCG LTM procedure. Since only intra-MN LTM is supported in Rel-18, there should also be no need for data forwarding involved. If SCG is kept unchanged during MCG LTM, then DRBs mapped to SCG are not affected. So keeping the SCG during MCG LTM seems feasible.

- Option 3 requires the MN to ensure that there is no capability conflict with the configured SCG in case there is a change in the MCG configuration during the MCG LTM cell switch. This means that if there is a change in the SCG configuration that is in conflict with any MCG configuration in the LTM candidate configurations, it is the network responsibility to update the LTM candidate configurations to avoid capability conflict when the LTM cell switch is triggered.

- Combining option 3 with option 2 allows the network to decide at LTM candidate configuration, whether to keep the SCG or not during MCG LTM cell switch. Since this has to be done at time of the MCG LTM candidate configuration, it needs to be done blindly, i.e. based on measurement results the network has at that point of time. However, based on received measurement results the network can at any time release or modify the SCG in a separate configuration procedure. Then at the LTM cell switch, this SCG configuration will remain, or if it was released before the MCG LTM, it will remain released, so there should not be any issue.

In our view, all these options are feasible and which one to use can be left to network implementation. Therefore, we propose:

For UE configured with NR-DC, whether SCG is released before or during LTM execution or kept unchanged is left to network implementation.

## 2.4	Support of SCG LTM

The support of NR-DC for LTM was discussed in the RAN#100 plenary meeting in Taipei in June. So far, the WID has described that LTM is performed within one CG in standalone, CA and NR-DC scenarios. After discussion in the plenary, it was further clarified to prioritize MCG for Rel-18 LTM (for all RAN WGs) [1]:

Note 4: The procedure of L1/L2 based inter-cell mobility are applicable to the following scenarios:

    - Standalone, CA and NR-DC case with serving cell change within one CG, prioritizing MCG

Following the agreement in RAN plenary to prioritize MCG in the Rel-18 LTM work, since only 3 WG meetings are left before WI closing, our proposal is that LTM in the SCG is not supported in Rel-18. This means that configuration of LTM candidate cells for the SCG is not supported. Furthermore, the MAC procedures for LTM are not supported by the SCG MAC entity, including reception of LTM cell switch command MAC CE and candidate cell TCI state activation MAC CE.

SCG LTM is not supported in Rel-18.

If agreed, the limitation to MCG for NR-DC could be captured in 38.300, see TP in Annex.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	The TCI-state information in the LTM cell switch command MAC CE refers to a TCI-state configured within the LTM candidate cell configuration.

Proposal 2	If RAN2 agrees not to include a MAC CE field for SCell activation/deactivation, RAN2 to send an LS to RAN1 to inform them about the decision and that no SCell specific L1 measurement configurations are needed.

Proposal 3	For UE configured with NR-DC, whether SCG is released before or during LTM execution or kept unchanged is left to network implementation.

Proposal 4	SCG LTM is not supported in Rel-18.

# References

RP-231475, Revised WID on Further NR mobility enhancements, MediaTek, Apple

R2-2308934, [Post122][055][Mob18] Discussion on RRC open issues list for LTM, Ericsson, 3GPP TSG-RAN WG2 Meeting #123, Toulouse, France, 21st – 25th August 2023

R2-2307962, Summary of [Post122][058][Mob18] Contents of Cell Switch MAC CE, Huawei, HiSilicon, 3GPP TSG-RAN WG2 Meeting #123, Toulouse, France, August 21 – 25, 2023

# Annex: Text Proposal for 38.300

#### 9.2.3.x	LTM

##### 9.2.3.x.1	General

[…]

LTM supports both intra-gNB-DU and intra-gNB-CU inter-gNB-DU mobility. LTM also supports inter-frequency mobility, including mobility to inter-frequency cell that is not a current serving cell. The following scenarios are supported:

-	PCell change in non-CA scenario,

-	PCell change in CA scenario,

-	Dual connectivity scenario, where LTM is supported only for the PCell. The network can release the SCG before or during the MCG LTM execution, or the SCG can be kept as is, but SCG configuration change is not supported during MCG LTM execution.