3GPP TSG-RAN WG2 Meeting #123	R2-2308445

Toulouse, France, 21st – 25th August 2023

Agenda Item:	7.12.2.2

Source:	Ericsson

Title:	Behaviour for RRC_IDLE and RRC_INACTIVE UEs under a mIAB node

Document for:	Discussion, Decision

# 1	Introduction

In the last RAN2#122 meeting, the following agreements have been reached regarding the mobility enhancements for mobile IAB:

R2 direction (solution agreements at later stage, no other directions will be considered):

- RAN2 acknowledges following two problems to be addressed for idle/inactive UEs:

- - Problem 1: For a UE that is physically on a moving vehicle but not camped on its mobile IAB-cell yet (i.e. the UE is camped on a stationary cell), how to help such UE(s) to identify a neighbour mobile IAB-cell, prioritize mobile IAB-cell (frequency and cell) and to be “pulled” into this mobile IAB-cell, especially for inter-frequency scenario where the mobile IAB-cell’s frequency priority is low.

- - Problem 2: After the UE physically on a moving vehicle is camped on the mobile IAB cell, how to avoid it reselecting other non-mIAB-(stationary) cells.

- - Such UE may prioritize a highest ranked cell at a frequency, if it broadcasts a mIAB-cell type indicator in SIB1 for cell reselection. UE may use the SIB4 assistance information to identify the presence of such mobile IAB-cell(s), if broadcasted. A SIB4 assistance information may include mIAB-cell frequencies. FFS on stage-2/3 to clarify the UE in problem 1 and 2.

In this contribution we will address further aspects regarding UE idle mode mobility behaviour for a mobile IAB.

# 2	Discussion

Similar to Problem 1, another problem that can also be formulated is as follows:

- Problem 3: For a UE that is physically on a moving vehicle and camped on its mobile IAB-cell (i.e. the UE is camped on a mobile IAB cell/bus), if the UE has left the bus, how to help such UE to identify a neighbour stationary cell, prioritize static cell (frequency and cell) and to be “pushed out” from this mobile IAB-cell, especially for inter-frequency scenario where the mobile IAB-cell’s frequency priority is high.

These seems to be deployment specific issues which can be solved by legacy cell reselection parameters or existing SIB4/5 configurations. If RAN2 starts to optimize the cell (re)selection procedure for all possible scenarios that involve UE(s) and a mobile IAB cell, the risk is that the list of problems to address will be much longer to those one already investigated by RAN2.

Problem 1 and 2 acknowledged by RAN2 for IDLE/INACTIVE UEs can be handled by network implementation/configuration.

Optimizing the cell (re)selection procedure for all possible scenarios that involve UE(s) and a mobile IAB cell may lead to address a list of problems that is much longer to those ones already investigated by RAN2.

The only real issue that may persist is that when UE is in Idle mode and during the migration procedure (i.e Inter-CU handover where two logical mobile DUs co-exist for certain duration), then how would these idle mode UE perform cell reselection to a new cell from the new logical DU. It can happen that logical DU1 power is slowly lowered and logical DU2 power is slowly ramped; in such case, the idle mode UE may select a third stationary cell as cell from DU1 may have poor RSRP and cell from DU2 may also have the same issue. In such case, to avoid any ping-pong and when UEs in RRC Connected mode are being handed over to new cell belonging to logical DU2, the CU can broadcast via logical DU1 cell that UEs in idle mode should perform cell reselection to a target cell. Thus, a cell that is indicating that it is a mobile IAB cell, can inform, via SIB4, the migration (between the two logical DUs) has been started and this UE should simply move to the specified target cell.

For the DU migration, target cell ID and frequencies for cell reselection that belongs to the new logical DU are provided via SIB for UEs in RRC_IDLE and RRC_INACTIVE.

According to what is proposed above, here below we provide a possible implementation of Proposal 1 for TS 38.331:

================================== TP to TS 38.331 ==================================

#### –	SIB4

SIB4 contains information relevant for inter-frequency cell re-selection (i.e. information about other NR frequencies and inter-frequency neighbouring cells relevant for cell re-selection), which can also be used for NR idle/inactive measurements. The IE includes cell re-selection parameters common for a frequency as well as cell specific re-selection parameters.

SIB4 information element

-- ASN1START

-- TAG-SIB4-START

SIB4 ::=                            SEQUENCE {

interFreqCarrierFreqList            InterFreqCarrierFreqList,

lateNonCriticalExtension            OCTET STRING                                OPTIONAL,

...,

[[

interFreqCarrierFreqList-v1610      InterFreqCarrierFreqList-v1610              OPTIONAL   -- Need R

]],

[[

interFreqCarrierFreqList-v1700      InterFreqCarrierFreqList-v1700              OPTIONAL   -- Need R

]],

[[

interFreqCarrierFreqList-v1720      InterFreqCarrierFreqList-v1720              OPTIONAL   -- Need R

]],

[[

interFreqCarrierFreqList-v1730      InterFreqCarrierFreqList-v1730              OPTIONAL   -- Need R

]],

[[

m-IAB-TargetCellReselection			M-IAB-TargetCellReselection					OPTIONAL   -- Need R

]]

}

InterFreqCarrierFreqList ::=        SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo

InterFreqCarrierFreqList-v1610 ::=  SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1610

InterFreqCarrierFreqList-v1700 ::=  SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1700

InterFreqCarrierFreqList-v1720 ::=  SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1720

InterFreqCarrierFreqList-v1730 ::=  SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1730

InterFreqCarrierFreqInfo ::=        SEQUENCE {

dl-CarrierFreq                      ARFCN-ValueNR,

frequencyBandList                   MultiFrequencyBandListNR-SIB                                OPTIONAL,   -- Cond Mandatory

frequencyBandListSUL                MultiFrequencyBandListNR-SIB                                OPTIONAL,   -- Need R

nrofSS-BlocksToAverage              INTEGER (2..maxNrofSS-BlocksToAverage)                      OPTIONAL,   -- Need S

absThreshSS-BlocksConsolidation     ThresholdNR                                                 OPTIONAL,   -- Need S

smtc                                SSB-MTC                                                     OPTIONAL,   -- Need S

ssbSubcarrierSpacing                SubcarrierSpacing,

ssb-ToMeasure                       SSB-ToMeasure                                               OPTIONAL,   -- Need S

deriveSSB-IndexFromCell             BOOLEAN,

ss-RSSI-Measurement                 SS-RSSI-Measurement                                         OPTIONAL,   -- Need R

q-RxLevMin                          Q-RxLevMin,

q-RxLevMinSUL                       Q-RxLevMin                                                  OPTIONAL,   -- Need R

q-QualMin                           Q-QualMin                                                   OPTIONAL,   -- Need S

p-Max                               P-Max                                                       OPTIONAL,   -- Need S

t-ReselectionNR                     T-Reselection,

t-ReselectionNR-SF                  SpeedStateScaleFactors                                      OPTIONAL,   -- Need S

threshX-HighP                       ReselectionThreshold,

threshX-LowP                        ReselectionThreshold,

threshX-Q                           SEQUENCE {

threshX-HighQ                       ReselectionThresholdQ,

threshX-LowQ                        ReselectionThresholdQ

}                                                                                               OPTIONAL,   -- Cond RSRQ

cellReselectionPriority             CellReselectionPriority                                     OPTIONAL,   -- Need R

cellReselectionSubPriority          CellReselectionSubPriority                                  OPTIONAL,   -- Need R

q-OffsetFreq                        Q-OffsetRange                                               DEFAULT dB0,

interFreqNeighCellList              InterFreqNeighCellList                                      OPTIONAL,   -- Need R

interFreqExcludedCellList           InterFreqExcludedCellList                                   OPTIONAL,   -- Need R

...

}

InterFreqCarrierFreqInfo-v1610 ::=  SEQUENCE {

interFreqNeighCellList-v1610        InterFreqNeighCellList-v1610                                OPTIONAL,    -- Need R

smtc2-LP-r16                        SSB-MTC2-LP-r16                                             OPTIONAL,    -- Need R

interFreqAllowedCellList-r16        InterFreqAllowedCellList-r16                                OPTIONAL,    -- Cond SharedSpectrum2

ssb-PositionQCL-Common-r16          SSB-PositionQCL-Relation-r16                                OPTIONAL,    -- Cond SharedSpectrum

interFreqCAG-CellList-r16           SEQUENCE (SIZE (1..maxPLMN)) OF InterFreqCAG-CellListPerPLMN-r16   OPTIONAL     -- Need R

}

InterFreqCarrierFreqInfo-v1700 ::=  SEQUENCE {

interFreqNeighHSDN-CellList-r17     InterFreqNeighHSDN-CellList-r17                             OPTIONAL,    -- Need R

highSpeedMeasInterFreq-r17          ENUMERATED {true}                                           OPTIONAL,    -- Need R

redCapAccessAllowed-r17             ENUMERATED {true}                                           OPTIONAL,    -- Need R

ssb-PositionQCL-Common-r17          SSB-PositionQCL-Relation-r17                                OPTIONAL,    -- Cond SharedSpectrum

interFreqNeighCellList-v1710        InterFreqNeighCellList-v1710                                OPTIONAL     -- Cond SharedSpectrum2

}

InterFreqCarrierFreqInfo-v1720 ::=  SEQUENCE {

smtc4list-r17                       SSB-MTC4List-r17                                            OPTIONAL     -- Need R

}

InterFreqCarrierFreqInfo-v1730 ::=  SEQUENCE {

channelAccessMode2-r17              ENUMERATED {enabled}                                        OPTIONAL     -- Need R

}

InterFreqNeighHSDN-CellList-r17 ::= SEQUENCE (SIZE (1..maxCellInter)) OF PCI-Range

InterFreqNeighCellList ::=          SEQUENCE (SIZE (1..maxCellInter)) OF InterFreqNeighCellInfo

InterFreqNeighCellList-v1610 ::=    SEQUENCE (SIZE (1..maxCellInter)) OF InterFreqNeighCellInfo-v1610

InterFreqNeighCellList-v1710 ::=    SEQUENCE (SIZE (1..maxCellInter)) OF InterFreqNeighCellInfo-v1710

InterFreqNeighCellInfo ::=          SEQUENCE {

physCellId                          PhysCellId,

q-OffsetCell                        Q-OffsetRange,

q-RxLevMinOffsetCell                INTEGER (1..8)                                              OPTIONAL,   -- Need R

q-RxLevMinOffsetCellSUL             INTEGER (1..8)                                              OPTIONAL,   -- Need R

q-QualMinOffsetCell                 INTEGER (1..8)                                              OPTIONAL,   -- Need R

...

}

InterFreqNeighCellInfo-v1610 ::=    SEQUENCE {

ssb-PositionQCL-r16                 SSB-PositionQCL-Relation-r16                                OPTIONAL    -- Cond SharedSpectrum2

}

InterFreqNeighCellInfo-v1710 ::=    SEQUENCE {

ssb-PositionQCL-r17                 SSB-PositionQCL-Relation-r17                                OPTIONAL    -- Cond SharedSpectrum2

}

InterFreqExcludedCellList ::=       SEQUENCE (SIZE (1..maxCellExcluded)) OF PCI-Range

InterFreqAllowedCellList-r16 ::=    SEQUENCE (SIZE (1..maxCellAllowed)) OF PCI-Range

InterFreqCAG-CellListPerPLMN-r16 ::= SEQUENCE {

plmn-IdentityIndex-r16              INTEGER (1..maxPLMN),

cag-CellList-r16                    SEQUENCE (SIZE (1..maxCAG-Cell-r16)) OF PCI-Range

}

M-IAB-TargetCellReselection    ::= SEQUNECE {

targetCellPCI						INTEGER (1..1007)

targetFrequencey					ARFCN-ValueNR

}

-- TAG-SIB4-STOP

-- ASN1STOP

==================================================================================

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	Problem 1 and 2 acknowledged by RAN2 for IDLE/INACTIVE UEs can be handled by network implementation/configuration.

Observation 2	Optimizing the cell (re)selection procedure for all possible scenarios that involve UE(s) and a mobile IAB cell may lead to address a list of problems that is much longer to those ones already investigated by RAN2.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For the DU migration, target cell ID and frequencies for cell reselection that belongs to the new logical DU are provided via SIB for UEs in RRC_IDLE and RRC_INACTIVE.

# 4	References

R2-2301613, Idle mode mobility for mIAB-MT and UE, and miscellaneous issue, LG Electronics, RAN2#121, February 2023

R2-2300437, Mobility Enhancement of mobile IAB-node and served UEs, Intel Corporation, RAN2#121, February 2023