3GPP TSG-RAN WG2 Meeting #123                               R2-2308453

Toulouse, France, Aug 21-25, 2023

Source: 			ZTE Corporation

Title: 	Support for BWP operation without restriction (Option C)

Agenda item:		7.25.1

Document for: 	Discussion and decision

# Introduction

At RAN#99 meeting, a new WID on completing the specification for supporting BWP operation without CD-SSB (FG 6-1a) was approved. According to the WID, one objective is to specify BM/RLM/BFD based on NCD-SSB (called “Option C”).

- For Option C - Specify support of BM/RLM/BFD based on NCD-SSB within active BWP for non-RedCap UEs (RAN4, RAN2, RAN1)

However, during the discussion in RAN1/2/4, companies showed different views on Option C. In addition, an LS [1] was sent from RAN4 to RAN, suggesting to clarify whether Option C also includes RRM measurements based on NCD-SSB. Based on the discussion at RAN#100, companies agreed that intra-freq RRM measurement is also applicable in Option C and the WID was updated in [2]. In this contribution, we further discuss the components of Option C and provide our views.

# Discussion

During previous RAN plenaries, multiple options were discussed for FG 6-1a capable UEs, in order to support BWP operation without CD-SSB. For Option B-1-1 and Option B-1-2, the UE still performs BM/RLM/BFD operations based on CD-SSB. For Option C, the intention is to reuse the NCD-SSB (which will be deployed for RedCap UEs) to facilitate non-RedCap UEs to perform BM/RLM/BFD, so that the network is able to configure the UE to operate in a BWP that does not contain CD-SSB and data interruption can be avoided.

NCD-SSB concept was defined in Rel-15, the definition is “an SSB that not associated with RMSI (SIB1)”. In Rel-17 RedCap, considering RedCap UEs only support limited bandwidth (20MHz for FR1 and 100MHz for FR2), the usage of NCD-SSB has been extended, so the network is able to schedule RedCap UEs in different frequency locations of a cell.

In general, for RedCap UEs, once NCD-SSB is configured for a BWP, the NCD-SSB can be used for:

- Beam management(BM) in this BWP;

- Radio link monitoring(RLM) in this BWP;

- Beam failure detection(BFD) in this BWP;

- DL synchronization when handover to this BWP;

- Serving cell RRM measurements in this BWP;

- QCL source in this BWP;

- RACH resource selection in this BWP

TS 38.331 Definition of NCD-SSB configuration for RedCap UEs nonCellDefiningSSB If configured, the RedCap UE operating in this BWP uses this SSB for the purposes for which it would otherwise have used the CD-SSB of the serving cell (e.g. obtaining sync, measurements, RLM). Furthermore, other parts of the BWP configuration that refer to an SSB (e.g. the "SSB" configured in the QCL-Info IE; the "ssb-Index" configured in the RadioLinkMonitoringRS; CFRA-SSB-Resource; PRACH-ResourceDedicatedBFR) refer implicitily to this NCD-SSB. The NCD-SSB has the same values for the properties (e.g., ssb-PositionsInBurst, PCI, ssb-periodicity, ssb-PBCH-BlockPower) of the corresponding CD-SSB apart from the values of the properties configured in the NonCellDefiningSSB-r17 IE.. TS 38.300 section 16.13.5 BWP operation for RedCap UEs A RedCap UE may be configured with multiple NCD-SSBs provided that each BWP is configured with at most one SSB. NCD-SSB may be configured for a RedCap UE in RRC_CONNECTED to perform RLM, BFD, and RRM measurements and RA resource selection when the active BWP does not contain CD-SSB.

Observation 1	In addition to BM/RLM/BFD, for RedCap UEs, NCD-SSB can also be used for DL synchronization, RRM, QCL and RACH resource selection.

Originally, for Option C for non-RedCap UEs, only BM, RLM and BFD support is mentioned for the NCD-SSB case. This is because these are the key functions that were discussed in RAN#99. After RAN#100, it is confirmed that intra-frequency measurements (including serving cell measurements) based on NCD-SSB is supported for Option C. In addition, in the last RAN1 meeting, RAN1 has just agreed that NCD-SSB can also be used as QCL source for other reference signal (see below the UE feature agreed in RAN1#113).

| 53. NR_BWP_wor | 53-3 | Support RLM/BM/BFD measurements based on NCD-SSB within active BWP | 1. UE performs RLM/BM/BFD measurements based on NCD-SSB, where the NCD-SSB is within the active DL BWP. 2. Bandwidth of UE-specific RRC configured BWP may not include bandwidth of the CORESET#0 (if CORESET#0 is present) and CD-SSB for PCell/PSCell (if configured) and bandwidth of the UE-specific RRC configured BWP may not include CD-SSB for Scell 3. NCD-SSB within the active DL BWP can be used as the QCL source for other reference signal. |  | Yes | n/a | UE cannot support RLM/BM/BFD measurements based on NCD-SSB within active BWP | Per band | No | No | n/a | This FG is not applicable to RedCap UEs. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Therefore, based on the discussion in RAN1 and RAN plenary, for non-RedCap UE, the NCD-SSB can already be used for BM/RLM/BFD/RRM. However, it is still unclear whether the NCD-SSB can be used for DL synchronization when handover to a BWP that contains only NCD-SSB, and whether it can be used for RACH resource selection. In our view, if we do not support these use cases, then the performance will be impacted. At the same time, the specification effort (at least in RAN2) will be increased as the specification for non-RedCap UEs supporting NCD-SSB will require a specific description, instead of reusing the one already available for RedCap UEs.

For example, if DL synchronization based on NCD-SSB is not supported, it means that during handover, the configured first active BWP in the target cell must contain the CD-SSB; and only after the handover succeeds, the network can switch the UE to a BWP that does not contain CD-SSB (but contains NCD-SSB). This extremely limits the network implementation.

For RACH resource selection, if the UE cannot select RACH resource based on the RSRP of NCD-SSB, it means the UE is forced to switch to CD-SSB before sending RACH preamble, or the network will be forced to not configure RACH in this BWP, both of them will cause additional RACH delay.

Therefore, to ensure the performance of Option C, we suggest to inherit all NCD-SSB based functions for non-RedCap UEs, including not only BM/RLM/BFD, RRM measurements and QCL source, but also DL synchronization and RACH resource selection.

Proposal 1	RAN2 confirms that for Option C, all the NCD-SSB based functions defined for RedCap UEs are inherited for non-RedCap UEs, i.e. including not only BM/RLM/BFD, RRM measurements and QCL source, but also DL synchronization and RACH resource selection.

If proposal 1 can be agreed, then for the specification change, the nonCellDefiningSSB-r17 and servingCellMO-r17 IEs defined in BWP-DownlinkDedicated can be completely reused, only the field description and text procedure need to be updated.

BWP-DownlinkDedicated information element

-- ASN1START

-- TAG-BWP-DOWNLINKDEDICATED-START

BWP-DownlinkDedicated ::=     SEQUENCE {

pdcch-Config                 SetupRelease { PDCCH-Config }     OPTIONAL,   -- Need M

pdsch-Config                 SetupRelease { PDSCH-Config }     OPTIONAL,   -- Need M

sps-Config                   SetupRelease { SPS-Config }       OPTIONAL,   -- Need M

radioLinkMonitoringConfig    SetupRelease { RadioLinkMonitoringConfig }  OPTIONAL,   -- Need M

...,

[[

sps-ConfigToAddModList-r16          SPS-ConfigToAddModList-r16    OPTIONAL,   -- Need N

sps-ConfigToReleaseList-r16         SPS-ConfigToReleaseList-r16   OPTIONAL,   -- Need N

sps-ConfigDeactivationStateList-r16 SPS-ConfigDeactivationStateList-r16   OPTIONAL,   -- Need R

beamFailureRecoverySCellConfig-r16  SetupRelease {BeamFailureRecoveryRSConfig-r16}                    OPTIONAL,   -- Cond SCellOnly

sl-PDCCH-Config-r16                 SetupRelease { PDCCH-Config }       OPTIONAL,   -- Need M

sl-V2X-PDCCH-Config-r16             SetupRelease { PDCCH-Config }       OPTIONAL    -- Need M

]],

[[

preConfGapStatus-r17                BIT STRING (SIZE (maxNrofGapId-r17))      OPTIONAL,   -- Cond PreConfigMG

beamFailureRecoverySpCellConfig-r17 SetupRelease { BeamFailureRecoveryRSConfig-r16}                   OPTIONAL,   -- Cond SpCellOnly

harq-FeedbackEnablingforSPSactive-r17 BOOLEAN                           OPTIONAL,   -- Need R

cfr-ConfigMulticast-r17             SetupRelease { CFR-ConfigMulticast-r17 }                          OPTIONAL,   -- Need M

dl-PPW-PreConfigToAddModList-r17    DL-PPW-PreConfigToAddModList-r17    OPTIONAL,   -- Need N

dl-PPW-PreConfigToReleaseList-r17   DL-PPW-PreConfigToReleaseList-r17   OPTIONAL,   -- Need N

nonCellDefiningSSB-r17              NonCellDefiningSSB-r17              OPTIONAL,   -- Need R

servingCellMO-r17                   MeasObjectId           OPTIONAL -- Cond MeasObject-NCD-SSB

]]

}

SPS-ConfigToAddModList-r16 ::=          SEQUENCE (SIZE (1..maxNrofSPS-Config-r16)) OF SPS-Config

SPS-ConfigToReleaseList-r16 ::=         SEQUENCE (SIZE (1..maxNrofSPS-Config-r16)) OF SPS-ConfigIndex-r16

SPS-ConfigDeactivationState-r16 ::=     SEQUENCE (SIZE (1..maxNrofSPS-Config-r16)) OF SPS-ConfigIndex-r16

SPS-ConfigDeactivationStateList-r16 ::= SEQUENCE (SIZE (1..maxNrofSPS-DeactivationState)) OF SPS-ConfigDeactivationState-r16

DL-PPW-PreConfigToAddModList-r17 ::=    SEQUENCE (SIZE (1..maxNrofPPW-Config-r17)) OF DL-PPW-PreConfig-r17

DL-PPW-PreConfigToReleaseList-r17 ::=   SEQUENCE (SIZE (1..maxNrofPPW-Config-r17)) OF DL-PPW-ID-r17

-- TAG-BWP-DOWNLINKDEDICATED-STOP

-- ASN1STOP

Proposal 2	Reuse existing nonCellDedefiningSSB-r17 and servingCellMO-r17 fields for configuring Option C for non-RedCap UE.

The CRs to TS 38.300 and TS 38.331 are provided in [3][4].

Proposal 3	Agree the CRs in [3][4].

# Conclusion

In this contribution, proposals and observations are:

Observation 1	In addition to BM/RLM/BFD, for RedCap UEs, NCD-SSB can also be used for DL synchronization, RRM, QCL and RACH resource selection.

Proposal 1	RAN2 confirms that for Option C, all the NCD-SSB based functions defined for RedCap UEs are inherited for non-RedCap UEs, i.e. including not only BM/RLM/BFD, RRM measurements and QCL source, but also DL synchronization and RACH resource selection.

Proposal 2	Reuse existing nonCellDedefiningSSB-r17 and servingCellMO-r17 fields for configuring Option C for non-RedCap UE.

Proposal 3	Agree the CRs in [3][4].

# Reference

[1] RP-230847	LS on L3 measurements for the options for BWP operation without restriction (R4-2310177; to: RAN; cc: -; contact: vivo)

[2] RP-231486	Complete the specification support for BandWidth Part operation without restriction in NR, Vodafone, vivo

[3] R2-2308454	Support for BWP operation without restriction	ZTE Corporation, Sanechips	CR	Rel-18	38.300	17.5.0	0665	1	B

[4] R2-2308455	Support for BWP operation without restriction	ZTE Corporation, Sanechips	CR	Rel-18	38.331	17.5.0	4057	1	B