3GPP TSG-RAN2#123	R2-2308460

Toulouse, France, August 21-25, 2023

Agenda item:		7.22.2

Source:		Lenovo

Title:		 LP-WUS in RRC Idle/ Inactive Mode

Document for:		Discussion and Decision

- Introduction

Currently, the UE’s Main Radio (MR) needs to periodically wake up once per DRX cycle, which dominates the power consumption in periods with no signaling or data traffic. If the UE’s MR can wake up only when they are triggered (e.g. paging), power consumption could be dramatically reduced. This can be achieved by using a wake-up signal to trigger the main radio and a separate receiver which has the ability to monitor wake-up signal with ultra-low power consumption. Main radio works for data transmission and reception, which can be turned off or set to deep sleep unless it is turned on [1]. The following agreements have been made at the RAN2 #122 meeting:

Agreements

- RAN2 expect that different coverage LR/MR may have RAN2 impact, e.g., UE need to stop using LP-WUS when moving out of LR coverage, other aspects FFS. What to cover (if anything) in TS 38.304 is FFS.

- For UE in RRC_IDLE/RRC_INACTIVE state, it is FFS to what extent the network is or need to be aware of which receiver the UE uses MR/LR or both (for paging reception etc). A potential drawback of not knowing could be increased LP WUS load, a potential drawback of awareness is increased signalling.

- RAN2 assumes that UE uses LP WUS when pre-configured condition(s) are fulfilled. 
(Other control methods not precluded)

- RAN2 assumes that using subgrouping for LP-WUS could be beneficial to reduce false alarms rate (depend on L1 capacity to carry payload).

This contribution looks at the possible RRC Idle/ Inactive procedures for an LP-WUR enabled UE including entry and exit conditions for LR activation, network awareness options about the UE’s active radio and possible paging enhancements.

- Discussion

  - General

As mentioned above, it has been agreed in the previous meeting that RAN2 can expect LR and MR to have different coverage, and the UE will use LP-WUS when some preconfigured condition(s) are fulfilled. RAN2 should discuss the possible entry and exit conditions for the UE to use LP-WUR, as well as how these conditions are configured. RAN2 needs to also consider what the MR behavior will be when the LR is in active reception status.

One possible solution to determine the activation/ deactivation of the LR is by measuring the power with which the SSB is received. A new RSRP threshold (configurable by the network) can be introduced for determining whether the UE’s LR is in active reception status or not. If the UE receives the SSB with a power higher than this threshold, then the UE’s LR is activated else the UE continues to use the MR. While the UE is within LR coverage, and the UE is in IDLE Mode, the UE may continue to use the LR once activated and allow the MR to stay in ultra-deep sleep. The LR may then be deactivated when the UE moves out of the LR coverage area which can be determined by the SSB power, i.e., if the SSB is received with a power lower than the threshold then LR may be deactivated, and the UE can start using the MR.

RAN2 to discuss and evaluate the use of a RSRP power threshold for activation/ deactivation of the LR.

  - Network Awareness

When the UE is in RRC_IDLE/ RRC_INACTIVE Mode, the network is unaware of the UE’s active radio status. This is a viable option when the UE only consists of one radio. In the case where the UE consists of two radios (LR and MR), network awareness becomes more crucial. For example, a radio network may page the UE differently depending on which radio is currently listening at the UE side. Or if the MR of the UE is asleep, and if the UE completely goes out of the LP-WUR coverage without informing the gNB, the UE will lose connectivity with the network and would no longer be reachable. This may be more aggravated when the UE is close to the LR coverage boundary and may consistently move in and out of the LR coverage. Hence, there needs to be some level of understanding between the UE and network about which radio is in active reception status and where the UE may be paged.

- There needs to be some level of understanding between the UE and network about which radio is in active reception status.

According to our study, there are two possible alternatives to achieve network awareness about the UE’s active radio in RRC_IDLE/ RRC_INACTIVE Modes. One possible way to notify the gNB of the UE’s active radio is by means of some explicit indication by the UE to the network. This could be done by means of some enhanced RACH procedure, for example (e.g., an Active Radio Notification RACH similar to the existing On-Demand SI Request RACH). Network Awareness enables better resource efficiency and reduced power consumption by the network.

Another possible alternative is for the network to configure the UE behavior such that the UE need not explicitly notify the network of its active radio. For example, the network may configure separate UE LR and MR Paging Occasions (POs). The total POs for the UE remains same as in the legacy behavior. This ensures that the network knows exactly where the UE may be paged at a given instant, without having to page UEs on any additional occasions. With this alternative, the UE need not constantly signal the network of its active radio and the MR can be configured to continue sleeping if the LR is in active reception status. Table 1 summarizes the pros and cons of the two alternatives.

Table 1: Alternatives for Network Awareness

| Alternative | Methodology | Advantages | Disadvantages |
| --- | --- | --- | --- |
| - Explicit Indication | Signaling an ‘Active Radio Notification’ to the network, e.g., by means of an enhanced RACH procedure. | - Better resource efficiency for the network. - Reduced power consumption by the gNB. | - Requires UE to do some additional signalling to the network. - New Network and UE behaviour. |
| - Implicit Awareness by Network Configuration | Network configuration of separate POs for LR and MR of the UE. | - Better power saving by the UE due to reduced signalling. - No additional signalling by the UE to the network. | - New Network and UE behaviour. |

Hence, RAN2 should discuss possible ways to maintain awareness in the network about the UE’s active radio to prevent loss of connectivity as well as the frequent wake-up of MR.

RAN2 to discuss possible network awareness options:

- Alt 1: Enable an explicit indication to the network about the UE’s active radio.

- Alt 2: Configuration of the UE behaviour by the network that avoids the need for an explicit indication.

  - Paging Enhancements

For an LR capable UE, the network may page the UE either on the MR or the LR depending on the active radio in the UE. In RRC_IDLE Mode, the UE’s current active radio is unknown to non-serving cells. This may lead to some additional discrepancies during paging if the active radio information of the UE is not uniform across different gNBs within a Registration Area/ RAN Notification Area. The serving cell and non-serving cells may page the UE non-uniformly leading to wasted resources. Hence, there needs to be some solution to ensure uniformity of radio information amongst all cells of a Registration Area/ RAN Notification Area.

There are two alternatives to prevent the issue of radio information discrepancy within the Registration Area/ RAN Notification Area of a UE. In the first alternative, the gNBs always page the UE on both radios simultaneously. This solution avoids the need for the UE to inform gNB of its active radio and also avoids the need for any radio information to be updated amongst other gNBs. But this has a major drawback of wastage of resources and additional power consumption by the network to transmit two paging indications to each UE, as well as additional signaling overhead due to the redundant paging indications.

In the second alternative, the surrounding gNBs are made aware of the UE’s most recent active radio information by means of some information update, for example. This explicit signaling ensures that all cells within the Registration Area/ RAN Notification Area maintain updated information about a UE’s active radio at all times, allowing for the UE to be paged in a resource efficient manner. The power saving gain is also maximized on the network as well as UE side when only one radio is active to receive paging at a time. Hence, RAN2 should discuss possible solutions for maintaining network awareness across different gNBs within a Registration Area/ RAN Notification Area, and in this contribution, we propose to use an explicit signaling to indicate the most recent radio information with surrounding cells.

- The most recent radio information must be uniform within the gNBs of a Registration Area/ RAN Notification Area.

- Transmitting a paging indication on both LR and MR simultaneously is a waste of resources and reduces the power saving gain of the network and UE.

RAN2 to discuss and evaluate information sharing amongst network nodes within UE’s Registration Area/ RAN Notification Area regarding UE’s current radio in use.

# Conclusion

This document discusses some procedures for a LR capable UE in the RRC_IDLE/ RRC_INACTIVE Mode. Following are the proposals and observations made in this document:

- Proposal 1: RAN2 to discuss and evaluate the use of a RSRP power threshold for activation/ deactivation of the LR.

- Observation 1: There needs to be some level of understanding between the UE and network about which radio is in active reception status.

- Proposal 2: RAN2 to discuss possible network awareness options:

- Alt 1: Enable an explicit indication to the network about the UE’s active radio.

- Alt 2: Configuration of the UE behaviour by the network that avoids the need for an explicit indication.

- Observation 2: The most recent radio information must be uniform within the gNBs of a Registration Area/ RAN Notification Area.

- Observation 3: Transmitting a paging indication on both LR and MR simultaneously is a waste of resources and reduces the power saving gain of the network and UE.

- Proposal 3: RAN2 to discuss and evaluate information sharing amongst network nodes within UE’s Registration Area/ RAN Notification Area regarding UE’s current radio in use.

# References

[1] 3GPP TR 38.869 v0.2.0: Study on low-power wake up signal and receiver for NR (Release 18)