3GPP TSG-RAN WG2 #123	Tdoc R2-2308467

Toulouse, France, August 21st-25th, 2023

Agenda Item:	7.8.3

Source:	Ericsson

Title:	UAV flight path reporting

Document for:	Discussion, Decision

# Introduction

The WID for supporting UAV in NR [1] contains an objective on enhancements to measurement reports, including flight path reporting.

The following agreement was made in the last meeting.

Agreements RAN2#122 - The network can configure the UE to trigger a flightpath update notification based on a configured delta time (when timestamp is configured to be reported) or distance configuration.

In this paper we discuss the remaining open issues for flight path reporting.

# Flight path reporting

## Triggering of Flight Path Update

The network can configure the UE to trigger a flightpath update notification based on a configured delta time (when timestamp is configured to be reported) or distance configuration.

In the previous meeting, it was agreed as shown above, the update notification is based on a configured delta time or distance configuration. Further clarification is necessary to the delta time and distance configuration as provided below.

We discuss these codepoints individually.

- Distance Configuration

Figure 1: Distance Configuration

As shown in Figure 1, the network can configure a distance threshold (for example) δx,y,z such that if the (distance) content of the waypoint is more than the configured threshold δx,y,z, this would trigger a flight path update. For example, as shown in Figure 1, if the wp3, wp4 and wp5 have changed more than the threshold δx,y,z, this triggers a flight path update indication.

- UE reports the flight path update when it detects a change in 3-D location of the waypoint exceeding a distance threshold configured by the network. FFS distance threshold for different 3-D location representations.

- Timestamp

Figure 2: Delta time

For the delta time (timestamp), a similar threshold can be configured. For example, as shown in Figure 2, if the timestamps of wp3, wp4 and wp5 have changed and greater than the configured threshold δt, this can trigger a flight path update.

- UE reports the flight path update when it detects a change in the timestamp of the waypoint exceeding a time duration threshold as configured by the network.

The intention of the flight path update is to indicate to the network of significant changes in the initial/original flight path.

An update to the flight path should be triggered only if there are actionable (at the network) changes to the initial flight path.

There are two issues from the agreed distance/time configurations which would not help the above observation.

- The network might not be interested in all timestamp-based updates.

- Likewise, the network might also not be interested to be notified of a single waypoint satisfying the time/distance thresholds and such updates might lead to frequent UL signaling.

For issue 2, it would help that the network can also configure the number of waypoints which should satisfy the distance/time threshold configuration to trigger a flight path update.

- In addition to the distance/time thresholds, network configures the number of waypoints which satisfy the distance/time thresholds to trigger a flight path update.

For issue 1, one way to circumvent this issue is to provide additional information in the indication of the flight path update (will be discussed in Section 2.2) to indicate ‘what’ triggered the flight path update.

## Indication of Flight Path Update

It was agreed in RAN2#121bis that UAI message is used to send the flight path update indication and that, as a baseline, a single indication can be used for both initial and updated flight path. As explained in Section 2.1 about the relevance of providing such flight path updates, a relevant issue is whether other types of information, such as cause of the flight path update should be included.

In our view, besides indicating that a flight path update is available, it would be beneficial to also include a codepoint in the flight path update indicator to indicate whether it is triggered by changes related to distance or timestamps or both.

The gNB with this extra information can decide whether it is worth polling the UAV UE. For example, if the network is only interested in knowing changes due to UE’s distance threshold (δd), the network may just ignore a flight path update indication caused by changes in timestamps.

Based on the additional information, the network can poll the UAV UE for flight path updates only when it is interested in the information provided by the UE.

This is useful since all UL traffic from the UAV is potential interference to other types of traffic and the size of flight path information can potentially be quite large.

- Include codepoints along with flight path update indicator to indicate the triggering condition(s) for the flight path update.

## Time Stamp

In our view, besides the baseline contents like in Rel-15, other alternatives or additions can be specified to improve the usability of the time and location information included in the flight path report. For example, instead of reporting a single timestamp, the UE can report a time window which indicates a period that the UAV is expected to be in or near a certain location, e.g., for a mission such as inspecting a construction site or part of a power grid.

Time stamp as UTC time does not well address the key point of GSMA of MNOs to plan the coverage and service reliability to ensure BVLOS flights.

The time window can be expressed using a validity timer to allow for flexibility of signalling. Similarly, instead of using 3-D waypoints, one can consider the notion of area or volume or 3-D zone.

- To increase the informativeness of flight path reporting, include hysteresis, or time window to the time stamp of a UTC waypoint.

## Waypoint Information

Additionally, some other types of information related to the flight path and operating condition of the UAV can also be reported, either in the flight path report or in a separate UE report. For example, information about default/emergency landing spots could be used by the network to secure good connectivity to those spots.

This indication about default/emergency landing spots is related to the final waypoint indication. It would be beneficial for the network to understand the initial and final waypoints to get a better idea of the flight path. In which case, for default/emergency landing spots, the UAV UE can just indicate a single (final) waypoint in the flight path update.

- Introduce the notion of an initial and final waypoint in the flight path report.

# Conclusion

In the previous sections we made the following observations:

Observation 1	An update to the flight path should be triggered only if there are actionable (at the network) changes to the initial flight path.

Observation 2	Based on the additional information, the network can poll the UAV UE for flight path updates only when it is interested in the information provided by the UE.

Observation 3	Time stamp as UTC time does not well address the key point of GSMA of MNOs to plan the coverage and service reliability to ensure BVLOS flights.

Based on the discussion in the previous sections we propose the following:

Proposal 1	UE reports the flight path update when it detects a change in 3-D location of the waypoint exceeding a distance threshold configured by the network. FFS distance threshold for different 3-D location representations.

Proposal 2	UE reports the flight path update when it detects a change in the timestamp of the waypoint exceeding a time duration threshold as configured by the network.

Proposal 3	In addition to the distance/time thresholds, network configures the number of waypoints which satisfy the distance/time thresholds to trigger a flight path update.

Proposal 4	Include codepoints along with flight path update indicator to indicate the triggering condition(s) for the flight path update.

Proposal 5	To increase the informativeness of flight path reporting, include hysteresis, or time window to the time stamp of a UTC waypoint.

Proposal 6	Introduce the notion of an initial and final waypoint in the flight path report.

# References

RP-223545, WID update: NR support for UAV (Uncrewed Aerial Vehicles), RAN#98, Dec 2022

# Annex

List of agreements from the previous meetings:

Agreements RAN2#119-bis-e: - The time information reported as part of flight path plan is optional. UE includes time info, if configured by the network and available at the UE. FFS on flight path details (waypoints and what is time information). - Allow the flight path to be updated. FFS on the details. - FFS on reporting format and initial flight path reporting (i.e. what information to report and how) – next meeting - Continue to study height-depending scaling, triggering and combinations - As in LTE, as a baseline, events A3, A4 and A5 can be configured with the configured number of cells (numberofTriggeringCells).

Agreements RAN2#120:

- A waypoint is a planned location for the UE along the flight path and is described via the existing parameter type LocationCoordinates defined in TS 37.355.

- A timestamp provides the UTC time associated with estimated time of arrival to a waypoint as baseline.  FFS on granularity

- No requirements are placed on spatial distribution of waypoints

- A UE indicates whether flight plan information is available within the RRCReconfigurationComplete, RRCReestablishmentComplete, RRCResumeComplete, or RRCSetupComplete message.   Flight path reporting uses at the UE Information request/response procedure as baseline.

- UE indicates to the network a new flight path is available in the UE (whether it is initial or update). Then, reuse the normal request/response procedure of flight path report.

- UAI message can also be used to indicate the UE has flight path availability.

- FFS whether and what triggering conditions are specified for flight update.  FFS The maximum number of waypoints within flight path plan is left FFS.

Agreements RAN2#121:

- The granularity of flightpath timestamp is 1s.

- Timestamp in flightpath is encoded using AbsoluteTimeInfo-r16 IE

Agreements RAN2#121bis:

- Flightpath update indication in UAI is configurable by the network

- Maximum number of waypoints is set to 20 same as in LTE and number of waypoints is configurable by network as in LTE

- Flightpath information should be forwarded from source gNB to target gNB during handover. Send LS to RAN3 to check for feasibility [LS to RAN3 over email 307]

- As a baseline, we can consider a simple network control mechanisms (e.g. a threshold(s)) that controls triggering the flightpath update indication in UAI. FFS if new threshold or the kind of threshold(s)

- As a baseline, single indication is used for both initial and updated flightpath available (i.e. same flag is used for initial and updated flight path indication.  FFS if further differentiation is needed if we decide to have delta signaling