3GPP TSG-RAN WG2 #123	R2-2308469

Toulouse, France, 21st August – 25th August 2023

Agenda Item:	7.9.2

Source:	Ericsson

Title:	Discussion on Relay (re)selection and Discovery

Document for:	Discussion, Decision

# 1 Introduction

In the WID [1], relay (re-)selection is one common objective for L2 and L3 U2U relays. In addition, the following are (selected) left-over proposals from the previous meetings that were not treated due to lack of time.

Proposals:

Proposal 12b: If multiple suitable U2U relay candidates which meet both the AS-layer and higher layer criteria are available, it is up to remote UE implementation to choose a U2U relay UE.

Proposal 9: Remote UE can trigger U2U relay selection when PC5 RLF of the direct link between the remote UE and the peer remote UE is detected.

Proposal 10: Remote UE can trigger relay reselection if the link quality of the second hop between the relay UE and peer remote UE is blow a threshold even the link quality of the first hop is good. FFS for the content of the link quality indication of the second hop.

Proposal 11: RAN2 to discuss whether AS criterion is needed for switching back from indirect to direct link.

Proposal 13: RAN2 to discuss whether/how to handle the case that remote UE and its peer remote UE may select two different relay UEs simultaneously for communicating with each other. Send LS to SA2 if necessary .

We discuss the above agreement, leftover proposals and express our views accordingly in this contribution.

# 2 Discussion

## 2.1 U2U Relay (Re-)Selection

In subsequent sections, we use the terms source remote UE, U2U relay UE and destination remote UE where, the terms source and destination denote the direction of traffic from source remote UE to destination remote UE. This traffic can either be carried over a direct (PC5) link between the source remote UE and destination remote UE or carried over an indirect (PC5+PC5) link between the source remote UE and destination remote UE via a U2U relay UE.

### 2.1.1 Relay (Re-)selection

- During the relay selection procedure, it is possible that the source remote UE finds more than one suitable candidate U2U relay UE that satisfy the AS-layer and higher layer criterion to establish an indirect link with the destination remote UE. In which case, as in U2N relays, it is left to source remote UE’s implementation to choose the suitable candidate U2U relay UE.

During relay selection, it is left to source/destination remote UE’s implementation to choose a U2U relay UE to perform PC5 connection establishment when more than one suitable candidate U2U relay UEs meet the AS-layer and higher layer criterion.

- Further, during the relay reselection procedure, if the source remote UE can see both a suitable direct link and an indirect link to the destination remote UE. In which case, it is left to the source remote UE’s implementation to choose on which link to perform connection establishment.

During relay reselection, it is left to source remote UE’s implementation to choose either the direct link or an indirect link. No AS criteria needs to be defined.

### 2.1.2 Simplified gNB Control

RAN2 will strive to simplify the gNB involvement in U2U-relay-specific operation as compared to the U2N case.  Details are FFS, including whether some gNB control is needed for the in-coverage scenario and how/whether the gNB involvement can be simplified compared to U2N.

- From the agreement above, it is evident for U2U relays, RAN2 should strive to simplify gNB involvement as compared to U2N relays primarily because the traffic does not go through the network. In terms of the details as to how the gNB involvement can be simplified, this would be dependent on the agenda item being discussed.

- For the case of relay (re-)selection, as compared to U2N relays, in a U2U relay, switching between a direct and indirect link and the relay (re-)selection procedure to change the indirect link (U2U relay change) are completely based on UE procedures and there is no gNB assistance/involvement required.

For in-coverage scenarios, the U2U relay relay (re-)selection procedure are purely UE-based procedures with no gNB assistance/involvement required.

- For the measurement configurations for e.g., in terms of the (pre-)configured thresholds used to trigger the path switch or relay (re-)selection can be obtained from the cell-specific configurations/preconfigurations. For U2U relays, for in-coverage UEs, the gNB does not provide dedicated measurement configurations including for e.g., UE-specific reselection triggers.

For in-coverage UEs in RRC_CONNECTED state, the gNB does not provide a dedicated configuration for relay (re-)selection. Such configurations can be acquired from the cell-specific configuration or preconfiguration.

## 2.2 U2U Relay Discovery Procedure

### 2.2.1 Coexistence between U2N and U2U

Proposal 11: RAN2 to agree that the same dedicated discovery resource pool (defined in Rel-17), if configured, can be used for non-relay discovery, U2N relay discovery and/or U2U relay discovery as baseline. Can be revisited if any impact on co-existence between U2N/U2U.

Proposal 12: RAN2 to further discuss whether to support co-existence between U2N relays and U2U relays and the potential specification impact if any.

In the above proposal is from the previous meeting, the impacts of allowing the U2U relay discovery and U2N relay discovery in the same dedicated discovery pool are yet to be studied. On the UE-side, it would require a prioritization and differentiation at the AS-layer between U2U/U2N relays. Similarly, from the network’s perspective, if the same/different UEs support U2N/U2U relaying at least would need to perform prioritization between transmissions.

Coexistence would require further study in terms of differentiation and prioritization of services in the AS-layer.

Such co-existence mechanisms between U2N relays and U2U relays is out of scope of the WI and should not be pursued.

RAN2 does not pursue the co-existence between U2N relays and U2U relays in this release as it is not scope of the work item.

### 2.2.2 Configuration Restrictions in RRC_CONNECTED State

- It was agreed that the gNB involvement will be simplified for U2U relay specific operations as compared to the U2N case. As the U2U traffic does not go through the network, there is no advantage in the gNB providing UE-specific configurations to the in-coverage UE. In addition, we need to have unified network behaviour regardless of RRC state of the remote UE or U2U relay UE.

We need to have unified network involvement/behaviour regardless of RRC state of the remote UE/U2U relay.

- Taking this into account, for an in-coverage UE in RRC_CONNECTED, the gNB does not provide a dedicated configuration for the discovery. The in-coverage UE in any RRC state i.e., IDLE/INACTIVE/CONNECTED can use the cell-specific configuration/pre-configuration to perform the discovery procedure.

In UE-to-UE relaying, the gNB does not provide a dedicated discovery configuration for an in-coverage UE in RRC_CONNECTED and can rely on cell-specific configuration/preconfiguration.

# 3 Conclusion

In the previous sections we made the following observations:

Observation 1	Coexistence would require further study in terms of differentiation and prioritization of services in the AS-layer.

Observation 2	We need to have unified network involvement/behaviour regardless of RRC state of the remote UE/U2U relay.

Based on the discussion in the previous sections we propose the following:

Proposal 1	During relay selection, it is left to source/destination remote UE’s implementation to choose a U2U relay UE to perform PC5 connection establishment when more than one suitable candidate U2U relay UEs meet the AS-layer and higher layer criterion.

Proposal 2	During relay reselection, it is left to source remote UE’s implementation to choose either the direct link or an indirect link. No AS criteria needs to be defined.

Proposal 3	For in-coverage scenarios, the U2U relay relay (re-)selection procedure are purely UE-based procedures with no gNB assistance/involvement required.

Proposal 4	For in-coverage UEs in RRC_CONNECTED state, the gNB does not provide a dedicated configuration for relay (re-)selection. Such configurations can be acquired from the cell-specific configuration or preconfiguration.

Proposal 5	RAN2 does not pursue the co-existence between U2N relays and U2U relays in this release as it is not scope of the work item.

Proposal 6	In UE-to-UE relaying, the gNB does not provide a dedicated discovery configuration for an in-coverage UE in RRC_CONNECTED and can rely on cell-specific configuration/preconfiguration.

# 4 References

- S2-2393381, Reply LS on Differentiation of Layer2 ID and Coexistence of U2N/U2U

- R1-2304211, Reply LS on Comparison of SL-RSRP and SD-RSRP Measurements, RAN1#112-e

- R4-2306366, LS on Comparison of SL-RSRP and SD-RSRP Measurements, RAN4#106bis-e