3GPP TSG-RAN WG2 #123	R2-2308470

Toulouse, France, 21st August – 25th August 2023

Agenda Item:	7.9.2

Source:	Ericsson

Title:	Control Plane Procedures for Layer 2 UE-to-UE Relays

Document for:	Discussion, Decision

# 1	Introduction

The following agreements were made in the RAN#122 meeting on the CP aspects for U2U relays:

Agreements:

For the E2E SL-SRB configuration of U2U relay, specified PDCP configuration is used. FFS for the SRAP and PC5 RLC channel configuration for SL-SRB.

AS layer is responsible for QoS split in L2 U2U relay.

Relay UE is responsible for AS layer QoS split in L2 U2U relay.

For OOC U2U relay/remote UE, pre-configuration is used for the E2E SL-DRB and per-hop PC5 RLC channel configuration.

For RRC_IDLE/INACTIVE U2U relay/remote UE, SIB is used for the E2E SL-DRB and per-hop PC5 RLC channel configuration.

For the possible use of a short ID in U2U relay, RAN2 will downselect between the following options for identifying the source and destination remote UEs at the SRAP layer:

a)	Single ID, identifying the source and destination remote Ues

b)	Source ID and Destination ID

For the possible use of a short ID in U2U relay, the U2U relay UE performs the ID assignment. FFS if this ID should be assigned hop-by-hop or globally.

These agreements do not imply agreement to use a short ID.

In this paper we’ll discuss the Layer-2 specific aspects and RAN2 details for the control plane (CP) procedures, adaptation layer design and QoS handling based on the leftover proposals and SA2’s progress.

# 2	Discussion

The descriptions below are written in the context of a source remote (SRC) UE communicating with one or more destination (DST) remote UEs via a relay UE (U2U relay).

## 2.1 SRAP Layer Design

Long-ID vs Short-ID:

For the long ID versus short ID to be used in the SRAP header. Primarily, due to the security aspects when the L2 ID or part of the L2 ID (24-bit) is exposure at the SRAP layer as the header is unprotected. Further, the long ID cannot be easily extended for forward compatibility with multi-hop relaying.

The information in the SRAP layer header is unprotected, therefore including the long ID in the header poses a security risk. In addition, usage of the long-ID is not easily extendable to the multi-hop relaying scenario.

In addition, when using a long-ID, the only option for designing the SRAP header is to include both the SRC and DST ID in the header. This would result in a 48-bit (SRC ID + DST ID) SRAP header leading to unnecessary overhead especially for public safety applications where packet sizes are generally small.

A 48-bit SRAP header leads to unnecessary transmission overhead especially in cases of small transmission packet sizes.

As a result, we think there is no need to deviate from Rel-17 U2N relaying in terms of including the local ID in the SRAP header. It should be noted that the local ID need not be unique on a global level (in the context of the link), it would just need to be unique to the relay connection at a particular U2U relay. Hence, fewer bits in the local ID would not be a limiting factor. Further, short ID helps to keep the transmission overhead at a minimum.

Short ID is to be included in the SRAP header for SRC/DST remote UE identification.

Short-ID:

As agreed in the last meeting, for short ID in U2U relays, RAN2 will down select from the following options for identifying the SRC and DST UEs at the SRAP layer:

- Single ID, identifying the source and destination remote UEs.

- Source (SRC) ID and Destination (DST) ID

We believe including both the SRC/DST ID in the SRAP header is reasonable for easy processing at the U2U relays and can be easily extended for multi-hop relaying scenarios. Further, most companies agreed in the last meeting that the single ID would represent both the SRC/DST ID pair at the U2U relay. Hence, this might require additional signaling exchange between the UEs. Furthermore, there are little gains to be obtained from a transmission overhead perspective by configuring a single ID.

The Single ID might require additional signaling exchanges between the UEs and there are little gains to be obtained from a transmission overhead perspective.

- The short-ID should be unique for a SRC, and DST UE at the U2U relay. By assigning different values, it is possible to reconfigure only one link without affecting the other link. For example, if there is a change in the short-ID or RLF declared over one link, this will not affect the other link. But if the short-ID was common as in a single ID for the SRC/DST pair, then both the first and second hop links would have to be reconfigured to use the new short-ID.

Both source and destination short-IDs are included in the SRAP header for identifying the corresponding source and destination remote UEs.

- Global vs hop-by-hop allocation:

For the possible use of a short ID in U2U relay, the U2U relay UE performs the ID assignment. FFS if this ID should be assigned hop-by-hop or globally.

- It was already agreed in the last meeting that the U2U relay performs the short-ID assignment for the SRC/DST UEs with FFS on how this ID would be assigned i.e., hop-by-hop or global.

- For a single-hop scenario, there is no difference between hop-by-hop and global ID assignment. It is only in the case of a multi-hop scenario that this assignment will come into play. As a result, for Rel-18 and in the single hop scenario, there is no need to consider this aspect.

Deprioritize the discussion on hop-by-hop vs global assignment in this release.

## 2.2 QoS Handling

AS layer is responsible for QoS split in L2 U2U relay.

Relay UE is responsible for AS layer QoS split in L2 U2U relay.

From the agreements above, the U2U relay is responsible for the QoS split. From Rel-16 NR V2X, it is the SRC/DST UE which determines the SRC/DST side QoS parameters. However, to avoid that the split QoS requirements from the U2U relay are unable to satisfy the E2E QoS requirements, the SRC/DST UE may provide some assistance information to the U2U relay which takes this into account when configuring the QoS split.

SRC/DST UE can provide the U2U relay with assistance info to assist in splitting the QoS. FFS signaling used to send this information.

## 2.3 Multiplexing Over First Hop

Multiplexing of different destinations in the same RLC channel of the first hop is supported.

- From the source remote UEs perspective, the multiplexing of the data over the first hop can happen in two layers:

- RLC-layer Multiplexing (RLC-Multiplex): This is related to the above agreement i.e., the data from the SRAP-layer can be multiplexed into the same RLC-channel.

- MAC-layer Multiplexing (MAC-Multiplex): In addition, in the RAN2#121 meeting, there was also a discussion about multiplexing the data for different final destinations at the MAC-layer i.e., data from the different LCHs (different final destinations) can be multiplexed into the same grant.

- In general, for sidelink without relaying, it would make sense to multiplex the data from the different logical channels as they are intended for the same destination. However, for U2U relaying, this is not the case. The different RLC channels/LCHs are associated with different (final) destinations in the second hop.

- As a result, we believe there are issues when the source remote UE is always allowed to multiplex the data intended for different destinations over the first hop:

- For both RLC-Multiplex and MAC-Multiplex, as the different logical channels are associated with different (final) destination remote UEs, it is possible that low priority transmissions get a treatment above its indicated priority. Thereby degrading the performance of the other UEs in the system. In addition, as the source remote UE can communicate with multiple final destinations further exacerbating the problem.

- For both RLC-Multiplex and MAC-Multiplex, in mode-2, the selection window (sl-SelectionWindow-r16) is configured independently for each priority value (sl-Priority-r16). In which case, it is possible that the sl-SelectionWindow-r16 are different for different (final) destination remote UEs i.e., T1 for destination remote UE1, T2 for destination remote UE2 and T1 < T2. The source remote UE can select resources for transmission using T2 in which case multiplexing would of data could result in the PDB not being satisfied for one of the (final) destination remote UEs. The same is also applicable for the case when a high priority data arrives for a different (final) destination UEs.

- Based on our concerns above, we think RAN2 should discuss the issue of multiplexing of data associated with different (final) destination remote UEs. Although it is up to the source UE’s implementation whether to perform multiplexing, there should be some restrictions for when the UE is allowed to do so.

RAN2 to discuss restrictions for multiplexing data from different final destinations in the first hop.

## 2.4 End-to-End Connection Establishment

As mentioned above, SA2 has concluded the following procedures for connection establishment in L2 and L3 U2U relays [2]:

- Standalone Discovery Procedure based PC5 end-to-end unicast link establishment

- Integrated PC5 end-to-end unicast link establishment

Although SA2 has specified the solutions, the details for the L2 U2U relaying have not yet been captured and is up to RAN2 to finalize the details. In addition, RAN2 also needs to discuss the details of the sidelink relay adaptation protocol (SRAP) layer and handling of end-to-end QoS. Figure 1 below illustrates the RAN2 related CP procedures in L2 U2U relaying for link establishment based on the details as captured in [2].

Although SA2 has specified the solutions for standalone and integrated end-to-end unicast link establishment in general, the details for L2 U2U relaying have not yet been captured. RAN2 needs to finalize the details and in addition, study the aspects of SRAP and QoS handling.

- Standalone Discovery Procedure based PC5 end-to-end unicast link establishment

- Integrated PC5 end-to-end unicast link establishment

Figure 1. Connection Establishment Procedure for L2 U2U relays

The following are the steps involved in the end-to-end connection establishment procedure for both cases (a) and (b). The order of these steps can vary but the intention is to use the following as a starting point for discussions.

- With a standalone procedure, the SRC UE performs a discovery procedure (using model A/B) to find the DST UE. In the integrated procedure, the SRC UE initiates a direct communication request message (contents of which are decided by SA2) to find the DST UE.

- In the integrated procedure, the DST UE then performs the relay selection. Then, in both procedures, a PC5 link establishment/modification can be performed.

- In both procedures, the U2U relay can then configure the SRC UE and DST UE with the SRAP configuration along with the local ID(s).

- Subsequent messages for establishing an end-to-end unicast link are transmitted using the SRAP layer.

- Once the end-to-end unicast link is established, the per-hop bearer mapping with the option to perform QoS split can be performed.

RAN2 to consider Figure 1. as the baseline for L2 U2U relaying in PC5 end-to-end link establishment for cases (a) and (b).

- It was agreed in the last meeting that E2E SL-SRB configuration of U2U relays uses specified PDCP configuration with an FFS on SRAP and PC5 RLC channel configurations.

- For SL-SRB0, SRAP configuration, we should follow the Rel-17 U2N relaying principles that SRAP layer is not present for transmission of PC5-S messages over SL-SRB0. For the integrated discovery procedure, the PC5-S message is broadcast and may include the UE ID in the DCR message. For the standalone discovery case, a DCR message need not be sent as the hop-by-hop link establishment is already completed. As a result, in both cases, there is no need to include the SRAP layer for E2E SL-SRB0.

SRAP layer is not present over both hops for E2E SL-SRB0.

- For SL-SRBs apart from SL-SRB0, the SRAP layer is present over both hops. It was already agreed that the remote UE determines the egress RLC channel based on mapping from the E2E bearer ID to egress RLC channel, for a particular target remote UE over the first hop. The same should also be applicable for the U2U relay, where it should be able to determine the egress RLC channel.

For SL-SRBs apart from SL-SRB0, the SRAP layer is present over both hops.

- For the RLC configuration, like in U2N relaying, a fixed or default configuration is used. This can also be extended to SL-SRB1 and SL-SRB2.

Fixed or default RLC configuration is used for all E2E SL-SRBs.

- Given that the PDCP configurations are specified and the RLC configurations are fixed/default, the SRAP configuration can also be fixed or default as the mapping of the SL-SRB ID to the corresponding egress RLC channel is obvious.

Specified SRAP configuration can be used for all E2E SL-SRBs.

RAN2 will strive to simplify the gNB involvement in U2U-relay-specific operation as compared to the U2N case.  Details are FFS, including whether some gNB control is needed for the in-coverage scenario and how/whether the gNB involvement can be simplified compared to U2N.

- For the remote UE/U2U relay in RRC_CONNECTED mode, it is sufficient that the E2E SL-DRB and per-hop PC5 channel configuration is provided in the SIB. In addition, it was also agreed that the gNB involvement in U2U relays will be simplified as the traffic does not go through the network.

For RRC_CONNECTED U2U relay/remote UE, the SIB or pre-configuration is used for E2E SL-DRB and per-hop RLC channel configurations.

- In keeping with the principle of simplified gNB control, it can also be confirmed that the gNB will not be involved in the QoS split aspects and it is left up to the U2U relay to perform the split.

For U2U relaying, gNB is not involved in the QoS split procedure.

- There was also some discussion in the last meeting about differentiation of SRB/DRBs at the SRAP layer. If fixed/specified configurations are used for the RLC channels, the U2U relay can easily recognize from the ingress RLC channel whether data is a SL-SRB or SL-DRB signaling. Hence, there is no need to differentiate between SL-SRB/DRBs at the SRAP layer.

RAN2 does not pursue the differentiation of SL-SRB/DRBs at the SRAP layer.

# 3 Conclusion

In the previous sections we made the following observations:

Observation 1	The information in the SRAP layer header is unprotected, therefore including the long ID in the header poses a security risk. In addition, usage of the long-ID is not easily extendable to the multi-hop relaying scenario.

Observation 2	A 48-bit SRAP header leads to unnecessary transmission overhead especially in cases of small transmission packet sizes.

Observation 3	The Single ID might require additional signaling exchanges between the UEs and there are little gains to be obtained from a transmission overhead perspective.

Observation 4	Although SA2 has specified the solutions for standalone and integrated end-to-end unicast link establishment in general, the details for L2 U2U relaying have not yet been captured. RAN2 needs to finalize the details and in addition, study the aspects of SRAP and QoS handling.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Short ID is to be included in the SRAP header for SRC/DST remote UE identification.

Proposal 2	Both source and destination short-IDs are included in the SRAP header for identifying the corresponding source and destination remote UEs.

Proposal 3	Deprioritize the discussion on hop-by-hop vs global assignment in this release.

Proposal 4	SRC/DST UE can provide the U2U relay with assistance info to assist in splitting the QoS. FFS signaling used to send this information.

Proposal 5	RAN2 to discuss restrictions for multiplexing data from different final destinations in the first hop.

Proposal 6	RAN2 to consider Figure 1. as the baseline for L2 U2U relaying in PC5 end-to-end link establishment for cases (a) and (b).

Proposal 7	SRAP layer is not present over both hops for E2E SL-SRB0.

Proposal 8	For SL-SRBs apart from SL-SRB0, the SRAP layer is present over both hops.

Proposal 9	Fixed or default RLC configuration is used for all E2E SL-SRBs.

Proposal 10	Specified SRAP configuration can be used for all E2E SL-SRBs.

Proposal 11	For RRC_CONNECTED U2U relay/remote UE, the SIB or pre-configuration is used for E2E SL-DRB and per-hop RLC channel configurations.

Proposal 12	For U2U relaying, gNB is not involved in the QoS split procedure.

Proposal 13	RAN2 does not pursue the differentiation of SL-SRB/DRBs at the SRAP layer.

# 4 References

[1] S2-2305915, Reply LS on 5G ProSe Layer-2 UE-to-UE Relay QoS enforcement, SA2#156e, April 17-21, 2023

[2] TS 23.304, V18.0.0, “Proximity based Service (ProSe) in the 5G System (5GS) (Release 18)”