3GPP TSG-RAN WG2 #123	R2-2308471

Toulouse, France, 21st August – 25th August 2023

Agenda Item:	7.9.3

Source:	Ericsson

Title:	Discussion on Inter-gNB Service Continuity

Document for:	Discussion, Decision

# 1	Introduction

The following agreements were made in the RAN3#119 meeting for the decision on the target U2N relay UE in a d2i and i2i path switch scenario:

During direct to indirect and indirect to indirect path switch procedures, the source gNB sends a list of candidate relay UEs belonging to the same target cell in the HO REQ message.

At least Remote UE L2 ID and a list of candidate target relay UE IDs should be included in the XnAP HANDOVER REQUEST message.

We discuss here further details on the above proposals and express our views accordingly in this contribution

# 2	Discussion

## 2.1 Source vs Target gNB decision on Target U2N relay UE

The decision on the target U2N relay UE is applicable to the d2i and i2i scenarios. The Rel-17 reporting mechanism can be reused and no enhancements are necessary.

- Reuse the Rel-17 measurement reporting framework i.e., report consists of candidate U2N relay UE IDs, candidate U2N relay UE’s serving cell ID and the sidelink measurement quantity information.

## 2.2 Triggering the U2N Relay UE to CONN State

The triggering of the U2N relay UE into the CONN state is relevant for the d2i and i2i scenarios.

The Rel-17 remote UE oriented solution to trigger the target U2N relay UE to the CONNECTED state should also be applicable to the Rel-18 inter/intra-gNB scenarios as a baseline for single-path relay.  Other mechanisms are not excluded if an issue is found with the baseline.

- The above agreement was made in the last meeting with the addition of the highlighted text. There was also some discussion about using a paging-based solution in addition to the trigger from the remote UE to transit the U2N relay UE in IDLE/INACTIVE to the CONN state.

- Although we foresee some potential benefits from the paging-based solution especially for inter-gNB path switch scenarios, in terms of reducing the latency of the overall path switch procedure by potentially transitioning the target U2N relay UE to the CONN state before remote UE initiates the PC5 communication. The following is our analysis:

- For the target U2N relay UE in the INACTIVE state, the paging-based solution is already feasible based on Rel-17 specifications and with no changes required to the legacy paging mechanism. That is, if the gNB can retrieve the target U2N relay UE’s context, it can page (using RAN-paging) the target U2N relay UE to initiate the random-access procedure. This can also be done before the path switch command is sent to the remote UE and as a result, potentially, the target U2N relay UE is already in the CONNECTED state when the remote UE initiates PC5 communication for relaying.

- For the target U2N relay UE in the IDLE state, additional specification work is required to support this state, and this would involve cross-WG interactions (SA2/RAN3). Given the current workload, such solutions should be down prioritized.

- For inter-gNB d2i and i2i scenarios, the following should be agreed about the paging-based mechanism to transit the target U2N relay UE in IDLE/INACTIVE state to the CONNECTED state:

  - In RRC_INACTIVE state, RAN2 to confirm that it is up to gNB implementation to page the target U2N relay UE before the path switch command is sent to the remote UE.

  - In RRC_IDLE state, RAN2 to not pursue the enhancements required for the paging solution.

# 3 Conclusion

In the previous sections we made the following observations:

Based on the discussion in the previous sections we propose the following:

- Reuse the Rel-17 measurement reporting framework i.e., report consists of candidate U2N relay UE IDs, candidate U2N relay UE’s serving cell ID and the sidelink measurement quantity information.

- For inter-gNB d2i and i2i scenarios, the following should be agreed about the paging-based mechanism to transit the target U2N relay UE in IDLE/INACTIVE state to the CONNECTED state:

- In RRC_INACTIVE state, RAN2 to confirm that it is up to gNB implementation to page the target U2N relay UE before the path switch command is sent to the remote UE.

- In RRC_IDLE state, RAN2 to not pursue the enhancements required for the paging solution.

# 4 References