3GPP TSG-RAN WG2 #123	R2-2308472

Toulouse, France, 21st August – 25th August 2023

Agenda Item:	7.9.4

Source:	Ericsson

Title:	Discussion on Multipath Relays

Document for:	Discussion, Decision

# 1 Introduction

The following agreements were made in the last meeting.

Agreements:

For Scenario-1/2, not pursue remote UE notifying network upon reception of notification message indicating relay UE handover. FFS whether rely on network to release configuration of relay UE at remote UE before relay UE handover, or rely on remote UE to suspend the indirect path upon reception of notification message indicating relay UE handover.

For Scenario-1, use T304-like timer for direct path addition and change. FFS on expiry behavior.

For Scenario-1, use T420-like timer for indirect path addition and change. FFS on stop condition and expiry behavior.

FFS if these two timers are new or reuse the existing timers

WA: For Scenario-1/2, MP remote UE is configured with a single cell group, i.e., MCG, for the direct path, and SL configuration, for the indirect path

WA: For scenario 1, primary path of the split SRB1 and SRB2 is always configured on direct path. This does not preclude having the case where the UE switches the primary path to the indirect path for reporting after direct path failure.

For scenario 1, SRB1 and SRB2 are not decoupled in terms of support of non-split SRB on indirect path; i.e., if SRB1 can be supported on indirect path, so can SRB2.

In this contribution, we discuss the proposals above and present our views.

# 2 Discussion

## 2.1 Scenario-1

FFS if there are cases where the configuration of non-split SRBs over indirect path is useful

For the FFS, some companies argue that this can be left to network implementation, and we need not capture any restrictions on how to configure SRB1/SRB2. From the network’s point of view, following the principles of MR-DC, CP signaling over SRB is usually configured on the most robust and reliable path. In addition, the network can deliberate at any point of time which of the two paths (i.e., direct, and indirect path) is more reliable. As a result, the network will configure all the SRBs to be transmitted on that path either the direct or indirect path. Configuring the SRB1/SRB2 on different paths with one being more reliable than the other is not a sensible configuration and there is no scenario in which such a configuration is required.

- For Scenario-1, SRB1 and SRB2 are always configured on the same path.

On which bearer type should be configured for SRBs, we believe that the non-split SRBs should always be configured on the direct path. This is because the direct path will always be more robust and reliable as opposed to the indirect path. However, there are situations where the indirect path can be better for e.g., in cell-edge cases. In those scenarios, a split SRB with duplication can be configured for the UE.

- For Scenario-1, non-split SRB1/2 can only be configured over the direct path.

WA: For Scenario-1/2, MP remote UE is configured with a single cell group, i.e., MCG, for the direct path, and SL configuration, for the indirect path

The above WA was made as there was some ambiguity in the interpretation of the MCG in a multipath relay scenario. With the understanding that the MCG is only for the direct path and has no relationship to the indirect path, the above WA can be confirmed with this clarification.

- Confirm the WA, for Scenario-1/2, MP remote UE is configured with a single cell group i.e., MCG for the direct path, and SL configuration, for the indirect path.

WA: For scenario 1, primary path of the split SRB1 and SRB2 is always configured on direct path. This does not preclude having the case where the UE switches the primary path to the indirect path for reporting after direct path failure.

In the above WA, the primary path of the split SRB1 and SRB2 should always be configured on the more robust path i.e., the direct path in a multipath relay scenario. As a result, the above WA should be confirmed.

- Confirm the WA, for Scenario 1, primary path of the split SRB1 and SRB2 is always configured on direct path. This does not preclude having the case where the UE switches the primary path to the indirect path for reporting after direct path failure.

For Scenario-1, use T304-like timer for direct path addition and change. FFS on expiry behavior.

For Scenario-1, use T420-like timer for indirect path addition and change. FFS on stop condition and expiry behavior.

FFS if these two timers are new or reuse the existing timers

There was also a discussion in the last meeting about the T304-like and T420-like timers for direct path and indirect path addition/change respectively. The following are the definitions of T304 and T420 with the corresponding start/stop and expiry conditions.

| Timer | Start | Stop | At Expiry |
| --- | --- | --- | --- |
| T304 | Upon reception of RRCReconfiguration message including reconfigurationWithSync or upon conditional reconfiguration execution i.e., when applying a stored RRCReconfiguration message including reconfigurationWithSync. | Upon successful completion of random access on the corresponding SpCell For T304 of SCG, upon SCG release | For T304 of MCG, in case of the handover from NR or intra-NR handover, initiate the RRC re-establishment procedure; In case of handover to NR, perform the actions defined in the specifications applicable for the source RAT. If any DAPS bearer is configured and if there is no RLF in source PCell, initiate the failure information procedure. For T304 of SCG, inform network about the reconfiguration with sync failure by initiating the SCG failure information procedure as specified in 5.7.3. |
| T420 | Upon reception of the RRCReconfiguration message including sl-PathSwitchConfig | Upon successfully sending RRCReconfigurationComplete message (i.e., PC5 RLC acknowledgement is received from target L2 U2N Relay UE) | Perform the RRC re-establishment procedure as specified in 5.3.7. |

As seen from the table, although the start conditions for T304/T420 will be the same in the case of direct/indirect path addition/change, the stop (at least for T420) and the UE behaviour upon expiry will be different. For the stop condition in T420, it is based on PC5 RLC acknowledgement from the target L2 U2N relay UE. However, in the case of an indirect path addition, depending on the SRB configuration, the RRCReconfigurationComplete can only be sent over the direct path. In both T420 and T304, upon expiry, the UE triggers RRC re-establishment. However, in the direct/indirect path addition/change procedures, the UE should not trigger re-establishment as it already has either a direct or indirect link to the gNB. The UE would need to report the failure of addition/change to the gNB either via the direct path or indirect path. Hence, it would be cleaner to define a couple of new timers.

- Define new T304-like and T420-like timers for direct and indirect path addition/change respectively.

Upon expiry of these new T304-like and T420-like timers,

- Upon expiry of T304-like and T420-like timers, the remote UE can send the addition/change failure to the gNB via the direct or indirect path. The remote UE should not trigger RRC re-establishment.

One of the basic aspects related to the setup of a multipath configuration i.e., addition of either an indirect path (on top of a direct path) or a direct path (on top of an indirect path) that is yet to be discussed are the events/conditions under which the remote UE can request for a multipath configuration. For example, for measurement events, in an indirect path addition, the remote UE can be configured with the Rel-17 measurement configurations used in the case of a path switch scenario i.e., Event Y2 the candidate relay UE becomes better than a threshold. Similar measurement events can be configured for the addition of the direct path.

- Discuss the measurement events/conditions under which a multipath connection can be setup i.e., indirect/direct path addition.

# 3 Conclusion

Based on the discussion in the previous sections we propose the following:

| Proposal 1 | For Scenario-1, SRB1 and SRB2 are always configured on the same path. |
| --- | --- |
| Proposal 2 | For Scenario-1, non-split SRB1/2 can only be configured over the direct path. |
| Proposal 3 | Confirm the WA, for Scenario-1/2, MP remote UE is configured with a single cell group i.e., MCG for the direct path, and SL configuration, for the indirect path. |
| Proposal 4 | Confirm the WA, for Scenario 1, primary path of the split SRB1 and SRB2 is always configured on direct path. This does not preclude having the case where the UE switches the primary path to the indirect path for reporting after direct path failure. |
| Proposal 5 | Define new T304-like and T420-like timers for direct and indirect path addition/change respectively. |
| Proposal 6 | Upon expiry of T304-like and T420-like timers, the remote UE can send the addition/change failure to the gNB via the direct or indirect path. The remote UE should not trigger RRC re-establishment. |
| Proposal 7 | Discuss the measurement events/conditions under which a multipath connection can be setup i.e., indirect/direct path addition. |

# 4 References