3GPP TSG-RAN WG2 Meeting #123	R2-2308483

Toulouse, France 21st – 25th, August 2023

Agenda Item:	7.2.5

Source:	Ericsson

Title:	Discussion based upon RAN1 agreements on CPP, RedCap, Bandwidth aggregation

Document for:	Discussion, Decision

# Introduction

One Rel-18 work item on expanded and improved NR positioning has been approved in RAN#98 [1], and its objective on reduced capability (RedCap) positioning, carrier phase positioning, and bandwidth aggregation for positioning are as follows:

- Specify support of positioning for UEs with Reduced Capabilities (RedCap UEs) - Specify support of Frequency Hopping (FH) beyond maximum RedCap UE bandwidth for reception of DL PRS and transmission of UL SRS for positioning [RAN1, RAN2]. - NOTE: The complexity of the corresponding capabilities for RedCap UEs should be addressed for the introduction of appropriate capabilities for RedCap UEs. - Specify RRM requirements for positioning including RRM measurements and procedures for RedCap UEs for both with and without frequency hopping [RAN4]. - Specify physical layer measurements and signalling to support NR DL and UL carrier phase positioning for UE-based, UE-assisted, and NG-RAN node assisted positioning [RAN1, RAN2, RAN3, RAN4]. - Existing DL PRS and UL SRS for positioning are used for NR carrier phase measurements. - Specify measurements that are limited to a single carrier/PFL. - Specify bandwidth aggregation for positioning measurements across up to three intra-band contiguous carriers [RAN1, RAN2, RAN4]. - Specify signalling and procedures to support aggregation of PRS/SRS (respectively) resources across PFLs/carriers (respectively) for positioning measurements under the assumption that the signals over aggregated resources are transmitted and received (respectively) using a single RF chain (same antenna) [RAN1, RAN2]. - NOTE: The support of bandwidth aggregation for positioning measurements applies only to timing related measurements (e.g., RSTD, RTOA, and UE/gNB Rx-Tx time difference).

In this contribution, we give our view on the possible LPP signaling on RedCap positioning, carrier phase positioning, and bandwidth aggregation for positioning from RAN2 part of view.

# Carrier phase-based positioning

RAN1#113:

Agreement

For UE-based carrier phase positioning, support enabling LMF to forward the DL carrier phase measurement reported by a PRU, with additional information of the same PRU to a target UE for UE-based carrier phase positioning in the positioning assistance data.

- Note: Whether the forwarded DL carrier phase measurement is DL RSCP and/or DL RSCPD depends at least on which of them is (are) supported by UE capability.

- additional information of the same PRU includes at least PRU location.

  - FFS: additional PRU information, e.g. the AoD of PRU to each TRP, etc.

LMF forwarding PRU measurements

Some parameters, like the PRU location mentioned in the RAN1 agreement, are not expected to change much, while the DL RSCP and/or DL RSCPD measurements vary over time due to for instance clock frequency errors. To avoid unnecessary signaling overhead, it should be possible for LMF to distribute updates to different parameters/measurements at different update rates.

Some parameters from the PRU are rather static while other change faster.

- It should be possible for LMF to provide updates on different parameters with different periodicity.

Another method to reduce the signaling overhead is to use event-triggered updates of parameters. For DL RSCPD measurements there can be a threshold specified that defines when updated correction factors need to be sent to the UE. The threshold may be defined in a statistical way related to the DL RSCPD uncertainty, so that only statistically verified changes are forwarded to the Target UE.

Example: A simple algorithm to provide updated correction factors to the UE can be as follows:

- Initial state: The DL RSCPD measurements of the PRU, ϕ, have been provided to the UE.

- Each time LMF retrieves new DL RSCPD measurements, ϕ(t),  from the PRU:

- if   ϕ(t)-ϕ >δ then ϕ≔ ϕ(t) and send the updated phase corrections ϕ to the UE. The parameter δ is a threshold.

- It should be possible to configure a DL RSCPD change threshold. When DL RSCPD measurements of a PRU has changed more than the threshold, LMF is triggered to send an update to the Target UE.

Concerning the specific signaling of PRU assistance data, a straightforward way to provide a target UE with PRU location information and PRU carrier phase measurements is to include it parallel to the TRP LocationInfo in NR-PositioningCalculationAssistance.

-- ASN1START

NR-PositionCalculationAssistance-r16 ::= SEQUENCE {

nr-TRP-LocationInfo-r16			NR-TRP-LocationInfo-r16				OPTIONAL,	-- Need ON

nr-UE-LocationInfo-r16			NR-UE-LocationInfo-r16				OPTIONAL,	-- Need ON

nr-DL-PRS-BeamInfo-r16			NR-DL-PRS-BeamInfo-r16				OPTIONAL,	-- Need ON

nr-RTD-Info-r16					NR-RTD-Info-r16						OPTIONAL,	-- Need ON

...,

[[

nr-TRP-BeamAntennaInfo-r17		NR-TRP-BeamAntennaInfo-r17			OPTIONAL,	-- Need ON

nr-DL-PRS-Expected-LOS-NLOS-Assistance-r17

NR-DL-PRS-ExpectedLOS-NLOS-Assistance-r17

OPTIONAL,	-- Need ON

nr-DL-PRS-TRP-TEG-Info-r17		NR-DL-PRS-TRP-TEG-Info-r17			OPTIONAL	-- Need ON

]]

}

-- ASN1STOP

- LMF should provide the PRU location information and PRU carrier phase measurements as assistance data to the target UE. One option is to include it in the NR-PositionCalculationAssistance data structure.

## Non-physical PRU

Non-physical reference stations is already supported by GNSS RTK, (see TS 37.355, GNSS-RTK-ReferenceStationInfo-r15 IE). We propose to use it for carrier phase-based positioning as well. A non-physical PRU is assigned a position nearby the expected position of the Target UE. The measurements of a non-physical PRU are obtained by combining/interpolating measurements from multiple physical PRU.

Similar to the GNSS-RTK solution, we propose to signal a flag that identifies whether a PRU is physical or non-physical. There could be less security and privacy concerns for non-physical PRUs since we are not sharing details of an actual device. It can also be the case that some parameters are only needed for physical PRUs.

LMF reports carrier phase measurements from a non-physical PRU, similar to    how it is possible to report for non-physical reference stations with GNSS-RTK.

## Simultaneous measurements

RAN1#113

Agreement

To enable simultaneous measurements on same DL PRS by a target UE and a PRU, support the following enhancements:

- Enabling LMF to request the UEs, including target UE and PRU(s), to perform measurements on indicated DL PRS resource sets occurring within indicated time window(s).

- FFS: the details of the configuration of the indicated time window(s), e.g., the start time, duration, periodicity for the time window(s), as well as the relationship with the Scheduled Location time.

To enable LMF to request the UEs, including target UE and PRU(s), to perform measurements on indicated DL PRS resource set(s) occurring within indicated time window(s), each time window is defined with the following parameters:

- The start of the time window, which is indicated by a combination of subframe number, slot offset and symbol index

- The duration of the time window, which is given by a number of consecutive slots/symbols

  - FFS: the number of consecutive slots/symbols

- (Optional) The periodicity of the time window, which is defined similar to IE NR-DL-PRS-Periodicity-and-ResourceSetSlotOffset in TS 37.355.FFS: the maximum number of the windows

TS 38.305 V17.4.0,

Section 7.3.2:

- If a scheduled location time is provided […], the LMF may schedule location measurements by the UE to occur at or near to the scheduled location time.

- If a scheduled location time is provided […], the LMF may schedule location measurements by the NG-RAN to occur at or near to the scheduled location time.

Section 6.17.4:

- A common scheduled location time may be used if the LMF determines that simultaneous measurements for UE and PRU(s) are desirable.

The Release 17 positioning feature “Scheduled location time T” was designed to enable synchronous positioning of multiple UEs.

The scheduled location time feature can be used to implement simultaneous measurements by different UEs in downlink and simultaneous transmissions by different UEs in uplink (and gNB measurements of those UL-signals). The feature does not provide any guarantees on the specific time-window then the measurements should be done, this is done on a best-effort basis. But rather than designing yet another mechanism for simultaneous measurements, we propose that “Scheduled Location Time T” should be used as baseline.

The “scheduled location time T” was designed to enable synchronous positioning of multiple UEs.

The “scheduled location time T” does not support stringent requirements on time-window for positioning measurements, it is on a best effort-basis.

- The “scheduled location time T” feature is used as baseline for enabling simultaneous measurements by different UEs in downlink and simultaneous transmissions by different UEs in uplink (and gNB measurements of those UL-signals).

To identify if there is a need for additional enhancements to the scheduled location time T, RAN1 need to clarify what “simultaneous” means. How separated (in time) can the PRU resp. Target UE measurements be, and still be considered “simultaneous”? One plausible output is that more stringent requirements need to be defined for the scheduled location time T feature.

Regarding the RAN1 agreement to enable LMF to indicate PRS Resources on which the different UEs should perform measurements, we note that LPP already supports LMF indicating PRS Resources to the UE as part of the assistance data. (To be specific, the nr-SelectedDL-PRS-IndexList-r16 field in NR-DL-<XXXX>-ProvideAssistanceData-r16.) This mechanism can be used also for the purpose of carrier phase based positioning

- LMF indicate the PRS-resources that a UE performs measurements on using the field nr-SelectedDL-PRS-IndexList-r16 in NR-DL-<XXXX>-ProvideAssistanceData-r16.

# RedCap positioning

## DL Frequency Hopping

### 3.1.1 Assistance Data: LMF to UE impacts

RAN1 has discussed Inter-slot and Intra-slot frequency hopping (FH). For Inter-slot frequency hopping, it is assumed that there will not be any specific assistance data or LPP signaling that would be required from LMF to the UE. However, for intra-slot FH it is expected that the assistance data needs to be provided on how UE can perform PRS measurement whilst PRS is being hopped between within different symbols of the same slot (within a slot). However, RAN2 needs to wait for RAN1 LS before proceeding on the assistance data design.

### 3.1.2 Measurement Report: UE to LMF impacts

When it comes to UE measurement with respect to FH, RAN1 has agreed that current signaling of measurement report can be reused whereas some enhancements may also be needed which appears to be FFS.

Agreement

For DL Rx hopping or UL Tx hopping, support the UE or gNB to report the following:

- A single measurement based on receiving multiple hops of the DL PRS or UL SRS for positioning

- One measurement where a measurement is associated with one received hop

- FFS: indication of how many received hops / which received hops where used in the measurement report.

- Note: no new measurement definition is introduced in RAN1

- FFS: conditions when the above measurements are reported, and whether the above measurements can be reported together

RAN2 needs to wait for RAN1 LS before proceeding on the signaling enhancement design for UE reporting.

RAN2 need to wait for RAN1 agreement on DL FH design for Assistance data and Measurement report signaling.

## UL Frequency Hopping

### 3.2.1 Assistance Data: gNB to UE impacts

gNB configures the SRS and thus gNB needs to configure SRS transmission with FH pattern. RAN1 is still discussing the FH pattern and thus RAN1 needs to wait before RAN2 can proceed with RRC signaling impacts.

### 3.2.2 Measurement Report: gNB to LMF impacts

This will be designed by RAN3 and thus it does not have any impacts on RAN2.

For UL SRS frequency hopping transmission, new or extended signaling to configure the UL SRS configuration would be needed between gNB and UE, and between LMF and gNB.

RAN2 need to wait for RAN1 agreement on UL FH pattern which would have the RRC impacts.

## Capability

To achieve the target positioning requirements for RedCap UEs, the recommended solutions from the corresponding SI in Rel-18 are partially overlapped DL PRS frequency hopping and partially overlapped UL SRS frequency hopping for DL-TDOA, UL-TDOA and multi-RTT. The partially overlapped DL PRS frequency hopping can be implemented via wideband PRS transmission with narrowband PRS frequency hopping reception. While regarding the partially overlapped UL SRS frequency hopping, narrowband SRS frequency hopping transmission at the UE side as well as corresponding reception at the network side will be expected.

The current LPP Capability IE does not support UE to characterize the capabilities of RX frequency hopping on DL-PRS or TX frequency hopping on UL SRS.

Since, there are two different NW nodes (gNB and LMF) involved for both DL and UL FH, it needs to be discussed if the capability needs to be signaled to both nodes or only one node is adequate. It may be desired that gNB which configures the SRS FH towards UE should be aware of detailed UE capability whereas LMF may need to know on high level whether UE supports UL FH or not. Similarly, LMF may have to know the detailed UE capability on performing PRS measurement with respect to FH. RAN2 may need to wait for RAN1 progress before concluding the signaling impacts for the UE capabilities.

gNB needs to know detailed UE capability on UL frequency hopping whereas LMF need to only know whether the feature is supported or not. However, RAN2 need to wait for overall progress on RAN1 before progressing.

RAN2 waits for RAN1 and RAN4 progress on RedCap positioning.

# Bandwidth aggregation for positioning

RAN1 has sent an LS to RAN2 asking for the Semi-Persistent Activation/Deactivation feasibility .

Working assumption For semi-persistent positioning SRS for bandwidth aggregation, a single MAC CE can activate or deactivate: - SRS resource set(s) in one or two or three of three aggregated carriers - SRS resource set(s) in one or two of two aggregated carriers. Note: the single spatial relation is indicated by the MAC CE for each of two or three aggregated SRS resources. Send an LS to RAN2 to confirm the feasibility.

Currently only one SRS Resource set can be indicated in the MAC CE for activation/deactivation. From TS 38.321

Positoining SRS Resource Set ID: This field indicates the SP Positioning SRS Resource Set identified by SRS-PosResourceSetId as specified in TS 38.331 [5], which is to be activated or deactivated. The length of the field is 4 bits;

Thus, if UE receives such indication and if there is UL BW aggregation configured using above SRS Resource set then UE may activate/deactivate other SRS resource set which should have been prior configured via RRC. Thus, one SRS Resource set activation/deactivation can implicitly work as a pointer or other SRS resource set. With this setup, there is no additional MAC CE design change needed.

One SRS resource set acts as pointer for the complete SRS BW aggregation resource set for activating/deactivating purpose.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Some parameters from the PRU are rather static while other change faster.

Observation 2	The “scheduled location time T” was designed to enable synchronous positioning of multiple UEs.

Observation 3	The “scheduled location time T” does not support stringent requirements on time-window for positioning measurements, it is on a best effort-basis.

Observation 4	RAN2 need to wait for RAN1 agreement on DL FH design for Assistance data and Measurement report signaling.

Observation 5	For UL SRS frequency hopping transmission, new or extended signaling to configure the UL SRS configuration would be needed between gNB and UE, and between LMF and gNB.

Observation 6	RAN2 need to wait for RAN1 agreement on UL FH pattern which would have the RRC impacts.

Observation 7	The current LPP Capability IE does not support UE to characterize the capabilities of RX frequency hopping on DL-PRS or TX frequency hopping on UL SRS.

Observation 8	gNB needs to know detailed UE capability on UL frequency hopping whereas LMF need to only know whether the feature is supported or not. However, RAN2 need to wait for overall progress on RAN1 before progressing.

Based on the discussion in the previous sections we propose the following:

Proposal 1	It should be possible for LMF to provide updates on different parameters with different periodicity.

Proposal 2	It should be possible to configure a DL RSCPD change threshold. When DL RSCPD measurements of a PRU has changed more than the threshold, LMF is triggered to send an update to the Target UE.

Proposal 3	LMF should provide the PRU location information and PRU carrier phase measurements as assistance data to the target UE. One option is to include it in the NR-PositionCalculationAssistance data structure.

Proposal 1	LMF reports carrier phase measurements from a non-physical PRU, similar to    how it is possible to report for non-physical reference stations with GNSS-RTK.

Proposal 4	The “scheduled location time T” feature is used as baseline for enabling simultaneous measurements by different UEs in downlink and simultaneous transmissions by different UEs in uplink (and gNB measurements of those UL-signals).

Proposal 5	LMF indicate the PRS-resources that a UE performs measurements on using the field nr-SelectedDL-PRS-IndexList-r16 in NR-DL-<XXXX>-ProvideAssistanceData-r16.

Proposal 2	RAN2 waits for RAN1 and RAN4 progress on RedCap positioning.

Proposal 3	One SRS resource set acts as pointer for the complete SRS BW aggregation resource set for activating/deactivating purpose.

# References

RP-223549, “New WID on expanded and improved NR positioning”, RAN#98-e, December 12-16, 2022.