3GPP TSG-RAN WG2 #123	R2-2308485

Toulouse, France, 21st – 25th August 2023

Agenda Item:	7.24.2

Source:	Ericsson, Deutsche Telekom, AT&T

Title:	Relay based Positioning posSIB forwarding

Document for:	Discussion, Decision

# Introduction

During last meetings, the below agreement took place:

Agreements:

CR in R2-2304318 is agreeable.

WA: Remote UE indication is introduced in LPP.

SFN-DFN offset is introduced into RRC.

posSIB forwarding is introduced into RRC; FFS how much control from network side over which posSIBs can be forwarded.

CRs on 37.355, 38.331, and 38.306 to be confirmed next meeting, based on the CRs submitted to this meeting.

Agreement:

RAN2 confirm that the posSIB is forwarded transparently by the relay UE.

In this paper, we discuss the FFS: how much control from network side over which posSIBs can be forwarded.

# 2	Discussion

## 2.1	Reason or Motivation for need of NW control

- posSIBs allows a UE in RRC IDLE or RRC INACTIVE Mode to obtain assistance data via broadcast and perform positioning. This allows UE to save power. Each posSIB contains information (assistance data) about certain positioning methods. The NW based upon deployment of various positioning methods knows the coverage of acceptable level of positioning.  Basically, the NW/operator is in charge of ensuring certain level of quality of service. If the user is experiencing poor quality of service, it would imply that the NW KPIs would fail. Hence the operator must have the possibility to control if the posSIBs are indeed forwarded or not. If the operator sees that forwarding of pos SIBs results in worsened QoS, the operator may want to disable forwarding of pos SIBs. Hence, NW knows the best as which positioning method is applicable while the UE is in coverage or is in coverage hole or out of coverage area.

Example: From below Figure: Location A where DL-TDOA would be applicable where in Location B DL-TDOA would not be applicable. Primarily because of the poor hearability and GDOP.

- Even for GNSS based positioning method, there can be multiple deployments GNSS RTK-OSR-MAC, GNSS-RTK-OSR-FKP, GNSS RTK-SSR. It could be that the quality level (integrity performance) differs while the UE is in-coverage and out of coverage (for large cells) and different set of GNSS AD is applicable while in coverage and while in out of coverage. An example based upon above Figure: It can be that at Location A; GNSS-RTK-OSR-MAC yields best result whereas at location B GNSS-RTK-SSR yields the best result.

- Upon positioning request by positioning client, it is LMF which decides the positioning method based upon the initial cell ID information where UE is connected to. These fundamental aspects should be retained even when UE is out of coverage.

- It has been proposed that remote UE indication is provided in LPP by CR [6]. One benefit is that LMF would determine which positioning method would be applicable for this remote UE when connected via relay UE operating in certain cell. This principle as such should be applicable also when posSIBs are being forwarded.

The fundamanetal principle of NW to decide the positioning method in a certain deployment or coverage scenario would be violated if UE autonmouly decide on forwarding. Positioning KPI such as accuracy requirement will degrade in Operator’s Network.

Considering above aspects and based upon the papers submitted in this area [1], it appears certain positioning methods would be applicable for relay UE whereas certain positioning methods would not yield satisfactory results. There is need to classify posSIBs in a way that certain posSIBs are allowed to be relayed whereas certain posSIBs are not.

The information on which posSIBs can be relayed is optionally provided by the NW to the UEs.

RAN2 to agree to the RRC CR as provided in R2-2308486.

# Conclusion

In the previous sections we made the following observations:

Observation 1	The fundamanetal principle of NW to decide the positioning method in a certain deployment or coverage scenario would be violated if UE autonmouly decide on forwarding. Positioning KPI such as accuracy requirement will degrade in Operator’s Network.

Observation 2	Considering above aspects and based upon the papers submitted in this area [1], it appears certain positioning methods would be applicable for relay UE whereas certain positioning methods would not yield satisfactory results. There is need to classify posSIBs in a way that certain posSIBs are allowed to be relayed whereas certain posSIBs are not.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The information on which posSIBs can be relayed is optionally provided by the NW to the UEs.

Proposal 2	RAN2 to agree to the RRC CR as provided in R2-2308486.

# References

R2-2207287, Positioning support for remote UEs, Media Tek Inc., CATT, 17-26 August 2022

R2-2301649 Positioning of remote UEs         MediaTek Inc., CATT, Huawei, HiSilicon           discussion       Rel-18 TEI18

R2-2301650 Uplink positioning restrictions for UE-to-network remote UE [PosL2RemoteUE]   MediaTek Inc., CATT, Huawei, HiSilicon CR       Rel-18 38.305 17.3.0  0122    -           C   TEI18

R2-2301651 Downlink positioning support and posSIB request for L2 UE-to-network remote UE (Alt 1) [PosL2RemoteUE]   MediaTek Inc., CATT, Huawei, HiSilicon       CR   Rel-18 38.331 17.3.0  3910    -           C         TEI18

R2-2301653 Downlink positioning support and posSIB request for L2 UE-to-network remote UE (Alt 2) [PosL2RemoteUE]   MediaTek Inc., CATT, Huawei, HiSilicon       CR   Rel-18 38.331 17.3.0  3911    -           C         TEI18

R2-2301655 Indication to positioning server of operation as a L2 UE-to-Network Remote UE [PosL2RemoteUE]      MediaTek Inc., CATT, Huawei, HiSilicon   CR       Rel-18 37.355   17.3.0  0414    -           C         TEI18