3GPP TSG-RAN WG2 Meeting #123                                    R2-2308498

Toulouse, France 21- 25 Aug, 2023

Agenda item:		7.17.2

Source:				Samsung

Title:					Discussion on temporary capability restriction

Document for:		Discussion & Decision

# 1 Introduction

In this contribution we discuss on the procedures for temporary capability restriction based on the agreements till now. We discuss various aspects of both ‘reactive’ and ‘proactive’ approach and suggest methods for making them work effectively for reporting the temporary capabilities.

# 2 Discussion

## 2.1 Difference with other UAI signalling

In RAN2#121 [1], It is agreed to use UAI for the signalling of temporary UE capability changes. We note that the UAI in the existing system is purely an ‘assistance information’. Network may decide not to configure the UE based on the UAI. Network also may configure the UE according to UAI, but it may change the configuration later disregarding UAI.

Observation 1: In existing specification, network may configure the UE according to UAI, but it may change the configuration later disregarding UAI.

When UAI is used for temporary capability restriction and network has informed the UE that it has accepted the request for the changes in the capability, UE reallocates the capabilities to the other UE, UE-B. Hence NW-A should not configure the UE based on original capabilities till the UE again informs that temporary capability restriction is removed. i.e. Once the temporary capability restriction is accepted, NW-A always configures the UE-A according to the restricted capabilities. This also means that UE-A will initiate RRCReestablishment if it receives some configuration not supported by temporary capability, even if they are supported by permanent capability.

Proposal 1: Once the temporary capability restriction requested through UAI is accepted, NW-A ensures the (re-)configuration for the UE-A according to the restricted capability until the restriction is removed.

Proposal 1a: Once the temporary capability restriction requested through UAI is accepted, UE-A initiates RRC Reestablishment if it receives some configuration not supported by restricted capability though it is supported by permanent capability.

## 2.2 Configuration to control temporary capability reporting

In current running CR [2], we have a FFS as below:

Editor note:	FFS whether one configuration to control all temporary capabilities update or introduce individual control for each temporary capability update.

Generally in OtherConfig, separate configuration is used for controlling different parameters. For e.g. there are different configurations for controlling maxBW-PreferenceConfig, maxCC-PreferenceConfig, maxMIMO-LayerPreferenceConfig and so on. But we also need to note that the purpose and the possible scearnios where each of these may be applied in the existing specification are quite different from the Dual Transmission/Reception (Tx/Rx) MUSIM usecase. We think that for the UE-B to move to RRC_CONNECTED in NW-B, UE-A need to relinquish some of the RF chains and associated hardware and software resources. Typically ,this would need restrictions in a number of capabilities rather than in a single capability. Hence it would be simpler to control all temporary capabilities using a single configuration.

Proposal 2: A single configuration is used to control all temporary capabilities.

## 2.3 Reactive approach

RAN2 has agreed for a reactive approach for requesting temporary capability restrictions [3]. In the reactive approach UE request capability restrictions based on current RRC configurations.i.e. if CA/DC is configured, UE-A can request the NW-A to release CA/DC so that the resources can be given to UE-B. However it is not clear if UE-A also reports the restrictions on the bandcombinations in such case. If the bandcombinations are not reported, can NW-A configure UE-A with SCG or SCells in other bands before the temporary restrictions are removed. RAN2 may also need to discuss the UE-A behaviour if such configurations are allowed.i.e. Whether the UE-A will fail the procedure and send RRC Reestablishment or UE-A will consider the procedure as successful.

Proposal 3: RAN2 to discuss the following for reactive approach about the release of SCells and SCG

- Does UE-A also need to report the bandcombination restrictions along with SCells/SCG to be released?

- If bandcombination restrictions are not reported, can NW-A can configure UE-A with SCG or SCells after release and whether UE-A fails the procedure if SCell/SCG for an unsupported bandcombination is configured?

## 2.1 Proactive approach

From RAN2#121-bis, RAN2 has discussed to consider proactive approach in addition to the reactive approach and made the following agreements.

- =>Consider “proactive” approach (wherein the UE can request capability restrictions which can be independent of current RRC configuration if allowed by the NW) to MUSIM capability restrictions in addition to the reactive approach (which has been agreed previously). Such a mechanism shall still be under NW control, i.e. it is up to network whether to allow such signalling. FFS on the details – should aim for a common framework for the reactive and proactive approach. FFS on UE capabilities

- 6: UE can explicitly request specific serving cells or serving cell group to be released for Rel-18 MUSIM purpose. FFS how/whether this works for the proactive case.

First of all, we would like to confirm the understanding of what is proactive approach. In our understanding, there is no impact on NW-A based on which actions of UE-B leads to the temporary capability reporting. i.e. NW-A need to be concerned only about the signalling from UE-A for MUSIM purpose and what action in UE-B causes this signalling is not really important to NW-A. Thus our understanding is that in proactive approach, UE can request capability restrictions which can be independent of current RRC configuration of NW-A, if allowed by the NW If the UE-A reports the temporary capabilities.

Proposal 4: RAN2 to clarify the definition of proactive approach: Proactive approach is the approach where UE-A can request capability restrictions which can be independent of current RRC configuration of NW-A, if allowed by the NW.

We had the below agreements on the capabilities [1] that could be requested to be restricted.

- A8: For dual-active MUSIM, at least the following type of UE capabilities can be expected to be impacted:

- •	Transmission and reception capabilities (e.g. MIMO layers)

- •	Measurement capabilities (e.g. gaps)

- •	Supported bandwidth

- •	Supported band-combinations (FFS whether this is CA or DC or both)

In proactive approach UE-A may report the restriction on all these capabilities irrespective fof current RRC configuration from NW-A. For reactive approach, UE-A may report restricition on a subset of these capabilities to the NW-A at a time based on the current RRC configuration of UE-A. But the UE needs to have a method to report the temporary capability restriction for all these capabilities (though they are not reported because current RRC configuration doesn’t need them) even for ar reactive approach, as sometimes all these capabilities need to reported if they are used for present configuration. Thus from a framework point of view, it makes sense to keep both the reactive and proactive approach similar. UE can use UAI to report the temporary capability restriction to NW-A even in proactive case.

Proposal 5: UAI based framework can be used for proactive reporting of temporary UE capability restriction.

There is also an FFS on how the release of serving cells or serving cell group works in proactive approach. We think that if the UE reports the supported/restricted bandcombinations, it is not complex for the network to release the SCell or SCG.

Proposal 6: Network can release the SCells/SCG based on the reported band combinations in the proactive reporting of temporary UE capability restriction.

# 3 Conclusion

Request RAN2 to consider the following observations and to discuss and agree to the following proposals:

Observation 1: In existing specification, network may configure the UE according to UAI, but it may change the configuration later disregarding UAI.

Proposal 1: Once the temporary capability restriction requested through UAI is accepted, NW-A ensures the (re-)configuration for the UE-A are according to the restricted capability until the restriction is removed.

Proposal 1a: Once the temporary capability restriction requested through UAI is accepted, UE-A initiates RRC Reestablishment if it receives some configuration not supported by restricted capability though it is supported by permanent capability.

Proposal 2: A single configuration is used to control all temporary capabilities.

Proposal 3: RAN2 to discuss the following for reactive approach about the release of SCells and SCG

- Does UE-A also need to report the bandcombination restrictions along with SCells/SCG to be released?

- If bandcombination restrictions are not reported, can NW-A can configure UE-A with SCG or SCells after release and whether UE-A fails the procedure if SCell/SCG for an unsupported bandcombination is configured?

Proposal 4: RAN2 to confirm the definition of proactive approach: Proactive approach is the approach where UE-A can request capability restrictions which can be independent of current RRC configuration of NW-A, if allowed by the NW.

Proposal 5: UAI based framework can be used for proactive reporting of temporary UE capability restriction.

Proposal 6: Network can release the SCells/SCG based on the reported band combinations in the proactive reporting of temporary UE capability restriction.

# 4 Reference

[1] RAN2#121 meeting report

[2] R2-2307689 Running RRC CR for NR MUSIM enhancements

[3] RAN2#121-bis-e meeting report