3GPP TSG-WG2 Meeting #123	R2-2308500

# Toulouse, France, August 21th - 25th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.320 | CR | 0127 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 37320 on RLF report and CEF report |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh-Core |  |  |  |  |  | Date: |  |  | 2023-07-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For CEF report It is stated in stage 2 that “Upon detecting a cell with a different RPLMN, the UE clears stored CEF report entries.” , which implies even when UE successfully establish connection in a new cell UE clears the CEF report. It is different from stage 3. Since the intention of the sentence is to emphasize that only connection failures of cells belong to the same RPLMN can be logged in the the CEF report list, which already covers in previous sentence therefore it is suggested to delete the misleading sentence and leave the details to stage 3. For RLF report: It is agreed that UE can report EUTRA RLF report to NR if UE has the capability, which this description is currently missing in stage 2 specs. Impact Analysis Impacted 5G architecture options: NG-RAN SA Impacted functionality: RLF report and CEF report Inter-operability: 1. If the UE is implemented according to the CR and the network is not, there is no inter-operability issues 2. If the network is implemented according to the CR and the UE is not, there is no inter-operability issues |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Delete “Upon detecting a cell with a different RPLMN, the UE clears stored CEF report entries.” from subclause 5.1.6 - Add reference to 38.300 in subclause 5 to indicate that UE can report EUTRA RLF report to NG-RAN as specified in TS 38.300. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 2 description for CEF report is not aligned with stage 3 and stage 2 description for RLF report is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.6, 5.2.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

5.1.6	Accessibility measurements

The UE logs failed RRC connection establishments for LTE, UMTS and NR, i.e. a log is created when the RRC connection establishment procedure fails. For NR, UE logs any failed connection establishment attempt, i.e. a log is created when the RRC setup or resume procedure fails. The UE logs failed RRC connection establishments without the need for prior configuration by the network.

The UE stores the Selected PLMN on the RRC connection establishment failure or RRC resume procedure failure. Only if that PLMN is the same as the RPLMN, the UE may report the log.

NOTE:	There is no expected performance degradation for networks using EPLMNs.

The trigger for creating a log related to a failed RRC connection establishment is for NR when timer T300 expires, for LTE when timer T300 expires and for UMTS when V300 is greater than N300. The trigger for creating log related to a failed RRC resume procedure is for NR when timer T319 expires.

The UE can store the following information related to the failed RRC connection establishment or failed RRC resume procedure:

-	Time stamp, which is the elapsed time between logging and reporting the log.

-	The global cell identity of the serving cell when the RRC connection establishment or resume fails, i.e. the cell which the UE attempted to access.

-	The latest available radio measurements for any frequency or RAT

-	The latest detailed location information, if available.

-	For LTE:

-	Number of Random Access Preambles transmitted;

-	Indication whether the maximum transmission power was used;

-	Contention detected;

-	The latest WLAN measurement results, if available;

-	The latest Bluetooth measurement results, if available.

-	For UMTS FDD:

-	Number of RRC Connection Request attempts (e.g. T300 expiry after receiving ACK and AICH)

-	For UMTS 1.28 Mcps TDD:

-	Number of RRC Connection Request attempts.

-	Whether the FPACH is received or whether the maximum number Mmax of synchronisation attempts is reached.

-	Failure indication of the E-RUCCH transmission. It is only applied when common E-DCH is supported by UE and network.

-	For NR:

-	SSB index of the downlink beams of serving cell;

-	The latest number of consecutive connection failures in the last failed cell the UE has experienced independent of RRC state transitions;

-	RACH failure report:

-	Tried SSB index and number of Random Access Preambles transmitted for each tried SSB in chronological order of attempts;

-	Contention detected as per RACH attempt;

-	Indication whether the selected SSB is above or below the rsrp-ThresholdSSB threshold, as per RACH attempt;

-	TAC of the cell in which the UE performs the RA procedure;

-	For 2-step RACH, the following information can be additionally included:

-	Indication that fallback from 2-step RA to 4-step RA was performed by the UE, as per RACH attempt.

-	The latest WLAN measurement results, if available;

-	The latest Bluetooth measurement results, if available;

-	The latest sensor information, if available.

In addition, the CEF report may include additional information required for RACH Optimization solutions, as specified in TS 38.300 [22].

For NR, the UE can store multiple CEF (up to 4) reports to solve the problem about UL/DL coverage imbalance. For the failures happening consecutively in different cells, the UE stores multiple CEF report entries in the CEF report list, as specified in TS 38.331 [15]. For the failures happening consecutively in the same cell, the UE stores only one CEF report entry in the CEF report list, and replaces the last information related to the failed RRC connection establishment or failed RRC resume procedure with the new one, while the number of consecutive connection failures is increased. All the entries in the multiple CEF report list correspond to one PLMN.

NEXT CHANGE

5.2.1.2	Enhancement to Radio Link Failure report

The Radio Link Failure report contains information related to the latest connection failure experienced by the UE. The connection failure can be Radio Link Failure (RLF) or Handover Failure (HOF). The contents of the RLF report and the procedure for retrieving it by an eNB are described in TS 36.300 [12] or by an NG-RAN are described in TS 38.300 [22].

RLF reports can be collected by OAM. Upon RLF/HOF detection in the UE, rlfReport defined in TS 36.331 [5] also includes available location information on where RLF occurred, i.e. if detailed location information (e.g. GNSS location information) is available the reported location information in rlfReport consists of:

-	Latitude, longitude (mandatory)

-	Altitude (conditional on availability)

-	Velocity (conditional on availability)

-	Uncertainty (conditional on availability)

-	Confidence (conditional on availability)

-	Direction (conditional on availability).

As an indication of impact to MMTEL calls the UE indicates in the radio link failure report whether a radio bearer with QCI 1 was established when radio link failure was detected.

RLF reports may also include available WLAN measurement results and/or Bluetooth measurement results for calculating UE location.

If available, the UE can indicate NR neighbor cell measurements in measurements results.

END OF CHANGE