3GPP TSG-RAN WG2 Meeting #123	R2-2308512

Toulouse, France, 21 – 25 August 2023

Agenda item:	5.1.3.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on interBandMRDC-WithOverlapDL-Bands-r16

WID/SID:	TEI16 - Release 16

Document for:	Discussion and Decision

# 1	Introduction

During RAN4#107, RAN4 discussed the applicability of MTTD/MRTD requirements for inter-band EN-DC/NE-DC with overlapping or partially overlapping DL frequency (R4-2310089). Following the meeting, RAN4 sent an LS (R2-2307043 (R4-2310170)) to inform RAN2 of the outcome of that discussion so RAN2 could update the UE capability description for interBandMRDC-WithOverlapDL-Bands-r16 if necessary. In this contribution we discuss the impacts to interBandMRDC-WithOverlapDL-Bands-r16 based on RAN4’s discussion from RAN4#107 and the LS to RAN2.

# 2	Impacts to interBandMRDC-WithOverlapDL-Bands-r16

## 2.1	RAN4#107 ad hoc discussion on MRTD/MTTD requirement applicability for EN-DC/NE-DC (R4-2310089)

The relevant content from RAN4’s discussion on MTTD/MRTD requirements for inter-band EN-DC/NE-DC with overlapping or partially overlapping DL can be found under Issue 1-1-2 and Issue 1-1-3 in R4-2310089.

From Issue 1-1-2 (Update of MRTD/MTTD requirement applicability in Rel-16):

For this issue, RAN4 discussed how the relevant MTTD/MRTD requirements when interBandMRDC-WithOverlapDL-Bands-r16 is or is not supported by a UE, as well as the applicability of the requirements/capability to NE-DC.

- Proposals

  - Case 1: If UE indicates that it is capable of interBandMRDC-WithOverlapDL-Bands-r16

    - Option 1 (Samsung, Apple, Nokia, Ericsson):

      - apply the MTTD/MTRD requirement for synchronous operation in 7.5.2.1/7.6.2.1 for EN-DC and in 7.5.5.1/7.6.5.1 for NE-DC

  - Case 2: If UE indicates that it is not capable of interBandMRDC-WithOverlapDL-Bands-r16

    - Option 1 (Samsung, Apple):

      - apply the MTTD/MTRD requirement for synchronous operation in 7.5.3/7.6.3 for EN-DC and NE-DC

- Recommended WF

  - For Case 1, all companies agree on option 1, so it is suggested to agree on option 1.

  - For Case 2, check if option 1 is agreeable

- Discussion

  - Nokia: need to consider FDD, which should apply the current inter-band ones.

  - Nokia: there is no combination for NE-DC

  - Apple: prefer to define generic requirements, no big difference btw NE-DC and EN-DC.

    - Nokia: we can extend when there is combination

    - Apple: OK

- Agreement

  - if UE indicates that it is capable of interBandMRDC-WithOverlapDL-Bands-r16

    - For TDD-TDD inter-band EN-DC with overlapping frequency,

      - apply the MTTD/MTRD requirement for synchronous operation in 7.5.2.1/7.6.2.1 for EN-DC

    - For FDD-FDD inter-band EN-DC with overlapping frequency,

      - apply the MTTD/MTRD requirement for synchronous/asynchronous operation in 7.5.2/7.6.2 for EN-DC

  - if UE does not indicate that it is capable of interBandMRDC-WithOverlapDL-Bands-r16

    - For TDD-TDD inter-band EN-DC with overlapping frequency,

      - apply the MTTD/MTRD requirement for synchronous operation in 7.5.3/7.6.3 for EN-DC

    - For FDD-FDD inter-band EN-DC with overlapping frequency,

      - apply the MTTD/MTRD requirement for synchronous/asynchronous operation in 7.5.2/7.6.2 for EN-DC

From the above discussion, it can be seen that RAN4 only agreed on the applicability of interBandMRDC-WithOverlapDL-Bands-r16 to the EN-DC case, as there are currently no relevant NE-DC band combinations related to the requirements associated with this capability. In the future, RAN4 may revisit the applicability of this capability to NE-DC if there is an applicable band combination.

Observation 1: Currently there are no NE-DC band combinations where interBandMRDC-WithOverlapDL-Bands-r16 is applicable.

From Issue 1-1-3 (From Issue 1-1-2 (Update of MRTD/MTTD requirement applicability in Rel-16):

This part of RAN4’s discussion is shown below for sake of completeness, the main takeaway being that RAN4 decided to send an LS to inform RAN2 of their agreements and allow RAN2 to determine whether an update is needed to the description of interBandMRDC-WithOverlapDL-Bands-r16.

- Proposals

  - Option 1 (Apple): update the description for interBandMRDC-WithOverlapDL-Bands-r16 as following

- Recommended WF

  - Discuss whether to send LS to RAN2 to update the UE capability in 36306, and if so, whether wording in option 1 (draft LS R4-2307664) is agreeable.

- Discussion

  - SS: people will finally check RAN4 spec on which requirements apply. Without the change to the capability description it is still clear. Prefer no change at this stage.

  - E///: same view as SS.

  - Nokia: we can send LS to inform RAN2 the RAN4 agreements, but do not ask them to change

  - Apple: we can compromise to Nokia suggestion. RAN2 can decide whether to change.

  - SS: we can also support this approach,

- Agreement

  - Send LS to inform RAN2 the RAN4 agreements. It is up to RAN2 to decide whether and how to change their spec.

## 2.2	LS from RAN4 to RAN2 (R2-2307043/R4-2310170)

During 107th meeting, RAN4 discussed the MRTD/MTTD requirement applicability for inter-band EN-DC/NE-DC with overlapping DL frequency. After discussion, RAN4 reached the following consensus:

- For TDD-TDD inter-band EN-DC with overlapping frequency,

  - apply the MTTD/MTRD requirement for synchronous operation in 7.5.2.1/7.6.2.1 of 38.133 for EN-DC

- For FDD-FDD inter-band EN-DC with overlapping frequency,

  - apply the MTTD/MTRD requirement for synchronous/asynchronous operation in 7.5.2/7.6.2 of 38.133 for EN-DC

Furthermore, “Type 1 UE” and “Type 2 UE” are not defined terminologies in RAN4 specification. Instead, Type 2 UE is described as “a UE indicating capable of interBandMRDC-WithOverlapDL-Bands-r16” and Type 1 UE is the conventional UE supporting general requirements for inter-band collocated EN-DC in RAN4 specifications.

RAN4 would like to share the above agreements which is thought to be relevant for the signalling description “interBandMRDC-WithOverlapDL-Bands-r16” with RAN2. RAN2 is invited to consider whether and how to update the signalling in 38.306.

RAN4’s LS provides the following information to RAN2:

Consistent with the discussion from RAN4#107, the LS does not point to any requirements or relevant clauses in 38.133 that are related to the case of NE-DC. So, at least for the time being, it seems like RAN4 intends for this capability to be limited to the EN-DC case.

On the other hand, the current description of interBandMRDC-WithOverlapDL-Bands-r16 from 38.306 refers to both EN-DC and NE-DC. Considering, however, that there are no NE-DC combinations where this capability is applicable, these references to NE-DC could be deleted without impact.

Observation 2: References to NE-DC can be removed from the capability description of interBandMRDC-WithOverlapDL-Bands-r16 without causing any ambiguity.

Proposal 1: Delete references to NE-DC from the description of interBandMRDC-WithOverlapDL-Bands-r16.

The LS points to the relevant clauses of 38.133 where the MTTD/MRTD requirements can be found. These sections are stated separately in the LS for the TDD-TDD and FDD-FDD cases. However, we do not see a strong reason to state this information separately in the capability description: if one refers to sections 7.5.2/7.6.2 of 38.133, it is already clear that only synchronous operation is assumed for TDD-TDD (i.e. only sub-clauses 7.5.2.1/7.6.2.1 apply). Thus, without ambiguity, the description only needs to refer to 7.5.2/7.6.2 of 38.133. This is consistent with the current capability description and no update is needed in this regard.

Observation 3: It is already clear from 38.133 7.5.2/7.6.2 which subclauses apply to TDD-TDD.

Proposal 2: The capability description of interBandMRDC-WithOverlapDL-Bands-r16 does not need to distinguish between TDD-TDD and FDD-FDD cases separately.

Likewise, considering that the relevant sections of 38.133 already clearly state the specific values of the MTTD/MRTD requirements, we think it is redundant to include this information in the capability description for the cases where the capability is or is not signalled. (RAN4 also omitted these specific details in their LS, meaning they most likely do not think it is necessary to include these values in the capability description.)

Observation 4: Specific values for the MTTD/MRTD requirements are already clear from the relevant sections of 38.133 and do not need to be re-stated in the capability description.

Proposal 3: Omit values for MTTD requirements and delete any values for MRTD requirements from the description of interBandMRDC-WithOverlapDL-Bands-r16.

Lastly, RAN4 commented that the terms “Type 1 UE” and “Type 2 UE” are not defined in any RAN4 specifications. Indeed, our understanding is that these terms are only otherwise used in 38.833 for the feature group index related to this capability (i.e. FGI 2-19 “FDD-FDD or TDD-TDD inter-band MR-DC with overlapping or partially overlapping DL spectrum”). Therefore, we think this terminology can be deleted from the description without ambiguity.

Observation 5: The terms “Type 1 UE” and “Type 2 UE” are not defined in the RAN4 specifications (they are only used in the feature group index related to this capability) and can be removed from the description without ambiguity.

Proposal 4: Delete the terms “Type 1 UE” and “Type 2 UE” from the description of interBandMRDC-WithOverlapDL-Bands-r16.

Based on the above discussion, we propose to update interBandMRDC-WithOverlapDL-Bands-r16 as shown below. We have also submitted CRs for 38.306: R2-2308510 (Rel-16, Cat-F) and R2-2308511 (Rel-17, Cat-A).

| interBandMRDC-WithOverlapDL-Bands-r16 Indicates the UE supports FDD-FDD or TDD-TDD inter-band (NG)EN-DC operation with overlapping or partially overlapping DL bands with an (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in 38.133 [5] and inter-band RF requirements. If the capability is not reported, the UE supports FDD-FDD or TDD-TDD inter-band operation with overlapping or partially overlapping DL bands with (NG)EN-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in 38.133 [5] and intra-band RF requirements. | BC | No | N/A | FR1 only |
| --- | --- | --- | --- | --- |

Proposal 5: RAN2 adopt above changes for interBandMRDC-WithOverlapDL-Bands-r16 in TS 38.306.

# 3	Conclusion

This document has made the following observations:

Observation 1: Currently there are no NE-DC band combinations where interBandMRDC-WithOverlapDL-Bands-r16 is applicable.

Observation 2: References to NE-DC can be removed from the capability description of interBandMRDC-WithOverlapDL-Bands-r16 without causing any ambiguity.

Observation 3: It is already clear from 38.133 7.5.2/7.6.2 which subclauses apply to TDD-TDD.

Observation 4: Specific values for the MTTD/MRTD requirements are already clear from the relevant sections of 38.133 and do not need to be re-stated in the capability description.

Observation 5: The terms “Type 1 UE” and “Type 2 UE” are not defined in the RAN4 specifications (they are only used in the feature group index related to this capability) and can be removed from the description without ambiguity.

And proposed the following:

Proposal 1: Delete references to NE-DC from the description of interBandMRDC-WithOverlapDL-Bands-r16.

Proposal 2: The capability description of interBandMRDC-WithOverlapDL-Bands-r16 does not need to distinguish between TDD-TDD and FDD-FDD cases separately.

Proposal 3: Omit values for MTTD requirements and delete any values for MRTD requirements from the description of interBandMRDC-WithOverlapDL-Bands-r16.

Proposal 4: Delete the terms “Type 1 UE” and “Type 2 UE” from the description of interBandMRDC-WithOverlapDL-Bands-r16.

Proposal 5: RAN2 adopt above changes for interBandMRDC-WithOverlapDL-Bands-r16 in TS 38.306.