3GPP TSG-RAN WG2 Meeting #123	R2-2308515

Toulouse, France, 21 – 25 August 2023

Agenda item:	7.15.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Open issues on consistent LBT failure handling and recovery

WID/SID:	NR_SL_enh2 - Release 18

Document for:	Discussion and Decision

# 1	Introduction

In this contribution, we discuss aspects related to the consistent LBT (C-LBT) failure handling, namely reporting of C-LBT failure to peer UEs, the impact of C-LBT failure in HARQ feedback, cancellation of C-LBT failure and finally on the time aspects related to resource exclusion due to C-LBT failure in the resource (re)selection procedure.

# 2	Discussion

## 2.1	On consistent LBT failure reporting to peer UEs

It was agreed in RAN2#119bis, and RAN2#120, and confirmed in RAN2#122, that a UE in RRC_CONNECTED can indicate the SL-specific consistent LBT failure detection to the gNB (regardless if it is operating in Mode 1 or 2). More specifically, through discussion in RAN2#122 it was confirmed that the mode-2 solution should at least follow the mode-1 solution i.e. to report the SL C-LBT failure indication to gNB at least for RRC connection.

Agreements on SL C-LBT failure recovery (mode 2, RRC connected UE) - RAN2 confirms that SL C-LBT failure indication is reported to the gNB also for mode 2, RRC connected UE.

Observation 1: C-LBT indication will be reported to gNB in both mode-1 and mode-2 (when in RRC Connected) cases.

Considering the SL-specific consistent LBT failure detection (and recovery procedure) is a device centric occurrence and therefore has impact to any procedures where two or more devices interact. In some cases the Tx UE may not be experiencing consistent LBT failure in a specific RB set (or resource pool) and therefore it is able to access the resources in the RB set (or resource pool), i.e. by successfully performing an LBT (either Type 1 or 2, depending on if a valid COT is available), and transmit its PSCCH/PSSCH which the associated SCI requests for a HARQ feedback via a PSFCH resource mapped to the PSCCH/PSSCH resources. In such cases, the Rx UE is experiencing consistent LBT failure in the RB set in which the PSFCH is mapped to (or resource pool) and therefore is unable to access the mapped PSFCH resource to provide the HARQ feedback. This means that the Tx UE will not receive the HARQ feedback and therefore not know if it should or not perform an HARQ retransmission, so the communication cannot be assumed to be reliable.

Observation 2: The Tx UE is not aware of the Rx UE(s) C-LBT state for any RB set or resource pools.

However, in mode 2, it might be worthwhile to allow a UE to report C-LBT failure to its peer(s) since this information can be used to prevent at least issues related to PSFCH feedback (as we discuss in the section 2.2). This is true at least for unicast, but sharing C-LBT failure information with peers in the case of broadcast is less useful.

Proposal 1: RAN2 to support for at least unicast that mode-2 UEs can report consistent LBT failure to their peer(s). FFS for groupcast.

## 2.2	On HARQ feedback in RB set(s) with C-LBT failure

To cope with the impact of SL-specific C-LBT failure in the SL HARQ procedure:

- The Rx needs to be able to report, to either the gNB or directly to the Tx UE, that it is experiencing consecutive LBT failure; and

- The Tx UE needs to be able to react to that information when receiving it directly from the Rx UE or via the gNB.

If the Tx UE is made aware that the Rx UE is experiencing C-LBT failure in the RB set where the Tx UE plans to transmit; then the Tx UE should be able to decide based on this information if it should or not request HARQ feedback (i.e. using the HARQ feedback indicator in the 2nd stage SCI). However, in the MAC specification, a UE when performing a transmission is not allowed to decide if it should request HARQ feedback and instead the need for HARQ feedback is configured by logical channel. In addition to the impact of C-LBT failure in general unlicensed operation, it is worthwhile to relax this restriction as the transmitting device can be aware how difficult it is to acquire access to the channel (i.e., to successfully complete the LBT procedure). Therefore, in situations where it is likely that the Rx UE will not be able to provide HARQ feedback due to expected LBT failure, then the Tx UE should be allowed to not request for HARQ feedback and adjust its transmission parameters to avoid a HARQ retransmission instead.

Proposal 2: RAN2 to allow a Tx UE to be able to decide (irrespective of the logical channel) on whether to request a HARQ feedback, e.g. based on the acquired information from the Rx UE C-LBT status.

However, when the Tx UE was not able to acquire information in which RB sets the Rx UE is experiencing C-LBT failure, the Tx UE will still be requesting for HARQ feedback. Upon receiving this request, the Rx UE should still be allowed to attempt PSFCH transmission even though the RB set where the PSFCH resource is situated has been declared to be experiencing consistent LBT failure.

Proposal 3: RAN2 to allow a Rx UE to attempt PSFCH transmission in an RB set where C-LBT failure has been declared.

## 2.3	On the cancellation of C-LBT failure

In RAN2#122 it was discussed whether the gNB can really perform the C-LBT failure recovery based on the reported C-LBT indications from the UEs at least in mode 1. The conclusion and agreement was to leave any recovery action up to gNB implementation.

Agreements on SL C-LBT failure recovery (mode 1) - Leave it to gNB implementation after UE reporting SL C-LBT failure indication. No spec change.

Furthermore, it was brought up how to perform the recovery action, with various solutions on the table. The suggestions were the three options seen below:

- Option1: Timer based cancellation (LG, ZTE, Vivo, Huawei, Lenovo, Nokia: 6) - Option2: Measured channel condition based cancellation (FFS on what to be measured) (Ericsson, Xiaomi, Qualcomm, IDC: 4) - Option3: Rely on recovery action and no need for both option1 and option2 (OPPO, Spreadtrum, NEC, CATT, Apple, Intel: 6)

One of the discussed topics was whether option 1 or option 2 should be baseline, as either camp assumed one of them could replace the other. Furthermore, option 3 were perceived by some companies as similar to Uu, and thus neither option 2 or option 1 should be needed.

First, we need to investigate how the sidelink operation differs from the Uu case. As it has also been agreed in RAN2, a UE may choose a resource pool where at least one RB-set has not detected any SL C-LBT failures. This differs greatly from Uu, and may mean that a UE may have one or more resource pools which has only partially declared SL C-LBT, as some RB sets may be available.

Agreements on SL C-LBT failure recovery (mode 2, RRC ide/inactive UE) 4: It is up to UE implementation to select a resource pool out of resource pools that has at least one RB-set that SL C-LBT failure was not detected.

In a scenario where some resource pools may have only a subset of the RB-sets available, the case differs greatly from Uu, and thus it is beneficial if the UE can at least reset the C-LBT failure on the RB-sets within the pool, thus making the pool fully utilisable. As the congestion conditions in a given RB set can be quite dynamic, both due to independent traffic generation patterns of the devices transmitting in the RB set as well as their mobility, then it can be desirable to after some time to allow the Tx UE to attempt to use the RB set resources.

Observation 3: SL-U C-LBT operation differs greatly from Uu, as a UE may have one or more resource pools which has only partially declared SL C-LBT, as some RB sets may be available.

Observation 4: Allowing a UE reset RB-sets with SL C-LBT failure declared, without relying on recovery like RLF/gNB intervention is fundamental behaviour for SL-U.

Proposal 4: RAN2 to agree that C-LBT cancellation is needed for SL-U.

In case a UE applies option 2, it could be theoretically interpreted as there is no need for a C-LBT declaration, as the UE is allowed to measure the channel condition (i.e. LBT) whenever it sees fit. This is unless there is a prohibit timer, or similar functionality, which prevents this. However, in such case, this would be similar to option 1, as when the timer expires the UE would be allowed to perform LBT and thus measuring the channel conditions.

Observation 5: In case a UE applies option 2, it could be theoretically interpreted as there is no effect of the C-LBT declaration.

Proposal 5: RAN2 to agree using a timer for C-LBT cancellation as a baseline. FFS whether any other means of measurements are allowed after timer expiration than LBT.

## 2.4	On the resource re-selection due to C-LBT failure

In RAN2#122-bis, it was decided that mode 2 resource (re)selection is triggered when C-LBT failure is detected on a RB set, which resulted in the following agreements which some of them that have been shared with RAN 1 via an LS (R2-2306713):

Agreements on SL C-LBT failure recovery (mode 2, RRC ide/inactive UE): 1: Exclusion of RB set(s) that SL C-LBT failure was detected in candidate resource selection + resource pool (re)selection 2: The UE performs resource pool (re)selection - When SL C-LBT failure was detected for all RB-sets within a selected resource pool or; - Up to UE implementation although the above condition is not met 3a: MAC informs L1 of the RB set information where SL C-LBT failure was detected. 3b: L1 performs the resource exclusion for the RB set that SL C-LBT failure was detected. 3c: RAN2 will send a LS to RAN1 to ask to take it into consideration in their job.

However, one aspect that was not discussed was in during which time period the RB set resources are to be assumed to be excluded. In one approach, the MAC provides an indication that the RB set resources are excluded due to C-LBT failure and another indication when the RB set is no longer experiencing C-LBT failure. In another approach, the C-LBT failure declaration in one RB set is associated with a timer (as we discussed in section 2.3), which then indicates the time period where the RB set resources should be part of the resource exclusion.

Proposal 6: RAN2 to support the indication of the time period of when the resource associated with a RB set where C-LBT has been declared are to be excluded from the resource (re)selection procedure in the L1.

# 3	Conclusion

We made following observations and proposals:

Observation 1: C-LBT indication will be reported to gNB in both mode-1 and mode-2 (when in RRC Connected) cases.

Observation 2: The Tx UE is not aware of the Rx UE(s) C-LBT state for any RB set or resource pools.

Proposal 1: RAN2 to support for at least unicast that mode-2 UEs can report consistent LBT failure to their peer(s). FFS for groupcast.

Proposal 2: RAN2 to allow a Tx UE to be able to decide (irrespective of the logical channel) on whether to request a HARQ feedback, e.g. based on the acquired information from the Rx UE C-LBT status.

Proposal 3: RAN2 to allow a UE to attempt PSFCH transmission in an RB set where C-LBT failure has been declared.

Observation 3: Sidelink Uu C-LBT operation differs greatly from Uu, as a UE may have one or more resource pools which has only partially declared SL C-LBT, as some RB sets may be available.

Observation 4: Allowing a UE reset RB-sets with SL C-LBT failure declared, without relying on recovery like RLF/gNB intervention is fundamental behaviour for SL Uu.

Proposal 4: RAN2 to agree that C-LBT cancellation is needed for SL-Uu.

Observation 5: In case a UE applies option 2, it could be theoretically interpreted as there is no effect of the a C-LBT declaration.

Proposal 5: RAN2 to agree using a timer for C-LBT cancellation as a baseline. FFS whether any other means of measurements are allowed after timer expiration than LBT.

Proposal 6: RAN2 to support the indication of the time period of when the resource associated with a RB set where C-LBT has been declared are to be excluded from the resource (re)selection procedure in the L1.