3GPP RAN WG2 Meeting #123	R2-2308526

Toulouse, France August 21st – 25th, 2023

Agenda Item:	7.7.4.2

Source:	InterDigital

Title:	NTN mobility enhancements for RRC_CONNECTED

Document for:	Discussion, Decision

# Introduction

Adaptations to NR mobility must address the following issues in NTN: 1) increased signalling latency due to large propagation delay; 2) frequent cell change due to satellite movement; and 3) lower robustness of measurement-based mobility due to reduced signal strength variation between cell center and cell edge and large regions of cell overlap. Rel-17 NR NTN partially addressed these issues via enhancements to conditional handover, measurement reporting, and measurement rules for cell (re)selection.

An objective for Rel-18 NR NTN is to continue enhancements for both NTN-NTN and NTN-TN mobility and service continuity. Work will consider existing methods from NR TN and Rel-17 NR NTN as baseline for further enhancements.

The following agreements have been made regarding handover enhancements in Rel-18 NTN. FFS topics which are still open are highlighted in yellow.

In RAN2#121 [1], the following was agreed:

- For location-based CHO for earth-moving cells we follow the solution being investigated for cell reselection to allow the UE to derive the serving cell’s reference locations as the cells move. FFS whether the same mechanism can also be used for the candidate cell’s reference location

- Support RACH-less Handover in Rel-18.

- RACH-less Handover in NR NTN is a L3 mobility procedure (FFS if this is combined with the unchanged PCI approach, if supported) and uses the LTE’s RACH-less Handover procedure as a baseline. FFS on TA acquisition

- In NTN RACH-less handover, network indicates (implicitly or explicitly) whether NTA in the target cell is identical to the source cell or explicitly provided by the NW.

- Support dynamic grant from the target cell for RACH-less PUSCH transmission to reduce random access congestion in the target cell. FFS whether to limit the solution to same feeder link/gateway scenario

In RAN2#121bis-e [2], the following was agreed:

- In Rel-18 we don’t aim at RACH-less HO for NTN-TN mobility

- For initial UL transmission in RACH-less HO, support pre-allocated grant in RACH-less HO command

- NTN RACH-less HO is supported for Intra-satellite handover with the same feeder link. i.e., with same gateway/gNB;

- NTN RACH-less HO can be supported for intra-satellite handover with different feeder links, i.e., with gateway/gNB switch, inter-satellite handover with gateway/gNB switch, and inter-satellite handover with same gateway/gNB.

- RAN2 confirms the general UE procedure for NTN RACH-less HO

  - receive a RACH-less HO command which can include pre-allocated grant optionally. FFS N_TA is optional. (RRC)

  - start timer T304 for the target cell (RRC)

  - perform DL and UL synchronization, and start timer T430. FFS how to perform RACH-less UL synchronization to NTN target cell. (RRC, MAC)

  - start time alignment timer (MAC)

  - monitor target cell PDCCH for dynamic grant if pre-allocated grant is not configured in RACH-less HO command (MAC, PHY)

  - send initial UL transmission including RRCReconfigurationComplete message using the available UL grant (RRC, MAC, PHY)

  - consider RACH-less HO is completed upon receiving NW confirmation. FFS how to confirm RACH-less HO is successfully completed. (RRC, MAC)

  - stop timer T304 for the target cell. (RRC)

    - FFS whether to release UL grant if pre-allocated after RACH-less HO completion

    - FFS RACH-less HO failure handling, e.g. whether UE fallback to RACH-based HO to the target cell

    - FFS procedure for RACH-less HO combined with PCI unchanged or CHO if supported

- The pre-allocated grant is provided as type-1 CG

- Send an LS to RAN1 informing RAN2 agreements on NTN RACH-less HO and check RAN1 views on the following aspects:

  - whether the pre-allocated grant is provided with association to SSBs; if so, whether a RSRP threshold is configured for SSB selection.

  - to monitor target cell PDCCH for dynamic grant for initial UL transmission, whether beam indication can be provided in RACH-less HO command.

  - power control for initial UL transmission

- At least for pre-allocated grant, for the confirmation of RACH-less HO completion we reuse of LTE approach, i.e., UE Contention Resolution Identity MAC CE is used but UE ignores the content of this field. FFS if anything else is needed for dynamic grant

- Consider to support combining RACH-less HO with time-based CHO for NTN, taking into account the 1) validity of pre-allocated grant and potential waste of reserved resource; 2) when/how to provide dynamic grant in PDCCH.

In RAN2#122 [3], the following was agreed:

- Group handover related to P1~P4 from R2-2304736 is not supported in Rel-18.

- In NTN RACH-less handover, NW either indicates NTA in the target cell is identical to the source cell, or the NTA explicitly provided by the NW is 0. RAN2 will not discuss the case where NTA does not equal to 0

- From RAN2 perspective synchronization among source and target cells is not an issue in NTN RACH-less HO

- Release pre-allocated UL grant after RACH-less HO completion

- LTE approach (of confirming the HO completion) is reused for both pre-allocated grant and dynamic grant. FFS any enhancement to the confirmation of RACH-less HO completion, e.g. the NW does not send the UE Contention Resolution Identity MAC CE, and sends PDCCH/PDSCH addressed to C-RNTI

- Remove “FFS how to perform RACH-less UL synchronization to NTN target cell”, RAN2 assumes the UL sync handling in the target cell is the same in RACH-based HO and RACH-less HO, except how to acquire NTA (FFS on the spec impact , if any)

Note: aspects related to common configuration (e.g., target cell’s servingCellConfigCommon) are not included pending feedback from RAN3, and aspects related to same PCI switch can be found in a companion contribution [4]

# Mobility enhancements: Remaining open issues

## Candidate cell reference location derivation for location-based CHO in earth-moving cell

Location-based measurement initiation is supported for earth-moving cells re-using the measurement rules from the quasi-earth fixed cells (defined in Rel-17). One difference is the Serving Cell reference location moves continuously over earth. To compensate, the UE roughly derives the reference location trajectory based on the serving satellite ephemeris and epochTime.

Location-based CHO requires the UE to monitor its distance to both the Serving Cell and Candidate Cell reference locations. For earth-moving cells a similar solution as above is used to derive the Serving Cell reference location, however it is currently FFS whether the same mechanism can be used for the candidate cell’s reference location.

The three components needed to support reference location derivation are: 1) the reference location at epochTime; 2) epochTime; and 3) the satellite ephemeris. Assuming this is information is available and up-to-date for a candidate cell, it seems the same procedure can be used for candidate cell reference location derivation.

Proposal 1: 	For location-based CHO for earth-moving cells, re-use the procedure from cell relection to derive the candidate cell’s reference location as the cell moves.

## RACH-less HO failure handling

Based on RAN1 requirements from [5], at the time of RACH-less HO execution the UE has valid assistance information and will have performed necessary pre-compensation procedures to access the target cell. In this case RACH failure would more likely be due to radio-link issues rather than time/frequency pre-compensation.

If the radio-link issues are severe, falling back to RACH-based HO would not do much, and could increase recovery delay due to the large RTT. Legacy RACH-less HO already supports failure recovery via the RRC-Reestablishment procedure, which is also agreed as part of the NR procedure. This seems sufficient to resolve RACH-less HO failure without introducing additional delay by first attempting RACH.

Proposal 2: 	Upon RACH-less HO failure, UE does not fallback to RACH-based HO.

## Enhancements to RACH-less HO completion confirmation

In RACH-less HO, after transmitting the RRCReconfigurationComplete in the initial UL transmission the UE monitors for the UE Contention Resolution Identity MAC CE to confirm HO success, ignoring the contents if received. This is the same procedure used in LTE, and applies to both dynamic and preallocated grant case.

Since the UE ignores the MAC CE contents, it is essentially a wasted transmission. One possibility is to use any general PDCCH/PSDCH addressed to the UE’s C-RNTI as confirmation. Since both transmission would be addressed to the UE’s C-RNTI, it would essentially accomplish the same thing, but allow the network to transmit meaningful data. It should be noted that this is under the assumption that the network has DL data available. In the case that no data is available, the network can simply re-use the existing solution of transmitting a Contention Resolution MAC CE.

Proposal 3: 	NW can send PDCCH/PDSCH addressed to C-RNTI to confirm RACH-less HO completion instead of UE Contention Resolution MAC CE (e.g., if DL data is available).

## Specification impact of N_TA acquisition

In RACH-less HO the network either indicates the N_TA value in the target cell is identical to the source cell, or explicitely provides an N_TA value of 0. It is currently FFS what spec impact is associated with N_TA acquisition.

In LTE, the N_TA value is explicitly provided in the RACH-less HO command via the targetTA IE. NR supports the same possible N_TA values and any signalling gains by implicitely signalling N_TA would be minimal (e.g., by indicating that N_TA is the same as source by excluding the explicit indication that N_TA = 0). Introducing an explicit indication within the RACH-less HO command of N_TA is straightforward and in-line with LTE specification. Apart from providing the N_TA value, no other specification impacts are foreseen.

Proposal 4: 	N_TA value (i.e., identical to source or 0) is explicitly provided in the RACH-less HO command as in LTE.

# Mobility enhancements: Other aspects

### Conditional handover configuration for upcoming NTN cells

In RAN2#119bis-e it was agreed that at least for the Earth moving cell case, the next serving cells can be largely predicted in NTN for UEs not at cell edge. A consequence of this is that for a UE in a given location, the NW can estimate the likely upcoming sequence of cells and provide a series of upcoming target cell configurations, for example, with associated time-based triggering conditions, which the UE can activate at the appropriate time/location

One objective of the Rel-18 WI is to specify CONNECTED handover enhancements for a UEs to reduce the signalling overhead. Although a target cell configuration is required for each upcoming cell, the ability to simultaneously provide multiple upcoming configurations at once increases network flexibility to load balance by freeing up resources in future cells (which would otherwise be needed to provide future CHO configurations).

Observation 1:	Providing multiple CHO configurations for upcoming cells at once allows the network greater flexibility to load balance by freeing up future resources

One potential drawback is that the target cell configuration may become invalid, e.g. based on UE movement, in which case the signalling would have been wasted. However, as per the TR UE movement accounts for only a maximum 4% variation in the HO rate, and is thus neglected based on the relative speed of satellites. This can be further reduced by limiting the maximum number of provided target configurations at one time.

Proposal 5: 	A UE can be provided with CHO configurations for multiple future NTN cells based on the predictability of NGSO satellite movement.

# Conclusion

In this contribution the following observations and proposals were made concerning RRC_CONNECTED measurement, mobility, and service continuity enhancements in Rel-18 NR NTN:

Observation 1:	Providing multiple CHO configurations for upcoming cells at once allows the network greater flexibility to load balance by freeing up future resources

Proposal 1: 	For location-based CHO for earth-moving cells, re-use the procedure from cell relection to derive the candidate cell’s reference location as the cell moves.

Proposal 2: 	Upon RACH-less HO failure, UE does not fallback to RACH-based HO.

Proposal 3: 	NW can send PDCCH/PDSCH addressed to C-RNTI to confirm RACH-less HO completion instead of UE Contention Resolution MAC CE (e.g., if DL data is available).

Proposal 4: 	N_TA value (i.e., identical to source or 0) is explicitly provided in the RACH-less HO command as in LTE.

Proposal 5: 	A UE can be provided with CHO configurations for multiple future NTN cells based on the predictability of NGSO satellite movement.

# References

R2-2302401 – RAN2#121 Meeting Report

R2-2306553 – RAN2#121bis-e Meeting Report

Draft_R2_122_meeting_report_v2

R2-2308527 – Satellite switching without PCI change – InterDigital

- [5]      R2-2300020 - Reply LS on RACH-less handover in NTN –  RAN1