3GPP RAN WG2 Meeting #123	R2-2308527

Toulouse, France August 21st – 25th, 2023

Agenda Item:	7.7.4.2

Source:	InterDigital

Title:	Satellite switching without PCI change

Document for:	Discussion, Decision

# Introduction

Adaptations to NR mobility must address the following issues in NTN: 1) increased signalling latency due to large propagation delay; 2) frequent cell change due to satellite movement; and 3) lower robustness of measurement-based mobility due to reduced signal strength variation between cell center and cell edge and large regions of cell overlap. Rel-17 NR NTN partially addressed these issues via enhancements to conditional handover, measurement reporting, and measurement rules for cell (re)selection.

An objective for Rel-18 NR NTN is to continue enhancements for both NTN-NTN and NTN-TN mobility and service continuity. Work will consider existing methods from NR TN and Rel-17 NR NTN as baseline for further enhancements. This document focuses on NTN-NTN mobility enhancements for RRC_Connected UEs, specifically satellite switching without PCI change.

# Satellite switching without PCI change

In Rel-17 NTN, cells originating from different satellites are associated with different PCIs. A stationary UE will thus experience continuous L3 mobility as the serving satellite moves overhead and out of coverage (e.g., due to the curvature of the earth) and a new satellite takes over coverage of the geographic area.

In RAN2#121bise [2], the following agreement was captured:

- In quasi-earth fixed cell case, for hard satellite switch in the same SSB frequency and same gNB (no key change), satellite switching without PCI changing (not requiring L3 mobility) is supported, unless major technical issues are identified by RAN1 (as usual RAN2 will aim at minimizing the specification impact so that it fits in Rel-18)

In this case, a geographic area is associated with a PCI and gNB. At time T0, the PCI is served by a first satellite. As the first satellite moves out of range, a second (incoming) satellite connected to the same gNB will takeover servicing the PCI at a pre-determined transition point.

This solution avoids the need for L3 mobility, which can reduce signalling overhead considering the large number of UEs served within an NTN cell. Furthermore, restricting the scenario to quasi-earth-fixed hard-switch in the same SSB frequency and same gNB removes possible interference concerns of a soft switch (where both gNBs can simultaneously serve the PCI for short time). RAN2 has sent an LS to RAN1 for input for both hard and soft switches in [3], who in response have confirmed the feasibility of the PCI unchanged scenario.

Figure 1: A different satellite serving the same PCI.

## Synchronization during satellite switch without PCI change

Although same-PCI satellite switch avoids L3 mobility, the UE must still re-synchronize to the incoming satellite since the satellite location and channel conditions may be very different. Since the PCI, gNB, and SSB frequency remain the same, the satellite switch will be mostly transparent to the UE. To avoid service interruptions (e.g., due to timing mis-synchronization), some assistance information is needed to inform the UE of the satellite switch event.

Observation 1: 	Assistance information is helpful to inform/aid the UE to resynchronize to the incoming satellite, since in this scenario satellite switch is mostly transparent to UE.

In email discussion [Post 122][114] [4] the following options were discussed to determine when the the re-syncronization time should be:

- Option 1: Introduce a new “t-Start” (corresponding to incoming satellite starts providing coverage for the serving cell)

- Option 2: Introduce a new t-gap (time duration between the current serving satellite and the incoming satellite starts providing coverage for the serving cell)

- Option 3: reuse t-service (i.e. no other IE) with the condition that the gap is very short/zero

Regardless of how the UE determines when the serving satellite ends coverage and incoming satellite starts coverage (e.g., t-service + t-gap, t-service and t-start etc.) unless the incoming satellite coverage starts immediately after the end of the former satellite coverage (i.e., exactly at t-Service), there will be a coverage gap between the former and incoming satelite.

Observation 2: 	Unless the incoming satellite starts coverage immediately at t-Service of the former satellite, there will be a gap with undefined UE behaviour.

As mentioned in the email discussion, there should at least be some small gap due to RF switching. RAN2 should discuss whether this time duration is large enough to merit consideration in specification.

Proposal 1a: 	RAN2 to decide if a potential service gap between the incoming and former satellite (e.g., due to RF switching) needs to be addressed during same-PCI satellite switch.

If there is some need to specifiy UE behaviour, there are several options that RAN2 could choose from. For example, the UE could use discontinuous coverage behaviour from IoT NTN where aspects like timer handling or during the coverage gap is left to UE implementation. However, this behaviour is specified for IDLE-mode handling, and may not be suitable for CONNECTED UEs. Alternatively there could be some other specified behaviour to avoid timer expiry or TX during the gap.

Proposal 1b:	If the answer to Proposal 1a is “yes”, RAN2 to discuss whether UE behaviour during gap should be left to UE implementation or require explicitely specified behaviour.

## RACH-less satellite switch without PCI change

Another aspect discussed within [4] is support for RACH-less HO. Extending RACH-less handover to a same-PCI satellite switch scenario would greatly help in reducing service interruption time, which is the primary downside of a hard-switch.

In RAN2#121bis-e [2] and RAN2#122 [5], the RACH-less HO procedure was more or less defined as follows:

- receive a RACH-less HO command which can include pre-allocated grant optionally. (RRC)

- start timer T304 for the target cell (RRC)

- perform DL and UL synchronization, and start timer T430. (RRC, MAC)

- start time alignment timer (MAC)

- monitor target cell PDCCH for dynamic grant if pre-allocated grant is not configured in RACH-less HO command (MAC, PHY)

- send initial UL transmission including RRCReconfigurationComplete message using the available UL grant (RRC, MAC, PHY)

- consider RACH-less HO is completed upon receiving NW confirmation. (RRC, MAC)

- stop timer T304 for the target cell. (RRC)

Since RAN2 has explicitly agreed that satellite switching without PCI change does not require L3 mobility, then aspects of the above procedure (e.g., related to RRC reconfiguration) are not needed. RAN2 threrefore needs to discuss the possible changes to the baseline RACH-less HO procedure to remove aspects of the L3 mobility procedure.

Observation 2: 	RACH-less HO is an L3 procedure which contains aspects not relevant to satellite switching without PCI changing (e.g., T304, aspects related to RRCReconfiguration etc.).

For example, the T304 timer which is triggered based on the use of RRC Reconfiguration is not applicable to this case. Neither would be inclusion of the RRCReconfigurationComplete message in the initial UL grant. Another aspect would be the use of preallocated grant for the initial UL transmission. In legacy RACH-less HO, the preallocated UL grant is provided in the RACH-less HO command, however the UE may already have configured grant configurations for the cell which can be used instead.

Observation 3: 	If the UE maintains CG configurations during a satellite switch, the UE can use a sutiable CG configuration similar to a preallocated grant for initial transmission after synchronization.

If the UE is able to maintain CG configurations provided before the satellite switch to after the satellite switch, the UE may be able to substitute a “preallocated grant” with regular configured grant provided prior to satellite switch to perform the initial UL transmission.

Proposal 2: 	RAN2 to discuss adaptations to RACH-less HO procedure to support satellite switching with PCI unchanged scenario (e.g., remove L3 mobility aspects like T304 handling, or transmission of RRCReconfigurationComplete).

## Measurements and satellite switch without PCI change

In legacy RRM UE averages cell measurements over time to obtain L3 cell quality. However, since measurements associated with the former satellite (pre-switch) are no longer valid after satellite switch, the current behaviour could delay detection of radio issues to incoming satellite after satellite switch by averaging out the new values with a channel that no longer exists.

Observation 4: 	Not resetting L3 measurement window after a satellite switch delays detecting issues with the incoming satellite channel due to averaging with measurements to the former satellite that are no longer valid.

This could be partially resolved by acquiring many new measurements quickly after satellite switch, but acquiring new measurements to incoming satellite would require a measurement reconfiguration after satellite switch, which could take time considering UE-gNB RTT and risk RLF. The most straightforward option is to simply reset the L3 measurement window and discard measurements associated with the former satellite.

Proposal 3: 	UE resets L3 measurement window at time of satellite switching without PCI change.

Until the UE has an accurate understanding of the channel to the incoming satellite, any measurement report sent to the network may be unreliable, leading to an incorrect mobility decision. UE may temporarily suspend L3 event-based reporting some offset prior to satellite switch (if channel conditions are about to change, no need to use resources to report a cell no longer available) and/or after the satellite switch (to allow time to properly measure new channel conditions) to avoid unnecessary reporting and mobility.

Proposal 4: 	RAN2 to discuss if UE should temporarily suspend measurement reporting after satellite switching without PCI change.

# Conclusion

In this contribution the following observations and proposals were made concerning satellite switching without PCI change:

Observation 1: 	Assistance information is helpful to inform/aid the UE to resynchronize to the incoming satellite, since in this scenario satellite switch is mostly transparent to UE.

Observation 2: 	RACH-less HO is an L3 procedure which contains aspects not relevant to satellite switching without PCI changing (e.g., T304, aspects related to RRCReconfiguration etc.).

Observation 3: 	If the UE maintains CG configurations during a satellite switch, the UE can use a sutiable CG configuration similar to a preallocated grant for initial transmission after synchronization.

Observation 4: 	Not resetting L3 measurement window after a satellite switch delays detecting issues with the incoming satellite channel due to averaging with measurements to the former satellite that are no longer valid.

Proposal 1a: 	RAN2 to decide if a potential service gap between the incoming and former satellite (e.g., due to RF switching) needs to be addressed during same-PCI satellite switch.

Proposal 1b:	If the answer to Proposal 1a is “yes”, RAN2 to discuss whether UE behaviour during gap should be left to UE implementation or require explicitely specified behaviour.

Proposal 2: 	RAN2 to discuss adaptations to RACH-less HO procedure to support satellite switching with PCI unchanged scenario (e.g., remove L3 mobility aspects like T304 handling, or transmission of RRCReconfigurationComplete).

Proposal 3: 	UE resets L3 measurement window at time of satellite switching without PCI change.

Proposal 4: 	RAN2 to discuss if UE should temporarily suspend measurement reporting after satellite switching without PCI change.

# References

RP-223534– 3GPP Work Item Description: NR NTN enhancements (Rel-18)

R2-2306552 – RAN2#121bis-e Meeting Report

R2-2304273	LS on unchanged PCI	CATT	LSout	To: RAN1

R2-230xxxx - Report of [Post122][114][NR NTN Enh] Unchanged PCI – CMCC

draft_R2_122_meeting_report_v2