3GPP RAN WG2 Meeting #123	R2-2308528

Toulouse, France August 21st – 25th, 2023

Agenda Item:	7.8.3

Source:	InterDigital

Title:	Flightpath update notification for UAV

Document for:	Discussion, Decision

# Introduction

A new work item on NR support for aerial UEs (UAVs) was agreed for Rel-18. Since aerial UEs are supported via LTE as from Rel-15, work done in LTE is a starting point and NR solutions should be harmonized as much as possible. Among detailed objectives for Rel-18 is the introduction of flight path reporting to NR.

RAN2#119bis-e has captured the following agreement relevant to flightpath notification update [2]:

- Allow the flight path to be updated.  FFS on the details.

RAN2#120 has captured the following [3]:

- A UE indicates whether flight plan information is available within the RRCReconfigurationComplete, RRCReestablishmentComplete, RRCResumeComplete, or RRCSetupComplete message.   Flight path reporting uses at the UE Information request/response procedure as baseline.

- UE indicates to the network a new flight path is available in the UE (whether it is initial or update). Then, reuse the normal request/response procedure of flight path report.

- UAI message can also be used to indicate the UE has flight path availability.

- FFS whether and what triggering conditions are specified for flight update.  FFS The maximum number of waypoints within flight path plan is left FFS.

RAN2#121bis-e has further captured the following [4]:

- Flightpath update indication in UAI is configurable by the network

- As a baseline, we can consider a simple network control mechanisms (e.g. a threshold(s)) that controls triggering the flightpath update indication in UAI. FFS if new threshold or the kind of threshold(s)

- As a baseline, single indication is used for both initial and updated flightpath available (i.e. same flag is used for initial and updated flight path indication.  FFS if further differentiation is needed if we decide to have delta signaling

And RAN2#122 has further captured the following [6]:

- The network can configure the UE to trigger a flightpath update notification based on a configured delta time (when timestamp is configured to be reported) or distance configuration.

This document addresses remaining aspects for UAVs, specifically the content and procedure for flightpath notification update.

# Flightpath update notification for UAV

Recent agreements regarding the contents of a UAV flight path and reporting procedure largely align with LTE. One exception is support for updating a previously reported flight path, which is a new enhancement unique to NR. Summarizing agreements from past meetings, once an update to a previously reported flight path is available, the UE can notify the network via the UE Assistance Information (UAI) message. An updated flight path is retrieved via the UE information request/response procedure like the initial flight path. Remaining details of flightpath notification are addressed in this contribution, and support for delta signalling is discussed in a companion contribution [5]

## Flightpath update notification triggering

One reason flight path reporting is under network control is to minimize possible interference caused by frequent and unnecessary flight path updates. A UE sending frequent update notifications (and thus triggering a flight path reporting procedure) over minor changes is contrary to the intention of the current procedure.

In RAN2#121bis-e, the following was agreed:

- As a baseline, we can consider a simple network control mechanisms (e.g. a threshold(s)) that controls triggering the flightpath update indication in UAI. FFS if new threshold or the kind of threshold(s)

Which was further refined in RAN2#122 in the following agreement:

- The network can configure the UE to trigger a flightpath update notification based on a configured delta time (when timestamp is configured to be reported) or distance configuration.

A simple place to provide the thresholds to control flightpath update indication triggering would be within the flightpath report configuration (i.e., equivalent to the IE FlightPathInfoReportConfig specified within TS 36.331).

Proposal 1:	UE is provided with thresholds controlling flightpath update indication triggering in the flight path report configuration (e.g., the NR equivalent to FlightPathInfoReportConfig).

If a threshold is configured, the UE will compare the current value to the value from the previously reported flightpath. If this value exceeds the configured threshold, the condition is satisfied. As this has not been explicitly agreed, RAN2 should confirm this is the intended behaviour.

Proposal 2:	The threshold condition is satisfied if the current waypoint/timestamp value exceeds the value from the previous flightpath report by the configured threshold.

Furthermore, considering both time and distance configurations are supported, RAN2 should discuss whether both can be simultaneously configured. If possible, RAN2 should confirm the intended triggering behaviour (e.g., if just one or both need to be triggered to trigger the flightpath update indication in UAI).

Proposal 3:	RAN2 to discuss if both waypoint/timestamp thresholds can be simultaneously configured, including triggering details.

The network may similarly restrict a flight path update notification to only being triggered if a certain number of waypoints and/or timestamps are considered changed or invalid. This may be configured independently or jointly with the above distance/time thresholds.

Proposal 4:	UAV can be configured to trigger a flightpath update notification only if a number of updated waypoint/timestamps simultaneously satisfy the threshold condition.

## Flightpath update notification assistance information.

In RAN2#121bis-e, The following was agreed regarding the format of the UAI flag:

- As a baseline, single indication is used for both initial and updated flightpath available (i.e. same flag is used for initial and updated flight path indication.  FFS if further differentiation is needed if we decide to have delta signaling

Based on current agreements, UE uses the same flag to indicate availability of both initial and updated flightpath. However, what constitutes a “new” flight path can vary significantly. For example, a new flight path could be a modification to a previously reported value or an entirely new value. It could be one change or a change to the entire flight path, and could impact only time information, location information, or both. How the flight path changes can affect resource provisioning and mobility planning or have no impact at all. A simple flag is insufficient to differentiate between these cases, especially if delta signalling is supported.

Observation 1:	How and to what degree a flight path has changed can vary significantly and can impact how the update notification is treated by the network.

Instead, the UE can include additional assistance information to support NW prioritization. Whether or not to include additional assistance information could be configured by the network (e.g., within the flight path reporting configuration), as well as what type of information to include.

Proposal 5:	NW can configure UE to report assistance information about the flight path update.

For example, if timestamp-only flight path reporting is supported (refer to companion contribution in [5]), a UE may optionally indicate that only time information has changed in an update notification to support whether the network requests the full or timestamp-only flightpath

Proposal 6:	Flight path update assistance information can include if only time information has changed.

# Conclusion

In this contribution the following proposals are made concerning updated flightpath update notification for aerial UEs in NR:

Observation 1:	How and to what degree a flight path has changed can vary significantly and can impact how the update notification is treated by the network.

Proposal 1:	UE is provided with thresholds controlling flightpath update indication triggering in the flight path report configuration (e.g., the NR equivalent to FlightPathInfoReportConfig).

Proposal 2:	The threshold condition is satisfied if the current waypoint/timestamp value exceeds the value from the previous flightpath report by the configured threshold.

Proposal 3:	RAN2 to discuss if both waypoint/timestamp thresholds can be simultaneously configured, including triggering details.

Proposal 4:	UAV can be configured to trigger a flightpath update notification only if a number of updated waypoint/timestamps simultaneously satisfy the threshold condition.

Proposal 5:	NW can configure UE to report assistance information about the flight path update.

Proposal 6:	Flight path update assistance information can include if only time information has changed.

# References

RP-223545 – WI on NR support for UAV

R2-2211101 – RAN2#119bis-e Meeting Report

R2-2300002 – RAN2#120 Meeting Report

R2-2306553 – RAN2#121bis-e Meeting Report

R2-2308529 – Flightpath reporting for UAV – InterDigital

Draft_R2_122_meeting_report_v2