3GPP RAN WG2 Meeting #123	R2-2308530

Toulouse, France August 21st – 25th, 2023

Agenda Item:	7.20.2

Source:	InterDigital

Title:	UL time alignment in multi-DCI based multi-TRP with two TAs

Document for:	Discussion, Decision

# Introduction

The Rel-18 WI on MIMO evolution for downlink and uplink has been ongoing in RAN1 since RAN1#109-e, focusing on many objectives including support for two TAs for UL multi-DCI based multi-TRP operation.

In RAN1#110, the following was agreed [1]:

- For multi-DCI based multi-TRP operation with two TAs, support configuring two TAGs belonging to a serving cell.

As a consequence of the above, RAN2 agreed to the following in RAN2#122 [2]:

- Configure one TAT per TAG to support two TAs for a serving cell, i.e., in this case 2 TAGs are configured for the serving cell.

This contribution discusses RAN2 impacts of the above agreement, specifically potential changes to how UL time alignment is maintained in MAC in the case of multi-DCI based multi-TRP with two TAs.

# UL time alignment

UL time alignment is maintained per-TAG in MAC by timeAlignmentTimer (TAT), which is restarted upon reception of a timing advance update addressed to that TAG. While the timer is running, L1 is considered time-synchronised, otherwise (e.g., once the timer expires), L1 is considered non-synchronised and uplink transmission is restricted to only MSG1/MSGA. In addition to restricting UL transmission, HARQ buffers are flushed and PUCCH, SRS, configured DL assignments/UL grants, and PUSCH resources for semi-persistent CSI are released for Serving Cells associated with the TAG. If the TAT associated with the PTAG expires, the above expiry procedures are performed for all Serving Cells.

Legacy specification assumes each cell is associated with one TAG, and in almost all cases the PTAG is associated with the PCell. However, for multi-DCI based multi-TRP operation with two TAs, RAN1 has agreed that a Serving Cell can be configured with two TAGs. Consequently, RAN2 agreed that a two TATs can now be configured for a Serving cell, with impacts on UL time alignment in the case of multiple TAGs configured for a Serving Cell discussed in [POST122][852] [3].

One outstanding issue is whether a PCell configured with two TAGs is considered as having one PTAG and one STAG or two PTAGs. If only one PTAG is supported and the associated TAT expires, the UE would have to perform recovery actions for all Serving Cells even though the UE may maintain UL/DL operation with the PCell via the other TRP, since RAN1 has confirmed that when the TAT associated to one TRP expires, UL or DL operation associated to the another TRP is not impacted.

Observation 1:	If a PCell is configured with two TAGs, but only one is considered a PTAG and its associated TAT expires, the UE would have to perform recovery actions for all Serving Cells even though the UE might still be able to maintain UL/DL operation with the PCell via the other TRP.

Alternatively, if both TAGs are considered as “PTAGs”, then if at least one of the TATs is still running then the UE would be able to avoid performing recovery actions for all Serving Cells. This is obviously beneficial to reduce service interruption.

Observation 2:	If a PCell is configured with two TAGs and both are considered PTAGs, as long as one of the associated TATs is running the UE can avoid performing recovery actions for all Serving Cells.

If the TATs associated with both PTAGs are expired concurrently, then the UE cannot maintain connection with the PCell and legacy operation associated with PTAG expiry should apply.

Proposal 1:	If a PCell is configured with two TAGs, both TAGs are considered as “PTAG”. Legacy procedures associated with PTAG TAT expiry require both PTAG TATs to be expired.

As previously mentioned, legacy specification assumes a single PTAG. To simplify specification and reduce possible impact to legacy procedures in the case of multi-DCI based multi-TRP operation with two TAs, a new TAG identifier (e.g., PSTAG) can be introduced to differentiate the second PTAG from the first PTAG.

Observation 3:	To simplify specification and avoid impact to legacy procedures (which assumes only one PTAG), a new TAG identifier (e.g., PSTAG) can be introduced to differentiate the second PTAG from the first PTAG.

An added advantage of defining a new type of PTAG identifier is that it allows differentiated treatment of timer expiry between the PTAG and PSTAG, should RAN2 choose to agree to such behaviour.

Proposal 2:	If a PCell is configured with two TAGs, introduce a new TAG identifier (e.g., “PSTAG”) to differentiate the second PTAG from the first PTAG.

A possible impact of introducing “PSTAG” (for at least the inter-cell case) is if the TRP associated with the PTAG is de-configured, then there is no “PTAG” for the UE. A simple solution in this case is the UE automatically promotes the PSTAG to the PTAG.

# Conclusion

In this contribution the following proposals are made concerning uplink time synchronization in a multi-DCI based multi-TRP with two TAs scenario:

Observation 1:	If a PCell is configured with two TAGs, but only one is considered a PTAG and its associated TAT expires, the UE would have to perform recovery actions for all Serving Cells even though the UE might still be able to maintain UL/DL operation with the PCell via the other TRP.

Observation 2:	If a PCell is configured with two TAGs and both are considered PTAGs, as long as one of the associated TATs is running the UE can avoid performing recovery actions for all Serving Cells.

Observation 3:	To simplify specification and avoid impact to legacy procedures (which assumes only one PTAG), a new TAG identifier (e.g., PSTAG) can be introduced to differentiate the second PTAG from the first PTAG.

Proposal 1:	If a PCell is configured with two TAGs, both TAGs are considered as “PTAG”. Legacy procedures associated with PTAG TAT expiry require both PTAG TATs to be expired.

Proposal 2:	If a PCell is configured with two TAGs, introduce a new TAG identifier (e.g., “PSTAG”) to differentiate the second PTAG from the first PTAG.

# References

RAN1#110 chair notes

RAN2#122 chair notes

R2-230xxxx [Post122][852][MIMOevo] 2TAs for mDCI mTRP – Samsung