3GPP TSG-RAN WG2 #123	Tdoc R2-2308535

Toulouse, France, Aug 21st – 25th, 2023

Agenda Item:	7.11.2.2

Source:	Ericsson

Title:	MBS remaining issues on DRX

Document for:	Discussion

# 1	Introduction

In this document, handling of open issues w.r.t DRX and retransmissions such as the impact for HARQ and DRX timers are discussed.

# 2	Discussion

In MBS multicast reception in RRC INACTIVE, the handling of DRX and retransmissions such as the impact for HARQ and DRX timers has been defined (RAN2#121bis):

RAN2 Agreement: “On DRX operation for multicast in RRC_INACTIVE, take the multicast DRX as baseline. FFS handling on PTM related HARQ RTT Timer and DRX Retransmission Timer.”

For 38.321, this leads to that a remaining open issue needs to be resolved:

Section 5.7b, 38.321:

“Editor Note: FFS on how to handle PTM related HARQ RTT Timer and DRX Retransmission Timer when UE is in RRC_INACTIVE.”

In order to support receiving retransmissions one can assume that support of drx-HARQ-RTT-TimerDL-PTM or drx-RetransmissionTimerDL-PTM is needed.

This is important especially for the case where the same PTM configuration is used in both RRC CONNECTED and RRC INACTIVE, that is to support a single PTM transmission received in both RRC states.

UEs in RRC_CONNECTED state sending HARQ feedback with retransmissions sent by the network may then benefit also UEs in RRC_INACTIVE.

RAN2 assumes RRC_INACTIVE UE receiving multicast also may receive retransmissions because of HARQ feedback from UEs receiving multicast in RRC_CONNECTED.

Concerning DRX and since multicast PTP is not supported in INACTIVE, the timers drx-HARQ-RTT-TimerDL or drx-RetransmissionTimerDL are not useable and handling in this state is thus not needed (not started or acted upon).

UE receiving MBS multicast in RRC_INACTIVE start drx-HARQ-RTT-TimerDL-PTM and drx-RetransmissionTimerDL-PTM.

The use of these timers are defined in 38.321, 5.7b and for supporting this in also in INACTIVE, one could base the UE use of these times on the RRC CONNECTED configuration on HARQ feedback.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	UE receiving MBS multicast in RRC_INACTIVE start drx-HARQ-RTT-TimerDL-PTM and drx-RetransmissionTimerDL-PTM.

# References

TS 38.321 v xxx