3GPP TSG-RAN WG2 #123	R2-2308539

Toulouse, France, Aug 21st – 25th, 2023

Agenda Item:	7.7.2

Source:	Ericsson

Title:	R18 NR NTN Coverage enhancements

Document for:	Discussion, Decision

# 1	Introduction

RAN2 made this agreement at RAN2#121bis:

Agreements:

- Rel-18 NTN coverage enhancements work will focus on addressing the RAN2 impact (if any) from RAN1 agreements on PUCCH enhancements for MSG4 HARQ-ACK and DMRS bundling for PUSCH. No further enhancements are pursued in this release

1. Overall Description:

RAN1 has discussed how UE reports information related to PUCCH repetition for Msg4 HARQ-ACK and reached the following working assumption. It is noted that this working assumption will be confirmed if Option B is feasible from RAN2 perspective.

Working assumption For PUCCH repetition for Msg4 HARQ-ACK, support Option B as container of the repetition request or capability report indicated by UE. - Option B: Higher layer signaling in Msg3 PUSCH

It is noted that an additional working assumption was reached for repetition request or capability report.

Working assumption For PUCCH repetition for Msg4 HARQ-ACK, - Two-state information is transmitted as ‘repetition request or capability report’ in the existing agreements/working assumptions. - The two-state information represents state 1: ‘repetition request or capability report’ or state 2: no indication. - How to transmit the two-state information is up to RAN2 when higher layer signaling is used for the transmission. - In state 1, only either repetition request or capability report is transmitted from each UE when transmitted, and they are not differentiated in the signaling. - Note: repetition request and capability report are defined as in the working assumption reached at RAN1#112.

2. Actions

To TSG RAN WG2

ACTION: 	RAN1 respectfully asks RAN2 to provide feedback on the feasibility of Option B, and if feasible, to specify the details of Option B.

Then at RAN2#122 we received an LS from RAN1 [1]:

In this paper we address the RAN2 aspects of this LS.

# 2	Discussion

Three solutions have been discussed:

- Letting the network store the UE capabilities and using the UE ID in Msg3 to retrieve them

- Using reserved LCIDs for indicating the request/capability of Msg4 PUCCH HARQ-ACK repetition

- Using one reserved R bit in one MAC subheader of Msg3

After some further investigation we think solution 1 is infeasible, as the UE ID in Msg3 will in some cases not contain a unique identifier for the UE, and this will put more stringent timing requirements on the signalling at a critical phase of the UE setup.

Further, solution 2 is feasible besides there is very few reserved LCIDs left (as eLCIDs means adding one extra byte to the message size which not acceptable for Msg3). The few remaining LCIDs need to be reserved for future important functionalities – this feature is just a minor addition (compare the use of LCIDs to indicate RedCap capabilities which has a much higher usefulness).

Solution 3 is also bad in the sense it leaves us with only one remaining reserved bit in the MAC subheader for fixed size MAC CEs, however if we instead introduce configurability for one of the R bit in the subheader – it makes future additions possible when the future addition is not needed at the same time as Msg3 indication of Msg4 PUCCH HARQ-ACK repetition request/capability.

Therefore, we propose a configuration possibility for one of the R bits in the fixed size MAC subheader. If indicated in the SIB that the feature is enabled (RAN1 have been discussing providing repetition factors in SIB as an indication that the feature is enabled), the UE can use one of the R bits of the CCCH LCH MAC CE subheader in Msg3 to indicate Msg4 PUCCH HARQ-ACK repetition request/capability.

There shall be an indication in SIB to enable the feature Msg4 PUCCH HARQ-ACK repetition, possibly by a separate flag or by presence of a configuration parameter for the feature.

In NTNs, when Msg4 PUCCH HARQ-ACK repetition feature is enabled in SIB, one of the R bits in the MAC subheader of CCCH LCH in Msg3 is used to indicate a request/capability for Msg4 PUCCH HARQ-ACK repetition for the UEs that support the feature. Setting this bit to 1 indicates request/capability for Msg4 PUCCH HARQ-ACK repetition; and setting it to 0 indicates no request or no capability for Msg4 PUCCH HARQ-ACK repetition.

# 3	Conclusions

Based on the discussion in the previous sections we propose the following:

Proposal 1	There shall be an indication in SIB to enable the feature Msg4 PUCCH HARQ-ACK repetition, possibly by a separate flag or by presence of a configuration parameter for the feature.

Proposal 2	In NTNs, when Msg4 PUCCH HARQ-ACK repetition feature is enabled in SIB, one of the R bits in the MAC subheader of CCCH LCH in Msg3 is used to indicate a request/capability for Msg4 PUCCH HARQ-ACK repetition for the UEs that support the feature. Setting this bit to 1 indicates request/capability for Msg4 PUCCH HARQ-ACK repetition; and setting it to 0 indicates no request or no capability for Msg4 PUCCH HARQ-ACK repetition.

# References

R1-2306105 LS on higher layer signaling in Msg3 PUSCH for PUCCH repetition for Msg4 HARQ-ACK (R1-2306105; contact: NTT DOCOMO)	RAN1	LS in	Rel-18	NR_NTN_enh-Core	To:RAN2