3GPP TSG-RAN WG2 Meeting #123	R2-2308543

# Toulouse, France, August 21 – 25, 2023

Agenda Item:	7.5.4.3

Source:	Ericsson

Title:	Configured Grant enhancements for XR

Document for:	Discussion, Decision

# 1	Introduction

In the Rel-18 XR WID [1] we have two objectives related to configured grant capacity enhancements:

Specify the enhancements related to capacity: - Multiple Configured Grant (CG) PUSCH transmission occasions in a period of a single CG PUSCH configuration (RAN1, RAN2); - Dynamic indication of unused CG PUSCH occasion(s) based on Uplink Control Information (UCI) by the UE (RAN1, RAN2); - Buffer Status Report (BSR) enhancements including at least new Buffer Status Table(s) (RAN2); - Delay reporting of buffered data in uplink (RAN2); - Discard operation of PDU Sets for DL and UL (RAN2, RAN3);

In this paper we discuss the WID’s two highlighted configured grant enhancements and the topics raised in RAN1 LS on XR capacity enhancements R1-2306233 [3].

# 2	Configured Grant enhancement

## 2.1 Multi-PUSCH CG

In RAN1 #113 [2] there were agreements relevant for multi-PUSCH CG.

RAN1 have finally agreed on the configuration possibilities of the multi PUSCH CG, we believe those will (if any change needed) only affect the configuration parameters in RAN2 spec and which are normally sent from RAN1 to RAN2 in an excel list common for all features close to the end of the release.

One of the agreements were included in the RAN1 LS on XR capacity enhancements R1-2306233 [3]:

Agreement From RAN1 perspective, for determination of HARQ process IDs associated to PUSCHs in multi-PUSCHs CG assuming one TB per PUSCH: - The HARQ process ID for the first configured/valid PUSCH in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured, and applying the following formula, whichever is applicable - HARQ Process ID = [X*floor( (CURRENT_symbol ) / periodicity)] modulo nrofHARQ-Processes - HARQ Process ID = [X*floor((CURRENT_symbol ) / periodicity)] modulo nrofHARQ-Processes + harq-ProcID-Offset2 - X= the number of configured PUSCHs in the CG period - The HARQ process ID of the remaining configured/ and valid CG PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period by one with module operation with nrofHARQ-Processes or module operation with (nrofHARQ-Processes + harq-ProcID-Offset2), whichever applicable. - Note: A configured CG PUSCH is invalid if the CG PUSCH is dropped due to collision with DL symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or SSB.

The HARQ process IDs for the dynamic grant multi PUSCH scheduling in Release-17, the HP ID for the first transmission is indicated in the DCI and the remaining HP IDs are calculated as described in 38.214 section 6.1.

For legacy CGs however, the HP IDs are all described in the MAC spec and we think it is fine to keep it that way for multi PUSCH CGs. Therefore, we propose to capture the yellow highlighted part and the grey highlighted part directly in the MAC spec (section 5.4.1, as for legacy CGs). For the turquois highlighted part, which is legacy, we can add a reference to the RAN1 spec 38.214.

We note that the RAN1 agreement uses a “in a period” that is not consistent with the MAC spec and the RRC parameter periodicity that describes the period of a CG.

For multi PUSCH CGs, capture the formula for the HARQ process ID for the first and the remaining configured grant transmission opportunities within one periodicity in the MAC spec section 5.4.1. Add a reference to 38.214 for the RAN1 specified valid/invalid transmission opportunities.

Discuss whether RAN1 needs to be informed about the RAN2 agreement on where to capture HARQ process IDs for multi PUSCH configured grants.

## 2.2 Dynamic indication of unused CG

RAN1 has introduced the terminology unused transmission opportunity (UTO) in uplink control information (UCI), or UTO-UCI, for the indications from a UE to the gNB on CG transmission opportunities (TOs) that will be unused.

RAN1 is discussing solutions for overprovisioning the UE with configured grants (multiple CGs or CGs with multiple transmission opportunities), and then letting the UE indicate to the gNB which of the future configured grant TOs that it will not use with an UTO-UCI.

The discussions of scheduling and use or not use of CGs is a RAN2 issue and not for RAN1 to decide since the decision of whether to use or not use a CG resource depends on available data status which is today available in MAC.

The latest agreements made in RAN1 #113 regarding UTO-UCI are found in [2].

RAN1 have not concluded on the functionality yet as some FFS remains, but there are some issues that need RAN2 attention now:

- MAC shall be aware of TOs that will not be used such that MAC PDUs obtained are not dropped in the PHY layer.

- To decide if a future TO is needed to be used, the PHY is not aware of data volume available for transmission.

- MAC is aware of data volume available for transmission in the buffer status reporting procedure.

- RLC and PDCP has data volume calculation functionality for the purpose of reporting to MAC in the buffer status reporting.

The MAC entity shall be aware of configured grant transmission opportunities that will not be used and not generate MAC PDUs for them. This includes transmission opportunities that PHY determines to be invalid.

- PHY is not aware of data volume available for transmission, MAC is aware of data volume available for transmission for the purpose of buffer status reporting.

Therefore, we think the best place for the functionality to select future TOs to be indicated as unused is in the MAC layer.

MAC entity select future configured grant transmission opportunities that are to be indicated as unused.

The MAC selection algorithm for UTO-UCI can be describe in a new section for example 5.8.2x or 5.4.x.

MAC layer indicates, to lower layers, the future transmission opportunities that shall be unused and that shall be indicated as unused in the UTO-UCI.

RAN1 have agreed that a TO that has been indicated as “unused” TO in an UTO-UCI at a later occasion cannot be used for transmission. While for TOs that has been indicated as “NOT unused” in a UTO-UCI can later be indicated as “unused” TO (which is similar to skip uplink functionality, while the UTO-UCI is sent earlier than the actual TO).

MAC entity shall consider all CG TOs indicated as “unused” as unavailable for transmissions. That is, no overriding for TOs that have been indicated as “unused”.

MAC entity is allowed to indicate a TO as “unused” even if it has previous been indicated as “used”.

### 2.2.1 Data volume available for transmission

The selection algorithm will at least need to be aware of how much data that is currently available for transmission.

Therefore, we need to update the purpose of data volume reporting in RLC and PDCP from “for MAC buffer status reporting” to include “for the purpose of decision on future unused transmission opportunities” or change to “for the purpose of informing MAC entity of data volume available in PDCP/RLC. We propose:

The purpose of data volume calculation functionality in RLC and PDCP is updated to say “for the purpose of informing the MAC entity of data volume available in RLC respectively in PDCP”

### 2.2.2 Selection algorithm for UTO-UCI indication

The network shall be able to adapt unused TO selection depending on the network load, the QoS requirements of the UEs, the traffic behaviour for a UE, or depending on the mix of UEs in the system. Otherwise, the feature may not show any gain or there may only be gain in certain scenarios. There is also a risk that UEs becomes greedy and indicates all future TOs as “used” to achieve better QoS which would be detrimental to network capacity.

Therefore, the network shall be in control of the UEs selection of “unused” TOs.

The network controls the UE algorithm for unused TO selection that are indicated in the UTO-UCI.

There are many possible algorithms, two useful may be:

- The simplest useful algorithm may be that the UE indicate as “used” the TOs needed to empty the current buffer, and future TOs are indicated as “unused”. This can be further improved by complementing with a timer:

  - A timer is started for each new data arrival. While the timer is running the UE shall indicate the earliest TOs as “used” until the TOs empty the buffer and the remaining TOs before the timer would expire shall be indicated as “unused”. For TOs after the timer would expire the UE can itself select to indicate as “used” or “unused”.

- Another simple algorithm for selecting “used” and “unused” can be that the UE is configured with a sliding window, and within that sliding window, the UE must indicate at least a percentage of the TOs as “unused”. For TOs outside of the current window, the UE itself may choose between “used” and “unused” indication.

The algorithms can be combined, and the restriction of not changing from “unused” to “used” must be observed.

The algorithm A1 is good if data in a periodicity always arrive at the same time.

The algorithm B is good if the networks need to ensure that there is a percentage of the resources allocated to CGs that can be used for dynamic scheduling.

RAN2 to discuss the selection algorithm for indicating future used and unused TOs.

# 3 Conclusion

In the previous sections we made the following observations:

Observation 1	PHY is not aware of data volume available for transmission, MAC is aware of data volume available for transmission for the purpose of buffer status reporting.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For multi PUSCH CGs, capture the formula for the HARQ process ID for the first and the remaining configured grant transmission opportunities within one periodicity in the MAC spec section 5.4.1. Add a reference to 38.214 for the RAN1 specified valid/invalid transmission opportunities.

Proposal 2	Discuss whether RAN1 needs to be informed about the RAN2 agreement on where to capture HARQ process IDs for multi PUSCH configured grants.

Proposal 3	The MAC entity shall be aware of configured grant transmission opportunities that will not be used and not generate MAC PDUs for them. This includes transmission opportunities that PHY determines to be invalid.

Proposal 4	MAC entity select future configured grant transmission opportunities that are to be indicated as unused.

Proposal 5	The MAC selection algorithm for UTO-UCI can be describe in a new section for example 5.8.2x or 5.4.x.

Proposal 6	MAC layer indicates, to lower layers, the future transmission opportunities that shall be unused and that shall be indicated as unused in the UTO-UCI.

Proposal 7	MAC entity shall consider all CG TOs indicated as “unused” as unavailable for transmissions. That is, no overriding for TOs that have been indicated as “unused”.

Proposal 8	MAC entity is allowed to indicate a TO as “unused” even if it has previous been indicated as “used”.

Proposal 9	The purpose of data volume calculation functionality in RLC and PDCP is updated to say “for the purpose of informing the MAC entity of data volume available in RLC respectively in PDCP”

Proposal 10	The network controls the UE algorithm for unused TO selection that are indicated in the UTO-UCI.

Proposal 11	RAN2 to discuss the selection algorithm for indicating future used and unused TOs.

# 4 References

- RP-230786, “Updated WID on XR Enhancements for NR”, 3GPP TSG RAN Meeting #99, 20-23 Mar. 2023.

- RP-231056, “Status Report to TSG”, 3GPP TSG RAN Meeting #100, 12-14 June 2023.

- R1-2306233, “LS on XR capacity enhancements”, 3GPP TSG RAN WG1 #113, 22-26 May 2023.