3GPP TSG-RAN WG2 Meeting #123											R2-2308562

Toulouse, France, 21– 25 August, 2023

Agenda item:       6.6.1

Source: 	Huawei, HiSilicon

Title: 	Summary on RRC corrections for SL enhancements

Document for:	Discussion and Decision

# Introduction

This document summarizes the contributions submitted to AI 6.6.1 on RRC corrections. Rapporteur’s recommendations on how to treat them are provided.

# Correction CRs to be discussed together with Rapporteur’s Misc CR

The following correction CRs could be discussed together. The agreed changes for RRC spec could be merged into a Rapporteur’s miscellaneous Rel-17 correction CR.

Table 1: Correction CRs

| Tdoc | Title | Company | Summary of changes/proposals | Rapp’s remark |
| --- | --- | --- | --- | --- |
| R2-2307561 | Miscellaneous corrections for SL enhancements | Huawei, HiSilicon, OPPO | 1. Adding DCR message for ProSe service, as specified in 24.554, into the field description of default DRX parameter. 2. For IE SL-InterUE-CoordinationConfig, add one sentence description that IUC is "between a UE, UE-A, and a peer UE, UE-B". 3. Editorial corrections. |  |
| R2-2307981 | Correction to 38.331 on IUC | vivo | In the IE SL-InterUE-CoordinationConfig, remove terminologies “UE-A” and “UE-B” from related field descriptions.Some editorials still exist. | The changes are to solve the same issue as the 2nd change in Rapp CR, however with the opposite approach (to clear the terms of UE-A/UE-B instead of clarifying them). Rapp thinks, even we can clear the terms UE-A/UE-B in field descriptions as we did in 300, "uea/ueb" are still embedded in ASN.1 values in sl-DetermineResourceType-r17, UE-A or UE-B are still in names sl-IndicationUE-B, sl-TypeUE-A. Rapp recommends to use 2nd change in Rapp Misc CR with much less changes and still achieve the purpose (to clarify the terms), i.e. to keep terms UE-A and UE-B in 331 with the added clarification. |
| R2-2307483 | Correction on SUI transmission for sidelink DRX | ZTE Corporation, Sanechips | According to current description on how to set the contents of the SidelinkUEInformationNR message procedure, the parent IE of sl-Failure is sl-sl-TxResourceReqList and/or sl-TxResourceReqListCommRelay. However, in fact, the sl-Failure is included in SL-failurelist, not sl-TxResourceReqList. Modify the normative text of transmission of SUI to include sl-failure into correct parent IE. Rapp's implementation option 1: Rapp's implementation option 2: | Rapp agrees with the motivation of this change, and field "sl-Failure" should be set in " sl-FailureList". On the other hand, this part of procedure texts are added upon R16 procedure texts starting with "3> if configured by upper layers to transmit non-relay NR sidelink communication:", and are used when DRX is enabled. Thus, no description on setting destination identity is needed for "sl-FailureList" or "sl-TxResourceReqList" or "sl-TxResourceReqListCommRelay". (Rapp option 1) Further, Rapp thinks the alternative option 2 could be viable implementation as well. |

Proposal: Discuss the CRs listed in Table 1 offline, the agreed changes for RRC spec could be merged into Rapporteur’s miscellaneous correction CR.

# Conclusions

The contributions submitted to AI 6.6.1 are summarized, Rapporteur’s recommendations are as follows.

Proposal: Discuss the CRs listed in Table 1 offline, the agreed changes for RRC spec could be merged into Rapporteur’s miscellaneous correction CR.