3GPP TSG RAN WG2 #123	                                R2-2308576

Toulouse, France, 21st – 25th August, 2023

Agenda Item:	7.6.2.1

Source:	InterDigital, Inc.

Title:	Remaining Issues on Disabling HARQ feedback for IoT-NTN.

Document for:	Discussion

# Introduction

The WI on IoT NTN enhancements [1] includes the following objectives for performance enhancement:

- Disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates [RAN1,RAN2]

In RAN2#119-e the following agreements were made:

Agreements:

- Disabling DL HARQ feedback is supported for NB-IoT and eMTC NTN. FFS on UE capability

- For UL HARQ operation, introduce two HARQ modes, i.e., HARQ mode A and HARQ mode B in IoT NTN (both NB-IoT and eMTC NTN), similarly to NR NTN

- From RAN2 perspective, at least for eMTC, enabling/disabling HARQ feedback can be configured per DL HARQ process at least via UE specific RRC signalling. FFS for NB-IoT (and especially for CP solution for NB-IOT).

In RAN2#119bis-e the following agreements were made:

Agreements:

- For NB-IoT, enabling/disabling HARQ feedback can be configured per DL HARQ process at least via UE specific RRC signaling (e.g. RRCConnectionSetup). This does not preclude other options (e.g. DCI-based). We can also revert this decision if requested by RAN1.

- Disabling HARQ feedback is supported for NB-IoT with single HARQ process, and it is up to eNB implementation whether to disable the HARQ feedback

Working Assumption:

- Blind retransmission can be used in IoT NTN when HARQ feedback is disabled and when HARQ mode B is used (RAN2 assumes there is no spec change for this)

- HARQ mode A/B for uplink transmission may be configured per UL HARQ process at least via UE specific RRC signalling for eMTC and NB-IOT NTN. We can also revert this decision if requested by RAN1

- RAN2 agree to take R17 NR NTN DRX solution as baseline for IoT NTN, e.g. for HARQ process with DL HARQ feedback disabled, the UE will not start the corresponding DL HARQ RTT timer.

- For NB-IoT NTN with single HARQ process when the HARQ feedback is disabled, the UE will start/restart drx-inactivity timer in the subframe containing the last repetition of the corresponding PDSCH reception (can still check whether the alternative to set the HARQ RTT timer to 0 also works)

- RAN2 agree to take R17 NR NTN DRX solution as baseline for IoT NTN, e.g. for HARQ process in HARQ mode B, the UE will not start the corresponding UL HARQ RTT timer.

- For NB-IoT NTN with single HARQ process in HARQ mode B, the UE will start/restart drx-inactivity timer in the subframe containing the last repetition of the corresponding PUSCH transmission (can still check whether other alternatives also work)

- The solutions of LCP restriction on allowed HARQ mode in NR NTN can be reused for eMTC NTN.

In RAN2#120 the following agreements were made:

Agreements: 1. For eMTC, the following LCH to HARQ process mapping rules are supported: 1) LCH is mapped only to a HARQ process configured with HARQ mode A; 2) LCH is mapped only to a HARQ process configured with HARQ mode B; 3) If an LCH is not configured with a mapping rule, it may be mapped to any HARQ process (HARQ mode A or B). 4) If UL HARQ mode is not configured, LCH mapping rules are not supported (legacy behaviour) 2. For eMTC, introduce allowedHARQ-mode for each logical channel, e.g. included in LogicalChannelConfig IE.

In RAN2#121bis-e the following agreements were made:

- RAN2#121’s agreement is revised to “For NB-IoT NTN with single HARQ process when the HARQ feedback is disabled, the UE will start/restart drx-inactivity timer in the subframe containing the last repetition of the corresponding PDSCH reception plus 12 subframes plus deltaPDCCH” (Can further check in the NB-IoT session if anything needs to be done for legacy NB-IoT as well, as some timers don’t take deltaPDCCH into account) - Wait for RAN1’s decision on the RRC signalling of enabling DCI-based solution to indicate HARQ feedback enabled/disabled, and the signalling granularity, e.g. per UE or per HARQ process - On DCI indication overriding RRC configuration for the HARQ feedback enabled/disabled, wait for RAN1’s progress on DCI-based solution before discussing related DRX impact in RAN2. - On DL multiple TB scheduling, wait for RAN1’s progress before discussing related DRX impact in RAN2. - P4 in R2-2302557 is not agreed, i.e. no special handling for single HARQ process for eMTC. - For eMTC NTN, a parameter harq-FeedbackEnablingforSPSactive could be configured for a UE. If harq-FeedbackEnablingforSPSactive is configured to enable HARQ feedback, UE reports ACK/NACK for the first SPS PDSCH after activation, regardless of if HARQ feedback is enabled or disabled corresponding to the first SPS PDSCH after activation. - For a NB-IoT UE configured with a single HARQ process in HARQ mode B, send LS to RAN1 and ask for the “processing time for starting drx-InactivityTimer (i.e. start to monitor NPDCCH)”. (can further check the detailed wording of the question) - Network implementation resolves the issue of ambiguity on start of DRX inactivity timer after the PUSCH transmission by not scheduling the NPDCCH back-to-back during the ambiguity period (i.e., Koffset – UE’s TA) - Send LS to RAN1 to check for UL multiple TB scheduling, which UL HARQ mode combination(s) are to be supported. - In the LS to RAN1, we don’t include a question on whether RAN1 intends to introduce the DCI-based solution for the UL HARQ mode - UL transmission using SPS can be configured with HARQ mode B - P1 in R2-2303713 is not agreed, i.e. do not enhance the LCP restriction based on uplinkHARQ-Mode for different RLC PDU types - Send LS to RAN1 informing RAN2’s agreements and also including potential questions to be checked with RAN1 - Add one more question in the LS to check with RAN1 which of the below understandings is correct for the RAN1 agreement. - Understanding 1: For a DL HARQ process with disabled HARQ feedback in NB-IoT, UE is not required to monitor NPDCCH for the same HARQ process in a period of Y=12(ms) from the end of reception of the NPDSCH. - Understanding 2: For a DL HARQ process with disabled HARQ feedback in NB-IoT, UE is not required to monitor NPDCCH for all the HARQ processes in a period of Y=12(ms) from the end of reception of the NPDSCH. - RAN2 further discuss whether UL transmission using PUR can be configured with HARQ mode B.

In addition, RAN1 has made the following agreements.

Conclusion For eMTC HD-FDD single TB scheduled by single DCI, UE is not expected to receive a DCI with “HARQ-ACK bundling flag” field set to 1 in case the corresponding HARQ process is configured with HARQ feedback disabled by RRC signaling. Agreement For a DL HARQ process with disabled HARQ feedback in eMTC, UE is not expected to receive another MPDCCH carrying a DCI scheduling a PDSCH for a given HARQ process or to receive another PDSCH without corresponding MPDCCH for the given HARQ process that starts at a BL/CE DL subframe until X=3 (ms) have passed after the end of the reception of the last PDSCH for that HARQ process. Agreement For HARQ feedback for eMTC SPS PDSCH, at least the following is supported: UE follows the per-process HARQ feedback enabled/disabled configuration for the associated HARQ process except for the first SPS PDSCH after activation - for the first SPS PDSCH after activation, - Option 1: If HARQ feedback for SPS activation is additionally enabled, ACK/NACK is reported by UE for the first SPS PDSCH after activation regardless of network configuration of enabled/disabled for this HARQ process, and follow per-process HARQ feedback enabled/disabled configuration otherwise. Conclusion For DCI indicating SPS PDSCH release, HARQ-ACK report is performed as legacy in eMTC, regardless of HARQ feedback enabled/disabled configuration. Agreement For DCI-based overridden mechanism/indication in single TB scheduled by DCI, down select one of the following alternatives based on the criteria DCI overhead, PDCCH monitoring/power consumption, HARQ timer, impact on scheduling flexibility, UE implementation complexity - Alternative 1: applies to both semi-statically HARQ enabled and disabled processes - Alternative 2: only applied to semi-statically HARQ disabled processes - Alternative 3: only applied to semi-statically HARQ enabled processes Agreement Confirm the following working assumption with the following update: Working assumption For NB-IoT NTN and eMTC NTN for CE Mode B, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission: - Support Option 1 in case only per-HARQ process bitmap signaling is configured - Support Option 3 DCI direct indication of HARQ feedback enable/disable in case only DCI solution enabling/disabling signaling is configured - Support Option 3 DCI indication to override Option 1 configuration for corresponding transmission in case both per-HARQ process bitmap and DCI solution enabling/disabling signaling are configured - FFS #1: Option 3 DCI-based overridden mechanism is applied to both semi-statically HARQ feedback enabled and disabled processes or only applied to semi-statically HARQ feedback disabled processes or only applied to semi-statically HARQ feedback enabled processes. - FFS #2: whether/how to support Option 3 overriding Option 1 configuration for corresponding transmission for multiple TBs scheduled by single DCI - FFS#3：Option 3 DCI-based overridden mechanism is DCI signaling to reverse the HARQ feedback enable/disable for the corresponding transmission from per-HARQ process RRC configuration or DCI signaling to directly indicate the HARQ feedback enable/disable for the corresponding transmission regardless of per-HARQ process RRC configuration. RAN1 strives to have a common design (in terms of DCI design, PDCCH monitoring, etc.) for “Option 3” and “Option 3 + Option 1”. For eMTC NTN, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission, take Option 1 for CE Mode A. Agreement For DCI-based overridden/direct indication, down select one of the following based on the criteria DCI overhead, PDCCH monitoring behavior, impact on scheduling flexibility, UE implementation complexity, etc - Option 1: Indication by adding one field in DCI - Option 2: Indication by reusing/reinterpreting existing field in DCI 112bis-e : Agreement For Option 3 DCI indication: - Option A: when both per-HARQ process bitmap and DCI solution enabling/disabling signaling are configured - DCI-based overridden mechanism is DCI signaling to reverse the HARQ feedback enable/disable for the corresponding transmission from per-HARQ process RRC configuration - For single TB scheduled by DCI, the DCI based overridden indication is applied to one of the following options (to be down-selected): - Option A-1: only applied to semi-statically HARQ disabled processes - Option A-4: applied to both semi-statically HARQ disabled and enabled processes - FFS for multiple TBs scheduled by single DCI - Option B: DCI-based HARQ enabling/disabling direct indication in case DCI solution enabling/disabling signaling is configured and per-HARQ process bitmap signaling is not configured (i.e. no bitmap is configured) - DCI-based mechanism is DCI signaling to directly indicate the HARQ feedback enable/disable for the corresponding transmission - For single TB scheduled by DCI, DCI-based direct indication is applied to the scheduled TB - FFS for multiple TBs scheduled by single DCI Agreement For single TB scheduled by DCI, for DCI-based direct indication, down select one of the following based on the criteria DCI overhead, PDCCH monitoring behavior, impact on scheduling flexibility, UE implementation complexity, etc - Option 1: Indication by adding one field in DCI (e.g., 1-bit) - Note: Other fields in DCI are the same as legacy. - Option 2: Indication by reusing/reinterpreting existing field in DCI - Option 2A: HARQ-ACK related field - For eMTC CE mode B, one state of “HARQ-ACK resource offset” field in DCI format 6-1B is used for indication of HARQ feedback disabled, other states are used for indication of HARQ feedback enabled and corresponding HARQ-ACK resource. - FFS: detailed state - For NBIoT, one state of “HARQ-ACK resource” field in DCI format N1 is used for indication of HARQ feedback disabled, other states are used for indication of HARQ feedback enabled and corresponding HARQ-ACK resource. - FFS: detailed state - Option 2B: MCS or repetition number field - Reduce 1bit of legacy MCS or repetition number field and add 1bit new field in DCI format 6-1B and N1 to indicate the HARQ feedback enabled/disabled - FFS: detailed for interpreting of the reduced MCS or repetition number field - Option 2C: HARQ-ACK related field v2 - For eMTC CE mode B, reduce 1bit of legacy “HARQ-ACK resource offset” field and add 1bit new field in DCI format 6-1B to indicate the HARQ feedback enabled/disabled - FFS: detailed for interpreting of the reduced “HARQ-ACK resource offset” field - For NBIoT, reduce 1bit of legacy “HARQ-ACK resource” field and add 1bit new field in DCI format N1 to indicate the HARQ feedback enabled/disabled - FFS: detailed for interpreting of the reduced “HARQ-ACK resource” field - Option 2D: Other indication by reusing/reinterpreting existing field RAN1#113 Agreement For single TB scheduled by DCI, - Working assumption 1 DCI based overridden indication is applied to both semi-statically HARQ disabled and enabled processes - For DCI based overridden indication, adopt indication by reusing/reinterpreting HARQ-ACK related field in DCI - For eMTC CE mode B, “HARQ-ACK resource offset” field in DCI format 6-1B is used for indication of maintaining/reversing the HARQ feedback enable/disable for the corresponding transmission from per-HARQ process RRC configuration and corresponding HARQ-ACK resource in case of indication of HARQ feedback enabled. - HARQ feedback disabled is reversed to enabled in case of any states other than state A in “HARQ-ACK resource offset”, otherwise is maintained as disabled. - HARQ feedback enabled is maintained in case of any states other than state A in “HARQ-ACK resource offset”, otherwise is reversed to disabled. - FFS: detailed state A, and whether this state A is different across different UEs - For NBIoT, “HARQ-ACK resource” field in DCI format N1 is used for indication of maintaining/reversing the HARQ feedback enable/disable for the corresponding transmission from per-HARQ process RRC configuration and corresponding HARQ-ACK resource in case of indication of HARQ feedback enabled. - The same DCI indication functionality as eMTC is adopted. - Working assumption 2 For Option 1 + Option 3 DCI based overridden mechanism, for a HARQ process configured as HARQ feedback disabled by per-HARQ process bitmap signaling and further reversed to HARQ feedback enabled by DCI, the NBIoT UE does not wait for an RTT+3ms (i.e., till subframe n+Kmac+3 in TS36.213 section 16.6) before monitoring NPDCCH for the same HARQ process (or monitoring any NPDCCH for the case of single HARQ process configuration). - Send an LS to RAN2 with the following contents: - RAN1 respectfully ask RAN2 for the feasibility of Working assumption 2 (taking into account potential RAN2 spec impact). Agreement The draft LS in R1-2306205 is endorsed. Final LS in R1-2306245. Agreement For the RRC configuration of DCI solution enabling/disabling of HARQ feedback for NB-IoT and LTE-MTC in CE Mode B, the RRC configuration is UE-specific. Agreement for NB-IoT and LTE-MTC in CE Mode B, if multiple TBs is configured, for DCI-based HARQ enabling/disabling direct indication in multiple TBs scheduled by single DCI, the same indication is applied to all scheduled TBs, i.e. HARQ is enabled or disabled for all TBs. Agreement Adopt the response below to RAN2’s question 3: - Understanding 2 is the correct understanding. Agreement For a NB-IoT UE operating with two HARQ processes, for an UL HARQ process with HARQ mode B, the minimum time between the end of NPUSCH transmission and the start of NPDCCH monitoring for the same HARQ process is 1 ms. - Note: this implies a RAN1 specification change in Rel-18 Agreement For the response to RAN2’s question 1a: provide the agreement above. Agreement Adopt the response below to RAN2’s question 1b: - For an UL HARQ process with HARQ mode B for eMTC UEs, the minimum time between the end of PUSCH transmission and the start of MPDCCH monitoring for the same HARQ process is 1 ms for Type B half-duplex FDD operation, and there is no minimum time restriction between the end of PUSCH transmission and the start of MPDCCH monitoring for full-duplex FDD operation. Agreement Adopt the response below to RAN2’s question 2: - Whether/how to support case 1, case 2 and/or case 3 for UL multiple TB scheduling for eMTC and NB-IoT is transparent to RAN1 and can be left to RAN2 decision. Agreement For a NB-IoT UE operating with one HARQ process, for an UL HARQ process with HARQ mode B, the minimum time between the end of NPUSCH transmission and the start of NPDCCH monitoring is 1 ms. - Note: this implies a RAN1 specification change in Rel-18 Agreement - For the response to RAN2’s question 1a: provide the agreement above.

In this contribution we elaborate the potential impact.

# Background

## 2.1	Disabled HARQ feedback in Rel-17 NR NTN

Configuration of HARQ characteristics per HARQ process ID

A UE may be semi-statically configured via RRC to apply a specific HARQ behaviour to a set of HARQ process IDs. This configuration is per serving cell, and can be optionally configured for both UL and DL HARQ processes via the following optional configurations:

- downlinkHARQ-feedbackDisabled: configured per HARQ process ID, and indicates whether DL HARQ feedback is enabled or disabled.

- uplinkHARQ-Mode: Configured per HARQ process ID, and indicates whether an UL HARQ process uses HARQModeA or HARQmodeB.

  - RAN2 understanding is that HARQmodeA best applies to transmissions with UL HARQ retransmission enabled, and HARQmodeB best applies to transmissions with UL HARQ retransmission disabled or with blind UL retransmission.

HARQ-based LCP restriction

A UE may be configured to apply an LCP restriction based on the UL HARQ mode configured for the HARQ process ID an UL grant is assigned to. UE behaviour is specified via two optional RRC configurations:

- uplinkHARQ-Mode, which configures each HARQ process ID as either HARQModeA or HARQModeB, and;

- allowedHARQ-Mode, which is configured per logical channel and sets the allowed HARQ mode of a HARQ process mapped to this logical channel.

Upon reception of a new UL grant, the UE will determine if allowedHARQ-mode is configured for this LCH, and if a HARQ Mode has been configured for the HARQ process the UL grant is assigned to. If both are configured and the LCH is allowed to be mapped to the HARQ mode, the restriction is satisfied and data from this logical channel may be mapped to the UL grant. If either uplinkHARQ-Mode or allowedHARQ-Mode are not configured, the UE may map this logical channel to any HARQ process.

Adaptation of DRX based on configured HARQ mode

A UE adapts DRX timers based on the configured HARQ characteristics of a HARQ process. DRX is adapted for both UL and DL to either adapt DRX active time account for additional propagation delay (e.g. if HARQ feedback is enabled) or to enabled additional UE power saving (e.g. if HARQ feedback is disabled). Operation is adapted as follows:

- DL: if downlinkHARQ-FeedbackDisabled is configured for this serving cell, upon DL reception:

  - if HARQ feedback is enabled for the HARQ process the UE will extend the length of the DL HARQ RTT Timer by the UE-gNB RTT (i..e propagation delay).

  - If HARQ feedback is disabled for the HARQ process, the UE will not start drx-RetransmissionTimerDL to enable additional power saving.

  - If  downlinkHARQ-FeedbackDisabled is not configured, the UE will apply legacy behaviour (i.e. start drx-RetransmissionTimerDL after expiry of drx-HARQ-RTT-TimerDL).

- UL:  if uplinkHARQ-Mode is configured for this serving cell, upon UL transmission:

  - if HARQ process is configured as HARQModeA the UE will extend the length of the DL HARQ RTT Timer by the UE-gNB RTT (i..e propagation delay).

  - If  HARQ process is configured as HARQModeB, the UE will not start drx-RetransmissionTimerDL to enable additional power saving.

  - If   uplinkHARQ-Mode is not configured, the UE will apply legacy behaviour (i.e. start drx-RetransmissionTimerUL after expiry of drx-HARQ-RTT-TimerUL).

## 2.2	HARQ for NB-IoT/eMTC

Asynchronous HARQ is used in uplink for NR, however in LTE uplink HARQ can be synchronous or asynchronous:

- For asynchronous HARQ, the network may schedule any HARQ process by indicating a HARQ process ID in the DCI scheduling the HARQ transmission. The DCI further indicates whether a new transmission or a retransmission is scheduled for that HARQ process via the NDI bit.

- For synchronous HARQ, the HARQ process is determined via uplink timing. For example, when 8 HARQ processes are configured, a given HARQ process will transmit every 8ms (TTIs). Whether a MAC PDU can be (re)transmitted depends on the PHICH, which carries HARQ ACK/NACK information for PUSCH transmissions.

For eMTC and NB-IoT, uplink HARQ operation is asynchronous except for repetitions within a bundle. The number of UL repetitions in a bundle is given by parameter UL_REPETITION_NUMBER, and is set (per bundle) via indication from lower layers. The UL bundling operation has the following characteristics:

- The bundling operation uses the same HARQ process for each transmission belonging to the same bundle;

- Within a bundle, HARQ retransmissions are non-adaptive and triggered without waiting for HARQ feedback for previous transmissions.

- An uplink grant corresponding to a new transmission or a retransmission of the bundle is only received after the last repetition of the bundle.

- A retransmission of a bundle is also a bundle.

Similarly, in the DL the parameter DL_REPETITION_NUMBER provides the number of transmissions repeated in a bundle and is also set per bundle by lower layers. The DL bundling operation has the following characteristics:

- Within a bundle, after the initial (re)transmission, DL_REPETITION_NUMBER-1 HARQ retransmissions follow.

- HARQ feedback is transmitted for the bundle

- A downlink assignment corresponding to a new transmission or retransmission of the bundle is received after the last repetition of the bundle.

- A retransmission of a bundle is also a bundle.

# Adaptation of DRX based on DCI configured HARQ mode

Since enabling/disabling of HARQ for the associated DL transmission with the DCI is not specified in NR we cannot exactly use this as a baseline. Adaptation of the DRX timers needs to take into account a potential dynamic change of setting of DL HARQ mode by L1. This behaviour needs to be specified in MAC. However, based on the current NR specification, the procedure sets the value of the relevant timers based on RRC setting only. For example, the NR text is as follows:

When DRX is configured, the MAC entity shall: - if a MAC PDU is received in a configured downlink assignment for unicast: 2> if this Serving Cell is configured with downlinkHARQ-FeedbackDisabled: 3> if the corresponding HARQ process is configured with HARQ feedback enabled: 4> set HARQ-RTT-TimerDL-NTN for the corresponding HARQ process equal to drx-HARQ-RTT-TimerDL plus the latest available UE-gNB RTT value; 4> start the HARQ-RTT-TimerDL-NTN for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback. 2> else: 3> start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

Regardless of the outcome of RAN1 discussions for the method of DCI signalling, RAN1 specification should capture that an indication is provided to upper layer (i.e. MAC) when a DCI indicating HARQ feedback enable/disable, and the details of how this indication is determined from DCI is RAN1 scope, both in terms of discussion and specification impact. From RAN2 point of view, all that is needed is to re-use the text from NR MAC specification can be re-used, with some additional behaviour compared to NR specified such that the configuration of HARQ feedback enabled/disabled can be done via RRC or via DCI.

One way would be to use a similar sentence as in NR, but rather than “configured” which implies RRC, to use a different wording. For example:

3>	if the corresponding HARQ process has HARQ feedback enabled

Some additional text would be needed (e.g. in a separate section) to specify that a HARQ process can have HARQ feedback enabled or disabled based on RRC configuration or DCI indication. Any final adjustments (e.g. to make the NR text compatible/aligned with LTE specification) can be made during stage 3 review.

Proposal 1a: The text from NR MAC spec is used as a baseline.

Proposal 1b: Specify in MAC that a HARQ process can have HARQ feedback enabled or disabled, based on RRC configuration or explicit indication from physical layer.

Proposal 1c: Any further changes (in addition to the above) can be discussed in stage 3 review.

RAN1 agreed that for the RRC configuration of DCI solution enabling/disabling of HARQ feedback for NB-IoT and LTE-MTC in CE Mode B, the RRC configuration is UE-specific.

Proposal 2a: A single flag is introduced for configuring DCI based HARQ feedback enable/disable.

For the cases where RRC is used, an initial setting needs to be provided. It has already been agreed that HARQ feedback can be configured per DL HARQ process at least via UE specific RRC signaling (e.g. RRCConnectionSetup). Since it is possible to configure DCI based only then this setting needs to be an optional field, and the most efficient signalling method would be to use a bitmap.

Proposal 2b: Introduce an optional RRC bitmap with a value per HARQ process to indicate the initial setting (enabled/disabled) for each HARQ process. (Similar to NR)

For the case that DCI based indication is not enabled, but RRC bitmap is signalled, then the only possibility is that the HARQ mode is based on the configuration in RRC.

For the case that DCI based indication is used, but RRC is not used, then RAN1 have agreed that DCI signaling is used to directly indicate the HARQ feedback enable/disable for the corresponding transmission. In our understanding this means that every transmission will include an indication from the physical layer which HARQ mode to use.

For the case that DCI based indication is used and RRC is used, each transmission may or may not include an indication from lower layers. RAN1 agreed for this case that the DCI indication will toggle/reverse the RRC setting. In our understanding, such behavior shall be captured in RAN1 specification such that lower layers indicate an explicit HARQ mode regardless of whether RRC has been used to provide an initial setting or not. This simplifies RAN2 specification somewhat.

Proposal 3: RAN2 understanding of the variety of options agreed in RAN1:

- If DCI based signalling is disabled and the RRC bitmap is absent then this indicates the behaviour is as per legacy (i.e. HARQ feedback is enabled for all HARQ processes).

- If DCI based signalling is disabled and the RRC bitmap is present then this indicates the HARQ mode setting is always based on the RRC setting (like in NR).

- If DCI based signalling is enabled and the RRC bitmap is absent then this indicates the HARQ mode setting will be explicitly provided by DCI for every transmission.

- If DCI based signalling is enabled and the RRC bitmap is present then this indicates the HARQ mode setting provided by RRC may be toggled by DCI for some transmissions, otherwise the RRC setting is used.

From RAN2 point of view, the variety of configuration options agreed in RAN1 can be simplified such that the behaviour can be specified in a straightforward way in MAC to cover all of the cases RAN1 have agreed, as long as RAN1 specifies the behaviour for determining the HARQ mode setting rather than leaving this complication to RAN2.

Proposal 4a: From RAN2 point of view, at least for the cases other than Option 1 + Option 3 DCI based overridden mechanism (DCI based signalling is enabled and the RRC bitmap is present):

- if the RRC bitmap is absent then HARQ feedback for all HARQ processes is enabled by default.

- if lower layer does not indicate a HARQ feedback setting for a particular transmission then the RRC setting is used.

With the above in mind, a question from RAN1 in [2] regarding feasibility of the following working assumption was received:

- For Option 1 + Option 3 DCI based overridden mechanism, for a HARQ process configured as HARQ feedback disabled by per-HARQ process bitmap signaling and further reversed to HARQ feedback enabled by DCI, the NBIoT UE does not wait for an RTT+3ms (i.e., till subframe n+Kmac+3 in TS36.213 section 16.6) before monitoring NPDCCH for the same HARQ process (or monitoring any NPDCCH for the case of single HARQ process configuration).

In our understanding, the main case for this operation is when HARQ feedback is disabled by RRC, but a MAC CE needs to be transmitted by the gNB and acknowledged by the UE – even though an ACK is requested when the DCI toggled HARQ feedback on for this (single) transmission, the gNB can continue immediately with transmitting more data with HARQ feedback disabled. In case the MAC CE is not acknowledged by HARQ then this may be transmitted again (i.e. transmitted again using a new transmission rather than a HARQ retransmission). In this case then any behaviour in MAC (i.e. timer handling) can follow the RRC configuration (i.e. HARQ feedback disabled behaviour).

This does, however, imply some special handling for the case that DCI based signalling is enabled and the RRC bitmap is present compared to other cases. Since NB-IoT is typically tolerant of some delays, data rates are typically very low, and the system is intended to be low complexity, the justification for adding special handling (i.e. complexity) for this particular optimisation is weak.

Proposal 4b: For Option 1 + Option 3 DCI based overridden mechanism (DCI based signalling is enabled and the RRC bitmap is present):

- It is feasible to support the RAN1 working assumption (NBIoT UE does not wait for an RTT+3ms before monitoring NPDCCH), if the NB-IoT UE timer handling is always according to RRC configured HARQ feedback state (i.e. timer handling is as per HARQ feedback disabled, even when lower layer indicates HARQ feedback is enabled for a transmission) when DCI based signalling is enabled and the RRC bitmap is present.

- However, from RAN2 perspective the benefit of this optimisation does not justify the additional complexity for NB-IoT and should therefore follow the same behaviour as other cases (as per proposal 4a)

It has already been agreed that HARQ mode A/B for uplink transmission may be configured per UL HARQ process at least via UE specific RRC signalling for eMTC and NB-IOT NTN. In addition, since DL HARQ feedback can be enabled/disabled by DCI, we should consider whether DCI can also be used to set the uplink HARQ mode. Currently the procedure in NR for UL is as follows:

1> if a MAC PDU is transmitted in a configured uplink grant and LBT failure indication is not received from lower layers: 2> if this Serving Cell is configured with uplinkHARQ-Mode: 3> if the corresponding HARQ process is configured as HARQModeA: 4> set HARQ-RTT-TimerUL-NTN for the corresponding HARQ process equal to drx-HARQ-RTT-TimerUL plus the latest available UE-gNB RTT value; 4> if drx-LastTransmissionUL is configured: 5> start the HARQ-RTT-TimerUL-NTN for the corresponding HARQ process in the first symbol after the end of the last transmission (within a bundle) of the corresponding PUSCH transmission. 4> else: 5> start the HARQ-RTT-TimerUL-NTN for the corresponding HARQ process in the first symbol after the end of the first transmission (within a bundle) of the corresponding PUSCH transmission

If this is the case, then a similar procedure needs to be introduced in MAC to set the uplink HARQ mode based on DCI.

Proposal 5a: RAN2 to consider whether uplink HARQ mode based on DCI should be supported.

If so, then:

Proposal 5b: Send an LS to RAN1 indicating RAN2 conclusion.

Proposal 5c: Specify in MAC that a HARQ process use Uplink HARQ mode A or B, based on RRC configuration or DCI indication.

Proposal 5d: Study whether any further changes are needed to support DCI based HARQ mode setting (E.g. timer handling, LCP restriction handling).

# Conclusion

In this contribution we discuss the impact of disabling HARQ feedback for IoT-NTN. The following is observed and proposed:

Proposal 1a: The text from NR MAC spec is used as a baseline.

Proposal 1b: Specify in MAC that a HARQ process can have HARQ feedback enabled or disabled, based on RRC configuration or explicit indication from physical layer.

Proposal 1c: Any further changes (in addition to the above) can be discussed in stage 3 review.

Proposal 2a: A single flag is introduced for configuring DCI based HARQ feedback enable/disable.

Proposal 2b: Introduce an optional RRC bitmap with a value per HARQ process to indicate the initial setting (enabled/disabled) for each HARQ process. (Similar to NR)

Proposal 3: RAN2 understanding of the variety of options agreed in RAN1:

- If DCI based signalling is disabled and the RRC bitmap is absent then this indicates the behaviour is as per legacy (i.e. HARQ feedback is enabled for all HARQ processes).

- If DCI based signalling is disabled and the RRC bitmap is present then this indicates the HARQ mode setting is always based on the RRC setting (like in NR).

- If DCI based signalling is enabled and the RRC bitmap is absent then this indicates the HARQ mode setting will be explicitly provided by DCI for every transmission.

- If DCI based signalling is enabled and the RRC bitmap is present then this indicates the HARQ mode setting provided by RRC may be toggled by DCI for some transmissions, otherwise the RRC setting is used.

Proposal 4a: From RAN2 point of view, at least for the cases other than Option 1 + Option 3 DCI based overridden mechanism (DCI based signalling is enabled and the RRC bitmap is present):

- if the RRC bitmap is absent then HARQ feedback for all HARQ processes is enabled by default.

- if lower layer does not indicate a HARQ feedback setting for a particular transmission then the RRC setting is used.

Proposal 4b: For Option 1 + Option 3 DCI based overridden mechanism (DCI based signalling is enabled and the RRC bitmap is present):

- It is feasible to support the RAN1 working assumption (NBIoT UE does not wait for an RTT+3ms before monitoring NPDCCH), if the NB-IoT UE timer handling is always according to RRC configured HARQ feedback state (i.e. timer handling is as per HARQ feedback disabled, even when lower layer indicates HARQ feedback is enabled for a transmission) when DCI based signalling is enabled and the RRC bitmap is present.

- However, from RAN2 perspective the benefit of this optimisation does not justify the additional complexity for NB-IoT and should therefore follow the same behaviour as other cases (as per proposal 4a)

Proposal 5a: RAN2 to consider whether uplink HARQ mode based on DCI should be supported.

If so, then:

Proposal 5b: Send an LS to RAN1 indicating RAN2 conclusion.

Proposal 5c: Specify in MAC that a HARQ process use Uplink HARQ mode A or B, based on RRC configuration or DCI indication.

Proposal 5d: Study whether any further changes are needed to support DCI based HARQ mode setting (E.g. timer handling, LCP restriction handling)

# References

- RP-223519,  “Revised WID on IoT NTN enhancements”

- R2-2307016, “LS on NPDCCH monitoring restriction for NB-IoT NTN”

- R2-2307005, “Reply LS on HARQ Enhancements”