3GPP TSG RAN WG2 #123	                                R2-2308577

Toulouse, France, 21st – 25th August, 2023

Agenda Item:	7.6.2.2

Source:	InterDigital, Inc.

Title:	GNSS acquisition and reporting for IoT NTN

Document for:	Discussion, Decision

# Introduction

Non-terrestrial networks are characterized by large moving cells and reduced effectiveness of measurement-based procedures due to reduced signal-strength variation between cell centre and cell edge. Rel-17 has partly addressed these issues by incorporating UE location into various procedures such as time pre-compensation, CHO, and measurement reporting. Since other RAT-dependent positioning methods are not currently supported, maintaining updated and accurate GNSS information is essential in NTN. It is assumed that all Rel-17/18 NTN capable devices are GNSS capable.

An objective for Rel-18 IoT NTN is to continue performance enhancements to address remaining issues from Rel-17. One of these remaining issues is improved GNSS operation [1]:

4.1.1 IoT-NTN Performance Enhancements in Rel-18 to address remaining issues from Rel-17 This work considers Rel-17 IoT-NTN as baseline as well as Rel-17 NR-NTN outcome and the further IoT-NTN performance enhancements objectives are listed below: - Disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates [RAN1,RAN2] - Study and specify needed improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption. Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed. [RAN1, RAN2]

RAN2 made the following agreements in RAN2#122

- Agreements: - Confirm the working assumption that GNSS validity duration UE reports is the remaining validity duration. - The UE triggers GNSS measurement reporting every time upon completing the GNSS fix operation. - When network triggers GNSS measurement initiation is up to network implementation. - Add a note to state some AS operations are suspended when UE is performing GNSS measurement during GNSS measurement gap. - GNSS fix time duration should be reported in 1) and 2): - RRCConnectionReestablishmentComplete and RRCConnectionReestablishmentComplete-NB - RRCConnectionReconfigurationComplete for HO case - (FFS whether there are some scenarios where this is not needed or whether there has to be some explicit NW indication to do so) - Working Assumptions: - An UL MAC CE for GNSS validity duration reporting is used for NB-IoT user plane solution and eMTC UE as well, in addition to previously agreed NB-IoT control plane solution - A new DL MAC CE is introduced to trigger connected UE to perform GNSS measurement.

RAN1 made the following agreements in RAN1#113

Agreement From RAN1 perspective, at least for the case when frequency error and timing error are within frequency and timing error requirements with legacy closed loop time correction, UL transmission can be allowed in a duration X after original GNSS validity duration expires without GNSS re-acquisition. RAN1 will decide further details of the above. Agreement UE reports one GNSS position fix time duration for GNSS measurement via a 4-bit field with component values [1,2,3,4,5,6,7,13,19,25,31] - FFS: other component values Agreement The UE is not required to transmit or receive any channel / signal within the aperiodic GNSS measurement gap duration before the UE reacquires GNSS successfully. FFS: UE’s behavior within the duration after UE reacquires GNSS successfully to the end of the gap if the UE reacquires GNSS successfully before the end of the gap. Agreement For the aperiodic GNSS measurement gap triggered by eNB with MAC CE, down select one of the alternatives for the start time of the gap: - Alt 1: should be at n+ X, where n is the end of MAC CE receiving subframe/slot and X>= 12ms for NB-IoT, X>= 3ms for eMTC - Note: X is one value regardless of HARQ feedback enabled or disabled for the MAC CE - FFS: details, e.g. X is predefined value or configured value - Alt 2: should be at n+ X1, where n is the end of MAC CE receiving subframe/slot when HARQ feedback for the MAC CE is disabled and X1>= 12ms for NB-IoT, X1>= 3ms for eMTC, or should be at p+ X2, where p is the end of HARQ feedback transmission subframe/slot when HARQ feedback for the MAC CE is enabled - FFS: details, e.g. X1 and X2 are predefined value or configured value, including whether X1 and X2 can be the same - Alt3: should be at p+ X, where p is the end of HARQ feedback transmission subframe/slot, where HARQ feedback for the MAC CE is always enabled - FFS: details, e.g. X is predefined value or configured value Agreement For NB-IoT and eMTC, at least for the case where the network configuration does not include a periodicity (if supported), for autonomous GNSS re-acquisition, the UE may re-acquire GNSS autonomously during GNSS measurement timer, the start time of the autonomous GNSS measurement timer is based on the original GNSS validity duration. - FFS: additional delay and details of delay (if any), e.g. delay can be zero or can be equal to/larger than the duration X where UL transmission can be allowed after original GNSS validity duration expires without GNSS re-acquisition. - Note1: Autonomous GNSS re-acquisition mechanism is enabled or disabled by network. - Note2: The length of GNSS measurement timer can be configured by network and the length of GNSS measurement timer is equal to the latest reported GNSS position fix time duration for measurement when the length of GNSS measurement timer is not configured - Note3: The autonomous GNSS re-acquisition can be periodic in certain conditions without further spec impact

This document continues the discussion on improved GNSS operations.

# GNSS acquisition and reporting in IoT NTN

Acquisition of GNSS is a time and power intensive process, which is particularly an issue for devices where low power consumption is a key concern. Furthermore, a subset of these devices (e.g. NB-IoT) may not capable of simultaneously supporting GNSS acquisition and data transmission/reception, so additional consideration must be given to trade off throughput vs. updated GNSS information.

In Release 17 it was assumed that NTN IoT operation would be limited to short and sporadic traffic. Few enhancements were therefore needed/adopted to reduce GNSS acquisition time since it was unlikely that a scheduled transmission/reception would overlap with GNSS acquisition. Release 18 IoT NTN removes the assumption of short-sporadic traffic, and although throughput is increased, so is the probability of collision between the GNSS acquisition window and UL transmission/DL receptions as shown below:

Figure 1: Rel-18 IoT NTN devices supporting longer & more frequent traffic can impact GNSS acquisition.

It was agreed in the previous meeting that UE reports GNSS validity duration after every time a GNSS measurement has been completed. This allows for the network to determine that a NW triggered GNSS measurement was successful as well as allowing for the network to determine whether a UE autonomous GNSS measurement has been performed.

Observation 1: Success of network triggered GNSS measurement is indicated when UE reports GNSS assistance information MAC CE containing validity duration after the measurement.

Observation 2: Completion of UE autonomous GNSS measurement is indicated when UE reports GNSS assistance information MAC CE containing validity duration after the measurement.

Most of the time, it is expected that the GNSS validity duration will be the same as the previously reported value, in case of a new measurement being completed. In this case it would be more optimal to use a shorter (e.g. 1 bit) indication for indicating that the measurement has been successfully completed and that the validity duration is unchanged compared to the previously reported value. Proposal 1:	Use a 1 bit indication for reporting successful GNSS measurement, validity duration is reported only if the value is changed from the previously reported value.

Since the validity of GNSS information may vary dynamically based on the UE circumstances, and the network may not be aware of when the UE circumstances changes, it may be necessary to support additional triggering conditions for GNSS assistance information reporting which are evaluated by the UE.

For example, should the UE detect that the GNSS information validity will expire sooner than previously reported (e.g., due to a change in UE speed), then the network can provision for a gap and trigger GNSS acquisition accordingly.

In fact, the NW can determine based on the reported validity time whether the report is due to an autonomous UE measurement (E.g. the reported validity time is relatively long, or similar to the last reported value) or due to a reduced validity determined by the UE (e.g. the new reported time is shorter than the expected remaining validity duration based on the previous reported value). In any case, the UE should be able to report a new validity time if the remaining time changes compared to what is expected from the last reported value whether this is due to UE autonomous measurement or another reason.

Proposal 2:	The GNSS assistance information MAC CE containing validity duration, may be reported when the UE detects that the remaining GNSS validity duration changes compared to what is expected based on the previously reported value, even if no new GNSS measurement has been performed.

In the previous meeting the following working assumptions were made:

- An UL MAC CE for GNSS validity duration reporting is used for NB-IoT user plane solution and eMTC UE as well, in addition to previously agreed NB-IoT control plane solution

- A new DL MAC CE is introduced to trigger connected UE to perform GNSS measurement.

Use of MAC CE has an advantage over RRC, not only because it allows for a consistent solution for the various cases, but also because it is more suitable for determining the correct timing of the necessary measurement gap as well as more suitable to dynamically indicate completion of  a UE autonomous measurement. RAN1 are discussing the options for determining the gap timing based on the assumption that a MAC CE is used. Hence, the working assumptions made in the last meeting should be confirmed.

Proposal 3:	The working assumptions made in the last meeting are confirmed.

      - An UL MAC CE for GNSS validity duration reporting is used for NB-IoT user plane solution and eMTC UE as well, in addition to previously agreed NB-IoT control plane solution

      - A new DL MAC CE is introduced to trigger connected UE to perform GNSS measurement.

Acquisition of GNSS may take considerable time and UE power consumption depending on aspects like UE capability and required accuracy. This may be especially important in IoT NTN scenarios, where devices may need to suspend transmission/reception to acquire GNSS (resulting in lower throughput), and where reduction of UE power consumption is an important consideration.

Additionally, one drawback of a UE trigger is that this may occur frequently, resulting in excessive GNSS acquisition and reporting.

To avoid this, a network configured prohibit timer could be used, which could be (re)started upon acquisition of GNSS or transmission of GNSS report. The duration of the prohibit timer could be configurable, or related to GNSS validity duration (e.g. shorter than the GNSS validity duration such that a change can be reported in time, but long enough to avoid excessive GNSS acquisition and reporting). While running the UE may not acquire or report GNSS information.

Proposal 4:	Introduce a prohibit timer to prevent excessive acquisition and reporting of GNSS information.

# Conclusion

In this contribution the following proposals were made concerning enhanced GNSS operation for IoT NTN devices:

Observation 1: Success of network triggered GNSS measurement is indicated when UE reports GNSS assistance information MAC CE containing validity duration after the measurement.

Observation 2: Completion of UE autonomous GNSS measurement is indicated when UE reports GNSS assistance information MAC CE containing validity duration after the measurement.

Proposal 1:	Use a 1 bit indication for reporting successful GNSS measurement, validity duration is reported only if the value is changed from the previously reported value.

Proposal 2:	The GNSS assistance information MAC CE containing validity duration, may be reported when the UE detects that the remaining GNSS validity duration changes compared to what is expected based on the previously reported value, even if no new GNSS measurement has been performed.

Proposal 3:	The working assumptions made in the last meeting are confirmed.

      - An UL MAC CE for GNSS validity duration reporting is used for NB-IoT user plane solution and eMTC UE as well, in addition to previously agreed NB-IoT control plane solution

      - A new DL MAC CE is introduced to trigger connected UE to perform GNSS measurement.

Proposal 4:	Introduce a prohibit timer to prevent excessive acquisition and reporting of GNSS information.

# References

- RP-223519,  “Revised WID on IoT NTN enhancements”

- R2-2304612, " LS on GNSS position fix during inactive state of Connected DRX for improved GNSS operations ", RAN1