3GPP TSG RAN WG2 #123	                                R2-2308579

Toulouse, France, 21st – 25th August, 2023

Agenda Item:	7.6.4

Source:	InterDigital, Inc.

Title:	Paging in discontinuous coverage

Document for:	Discussion

# Introduction

The proposal made as an outcome of the email discussion [8] is as follows:

Paging enhancements Proposal 1: Send an LS to SA2 to ask for: - SA2 opinion about the issue about misalignment between PTW and Coverage Window (e.g., whether it needs to be solved and if yes, whether it can be solved by NW implementation) - SA2 opinion about the possible enhancement to UE and NW behaviour to adjust PH and PTW starting point (e.g., based on an offset configured by eNB or CN, or based on an predefined rule between UE and eNB ) , if it cannot be solved by NW implementation

In this paper we provide some further input to the discussion, specifically what to ask SA2.

# Discussion

According to SA2 [2] “The AMF/MME configures the UEs mobility management and Power saving mode (PSM) parameters and sends the MICO/PSM parameters to UE before the satellite discontinuous coverage starts.” and “The mobility management and MICO/PSM parameters include eDRX parameters”

In case the UE is configured with a long DRX (i.e. eDRX) then it may be the case that the UE is configured with a paging cycle which is longer than a UE unreachability period. In some cases, the PTW may fall within the UE unreachability period, and in extreme cases e.g. when there are many relatively long UE unreachability periods this may result in the UE being unreachable for long periods of time (i.e. much longer than the configured eDRX cycle due to missed PTWs). It may be possible by configuration to avoid this some of the time, however this may not be possible in all cases.

In the last meetings, 4 potential approaches were provided in [4], [5], [6], [7]. Upon comparing the solutions, they are not so different.

Solution 1:

In [4] it was proposed to update the PTW calculation. To elaborate, the proposal is not to modify the PTW calculation itself, but rather to consider only the H-SFNs which fall into coverage and exclude those which fall in a coverage gap when performing the existing PH and PTW calculation. As an example, consider the illustration in figure 1.

Figure 1: “Remapping” the PH taking into account in coverage/out of coverage H-SFNs.

In the above example, 14 UEs are shown, each one of those would fall into a different PH based on UE_ID.

We can renumber H-SFNs to exclude the ones which are out of coverage – e.g. as follows

If H=SFNs are : 1, 2, 3, 4, 5, 6, 7, 8, 9, 10.

And if H-SFNs 8, 9, 10 are out of coverage, then the remapping would be as follows.

1, 2, 3, 4, 5, 6, 7, X, X, X = “H-SFN-remapped”

By using the remapped H-SFNs (instead of true H-SFN) compared to the UE-ID mod N (where N is the number of in coverage H-SFNs – in this example it’s 7) the UE would calculate a PH which falls into one of the in-coverage H-SFNs. In this example, UE 8 is shifted to the same H-SFN as UE 1, UE 9 is shifted to the same H-SFN as UE 2 – i.e. an offset of 7 ( = N ) is applied. One could also define a “re-mapping” which is applied by all UEs (not just the out of coverage ones), in order to spread the PHs evenly amongst available H-SFNs to minimise the impact on paging load. However, the most basic example as shown above is a simple offset determined for the UEs in the coverage hole, calculated based on the number of in-coverage H-SFNs.

In other words, this approach calculates an offset for the UEs with a PH in a coverage gap by removing the out of coverage H-SFNs used for the PH calculation.

Solution 2:

In [5], a configurable offset is proposed. The details are not provided, for example whether the offset is provided per UE in dedicated signalling, or whether this is enabled by the network and the UE calculates a UE specific offset based on UE-ID. If it is the latter then there is not much difference to the solution 1 above.

In other words, this approach configures an offset for the UEs with a PH in a coverage gap. The details are not provided e.g. whether UE determines a UE specific value based on a common configuration or whether the gNB configures an offset per UE.

Solution 3:

In [6] it is proposed to configure multiple DRX configurations. One would assume that the UE should not monitor multiple PTW all of the time, because this would be equivalent to just configuring a shorter eDRX cycle. The power consumption would be increased in this case. Rather, the UE would monitor the “additional PTWs” only in case a coverage gap is detected. In this case, this is equivalent to calculating an offset based on applying a different eDRX configuration while in a coverage gap.

In other words, this approach configures an offset for the UEs with a PH in a coverage gap by providing an additional DRX configuration.

Solution 4:

In [7] it is proposed that the UE autonomously extends the paging window, “if it detects a shift in the window” and reports to the NW.

In other words, this approach uses an offset determined by UE implementation and reported to the NW.

In summary, it seems all of the approaches use an offset, with differing details of how to configure or calculate the offset.

Observation: All of the proposed solutions for updating the PTW to receive paging outside of a coverage gap determine or configure an offset. Each solution may use a different approach to determine/configure the offset.

In the email discussion [8], the majority of companies support introducing an offset of some sort to align the PTW with coverage. In addition, a number of disadvantages have been pointed out with a purely CN based solution. In particular, some concerns have been raised in terms of power consumption impact (E.g. if a shorter DRX cycle is configured) and in terms of not being able to align based on a purely CN solution, due to the PTW calculation defined in RAN.

Therefore, we think that RAN2 should take a stronger position and make some concrete proposals and recommendations to send to SA2, we can provide some indication of what the concerns are.

Proposal 1: From a RAN2 perspective, a CN solution which relies on configuring a shorter DRX cycle is not preferred as it may have significant adverse impact on UE power consumption.

Proposal 2: From a RAN2 perspective, in order to minimise UE power consumption impact, it is beneficial to update the PTW if it falls in a coverage gap by applying an offset to the PTW calculation defined in 36.304. (FFS exactly how to determine/configure the offset).

For a RAN based solution (e.g. update the PTW defined in 36.304), we need to ask what the content/granularity of “UE unreachability period” is, whether it can be provided to the gNB, and whether adjusting the PTW is feasible from SA2 point of view.

Proposal 3: In addition to the above proposals, ask SA2 what the content/granularity of “UE unreachability period is, whether it can be provided to the gNB, and whether adjusting the PTW is feasible and necessary from SA2 point of view.

# Conclusion

In this contribution we discuss the potential paging enhancement for discontinuous coverage for R18 IoT-NTN and have the following proposals:

Observation: All of the proposed solutions for updating the PTW to receive paging outside of a coverage gap determine or configure an offset. Each solution may use a different approach to determine/configure the offset.

Proposal 1: From a RAN2 perspective, a CN solution which relies on configuring a shorter DRX cycle is not preferred as it may have significant adverse impact on UE power consumption.

Proposal 2: From a RAN2 perspective, in order to minimise UE power consumption impact, it is beneficial to update the PTW if it falls in a coverage gap by applying an offset to the PTW calculation defined in 36.304. (FFS exactly how to determine/configure the offset).

Proposal 3: In addition to the above proposals, ask SA2 what the content/granularity of “UE unreachability period is, whether it can be provided to the gNB, and whether adjusting the PTW is feasible and necessary from SA2 point of view.

# References

- RP-223519,  “Revised WID on IoT NTN enhancements”

- 23.700-28-i00, “Study on Integration of satellite components in the 5G architecture; Phase 2”

- R2-2304258, “[offline-115] Discontinuous coverage enhancements”, InterDigital, Inc.

- R2-2303716, “IoT-NTN discontinuous coverage enhancements”, InterDigital, Inc.

- R2-2302822, “RAN2 enhancements for discontinuous coverage”, ZTE Corporation, Sanechips.

- R2-2303963, “Discussion on discontinuous coverage”, Huawei, Turkcell, HiSilicon.

- R2-2303193, “On RAN impacts for Discontineous coverage enhancements”, Nokia, Nokia Shanghai Bell.

- Report of [Post122][113][IoT NTN Enh] Discontinuous coverage (Huawei)