3GPP TSG-RAN WG2 Meeting #123                           	R2-2308583

# Toulouse, France, August 21-25, 2023

Title:		Corrections for 38.331 Running CR for IDC Enhancements

Source:	Huawei, HiSilicon

Agenda item:	7.10

Document for:	Discussion and Decision

# Introduction

In the past RAN 2 meetings we had made excellent progress and had also endorsed the TS 38.331 running CR for introducing IDC enhancement. In last meeting, the WI on “In-device Co-existence (IDC) enhancements for NR and MR-DC” was completed.

In this paper we intend to further improve the running CR and discuss some of the issues we have identified in the running CR and suggest the text proposals for resolving these issues.

# Discussion

## 2.1 Misalignment between the procedural text and ASN.1 for the idc-AssistanceConfig-r18

Based on the current description in the TS 38.331 running CR there is no new addition for the procedural text in section 5.3.5.9 for the new configuration and only the legacy idc-AssistanceConfig is mentioned in the procedure text (as shown below in the highlighted text). Additionally, the idc-AssistanceConfig-r16 has a SetupRelease structure whereas the idc-AssistanceConfig-r18 is a SEQUENCE structure. This creates potential misalignment between the procedure text and the ASN1 structure.

One option to resolve this is that we could have a separate branch for idc-AssistanceConfig-r18 in section 5.3.5.9. The other option is to change the ASN.1 structure so that the idc-AssistanceConfig for R18 is also a SetupRelease structure as in Rel 16 so that idc-AssistanceConfig in the procedure text refers to both R16 and R18 configuration. We think both of these would work and the text proposals for both these options are given below.

Option 1 - Changes to the procedural text: Changes to the procedural text to have separate branch for idc-AssistanceConfig-r18 is given in the text proposal below.

START OF CHANGE – Option 1

#### 5.3.5.9	Other configuration

The UE shall:

1>	if the received otherConfig includes the delayBudgetReportingConfig:

2>	if delayBudgetReportingConfig is set to setup:

3>	consider itself to be configured to send delay budget reports in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to send delay budget reports and stop timer T342, if running.

1>	if the received otherConfig includes the overheatingAssistanceConfig:

2>	if overheatingAssistanceConfig is set to setup:

3>	consider itself to be configured to provide overheating assistance information in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide overheating assistance information and stop timer T345, if running;

1>	if the received otherConfig includes the idc-AssistanceConfig-r16:

2>	if idc-AssistanceConfig-r16 is set to setup:

3>	consider itself to be configured to provide IDC assistance information in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide IDC assistance information;

1>	if the received otherConfig includes the idc-AssistanceConfig-r18:

2>	if idc-FDM-AssistanceConfig-r18 is set to setup:

3>	consider itself to be configured to provide IDC assistance information based on idc-FDM-AssistanceConfig in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide IDC assistance information based on idc-FDM-AssistanceConfig;

2> if idc-FDM-AssistanceConfig-r18 is set to setup or idc-AssistanceConfig-r16 is set to setup:

3> if idc-TDM-AssistanceConfig-r18 is set to setup :

4>	consider itself to be configured to provide IDC assistance information based on

idc-TDM-AssistanceConfig in accordance with 5.7.4;

3> else:

4>	consider itself not to be configured to provide IDC assistance information based on idc-TDM-AssistanceConfig;

1>	if the received otherConfig includes the drx-PreferenceConfig:

2>	if drx-PreferenceConfig is set to setup:

3>	consider itself to be configured to provide its preference on DRX parameters for power saving for the cell group in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide its preference on DRX parameters for power saving for the cell group and stop timer T346a associated with the cell group, if running;

…

END OF CHANGE – Option 1

Option 2 - Changes to ASN.1 structure:  – Changes to the ASN.1 structure of idc-AssistanceConfig-r18 to align it with idc-AssistanceConfig-r16 is given below.

START OF CHANGE – Option 2

…..

OtherConfig-v18xy ::=                   SEQUENCE {

idc-AssistanceConfig-r18                SetupRelease {IDC-AssistanceConfig-r18}                       	 OPTIONAL -- Need M

}

IDC-AssistanceConfig-r18 ::=    SEQUENCE {

idc-FDM-AssistanceConfig-r18            	SetupRelease {IDC-FDM-AssistanceConfig-r18)                   OPTIONAL, -- Need M

idc-TDM-AssistanceConfig-r18            	ENUMERATED {setup}					                    		 OPTIONAL  -- Cond FDM

...

}

…..

END OF CHANGE – Option 2

Proposal 1- Update either the procedural text in section 5.3.5.9 as suggested in option 1 or adopt option 2 of the text proposal involving ASN.1 changes above.

If the Option 2 is agreed, minor changes in the procedural text may also be needed.

## 2.2  Using interferenceDirection in AffectedCarrierFreqRangeComb-r18

For the individual affected frequency range, the network need to know the interference direction to understand which side has been affected and accordingly resolve the IDC problem. Hence the UE needs to report the Interference direction for the AffectedCarrierFreqRange. The Value nr indicates that only NR is victim of IDC interference, value other indicates that only another radio is victim of IDC interference and value both indicates that both NR and another radio are victims of IDC interference. However, when the UE reports AffectedCarrierFreqRangeCombnation, the IDC issue experienced by the UE is due to Inter Modulation Distortion (IMD), which is due to the simultaneous UL transmission on the 3GPP part on the two frequency range combination when the UE is operating in UL CA or NR-DC scenario. It means that the interference is caused by the simultaneous UL transmission in NR to the non-3GPP system. For this combination case the interference direction will always be other” indicating that another radio is victim of IDC interference. Thus, interferenceDirection in AffectedCarrierFreqRangeComb-r18 will be ineffective and redundant and will cause unnecessary signaling overhead. Hence, we propose to remove interferenceDirection in AffectedCarrierFreqRangeComb-r18 in the running CR. The text proposal for the procedural text and the ASN .1 is given below.

START OF 1st CHANGE

…

2>	if there is at least one supported UL CA or MR-DC combinations comprising of frequency ranges included in candidateServingFreqRangeListNR, and each affected frequency range in the UL CA or MR-DC combination overlapping with one frequency range included in candidateServingFreqRangeListNR, and the center frequency of the affected frequency range is within the frequency range included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself:

3>	include the field affectedCarrierFreqRangeCombList with an entry for each supported UL CA or MR-DC combination comprising of frequency ranges that is affected by IDC problems;

3>	for each affected frequency range included in the field affectedCarrierFreqRangeCombList, include centerFreq and affectedBandwidth;

3>	for each UL CA or MR-DC combination included in the field affectedCarrierFreqRangeCombList, optionally include victimSystemType, and set it accordingly;

…

END OF 1st CHANGE

START OF 2nd  CHANGE

….

AffectedCarrierFreqRangeCombList-r18 ::= SEQUENCE (SIZE (1..maxCombIDC-r16)) OF AffectedCarrierFreqRangeComb-r18

AffectedCarrierFreqRangeComb-r18 ::=   SEQUENCE {

affectedCarrierFreqRangeComb-r18         SEQUENCE (SIZE (2..maxNrofServingCells)) OF AffectedCarrierFreqRangeComb-r18,

victimSystemType-r18                     VictimSystemType-r16                              OPTIONAL

}

….

END OF 2st CHANGE

Proposal 2 - Remove interferenceDirection in AffectedCarrierFreqRangeComb-r18 in the ASN.1 and the procedural text in running CR as proposed in the text proposal above.

## 2.3 Missing references to candidateServingFreqRangeListNR  and candidateServingFreqListNR in the procedural text

Based on the current description in the TS 38.331 running CR there are some missing references to candidateServingFreqRangeListNR  and candidateServingFreqListNR in the procedural text. These are proposed to be added in the procedural text in the text proposal below.

START OF 1st CHANGE

…

1>	if configured to provide IDC assistance information based on idc-AssistanceConfig-r16 of a cell group:

2>	if the UE did not transmit a UEAssistanceInformation message with idc-Assistance since it was configured to provide IDC assistance information:

3>	if on one or more frequencies included in candidateServingFreqListNR, the UE is experiencing IDC problems that it cannot solve by itself; or

3>	if on one or more supported UL CA combination comprising of carrier frequencies included in candidateServingFreqListNR, the UE is experiencing IDC problems that it cannot solve by itself:

4>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC assistance information;

2>	else if the current idc-Assistance information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC assistance information;

1>	if configured to provide IDC assistance information based on idc-FDM-AssistanceConfig of a cell group:

2>	if the UE did not transmit a UEAssistanceInformation message with idc-FDM-Assistance since it was configured to provide IDC assistance information:

3>	if on one or more frequency ranges included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself; or

3>	if on one or more supported UL CA or MR-DC combination comprising of frequency ranges included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself:

4>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC assistance information;

2>	else if the current idc-FDM-Assistance information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC assistance information;

1>	if configured to provide IDC assistance information based on idc-TDM-AssistanceConfig of a cell group:

2>	if the UE did not transmit a UEAssistanceInformation message with idc-TDM-Assistance since it was configured to provide IDC assistance information:

3>	if on one or more frequencies included in candidateServingFreqListNR or frequency ranges included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself; or

3>	if on one or more supported UL CA or MR-DC combination comprising of carrier frequencies included in candidateServingFreqListNR or frequency ranges included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself:

4>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC assistance information;

2>	else if the current idc-TDM-Assistance information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC assistance information;

…

END OF 1st CHANGE

START OF 2nd CHANGE

…

1>	if transmission of the UEAssistanceInformation message is initiated to provide IDC assistance information according to 5.7.4.2 or 5.3.5.3:

2>	if there is at least one carrier frequency included in candidateServingFreqListNR, the UE is experiencing IDC problems that it cannot solve by itself:

3>	include the field affectedCarrierFreqList with an entry for each affected carrier frequency included in candidateServingFreqListNR;

3>	for each carrier frequency included in the field affectedCarrierFreqList, include interferenceDirection and set it accordingly;

2>	if there is at least one supported UL CA combination comprising of carrier frequencies included in candidateServingFreqListNR, the UE is experiencing IDC problems that it cannot solve by itself:

3>	include victimSystemType for each UL CA combination included in affectedCarrierFreqCombList;

3>	if the UE sets victimSystemType to wlan or bluetooth:

4>	include affectedCarrierFreqCombList with an entry for each supported UL CA combination comprising of carrier frequencies included in candidateServingFreqListNR, that is affected by IDC problems;

3>	else:

4>	optionally include affectedCarrierFreqCombList with an entry for each supported UL CA combination comprising of carrier frequencies included in candidateServingFreqListNR, that is affected by IDC problems;

2>	if there is at least one affected frequency range overlapping with one frequency range included in candidateServingFreqRangeListNR, and the center frequency of the affected frequency range is within the frequency range included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself:

3>	include the field affectedCarrierFreqRangeList with an entry for each affected frequency range;

3>	for each affected frequency range included in the field affectedCarrierFreqRangeList, include centerFreq and affectedBandwidth;

3>	for each affected frequency range included in the field affectedCarrierFreqRangeList, include interferenceDirection and optionally victimSystemType, and set it accordingly;

2>	if there is at least one supported UL CA or MR-DC combinations comprising of frequency ranges included in candidateServingFreqRangeListNR, and each affected frequency range in the UL CA or MR-DC combination overlapping with one frequency range included in candidateServingFreqRangeListNR, and the center frequency of the affected frequency range is within the frequency range included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself:

3>	include the field affectedCarrierFreqRangeCombList with an entry for each supported UL CA or MR-DC combination comprising of frequency ranges that is affected by IDC problems;

3>	for each affected frequency range included in the field affectedCarrierFreqRangeCombList, include centerFreq and affectedBandwidth;

3>	for each UL CA or MR-DC combination included in the field affectedCarrierFreqRangeCombList, include interferenceDirection and optionally victimSystemType, and set it accordingly;

2>	if there is at least one carrier frequency included in candidateServingFreqListNR or frequency range included in candidateServingFreqRangeListNR or one supported UL CA or MR-DC combination comprising of carrier frequencies included in candidateServingFreqListNR or frequency ranges included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself, and affectedCarrierFreqList or affectedCarrierFreqCombList or affectedCarrierFreqRangeList or affectedCarrierFreqRangeCombList is included, and idc-TDM-AssistanceConfig is set to setup:

3>	include Time Domain Multiplexing (TDM) based assistance information as indicated by idc-TDM-Assistance that could be used to resolve the IDC problems;

…

END OF 2nd CHANGE

Proposal 3: Add the missing references to candidateServingFreqRangeListNR  and candidateServingFreqListNR in the procedural text as proposed in  the text proposal above.

# Conclusions

In this contribution we discuss some of the issues we have identified in the running CR and suggest the text proposals for resolving these issues.

Misalignment between the procedural text and ASN.1 for the idc-AssistanceConfig-r18

Proposal 1- Update either the procedural text in section 5.3.5.9 as suggested in option 1 or adopt option 2 of the text proposal involving ASN.1 changes above.

Using interferenceDirection in AffectedCarrierFreqRangeComb-r18

Proposal 2 - Remove interferenceDirection in AffectedCarrierFreqRangeComb-r18 in the ASN.1 and the procedural text in running CR as proposed in the text proposal above

Missing references to candidateServingFreqRangeListNR  and candidateServingFreqListNR in the procedural text

Proposal 3: Add the missing references to candidateServingFreqRangeListNR  and candidateServingFreqListNR in the procedural text as proposed in  the text proposal above.

# References

- RP-213589 New WI: In-Device Co-existence (IDC) enhancements for NR and MR-DC

- TS 38.331 NR Radio Resource Control (RRC) protocol specification v 17.5.0

- TS 36.331 LTE Radio Resource Control (RRC) protocol specification v 17.5.0

- R2-2306925 38.331 Running CR for introduction of IDC