3GPP TSG-RAN2 #123	R2- 2308593

# Toulouse, France, August 21st – 25th, 2023

Agenda item:		7.3.2 (Netw_Energy_NR-Core)

Source:	LG Electronics Inc.

Title: 	Discussion on DTX/DRX mechanism

Document for:	Discussion and Decision

1.	Introduction

This document discusses DTX/DRX mechanism for NES. The agreements related to DTX/DRX mechanism in R2-122 are as follows.

Agreements in R2-122 meeting

1	UE monitors PDCCH for RAR during Cell DTX non-active time. The ra-ResponseWindow could be started as legacy.

2	UE monitors PDCCH for msg4 during Cell DTX non-active time. The ra-ContentionResolutionTimer could be started as legacy.

3	Working assumption:  When the retransmission timer is running (if C-DRX is configured), the UE is expected to monitor PDCCH, like in legacy.  It is up to the network whether it schedules retransmissions out of the Cell DTX active period, i.e., when the DRX retransmission timer is running, the UE should monitor PDCCH regardless of the Cell DTX.

4	Once gNB recognizes there is an emergency call or public safety related service (e.g. MPS/MCS), the NW should ensure there is no impact to the emergency call (e.g. may deactivate Cell DTX/DRX).  The behavior is captured in stage 2 spec

5	When an DG grant is received, by the gNB during cell DRX/DTX, the UE follows the grant assignment (i.e. like in legacy).  This includes DL HARQ feedback.

In the post email discussion of [Post122][307][NES] DTX/DRX, the followings are discussed.

- Alignment between Cell DTX/DRX and UE C-DRX

- Whether there is one or there are multiple cell DTX/DRX configurations

- Cell DTX/DRX parameter value range

The related objective of the work item are as follows.

- Specify enhancement on cell DTX/DRX mechanism including the alignment of cell DTX/DRX and UE DRX in RRC_CONNECTED mode, and inter-node information exchange on cell DTX/DRX [RAN2, RAN1, RAN3] - Note: No change for SSB transmission due to cell DTX/DRX. - Note: The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided.

2.	Discussion

Referring to the objectives in the WID, cell DTX/DRX mechanism includes the alignment of cell DTX/DRX and UE DRX in RRC_CONNECTED. The alignment mechanism may be left to the network implementation without UE impact. However, it does not mean that the network can arbitrarily configure cell DTX/DRX pattern and UE C-DRX patterns, and any overlap patterns even including no overlap between cell DTX/DRX active period and UE C-DRX active time are allowed. Therefore, principle is needed to define what the alignment means, and to have the network implementation follow the defined alignment.

Power saving gain may be maximized when cell DTX/DRX pattern and UE C-DRX patterns are fully aligned or UE C-DRX active times falls within Cell DTX/DRX active period. In other words, when the misalignment such as overlap between cell DTX/DRX non-active period and UE C-DRX active time and overlap between cell DTX/DRX active period and UE C-DRX non-active time is minimized, unnecessary power consumption is minimized. It may be the best for power saving that the principle of the alignment pursues full alignment rather than partial alignment of cell DTX/DRX and UE C-DRX.

Observation 1. Power saving gain may be maximized when cell DTX/DRX pattern and UE C-DRX patterns are fully aligned or UE C-DRX active times falls within Cell DTX/DRX active period.

In the post email discussion of [Post122][307][NES] DTX/DRX, the following proposal are discussed. Those can be considered as basic rules for maximizing overlap between Cell DTX/DRX active period and UE C-DRX active time.

Proposal 6: An aligned UE C-DRX configuration with Cell DTX means that the on-duration of C-DRX falls within Cell DTX on-duration. FFS extension of Cell DTX active time beyond Cell DTX on-duration. (15/25) Proposal 7: The periodicity of UE C-DRX configurations in a cell should be the same or a multiple of the serving Cell’s DTX periodicity.

For Proposal 6, if “on-duration” of UE C-DRX is replaced with “Active time” of UE C-DRX, network implementation will perform scheduling smartly to control “Active time” of UE C-DRX such that the Active time of UE C-DRX falls within Cell DTX on-duration. For example, gNB can make Active time of UE C-DRX fall within cell DTX on-duration based on followings.

- For a new transmission case, gNB does not schedule PDCCH for a new transmission if drx-InactivityTimer is expected to be expired in cell DTX non-active period, or gNB can configure drx-InactivityTimer with 0, or gNB can send DRX Command MAC CE to stop Active Time of UE C-DRX.

- For a retransmission case, gNB does not schedule PDCCH for a retransmission if drx-RetransmissionTimer is expected to be expired in cell DTX non-active period.

Proposal 1. Alignment of cell DTX and UE C-DRX means that the Active Time of UE C-DRX falls within Cell DTX on-duration.

In spite of gNB’s smart scheduling, if an exceptional case such as overlap between UE C-DRX Active time and cell DTX/DRX non-active period happens, it can be handled by the previous agreements. For example, for new transmissions (R2#121bis-e agreement) the gNB doesn’t schedule UE-specific dynamic grants/assignments during Cell DTX non-active period, even if the UE is in C-DRX Active Time. UE doesn’t monitor PDCCH for dynamic grants/assignments for new transmissions during Cell DTX non-active period, even if the UE is in C-DRX Active time. For retransmissions (R2#122 agreements), there is a working assumption that when the retransmission timer is running (if C-DRX is configured), the UE is expected to monitor PDCCH, like in legacy. It is up to the network whether it schedules retransmissions out of the Cell DTX active period, i.e., when the DRX retransmission timer is running, the UE should monitor PDCCH regardless of the Cell DTX.

For the extension of Cell DTX active time beyond Cell DTX on-duration in Proposal 6, no extension of Cell DTX active time is preferred for clear cell DTX/DRX operation and maximizing network power saving gain.

Proposal 2. Cell DTX pattern is considered as a fixed pattern. (i.e., active period of cell DTX is not extended.)

For Proposal 7, the periodicity of UE C-DRX configurations in a cell should be the same or a multiple of the serving Cell’s DTX periodicity for the alignment between Cell DTX/DRX active period and UE C-DRX active time and for maximizing overlap between Cell DTX/DRX active period and UE C-DRX active time.

Proposal 3. Support that the periodicity of UE C-DRX configurations in a cell should be the same or a multiple of the serving Cell’s DTX periodicity.

It is agreed in R2-122 meeting that the network should ensure there is no impact to the emergency call once gNB recognizes there is an emergency call or public safety related service. The gNB may deactivate Cell DTX/DRX. Deactivation of Cell DTX/DRX may be effective when many UEs are involved in emergency call or public safety related service. However, when one or only few UEs request emergency call, deactivation of Cell DTX/DRX is not a good solution because the gNB loses the opportunity of network power saving to provide a small number of UEs with emergency call services. If the number of UEs requesting emergency call is small, the gNB consider to handover those UEs to another cell which is able to support emergency call or public safety related service without negative impact if there is a proper cell. Then, the gNB keeps achieving network power saving and the UEs are provided with emergency call service in the target cell without impact to the emergency call. Referring to the objectives in the WID, inter-node information exchange on cell DTX/DRX is considered. Therefore, it can be considered to handover the UEs to a proper cell, for example, a cell where cell DTX/DRX is not activated. Furthermore, gNB can configure a UE such that the UE performs handover to a cell when an emergency call is triggered.

Observation 2. gNB loses the opportunity of network power saving if Cell DTX/DRX is deactivated for no impact to the emergency call.

Observation 3. gNB can be aware of which neighbour cell is proper for emergency call services if inter-node information exchange on cell DTX/DRX is considered.

Proposal 4. Support that a UE performs handover to a cell available for emergency call when an emergency call arrives during non-active period of Cell DTX/DRX.

3.	Conclusion

This document discusses DTX/DRX mechanism for NES.

Observation 1. Power saving gain may be maximized when cell DTX/DRX pattern and UE C-DRX patterns are fully aligned or UE C-DRX active times falls within Cell DTX/DRX active period.

Proposal 1. Alignment of cell DTX and UE C-DRX means that the Active Time of UE C-DRX falls within Cell DTX on-duration.

Proposal 2. Cell DTX pattern is considered as a fixed pattern. (i.e., active period of cell DTX is not extended.)

Proposal 3. Support that the periodicity of UE C-DRX configurations in a cell should be the same or a multiple of the serving Cell’s DTX periodicity.

Observation 2. gNB loses the opportunity of network power saving if Cell DTX/DRX is deactivated for no impact to the emergency call.

Observation 3. gNB can be aware of which neighbour cell is proper for emergency call services if inter-node information exchange on cell DTX/DRX is considered.

Proposal 4. Support that a UE performs handover to a cell available for emergency call when an emergency call arrives during non-active period of Cell DTX/DRX.