3GPP TSG-RAN WG2 Meeting #123	R2-2308600

Toulouse, France, Aug 21 – 25, 2023

Title		Discussion on lower layer aspects for sidelink positioning

Source	LG Electronics

Document for	Discussion and Decision

Agenda Item	7.2.2

WID/SID	NR_pos_enh2

# 1	Introduction

In this paper, we discuss lower layer aspects for sidelink positioning.

# 2	Discussion

## 2.1		Aspect of SL-PRS priority

To meet higher-layer positioning QoS requirements, SLPP messages (i.e. PSSCH, SL-SCH) would be delivered between UEs within a certain time budget, which could be indicated by the priority field in 1st-stage SCI according to legacy rule (it can change upon RAN1 decision). The priority field with 3 bits can distinguish 8 different priorities mapping to the priority (i.e. sl-Priority) in MAC LCP procedure (it may be associated to SLPP transport QoS over PC5-U). Then it can reserve a set of resources for SLPP transmission according to the resource selection procedure.

RAN1 introduced SL-PRS priority for SL-PRS transmission prioritization for scheme 2 resource selection (i.e. UE autonomous resource allocation mode based on sensing or random selection), as below;

Agreement For the scheme 2 sensing-based resource allocation, - From RAN1 perspective, priority value for SL PRS should be provided by higher layers from Tx UE perspective Agreement For Scheme 2 SL-PRS resource allocation, specify congestion control mechanisms using the existing congestion control mechanisms as a starting point. - Study at least the following aspects on potential changes over the existing congestion control mechanisms: - CBR and CR definition for SL-PRS - Which parameters of a SL-PRS configuration could be impacted by the congestion control mechanism, the mapping between congestion measurements, SL-PRS priority and SL-PRS parameters - CR and CBR measurement time window - Congestion control processing time - Number of CBR ranges - Whether any proposed changes could be applicable to shared resource pools in addition to the dedicated resource pool Agreement For Scheme 2, in a dedicated resource pool, using Rel-16 resource (re)-selection procedure as the starting point, consider at least the following potential modifications: - Modification 3: For the SL-PRS priority: - Option 1: A single L1 SL-PRS priority is allowed in a resource pool - Option 2: Multiple L1 SL-PRS priority are allowed in a resource pool Agreement In the dedicated resource pool, - with regards to the SL-PRS time-domain resource allocation within the resource pool support a - SL-PRS-resource-based allocation - SCI for SL-PRS should at least indicate the following values: - Source ID - Destination ID - Resource reservation period - SL-PRS Priority - Cast type Agreement For Scheme 2, in a dedicated resource pool, - Multiple L1 SL-PRS priority are allowed in a resource pool

By introducing SL-PRS priority, it could be used for resource selection for SL-PRS transmission at least in a dedicated resource pool. According to RAN1 agreement, SL-PRS priority should be provided by higher-layers (e.g. SLPP and/or RRC) as well as lower layer (i.e. SCI). RAN2 therefore should discuss the SL-PRS priority configuration method via higher layers.

Proposal 1.	RAN2 to discuss the SL-PRS priority configuration via higher layers.

## 2.2		Aspect of SL-PRS filtering

According to TS 38.212 and TS 38.321, in current filtering mechanism on sidelink communication, to identify of full 24-bits Source ID and Destination ID can be achieved through combination of 2nd SCI information (i.e. Source ID 8 bits and Destination ID 16 bits) in PSSCH and MAC subheader information (i.e. Source ID 16 bits and Destination ID 8 bits) in SL-SCH (PSSCH).

RAN1 agreed that PSSCH (i.e. SL-SCH) is not included in a dedicated resource pool, so it is used only for SL-PRS transmission/reception.

Agreement PSSCH is not included in dedicated resource pool for SL positioning.

In a dedicated resource pool, RAN1 agreed to use a single stage SCI including Source ID and Destination ID in PSCCH, as below;

Agreement For a dedicated resource pool for SL positioning, only a single stage SCI is used. PSCCH and associated SL-PRS are TDMed in the same slot. Agreement In the dedicated resource pool, - SCI for SL-PRS should at least indicate the following values: - Source ID - Destination ID

If a single stage SCI in PSCCH includes full 24-bits Source ID and Destination ID in a dedicated resource pool, PSSCH (i.e. 2nd SCI and SL MAC subheader) transfer for SL-PRS filtering is not needed. Also, MAC layer can use this single stage SCI in PSCCH for this decoded packet delivering if needed.

In a shared resource pool, RAN1 agreed that PSCCH (i.e. 1st SCI), PSSCH (i.e. 2nd SCI and SL-SCH) and SL-PRS can be multiplexed in a slot, as below;

Agreement In a shared resource pool, SL-PRS, associated PSCCH and PSSCH scheduled by the PSCCH are included in the same slot: - The PSSCH is used for 2nd SCI and SL-SCH - Note: the UE may not have data available for transmission. Up to RAN2 how to define the specification support for this case.

If UE has data available for transmission in a shared resource pool, PSSCH (i.e. 2nd SCI and SL-SCH) is TDMed with PSCCH and SL-PRS, then receiving UE can deliver the decoded MAC PDU to the disassembly and demultiplexing entity as legacy rule. Otherwise (i.e., UE does not have data available for transmission), RAN2 should discuss how to define the specification support for this case, as RAN1 pointed out.

Proposal 2.	RAN2 to discuss how to support Source/Destination identification for the case that UE does not have data available for transmission.

## 2.3	Aspect of SL-PRS multiplexing

As seen as above, RAN1 agreed that PSCCH (i.e. 1st SCI), PSSCH (i.e. 2nd SCI and SL-SCH) and SL-PRS can be multiplexed (i.e. TDMed) in the same slot in a shared resource pool. It has not been decided that which layer performs multiplexing (i.e. TDMing) between LCH, MAC CE and SL-PRS in this case.

Not sure but RAN2 first could check if it is feasible to perform multiplexing in MAC layer, then may decide whether MAC layer to support or not. If decided to support it, a new multiplexing mechanism and priority rule between legacy LCP priority and SL-PRS priority is needed.

Proposal 3.	RAN2 to discuss whether MAC layer to support multiplexing of LCH, MAC CE and SL-PRS or not.

## 2.4		Aspect of SL-PRS triggering

According to TS 38.321, legacy resource selection procedure is triggered on conditions; when SL data is available in a logical channel, when MAC CE is triggered, or for retransmission of a MAC PDU. That is, current rule for resource selection procedure triggering relies only on SL-SCH data transmission. Due to that PSSCH (i.e. SL-SCH) is not included in a dedicated resource pool, new condition for resource selection procedure triggering according to SL-PRS triggering events is needed in MAC layer.

In RAN1 agreements, SL-PRS transmission could be triggered by higher layers (e.g. SLPP and/or RRC) and/or lower layer (i.e. SCI), as below;

Agreement In Scheme 2, with regards to the triggering of SL-PRS, support one or both of the following options: - Option 1: Support SL-PRS transmission triggering at the physical layer by the UE’s own higher layers. - Note: this also includes higher layer triggering from another UE - Option 2: Support UE-A to request UE-B to transmit SL-PRS via lower layer signaling sent by UE-A. - FFS: Whether lower-layer signaling is SCI or SL MAC-CE Agreement In Scheme 2, with regards to the triggering of SL-PRS, confirm the related WA for shared and dedicated resource pools. With regards to the lower-layer signalling, support SCI associated with SL-PRS transmission

Therefore, RAN2 should discuss resource selection procedure for SL-PRS transmission according to SL-PRS triggering.

Proposal 4.	RAN2 to discuss resource selection procedure for SL-PRS transmission according to SL-PRS triggering.

# 3	Conclusion

Proposal 1.	RAN2 to discuss the SL-PRS priority configuration via higher layers.

Proposal 2.	RAN2 to discuss how to support Source/Destination identification for the case that UE does not have data available for transmission.

Proposal 3.	RAN2 to discuss whether MAC layer to support multiplexing of LCH, MAC CE and SL-PRS or not.

Proposal 4.	RAN2 to discuss resource selection procedure for SL-PRS transmission according to SL-PRS triggering.

# 4	References

- R1-2302256, Final Report of 3GPP TSG RAN WG1 #112 v1.0.0 (Athens, Greece, 27th February – 3rd March 2023)

- R1-2304301, Final Report of 3GPP TSG RAN WG1 #112bis-e v1.0.0 (Online, 17th - 26th April 2023)

- Draft Report of 3GPP TSG RAN WG1 #113 v0.2.0 (Incheon, South Korea, 22nd – 26th May 2023)