3GPP TSG-RAN WG2 Meeting #123	R2-2308603

Toulouse, France, 21 – 25 August 2023

Agenda item:	7.12.2.1

Source:	Nokia, Nokia Shanghai Bell, LG Electronics Inc.

Title:	Way forward for RACH-less HO in mobile IAB

WID/SID:	NR_mobile_IAB-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

During RAN2#121bis and RAN2#122, RACH-less handover for UEs on-board a mobile IAB was discussed.

- Feasibility of beam handling during RACH-less HO in the mIAB WI is FFS (and this need to be addressed for RACH-less to be supported for mIAB).

- RAN2 discuss further the following options to support beam operation for the first UL transmission/DL reception towards the target logical DU in RACH-less HO during DU migration:

Option 1: (Explicit approach) Explicit beam information is included in HO command. FFS the details.

Option 2: (Implicit approach) UE re-uses the same beam status as in the source cell (the beam information is not carried explicitly in HO command).

- RACH-less HO with same TA with security key change is in scope for served UEs during mIAB DU migration. FFS UL grant and HO completion procedure in mIAB RACH-less HO.

RAN2#121bis:

- RAN2 think that to have a fast handover from UE point of view for legacy UEs it is important that the target cell is known to the UE (detected and measured).

- For RACH-less, if supported, there would need to be a beam indication (in RRC HO command), which seems feasible in this release from R2 perspective. R2 assumes that the network can know/select the beam, either from network impl specific knowledge or from UE measurement report (legacy report).

- for the UL grant and HO completion in RACH-less HO:

- 1. Both type-1 configured grant and dynamic grant are supported

- 2. FFS handling of supervision timer and when HO is considered successfully complete (expect to align with other WI).

- Send LS to RAN3 to check whether there are issues / feasibility concerns

RAN2#122:

Based on the discussions, RAN2 has still not agreed to support RACH-less HO during mobile IAB node migration. Despite certain directions being outlined, several issues remain to be addressed. In this paper we outline these issues and discuss the possible ways forward for this topic.

# 2	Discussion

## 2.1	Issues with RACH-less HO for mobile IAB

One of the key issues to be addressed for a mIAB-specific RACH-less HO solution is related to the beam handling.

To make progress on this issue, some companies proposed that RAN2 could focus on scenarios where the same beam configuration can be used for the source and target logical DU (i.e. the beam information for the target is known implicitly based on the beam information from the source). In our view, usage of the same beam for source and target DUs is relevant to the case where the logical DUs are using the same physical cell resources; however, RAN2 previously agreed not to focus on this scenario in RAN2#119bis (R2-2211101):

RAN2 focuses on the scenario where, during full migration, the UE sees the two logical DU cells as different physical cells (e.g. with different PCI if same carrier), and where the two logical DU cells use separate physical resources (i.e., different carriers, or orthogonal time and frequency resources of the same carrier, as supported by legacy L1).

For the case where the source and target cell use different physical cell resources, e.g. different carrier or BWP, the same beam configuration cannot be assumed in general between the DUs. In those cases, the implicit approach to determining beam information for the target DU cell could be suboptimal. For example, it may result in a UE not selecting the appropriate beam and falling back to RACH-based procedure anyway. Therefore, if RACH-less handover is supported in mobile IAB, it will need to be based on the UE obtaining explicit beam information for the target cell.

Observation 1: RACH-less HO, if supported for mobile IAB, will require the UE to obtain explicit beam information for the target DU cell; otherwise, the solution will not be practical.

Based on the discussions at RAN2#122, most companies also agreed that an explicit beam indication would need to be provided to a UE; however, despite being a more practical solution, adopting the explicit approach leads to additional issues that will need to be discussed, e.g. related to how beam information for the target DU is determined prior to handover and how the beam-related information is shared between target and source donor-CUs.

First, is the issue of how the target cell beam information is determined so that it can be indicated in the handover command to the UE. Some companies speculated that the beam information can be indicated blindly (perhaps based partly on network implementation-specific configuration). Depending on how blind beam indication is implemented, there are different issues: if the indication is too granular, there could be a similar limitation to the implicit beam approach in that it may result in suboptimal beam selection and cause failed RACH-less HO; alternatively, if the indication is not granular enough, it could require the UE to spend time taking more precise measurements to refine the beam selection, which could minimize the benefits of avoiding the RACH procedure anyway.

The alternative to blind beam indication is to require the UE to detect and measure the beams of the target DU cell as part of the handover preparation phase so that the target cell can determine the appropriate beam to be indicated in the handover command. Once again, requiring the UE to take beam-level measurements of the target DU cell in this way could offset the benefits of the RACH-less procedure.

Observation 2: RACH-less HO will be more reliable if the target cell beam indication is based on UE measurements (instead of blind beam indication); however, this introduces further delays to the handover procedure that could offset the benefits of the RACH-less approach.

Regardless of which approach would be adopted for determining the beam information (blind indication based on network-specific knowledge or UE measurement-based approach), coordination is required with other WGs in order to fully specify a RACH-less solution. If we assume a network-based approach, coordination can possibly be limited to RAN3; however, as mentioned above, a purely network-based approach to beam indication may not perform very well. If we instead adopt a measurement-based approach to beam indication, RAN1’s input would also be necessary; however, RAN1 does not have any TUs allocated for this WI, which could make coordination difficult.

Observation 3: If RAN2 aims to specify a mIAB-specific RACH-less solution where beam indications are based on UE measurements, coordination needs to be done with RAN1 and RAN3; however, RAN1 has no TUs for this WI.

Considering these issues, we view RACH-less HO as an optimization rather than a feature to be prioritized for mobile IAB. Indeed, compared to the overall effort required to specify a mIAB-specific RACH-less HO solution, the relative benefit could be quite small in terms of signalling offload:

-	mIAB-specific RACH-less HO would only be applicable to Rel-18 UEs in RRC_CONNECTED that support this capability, i.e. a small number of UEs relative of the total users on board the node, so the overall signalling offload would be minimal.

-	Requiring RACH-less UEs to measure and report the beams of the target DU cell in order to support explicit beam indication causes additional time interruptions (which would already be absorbed by RACH-based HO procedure anyway).

-	Signalling storms can already be mitigated without new enhancements in RAN2 based on network implementation, e.g. by delaying/spreading out UE handovers during migration (not an issue since the DUs use different physical resources) [1],[2] and/or by appropriate network planning [3], so RACH resources are unlikely to be very constrained anyway.

Observation 4: The benefits gained from RACH-less HO in mobile IAB seem quite low relative to the specification effort.

Proposal 1: RAN2 does not pursue further RACH-less handover enhancements specific to mobile IAB.

## 2.2	Possible way forward with RACH-less HO for mobile IAB

Based on the above discussion, our view is that RAN2 should not specify a RACH-less HO solution specific to mobile IAB in Release 18. However, given that many companies in RAN2 want RACH-less HO in mobile IAB, one way forward could be to optionally apply the RACH-less feature being developed under the Rel-18 NTN WI to on board UEs in some scenarios (if it is concluded that NTN RACH-less can be applied in those cases).

For Rel-18 NTN, RAN2 made considerable progress on inter-gNB RACH-less HO at RAN2#121bis and RAN2#122. Indeed, many of the agreements now reached for RACH-less HO in Rel-18 NTN address aspects left FFS or not even discussed yet for RACH-less in mIAB, e.g. handling of grants, timers, HO completion. Furthermore, RAN2 is already engaged in discussions with RAN1 regarding beam selection/grant monitoring, although RAN1 is still discussing the case where multiple beams could be indicated. (Relevant agreements and issues discussed the LSs are shown below in the Appendix.)

Observation 5: RAN2 has made good progress on RACH-less HO under the Rel-18 NTN WI.

Given the current issues/progress on RACH-less HO for mobile IAB in contrast to Rel-18 NTN, we think a more prudent way forward would be to discuss whether the RACH-less HO feature developed for Rel-18 NTN could optionally be supported by UEs in mobile IAB in some cases (for example, RAN2 already agreed that same TA case would be applicable to mIAB, which is one of the cases supported by NTN RACH-less). To determine this RAN2 may need to first progress more on NTN RACH-less, followed by some pipelining between NTN and mIAB WIs or perhaps a common NTN-mIAB session held in an upcoming meeting. In any case, if NTN RACH-less HO is adopted somehow in mobile IAB, RAN2 should avoid introducing mIAB-specific optimizations/UE capabilities in Rel-18, since it will create additional specification effort.

Proposal 2: RAN2 should confirm progress on Rel-18 NTN RACH-less and discuss whether it is applicable to mobile IAB in some cases.

# 3	Conclusion

This document has made the following observations:

Observation 1: RACH-less HO, if supported for mobile IAB, will require the UE to obtain explicit beam information for the target DU cell; otherwise, the solution will not be practical.

Observation 2: RACH-less HO will be more reliable if the target cell beam indication is based on UE measurements (instead of blind beam indication); however, this introduces further delays to the handover procedure that could offset the benefits of the RACH-less approach.

Observation 3: If RAN2 aims to specify a mIAB-specific RACH-less solution where beam indications are based on UE measurements, coordination needs to be done with RAN1 and RAN3; however, RAN1 has no TUs for this WI.

Observation 4: The benefits gained from RACH-less HO in mobile IAB seem quite low relative to the specification effort.

Observation 5: RAN2 has made good progress on RACH-less HO under the Rel-18 NTN WI.

And proposed the following:

Proposal 1: RAN2 does not pursue further RACH-less handover enhancements specific to mobile IAB.

Proposal 2: RAN2 should confirm progress on Rel-18 NTN RACH-less and discuss whether it is applicable to mobile IAB in some cases.

# 4		References

[1]	R2-2302784: “Mobile IAB mobility enhancement for connected UEs”, Intel Corporation

[2]	R2-2302929, “Mobile IAB connected mode issues and enhancements”, Nokia, Nokia Shanghai Bell

[3]	R2-2303852, “On the need for connected mode mobility enhancements for mIAB”, LG Electronics Inc.

# Appendix: Progress on RACH-less HO from Rel-18 NTN

Agreements after RAN2#121bis (EOM report R2-2304601):

- NTN RACH-less HO is supported for Intra-satellite handover with the same feeder link. i.e., with same gateway/gNB;

- NTN RACH-less HO can be supported for intra-satellite handover with different feeder links, i.e., with gateway/gNB switch, inter-satellite handover with gateway/gNB switch, and inter-satellite handover with same gateway/gNB.

- RAN2 confirms the general UE procedure for NTN RACH-less HO

1.   receive a RACH-less HO command which can include pre-allocated grant optionally. FFS N_TA is optional. (RRC)

2.   start timer T304 for the target cell (RRC)

3.   perform DL and UL synchronization, and start timer T430. FFS how to perform RACH-less UL synchronization to NTN target cell. (RRC, MAC)

4.   start time alignment timer (MAC)

5.   monitor target cell PDCCH for dynamic grant if pre-allocated grant is not configured in RACH-less HO command (MAC, PHY)

6.   send initial UL transmission including RRCReconfigurationComplete message using the available UL grant (RRC, MAC, PHY)

7.   consider RACH-less HO is completed upon receiving NW confirmation. FFS how to confirm RACH-less HO is successfully completed. (RRC, MAC)

8.   stop timer T304 for the target cell. (RRC)

FFS whether to release UL grant if pre-allocated after RACH-less HO completion

FFS RACH-less HO failure handling, e.g. whether UE fallback to RACH-based HO to the target cell

FFS procedure for RACH-less HO combined with PCI unchanged or CHO if supported

- The pre-allocated grant is provided as type-1 CG

- Send an LS to RAN1 informing RAN2 agreements on NTN RACH-less HO and check RAN1 views on the following aspects:

1.   whether the pre-allocated grant is provided with association to SSBs; if so, whether a RSRP threshold is configured for SSB selection.

2.   to monitor target cell PDCCH for dynamic grant for initial UL transmission, whether beam indication can be provided in RACH-less HO command.

3.   power control for initial UL transmission

- At least for pre-allocated grant, for the confirmation of RACH-less HO completion we reuse of LTE approach, i.e., UE Contention Resolution Identity MAC CE is used but UE ignores the content of this field. FFS if anything else is needed for dynamic grant

Agreements after RAN2#122 (EOM report R2-2307001):

- In NTN RACH-less handover, NW either indicates NTA in the target cell is identical to the source cell, or the NTA explicitly provided by the NW is 0. RAN2 will not discuss the case where NTA does not equal to 0

- From RAN2 perspective synchronization among source and target cells is not an issue in NTN RACH-less HO

- Release pre-allocated UL grant after RACH-less HO completion

- LTE approach (of confirming the HO completion) is reused for both pre-allocated grant and dynamic grant. FFS any enhancement to the confirmation of RACH-less HO completion, e.g. the NW does not send the UE Contention Resolution Identity MAC CE, and sends PDCCH/PDSCH addressed to C-RNTI

- Remove “FFS how to perform RACH-less UL synchronization to NTN target cell”, RAN2 assumes the UL sync handling in the target cell is the same in RACH-based HO and RACH-less HO, except how to acquire NTA (FFS on the spec impact , if any)

RAN2 questions to RAN1 after RAN2#121bis (LS R2-2304271):

RAN2 has also identified some issues which are more relevant to RAN1 and would like to check RAN1 views on the following aspects for NTN RACH-less HO.

1. Regarding the pre-allocated grant for initial UL transmission, considering the similarity to Msg1 in RACH and the similarity to the initial UL transmission in CG-SDT, where PRACH/PUSCH resource is mapped to SSBs, whether the pre-allocated grant is provided with association to SSB(s)? If yes, whether a RSRP threshold is needed for SSB selection for initial UL transmission?

2. To monitor target cell PDCCH for dynamic grant for initial UL transmission, whether beam selection is needed (e.g., performed by NW with selected beam(s) indicated, or performed by UE)?

3. Regarding the power control for initial UL transmission, whether it follows the rules specified for PUSCH scheduled by Random Access grant or by configured grant or others?

RAN1 response to RAN2 after RAN2#122 (LS R2-2307011):

1. Regarding the pre-allocated grant for initial UL transmission, considering the similarity to Msg1 in RACH and the similarity to the initial UL transmission in CG-SDT, where PRACH/PUSCH resource is mapped to SSBs, whether the pre-allocated grant is provided with association to SSB(s)? If yes, whether a RSRP threshold is needed for SSB selection for initial UL transmission?

RAN1 response: One company thinks that when the network knows the suitable DL beam for RACH-less handover, the pre-allocated grant can be associated with a SSB index of the target cell, and when the network does not know the suitable DL beam, RACH-based HO can be used instead of introducing beam-sweeped pre-allocated grants associated with multiple SSB indexes. Other companies think that the association between the pre-allocated grant for initial transmission and SSB index should be supported without any condition(s), and think that RSRP threshold may be helpful.

2. To monitor target cell PDCCH for dynamic grant for initial UL transmission, whether beam selection is needed (e.g., performed by NW with selected beam(s) indicated, or performed by UE)?

RAN1 response: If single beam is indicated, UE will monitor the target cell PDCCH scheduling the first PUSCH based on the indicated beam. RAN1 will further discuss the case where multiple beams are indicated.

3. Regarding the power control for initial UL transmission, whether it follows the rules specified for PUSCH scheduled by Random Access grant or by configured grant or others?

RAN1 response:  RAN1 will further discuss the issue.