3GPP TSG-RAN WG2 Meeting #123	R2-2308606

Toulouse, France, 21st-25th August 2023

Agenda item:	7.3.2

Source: 	Fujitsu

Title: 	Discussion on Cell DTX/DRX

Document for:	Discussion and decision

Introduction

In RAN2#122 meeting, RAN2 agreed the following on the Cell DTX/DRX [1].

Agreements

1	UE monitors PDCCH for RAR during Cell DTX non-active time. The ra-ResponseWindow could be started as legacy.

2	UE monitors PDCCH for msg4 during Cell DTX non-active time. The ra-ContentionResolutionTimer could be started as legacy.

3	Working assumption:  When the retransmission timer is running (if C-DRX is configured), the UE is expected to monitor PDCCH, like in legacy.  It is up to the network whether it schedules retransmissions out of the Cell DTX active period, i.e., when the DRX retransmission timer is running, the UE should monitor PDCCH regardless of the Cell DTX.

4	Once gNB recognizes there is an emergency call or public safety related service (e.g. MPS/MCS), the NW should ensure there is no impact to the emergency call (e.g. may deactivate Cell DTX/DRX).  The behavior is captured in stage 2 spec

5	When an DG grant is received, by the gNB during cell DRX/DTX, the UE follows the grant assignment (i.e. like in legacy).  This includes DL HARQ feedback.

In addition, RAN1 sent the LS reply regarding a feasibility of L1 signalling for Cell DTX/DRX activation/deactivation in [2]. In this LS, RAN1 supports the group common L1 signalling without HARQ feedback, but also asked RAN2 about MAC CE based indication as highlighted below.

- Support of L1 signaling at least for activation/deactivation of a cell DTX and/or DRX configuration is feasible (e.g., in terms of enabling/disabling the feature) from RAN1 perspective.

- RAN1 supports the group common L1 signaling using PDCCH for cell DTX/DRX activation and deactivation without HARQ feedback.

- subject to UE capability

- RAN1 asks RAN2 to consider the additional support of a MAC CE based indication.

In this contribution, we would like to discuss the remaining open issues on the Cell DTX/DRX.

Discussion

MAC CE based indication

As in RAN1 LS reply [2], they asked to the additional support of a MAC CE based indication. Because RAN1 agreed L1 signalling using PDCCH for Cell DTX/DRX activation/deactivation without HARQ feedback, in order to improve a reliability, it can be considered whether the MAC CE based indication is introduced similar to the CG confirmation. However, RAN2 already agreed Cell DTX/DRX is activated or deactivated implicitly by RRC signalling. If more reliable signalling is needed, RRC based activation/deactivation is suitable for this purpose. In addition, even if RAN2 does not introduce the MAC CE based indication and the UE fails to receive the DCI command, it does not cause negative impact on the network energy saving gains. In our views, when the UE fails to receive the DCI command, the UE performance might be decreased due to incorrect CSI-RS measurements during non-active period. But it can be acceptable because there are no high-speed data UEs in the cell, thus it is not a critical issue. Meanwhile, if the MAC CE based indication is introduced and the gNB cannot receive the confirmation MAC CE, the gNB may send the L1 signalling again or switch to send a dedicated RRC signalling for the UE. It will make the implementation complicated, therefore it should not be introduced.

Observation 1:	The additional MAC CE based indication may improve a reliability, however it seems too complicated from implementation perspective.

Observation 2:	The network can rely on the RRC based activation/deactivation if there is a concern about reliability.

Proposal 1:	It is not necessary to support the additional MAC CE based indication corresponding to PDCCH for Cell DTX/DRX activation/deactivation.

Support of multiple configurations

It is still an FFS whether multiple configurations of Cell DTX or DRX will be supported. A benefit of multiple configurations is that the network can quickly apply an appropriate Cell DTX/DRX pattern based on a cell traffic. For example, if there are only low priority QoS traffics, the network can use a very long Cell DTX/DRX pattern. However there are middle priority QoS traffics or cell load is increased, the network can switch a shorter Cell DTX/DRX pattern to meet the QoS requirement.

If RRC signalling is used to switch the Cell DTX/DRX patterns, it causes very high RRC signaling overheads. However, as in RAN1 LS [2], it is feasible to use L1 signalling to activate/deactivate Cell DTX/DRX. It means L1 signalling can be used to indicate for appropriate Cell DTX/DRX pattern timely. To make a specification simple and according to the agreement that pattern configuration is common for Rel-18 UEs in a cell, we think only one configuration can be activated, even if multiple configurations are configured.

Observation 3:	If L1 activation/deactivation signalling is adopted, the network can timely switch the appropriate Cell DTX/DRX pattern depend on the cell traffics.

Proposal 2:	RAN2 supports multiple Cell DTX/DRX configurations per serving cell.

Proposal 3:	Only one configuration can be activated, even if multiple Cell DTX/DRX configurations are configured.

SR transmissions

In RAN2#122 meeting, for an emergency call or public safety related service, it should be handled by the network implementation, e.g., deactivate Cell DTX/DRX after service recognition. However, it is still FFS whether an exceptional handling is necessary for SR transmissions during Cell DTX non-active period as follows:

FFS: whether we will allow to configure the UE per SR configuration with whether SR can be transmitted during Cell DRX non-active period to to support high priority traffic

FFS how to deal with any exceptions (e.g. SR if agreed and RACH).

From the network energy saving efficiency perspective, it is useful that any SR occasions are skipped during Cell DTX non-active period. But it may jeopardize the UE QoS requirements especially a delay sensitive service. Some companies proposed SR RACH can be used to request resources during Cell DTX non-active period. But RAN2 agreed that SR RACH is not triggered if SR is dropped, therefore it is not sure that the SR RACH can be ensured to fulfil the requirements due to contention.

For SR transmission, there are trade-offs between network energy saving gain and scheduling flexibility. Hence, it would be good to be configurable by the network. If the network wants to maximize the network energy saving gain, it can be simply set to “not allowed”. On the other hand, if a delay sensitive service can be accepted in the serving cell, immediate SR transmission should be allowed and indicated by SR configuration. As the SR is supposed to be configured per LCH, it is natural to indicate whether it is allowed per SR configuration.

Observation 4:	It is reasonable that the network allows an SR transmission during Cell DRX non-active period for a high priority traffic.

Proposal 4:	The network indicates per SR configuration whether an SR transmission is allowed during Cell DRX non-active period.

If the SR is transmitted during the Cell DRX non-active period, it is natural for the network to send a corresponding PDCCH for a new transmission immediately after the SR reception. Because the SR is triggered by the high priority traffic then it is urgent to allocate radio resources to archive the UE QoS requirement. Therefore, in this case, the network should not delay transmitting a scheduling grant until the next Cell DTX active period.

The UE behaviors are the same as the current C-DRX mechanism, i.e., the active period of the C-DRX is started after sending SR PUCCH and monitor the PDCCH for a new transmission during C-DRX active period even during Cell DTX non-active period.

Proposal 5:	If the SR transmission is allowed during Cell DRX non-active period, the UE should monitor a PDCCH after the SR transmission regardless of Cell DTX configuration.

Proposal 6:	If the SR with high priority is transmitted, the network can schedule a dynamic UL grant for a new transmission during C-DRX active period even during Cell DTX non-active period.

Conclusion

In this contribution, we have some discussions for remaining issues on the Cell DTX/DRX issues, and the following proposals are made:

MAC CE based indication:

Observation 1:	The additional MAC CE based indication may improve a reliability, however it seems too complicated from implementation perspective.

Observation 2:	The network can rely on the RRC based activation/deactivation if there is a concern about reliability.

Proposal 1:	It is not necessary to support the additional MAC CE based indication corresponding to PDCCH for Cell DTX/DRX activation/deactivation.

Support of multiple configurations:

Observation 3:	If L1 activation/deactivation signalling is adopted, the network can timely switch the appropriate Cell DTX/DRX pattern depend on the cell traffics.

Proposal 2:	RAN2 supports multiple Cell DTX/DRX configurations per serving cell.

Proposal 3:	Only one configuration can be activated, even if multiple Cell DTX/DRX configurations are configured.

SR transmissions:

Observation 4:	It is reasonable that the network allows an SR transmission during Cell DRX non-active period for a high priority traffic.

Proposal 4:	The network indicates per SR configuration whether an SR transmission is allowed during Cell DRX non-active period.

Proposal 5:	If the SR transmission is allowed during Cell DRX non-active period, the UE should monitor a PDCCH after the SR transmission regardless of Cell DTX configuration.

Proposal 6:	If the SR with high priority is transmitted, the network can schedule a dynamic UL grant for a new transmission during C-DRX active period even during Cell DTX non-active period.

References

- R2-2306543

- R2-2307017