3GPP TSG-RAN WG2 Meeting #123                                    R2-2308607

Toulouse, France, August 21-25, 2023

Agenda Item	: 7.5.4.2 (NR_XR_enh-Core)

Source	: LG Electronics Inc.

Title	: Discussion on the discard for XR

Document for	: Discussion and Decision

Introduction

In this contribution, we show our views on the discard operation for XR.

Discussion

In RAN2#122 meeting, the following agreements were made.

Network indicates UE to apply PSI-based XR discard mechanism via dedicated signalling. FFS how/whether to minimize additional UL signalling after this indication. FFS if the NW indication is a one-shot or also subsequent packets

However, how to perform PSI based discard and how to indicate PSI based discard were not concluded

According to the RAN2#122 meeting, the following options were proposed.

- Option 1. The UE immediately discards PDUs associated with a PDU set regardless of the remaining time of the discardTimer for the PDUs whose PSI is lower than the PSI threshold.

- Option 2. The network configures two dicardTimer values for a PDU set.

- Option 3. The network configures different discardTimer values based on the PSI for each PDU set.

For Option 1, the network should indicate a PSI threshold. Then, the PDCP entity discards all PDCP SDUs associated with a PDU set if the PSI for the PDU set is lower than the PSI threshold right after receiving the congestion indication. In this case, the network congestion can be easily relaxed by discarding the SDUs associated with the PDU set whose PSI is lower than the PSI threshold for the multiple UEs.

For Option 2, the PDCP entity is configured with two PDCP discardTimer values. More specifically, in case of non-congestion, the PDCP entity uses the long PDCP discardTimer value, and, in case of congestion, the PDCP entity uses the short PDCP discardTimer value. The congestion indication triggers the change between different PDCP discardTimer values.

This option is essentially same as the reconfiguration of PDCP discardTimer values. According to the current specification, the UE behaviour at PDCP discardTimer value reconfiguration is specified as follows.

NOTE 1: The state variables which control the transmission and reception operation should not be reset, and the timers including t-Reordering and discardTimer keep running during PDCP entity reconfiguration procedure.

If we follow the legacy procedure, at the time of receiving congestion indication, the already running PDCP discardTimer keeps running, and only the new PDCP SDUs are associated with the short PDCP discardTimers. Therefore, the congestion situation is not alleviated immediately, but gradually alleviated based on short PDCP discardTimer for new PDCP SDUs. Then, the usefulness of congestion indication is questioned.

For Option 3, a PDCP entity should maintain multiple PDCP discardTimer values at the same time because PDU sets with different PSIs can be mapped to a single DRB. When the PDCP entity receives a PDCP SDU from upper layers, the PDCP entity should check the PSI of the PDCP SDU and start the PDCP discardTimer with the value related to the PSI of the PDCP SDU. This option may be applied to the non-congestion and congestion situation without congestion indication.

According to the 38.300, the DRB serves the packet with the same QoS requirement, e.g., PSDB. In other words, the QoS flows with the same QoS requirement can be mapped with one DRB, and the QoS flows with different QoS requirements should be mapped with different DRBs.

At Access Stratum level, the data radio bearer (DRB) defines the packet treatment on the radio interface (Uu). A DRB serves packets with the same packet forwarding treatment. The QoS flow to DRB mapping by NG-RAN is based on QFI and the associated QoS profiles (i.e. QoS parameters and QoS characteristics). Separate DRBs may be established for QoS flows requiring different packet forwarding treatment, or several QoS Flows belonging to the same PDU session can be multiplexed in the same DRB.

However, according to the PSDB definition, a QoS flow is associated with only one PSDB. In other words, if different QoS flows have different PSDBs, different QoS flows should be mapped to the different DRBs. Considering that the PSDB is associated with the PDCP discardTimer, it may be a huge impact on the RAN2 specification as well as other working group specifications if we introduce option 3.

Considering the above, we think Option 1 has the practical benefit and lowest complexity among above options. Thus, we prefer to go for Option 1.

Proposal 1. If the network indicates the PSI based discard operation, the UE immediately discards PDUs associated with a PDU set whose PSI is lower than the PSI threshold regardless of the remaining time of the discardTimer for the PDUs.

In order to indicate the PSI threshold, there are two methods. The first method is that the network configures the PSI threshold in a semi-static manner, e.g., RRC signalling, and the second method is that the network indicates the PSI threshold in a dynamic manner, e.g., PDCP control PDU or MAC CE.

Considering that the congestion level can be changed, the network may want to change the PSI threshold based on the congestion level. In this case, the semi-static configuration is not suitable. Thus, we prefer to indicate the PSI threshold in a dynamic manner.

Observation 1. The PSI threshold should be indicated in a dynamic manner.

For observation 1, the PDCP control PDU and the MAC CE can be considered. In our view, since the XR traffic can be transmitted per DRB level and the PDU set discard is performed in PDCP, the network should indicate the PSI based SDU discard to a UE using PDCP control PDU.

Proposal 2. Network indicates the PSI based SDU discard to a UE using PDCP control PDU containing the PSI threshold.

In addition, since the UM DRB can be used to transmit the XR traffic, the PDCP control PDU for the PSI based SDU discard may be lost in the air. Therefore, the network may transmit multiple PDCP control PDUs for PSI based SDU discard to the UE. In this case, the start or stop indication for performing the PSI based SDU discard is required because the UE may receive multiple PDCP control PDUs for PSI based SDU discard.

Proposal 3. The start/stop of the PSI based SDU discard is indicated in the PDCP control PDU.

For proposal 1~3, we provide the text proposal in section 4.

Conclusion

Based on the above discussion, we made following proposals and observations.

Proposal 1. If the network indicates the PSI based discard operation, The UE immediately discards PDUs associated with a PDU set whose the PSI is lower than the threshold regardless of the remaining time of the discardTimer for the PDUs.

Observation 1. The threshold should be indicated in a dynamic manner.

Proposal 2. Network indicates the PSI based SDU discard to a UE using PDCP control PDU containing the threshold.

Proposal 3. The start/stop of the PSI based SDU discard is indicated in the PDCP control PDU.

Text proposal

- 5.3	SDU discard

When the discardTimer expires for a PDCP SDU, or the successful delivery of a PDCP SDU is confirmed by PDCP status report, the transmitting PDCP entity shall discard the PDCP SDU along with the corresponding PDCP Data PDU.

When the PDCP Control PDU for the PSI based SDU discard (i.e. discard all PDCP SDUs belonging to the PDU Set whose PSI is less than the RPSI) activation/deactivation is received, the transmitting PDCP entity shall:

-	if the PSD field is sets to 1:

- 	activate the PSI based SDU discard;

-	else:

- 	deactivate the PSI based SDU discard.

If the corresponding PDCP Data PDU has already been submitted to lower layers, the discard is indicated to lower layers.

For SRBs, when upper layers request a PDCP SDU discard, the PDCP entity shall discard all stored PDCP SDUs and PDCP PDUs.

NOTE:	Discarding a PDCP SDU already associated with a PDCP SN causes a SN gap in the transmitted PDCP Data PDUs, which increases PDCP reordering delay in the receiving PDCP entity. It is up to UE implementation how to minimize SN gap after SDU discard.

………………………………………………… omit ……………………………………………

- 6.1.2	Control PDU

The PDCP Control PDU is used to convey one of followings in addition to the PDU header:

-	a PDCP status report;

-	an interspersed ROHC feedback;

-	an EHC feedback;

-	a UDC feedback;

-	a PSI based SDU discard activation/deactivation.

………………………………………………… omit ……………………………………………

- 6.2.3.x	Control PDU for PSI based SDU discard activation/deactivation

Figure 6.2.3.x-1 shows the format of the PDCP Control PDU carrying PSI based SDU discard activation/deactivation. This format is applicable for UM DRBs and AM DRBs.

Figure 6.2.3.x-1: PDCP Control PDU format for PSI based SDU discard

………………………………………………… omit ……………………………………………

- 6.3.8	PDU type

Length: 3 bits

This field indicates the type of control information included in the corresponding PDCP Control PDU.

Table 6.3.8-1: PDU type

| Bit | Description |
| --- | --- |
| 000 | PDCP status report |
| 001 | Interspersed ROHC feedback |
| 010 | EHC feedback |
| 011 | UDC feedback |
| 100 | PSI based SDU discard activation/deactivation |
| 101-111 | Reserved |

- ………………………………………………… omit …………………………………………

- 6.3.x	PSD

Length: 1 bit

This field indicates whether to activate/deactivate the PSI based SDU discard.

Table 6.3.x-1: PSD field

| Bit | Description |
| --- | --- |
| 0 | Deactivaton the PSI based SDU discard |
| 1 | Activate the PSI based SDU discard |

- 6.3.x	RPSI

Length: 8 bits

This field indicates the reference PSI that is used for discarding PDCP SDUs belonging to a PDU Set whose PSI is less than the RPSI when the PSI based SDU discard is activated.