3GPP TSG-RAN WG2 Meeting #123	R2-2308609

Toulouse, France, 21st-25th August 2023

Agenda item:	7.7.4.2

Source: 	Fujitsu

Title: 	Discussion on NTN handover enhancements

Document for:	Discussion and decision

Introduction

In the RAN2#122 meeting, for the unchanged PCI scenario, the followings were agreed on NTN HO enhancements [1].

Agreements:

- t-Service in SIB19 can also be interpreted by Rel-18 UE in Connected mode to know that a satellite change or feeder link change happens

- In hard switch unchanged PCI scenario (i.e. no handover), the UE needs to know the time the UE attempts to re-synchronize. (FFS whether a new “t-Start” / a t-gap is needed or whether t-Service can be reused (i.e. no other IE) if the gap is very short/zero).

In addition, RAN1 didn’t identify any feasibility issues on unchanged PCI scenario with hard satellite switching in reply LS [2].

Reply:

RAN1 discussed the resynchronization of UE when hard switching, given that new common TA, K_mac, ephemeris and cell-specific K-offset are applied during resynchronization to new satellite.

From RAN1 perspective, no feasibility issue is identified for hard satellite switching without PCI change.

In this contribution, we would like to discuss remaining open issues on the satellite switching without PCI change for finalization.

Discussion

Remaining issues on the satellite switching without PCI change

Cell identification

In the unchanged PCI scenario, from the UE perspective, it is seen the same cell even if the satellite is changed, thus the UE does not have to initiate handover procedure during the switch. Meanwhile, from the network perspective, the leaving (old) satellite will stop the beam transmission to the UE and the next (new) satellite using the same gNB will start the beam transmission with the same PCI.

The UE needs the information that the cell belonging to the upcoming satellite uses the same PCI. Otherwise, the UE may lose the DL or UL synchronization at a boundary, and it causes RLF. Therefore, it is reasonable to add a new indication to the SIB19 that the UE can be aware of the unchanged PCI scenario.

Proposal 1: A new indication is introduced to the SIB19 that the next cell uses the same PCI after the satellite switching.

Satellite switch timing

Using the same PCI can reduce RRC signalling transactions during handover procedure, but it cannot reduce the RA procedure due to the UL timing lost. Thus, if the UE expects the next satellite uses the same PCI, the UE should perform DL/UL re-synchronization instead of handover after the satellite switching.

At the satellite switching, UL synchronization, i.e., TA adjustment is required after the satellite switching as well as the different PCI case. In general, the random access (RA) procedure can be used to re-synchronize with the new satellite. As in the Rel-17 NTN, the UE has to acquire TA related RRC information (e.g., TA common) via SIB19 after the satellite switch, therefore the UE will be provided the satellite switch timing from the network. Some companies proposed that the timing can be already indicated by the current SIB19 (i.e., t-service), or add a new parameter to indicate start timing (i.e., t-start) or gap duration between cells (i.e., t-gap). However, it has never discussed the unchanged PCI scenario is also applicable in the discontinuous coverage case. Therefore, given it is reasonable to assume there are almost no gaps between cells, the t-service is enough to indicate to the UE for DL re-synchronization timing.

Observation 1: In the unchanged PCI scenario, there are almost no gaps between cells.

Proposal 2: The UE can be aware of the satellite switch timing by the t-service. No need to introduce additional timing information in Rel-18.

UL synchronization

Combination of RA-less HO

In RAN2#122 meeting, it was proposed to combine the unchanged PCI with RA-less HO in NTN. Because RA-less HO can be supported for inter-satellite handover with the same gNB/Gateway, therefore it is considerable to combine both techniques to reduce signalling overheads. However, it is not clear the current RA-less HO agreements can be reused for the unchanged PCI scenario. Especially as the unchanged PCI does not require the L3 mobility, thus the RRCReconfigurationComplete message is not sent to the target satellite after switching. It means some modification is required to combine RA-less HO with the unchanged PCI scenario. Based on the aspects, it is difficult to discuss an applicability of RA-less HO for the unchanged PCI scenario and would wait for more progress on RA-less HO topics.

Observation 2: As L3 mobility is not required for unchanged PCI scenario, the same procedure of RACH-less HO is not used.

Proposal 3: RAN2 to postpone the discussion about a combination of the unchanged PCI and RA-less HO until RA-less HO mechanism is further clarified.

RA procedure

Given L3 mobility is not involve, the cell common configuration is not changed after switching to mitigate the RRC signalling overhead. It means the RA resources configured from the old satellite should be reused for RA attempt to the new satellite (except for RA-less HO). In general, because the UE identifies the cell by PCI, the UE may not clearly recognize whether the new cell belonging to the new satellite is available or not. Thus, for initiating RA procedure in the same PCI scenario, it is useful the UE needs to check the cell availability before RA attempt. We think at least it is possible and enough to re-acquire the SIB19 broadcasted from the new satellite. So, in the unchanged PCI scenario, the UE should initiate RA procedure after the UE re-acquires the SIB19 and identifies the cell availability via SIB19.

Observation 3: In the unchanged PCI scenario, the UE needs to identify the availability of the new cell for initiating RA procedure.

Proposal 4: In the unchanged PCI scenario, the UE should re-acquire the SIB19 belonging to the new cell and initiate RA procedure after receiving SIB19.

Conclusions

In this contribution, we have some discussions on NTN handover enhancements on the unchanged PCI scenario and the following observations and proposals are made:

Cell identification:

Proposal 1: A new indication is introduced to the SIB19 that the next cell uses the same PCI after the satellite switching.

Satellite switch timing:

Observation 1: In the unchanged PCI scenario, there are almost no gaps between cells.

Proposal 2: The UE can be aware of the satellite switch timing by the t-service. No need to introduce additional timing information in Rel-18.

Combination of RA-less HO:

Observation 2: As L3 mobility is not required for unchanged PCI scenario, the same procedure of RACH-less HO is not used.

Proposal 3: RAN2 to postpone the discussion about a combination of the unchanged PCI and RA-less HO until RA-less HO mechanism is further clarified.

RA procedure:

Observation 3: In the unchanged PCI scenario, the UE needs to identify the availability of the new cell for initiating RA procedure.

Proposal 4: In the unchanged PCI scenario, the UE should re-acquire the SIB19 belonging to the new cell and initiate RA procedure after receiving SIB19.

References

- R2-2306541

- R1-2306210