3GPP TSG-RAN WG2 Meeting #123                                     R2-2308610

Toulouse, France, August 21-25, 2023

Agenda item:		7.5.2 (NR_XR_enh-Core)

Source:	LG Electronics Inc.

Title: 	Discussion on XR awareness

Document for:	Discussion and Decision

1. Introduction

This document shows our views on XR awareness.

2. Discussion

2.1 Report of the periodicity

In RAN2#122 meeting, the following agreements was made for the indication of the end of data burst.

- 2. UL assistance information (burst arrival time, UL jitter, FFS on periodicity) is reported per QoS flow. Network can configure for which QoS flow UE should report assistance information.

As all known, the network is configured for the configured grant for the periodic uplink traffic. In other words, if the network knows the periodicity of the data burst, the network efficiently configures the configured grant. Thus, the UE should report the periodicity in the assistance information.

Proposal 1. The UE should report the periodicity in the assistance information.

2.2 PDCP duplication based on the PSI.

The PSI is introduced to enable the differentiated handling of the PDU set. For example, it is assumed that the I-frame has the high PSI than other frame. Since the P-frame and B-frame are decoded based on the I-frame, if the PDUs for I-frame are not successfully transmitted to the network, P-frame and B-frame do not need to be transmitted to the network. In other words, it is required that the transmission of the PDU set with the high PSI should be more reliable than the transmission of the PDU set with the low PSI.

Observation 1. The transmission of the PDU set with the high PSI should be more reliable than the transmission of the PDU set with the low PSI.

In addition, according to the latest agreements, all PDU sets associated with a single QoS flow would be transmitted through the RLC entities with the same priority, same as legacy. That is, the PDU sets belonging to a QoS flow are transmitted through the same RLC entity even if they have different PSI. In this case, reliable transmission of the PDU set with high PSI may not be ensured due to the transmission of the PDU set with low PSI, e.g. Head-of-Line blocking.

Observation 2. Reliable transmission of the PDU set with high PSI may not be ensured due to the transmission of the PDU set with low PSI.

In order to ensure the reliable transmission of the PDU set with high PSI, we can reuse the PDCP duplication introduced in Rel-15. However, if the PDCP duplication is applied to all PDU sets, the reliable transmission of the PDU set with high PSI still may not be ensured due to the transmission of the PDU set with low PSI.

In addition, considering that the PDCP duplication consumes much radio resource to perform duplicated transmission, applying PDCP duplication to high data rate such as XR traffic would increase huge signalling overhead

Observation 3. The use of the PDCP duplication for XR as it is may increase the signalling overhead significantly.

Based on above observations, the PDCP duplication can be reused with small enhancement in order to ensure reliable transmission of the PDU set with high PSI while minimizing signalling overhead. More specifically, the PDCP entity applies the PDCP duplication only for the PDCP PDUs associated with PDU set with the high PSI, and does not apply for the PDCP PDUs associated with PDU set with the low PSI.

Proposal 2. In order to ensure reliable transmission of the PDU set with high PSI within a DRB, the PDCP entity selectively duplicates the PDCP PDUs associated with PDU set with the high PSI.

3.	Conclusion

Based on the above discussion, we made following observations and proposals.

Proposal 1. The UE should report the periodicity in the assistance information.

Observation 1. The transmission of the PDU set with the high PSI should be more reliable than the transmission of the PDU set with the low PSI.

Observation 2. Reliable transmission of the PDU set with high PSI may not be ensured due to the transmission of the PDU set with low PSI.

Observation 3. The use of the PDCP duplication for XR as it is may increase the signalling overhead significantly.

Proposal 2. In order to ensure reliable transmission of the PDU set with high PSI within a DRB, the PDCP entity selectively duplicates the PDCP PDUs associated with PDU set with the high PSI.