3GPP TSG-RAN WG2 Meeting #123                                                             	  R2-2308614

Toulouse, France, August 21 – 25, 2023

Agenda item: 		7.4.2.1

Source: 		Qualcomm Incorporated

Title: 	LTM procedures

Document for:	Discussion

# 1	Introduction

In RAN2#122 the following agreements were achieved [1]:

Dynamic grant can be used for RACH-less LTM, for the first UL data transmission to the target cell: - the UE monitors PDCCH for dynamic scheduling from the target cell, upon LTM cell switch. - upon cell switch decision, R2 assumes that the source DU informs the target DU about the selected beam, so that the target DU can start scheduling dynamic UL grant. Configured grant can be used for RACH-less LTM, for the first UL data transmission to the target cell, the UE selects the configured grant occasion, which is associated with the beam indicated in the LTM MAC CE (as set by source cell). FFS further optimization For early TA acquisition for candidate Cells: - For PDCCH ordered early TA acquisition without RAR, there is no need for UE to maintain the TA timer for candidate cell (i.e. it is NW implementation to determine the TA validity), TA is given in the cell switch MAC CE (when available in the network). - RAN2 doesn’t see a need for a solution with RAR in for Rel-18. - Observation: Without RAR (without UE maintaining TA), the UE will need to do RACH for link recovery and/or conditional (if supported), which is acceptable in Rel-18 - The UE determines to trigger RACH-less cell switch in MAC layer, if the LTM cell switch MAC CE provides the TA value (no RAR is assumed). - We send LS to R1

In RAN2#121bis-e the following agreements were achieved [2]:

From RAN2 perspective, to enable shared preamble resource among multiple UEs, it is beneficial that the information that identifies the allocated CFRA resource (i.e., SS/PBCH index, RACH occasion, and Random Access Preamble index) can be indicated in the PDCCH order (as legacy intra-cell PDCCH order). RRC RACH configuration for early TA acquisition (e.g., including whether RAR needs to be received) is specific per target cell and is signalled separately (separate IEs) from the candidate cell configuration (the part that need to be applied at cell switch). It is FFS if the following TA values can be given to the UE: - Value 0, - Value indicating that the UE shall apply the TA of one source cell. R2 assumes RRCReconfigurationComplete message is always sent at each LTM execution. In RACH-based LTM, the target cell is aware of the UE’s arrival based on the reception of preamble in CFRA and on the reception of Msg3/MsgA in CBRA, like the legacy HO. In RACH-less LTM, the target cell is aware of the UE’s arrival based on reception of the first UL transmission from this UE In RACH-less LTM, RRCReconfigurationComplete can be the content of the first UL MAC PDU/transmission to indicate UE arrival, i.e. no need to introduce any new signaling to indicate UE arrival (for the MCG-switch case) For RACH-based LTM, the UE considers that LTM execution procedure is successfully completed when the RACH is successfully completed. For RACH-less LTM, the UE considers that LTM execution procedure is successfully complete when the UE determines the NW has successfully received its first UL data. Following behaviors of LTM supervisor timer are agreed: - The UE starts the LTM supervisor timer, upon reception of the LTM cell switch MAC CE; - The UE stops the LTM supervisor timer, upon successful completion of LTM cell switch; - If the LTM supervisor timer for MCG expires, as baseline, the UE considers LTM failure and initiates RRC re-establishment. (SCG switch case FFS) LTM supervisor timer is RRC layer timer. At RLF or LTM execution failure (for MCG), RAN2 intend to support fast recovery to a candidate cell by LTM execution. At least for pre-allocated grant, for the confirmation of RACH-less HO completion we reuse of LTE approach, i.e., UE Contention Resolution Identity MAC CE is used but UE ignores the content of this field. FFS if anything else is needed for dynamic grant

This contribution discusses early TA acquisition for LTM, aspects of RACH-less LTM, LTM failure handling, and LTM for SCG.

# 2	Early TA acquisition

## 2.1	Identification of the UE when preamble resources for early TA acquisition are shared in inter-DU scenario

Figure 1 – gNB-DU2/CU identify the serving DU of the UE that sent the PRACH based on PID and forward the TA result together with the PID. The serving DU associates the TA value to the right UE that was ordered to send the PRACH using that PID.

RAN2#121bis-e agreed that the preamble resource for PDCCH-ordered RACH for early TA acquisition can be shared among multiple UEs, where the PDCCH order would indicate the SSB index, RACH occasion and preamble ID. This implies that in the inter-DU case, the candidate DU cannot uniquely identify the UE that was ordered by the serving DU to send the PRACH.

Observation 1: In the inter-DU scenario, the candidate DU may not be able to identify the UE that sent the PRACH for early TA acquisition based on PID since the preamble resource can be shared among multiple UEs.

To resolve this issue, one solution would be that the serving DU and the candidate DU coordinate before the serving DU orders the UE to send the PRACH towards the candidate cell. However, this option incurs backhaul signaling overhead and latency.

Observation 2: Coordination between the serving DU and the candidate DU before triggering the UE to send the PRACH is one solution to enable sharing of preamble resources among UEs. However, this coordination incurs backhaul signaling overhead and latency.

Another solution is captured in Figure 1. In this solution, the set of dedicated preamble resources of a candidate cell used for early TA acquisition is partitioned among the various serving DUs. A serving DU always orders its own UEs to send PRACH towards the candidate cell using a member PID of the pool of PIDs allocated to that serving DU. In this manner, the candidate DU or the CU will be able to identify the serving DU of the UE that sent the PRACH based on the detected PID and forward the TA result along with the PID value towards that serving DU as a first step. In a second step, the serving DU associates the TA value to the UE that was ordered to send the PRACH using the latter PID value.

Proposal 1: For a given candidate cell, to enable sharing of preamble resources for early TA acquisition among UEs, these resources are partitioned into sets among the serving DUs:

- A serving DU only orders a UE to send PRACH to the candidate cell using a member PID of its set.

- The candidate DU/CU identify the serving DU from the received PID and forward the TA result with the PID value to the serving DU.

- The serving DU associates the TA value to the UE that was ordered to send the PRACH using the received PID value.

The above proposal requires RAN3 involvement.

Proposal 2: Send LS to RAN3 with proposal 1.

## 2.2	Support of early TA acquisition for deactivated SCell

In legacy, the UE does not have to retain UL synchronization for a deactivated SCell. In case the latter cell is an LTM candidate cell, early TA acquisition shall still be supported to enable RACH-less LTM execution towards that cell. In this case, the UE may transmit RACH towards the deactivated SCell based on PDCCH order.

Proposal 3: A UE may transmit RACH towards a deactivated serving SCell for early TA acquisition based on PDCCH order if the latter cell is an LTM candidate cell.

## 2.3	Supported TA values within the LTM cell switch command

If early TA acquisition via PDCCH-ordered RACH was performed prior to LTM triggering, the network includes the computed TA value into the LTM MAC CE based on PRACH reception.

If early TA acquisition via PDCCH-ordered RACH was skipped prior to LTM triggering, the network may still have TA value for the target cell available based on implementation. In this case, it should be possible that the network includes this TA value into the LTM MAC CE. From UE’s perspective, it is irrelevant whether this TA value is zero or same as a prior serving cell or any other value.

Proposal 4: The network may include a TA value into the LTM cell switch command even if PDCCH-ordered RACH towards the target cell was never performed prior to LTM triggering. How the network determines the TA value is up to implementation.

# 3	Aspects of RACH-less LTM

## 3.1	Which UL grant to use for RACH-less LTM?

RAN2#122 agreed two methods to provide an UL grant to the UE for the first UL data transmission to the target cell:

- Method 1: based on dynamic grant

- Method 2: based on configured grant

Which method the UE applies should be based on configuration:

- If the UE is configured with PDCCH search space of the target cell, Method 1 applies.

- If the UE is configured with a configured grant for the target cell, Method 2 applies.

- If the UE is configured with PDCCH search space of the target cell and a configured grant for the target cell, both methods apply.

Proposal 5: At LTM triggering, the UE may receive a dynamic grant and/or configured grant for the first UL data transmission based on whether the RRC configuration of the target cell includes a PDCCH search space of the target cell and/or a configured grant for the target cell respectively. The two configurations are not exclusive.

## 3.2	LTM enhancements for the configured grant used for the first UL data transmission

The UE may be configured with a configured grant for the first UL transmission. To reduce the interruption time, this configured grant may have short periodicity. Since it is unknown beforehand when LTM execution towards a candidate cell will be triggered, configuring a configured grant with short periodicity may be costly for the candidate cell (especially in the inter-DU scenario).

Observation 3: A configured grant for the first UL data transmission may be too costly for the candidate cell in terms of resource overhead since time of LTM triggering towards the candidate cell is unknown.

As an enhancement, the UE may only be allowed to use the configured grant to send the first UL data transmission. After the UE receives confirmation from the target cell that the transmission is successful, the configured grant is deactivated.

Proposal 6: As an enhancement, the configured grant for the first UL data transmission may be automatically deactivated after the UE receives confirmation from the target cell that the transmission is successful.

Another issue is that the UE’s first UL data transmission using the configured grant may fail. If the network detects the UE, the network may reschedule retransmission of the first UL data via a dynamic grant (as in legacy). Otherwise, the UE may be allowed to retransmit the first UL data using the configured grant itself.

Proposal 7: If the network detects but fails to decode the first UL data transmission of the UE, the network may schedule retransmission of the first UL data using a dynamic grant (as in legacy). As an enhancement, the UE may be allowed to retransmit the first UL data using the configured grant.

## 3.3	Determination of successful completion of RACH-less LTM by the UE

RAN2#121bis-e agreed that UE considers that LTM execution procedure is successfully complete when the UE determines the NW has successfully received its first UL data. Multiple options were discussed in [3]:

- Option 1: RLC ACK of RRCReconfigurationComplete

- Option 2: C-RNTI addressed PDCCH

- Option 3: UE Contention Resolution MAC CE

Option 3 is the solution for RACH-less handover in LTE. It should be noted though that for LTE the CRC for PDCCH is 16 bits, so just using PDCCH as in Option 2 increases the false alarm rate. For this reason, Option 3 provides additional reliability in the LTE context.

Observation 4a: In LTE, the CRC for PDCCH is only 16 bits, so the false alarm rate for determining successful RACH-less handover completion based on PDCCH addressing the UE’s C-RNTI would be of concern, so the UE Contention Resolution MAC-CE is needed.

On the other hand, the CRC for PDCCH in NR is 24 bits, so just using PDCCH addressing the UE’s C-RNTI as in Option 2 would yield a very low false alarm rate and is thus sufficient. Despite that NR NTN agreed to copy the LTE solution, there is no justification to do the same thing for LTM.

Observation 4b: In NR, the CRC for PDCCH is 24 bits, so the false alarm rate for determining successful RACH-less handover completion based on PDCCH addressing the UE’s C-RNTI is very low, so the UE Contention Resolution MAC-CE is NOT needed.

Option 2 is thus favored compared to Option 3. It is also favored compared to Option 1 which introduces latency due to the delay for the RLC ACK. We therefore support Option 2.

Proposal 8: [Different from RACH-less LTE and NTN] For RACH-less LTM, the UE determines successful reception of its first UL data based on receiving a PDCCH addressing the UE’s C-RNTI in the target cell.

## 3.4	Resume of DL scheduling by the target cell

The UE monitors for PDCCH from the target cell if the UE is configured with PDCCH search space of the target cell. PDCCH may schedule the UE in either UL or DL. It is up to the network whether PDCCH from the target cell schedules DL first before the UE sends its first UL data transmission.

Proposal 9: At LTM triggering, the UE may receive a dynamic grant from the target cell for either UL transmission or DL reception. It is up to the network whether DL scheduling is resumed before the UE sends its first UL transmission.

# 4	Failure handling

RAN2#121bis-e agreed to support fast recovery to a candidate cell by LTM execution in case of LTM execution failure or RLF. A similar solution to recovery by CHO can be used: the UE performs cell selection, and if the selected cell is the PCell for one of the LTM candidate configurations, the UE executes LTM towards that cell and applies the corresponding configuration. Otherwise, the UE performs RRC re-establishment at the selected cell.

Proposal 10: For MCG, at LTM execution failure or RLF, the UE performs cell selection:

- If the selected cell is a PCell for one of the LTM candidate configurations, the UE executes LTM towards that cell and applies the corresponding configuration as an MCG configuration.

- Else, the UE performs RRC re-establishment at the selected cell.

# 5	LTM for SCG

For the MCG switch case, RAN2#121bis-e assumed that the UE sends an RRC Reconfiguration Complete message to indicate its arrival to the target cell.

For the SCG switch case, neither SRB3 nor split SRB1 may be configured, so the RRC Reconfiguration Complete message goes via the MN. For RACH-less LTM, this implies a separate UL transmission is required by the UE to indicate arrival to the target SCG cell. Since the UE may have no UL data to send, a MAC CE should be used for this purpose.

Proposal 11: For RACH-less SCG switch case, the UE indicates arrival to the target SCG cell via a MAC CE.

# Conclusion

This contribution discussed early TA acquisition for LTM, aspects of RACH-less LTM, LTM failure handling, and LTM for SCG. The following observations and proposals have been made:

Early TA acquisition:

Observation 1: In the inter-DU scenario, the candidate DU may not be able to identify the UE that sent the PRACH for early TA acquisition based on PID since the preamble resource can be shared among multiple UEs.

Observation 2: Coordination between the serving DU and the candidate DU before triggering the UE to send the PRACH is one solution to enable sharing of preamble resources among UEs. However, this coordination incurs backhaul signaling overhead and latency.

Proposal 1: For a given candidate cell, to enable sharing of preamble resources for early TA acquisition among UEs, these resources are partitioned into sets among the serving DUs:

- A serving DU only orders a UE to send PRACH to the candidate cell using a member PID of its set.

- The candidate DU/CU identify the serving DU from the received PID and forward the TA result with the PID value to the serving DU.

- The serving DU associates the TA value to the UE that was ordered to send the PRACH using the received PID value.

Proposal 2: Send LS to RAN3 with proposal 1.

Proposal 3: A UE may transmit RACH towards a deactivated serving SCell for early TA acquisition based on PDCCH order if the latter cell is an LTM candidate cell.

Proposal 4: The network may include a TA value into the LTM cell switch command even if PDCCH-ordered RACH towards the target cell was never performed prior to LTM triggering. How the network determines the TA value is up to implementation.

RACH-less LTM:

Proposal 5: At LTM triggering, the UE may receive a dynamic grant and/or configured grant for the first UL data transmission based on whether the RRC configuration of the target cell includes a PDCCH search space of the target cell and/or a configured grant for the target cell respectively. The two configurations are not exclusive.

Observation 3: A configured grant for the first UL data transmission may be too costly for the candidate cell in terms of resource overhead since time of LTM triggering towards the candidate cell is unknown.

Proposal 6: As an enhancement, the configured grant for the first UL data transmission may be automatically deactivated after the UE receives confirmation from the target cell that the transmission is successful.

Proposal 7: If the network detects but fails to decode the first UL data transmission of the UE, the network may schedule retransmission of the first UL data using a dynamic grant (as in legacy). As an enhancement, the UE may be allowed to retransmit the first UL data using the configured grant.

Observation 4a: In LTE, the CRC for PDCCH is only 16 bits, so the false alarm rate for determining successful RACH-less handover completion based on PDCCH addressing the UE’s C-RNTI would be of concern, so the UE Contention Resolution MAC-CE is needed.

Observation 4b: In NR, the CRC for PDCCH is 24 bits, so the false alarm rate for determining successful RACH-less handover completion based on PDCCH addressing the UE’s C-RNTI is very low, so the UE Contention Resolution MAC-CE is NOT needed.

Proposal 8: [Different from RACH-less LTE and NTN] For RACH-less LTM, the UE determines successful reception of its first UL data based on receiving a PDCCH addressing the UE’s C-RNTI in the target cell.

Proposal 9: At LTM triggering, the UE may receive a dynamic grant from the target cell for either UL transmission or DL reception. It is up to the network whether DL scheduling is resumed before the UE sends its first UL transmission.

Failure handling:

Proposal 10: For MCG, at LTM execution failure or RLF, the UE performs cell selection:

- If the selected cell is a PCell for one of the LTM candidate configurations, the UE executes LTM towards that cell and applies the corresponding configuration as an MCG configuration.

- Else, the UE performs RRC re-establishment at the selected cell.

LTM for SCG:

Proposal 11: For RACH-less SCG switch case, the UE indicates arrival to the target SCG cell via a MAC CE.

# References

- Chairman Notes; TSG RAN WG2 Meeting #122

- Chairman Notes; TSG RAN WG2 Meeting #121bis-e

- R2-2304214, Summary of [AT121bis-e][018][eMob] Procedure Consolidation (Huawei), 3GPP TSG-RAN WG2 Meeting #121-bis-e