3GPP TSG-RAN WG2 Meeting #123	R2-2308626

Toulouse, France, 21 - 25 August 2023

Agenda Item:	7.13.6

Source:	Huawei, HiSilicon

Title:	Discussion on RACH enhancement

Document for:	Discussion and decision

1. Introduction

In RAN2#122 meeting, the following agreements and remaining issues were made for RACH enhancement:

Agreements:

RACH Partitioning

1	RAN2 confirms agreed “used feature combination” is all the features configured in the FeatureCombination applied for the RACH procedure.

2	Feature specific RACH information is included in RA-InformationCommon and is also included for RLF report and CEF report.

Msg3 repetition

3	Not include the number of Msg3 repetition applied in RACH procedure in RA report.

SN RACH Report

4	When reporting SN NR RA-report to LTE BS, the unique PSCell identities (i.e. if a PSCell occurs more than once in NR RA-ReportList, it is recorded only once in the list of PSCell identities) are included outside the NR RA report container.

5	Revert the agreement that UE does not support reporting NR RA report to LTE when it is in standalone LTE mode i.e., eNB may fetch the NR RA report irrespective to whether the UE is in single connectivity or dual connectivity.

6	No need to introduce availability bit to notify LTE BS there are available NR RA report for fetching.

7	Enhance the LTE UE information Request procedure with NR RA-Report request flag to fetch the NR RA-Report in LTE.

8	For NR RACH report, UE performs RPLMN checking before sending the NR RACH report to LTE BS.

9	A new UE capability is introduced to indicate whether UE supports NR RACH Report in LTE.

In this contribution, we take the agreed solutions and FFSs into account and discuss how to progress the work on this objective in the three aspects: RA partitioning leftover, SDT, and SN RA report.

2. Discussion

2.1 RA partitioning leftover

RACH partitioning specifies the selection of the set of RACH resources based on feature prioritization assigned in featurePriorities at the UE side when the intended feature or feature combination maps to more than one RACH resources. A mismatch might occur between the feature(s) which the selected RACH resource is indicated for and the feature(s) the UE intended to initiate via RA procedure. For instance, as illustrated in the below figure, When UE intends to initiate feature combination RedCap+SDT+CovEnh, UE select the RACH resource RedCap among the available sets of RACH resource based on feature prioritization, and finally initiate RA procedure for RedCap.

The mismatch happens mainly due to improper RA resource segmentation for feature(s) and inappropriate feature priorities, which need to be further optimized by network. When no exact matching RA resource is available for the feature combination, the UE will select a set of RA resources among the available sets of RA resources based on priority order. Knowing Feature priorities used by UE for selecting RA resource is useful for network. Currently, two possible options are discussed for UE to report the feature priorities to network.

Option 1: Include priorities information assigned for features;

Option 2: Implicitly indicated by the order of applicable features included in RA report

So one thing we would like to address is that, UE should report the feature priorities, not only just for the intended feature(s) but also for all the deployed feature(s) in order to help network decide which feature is in higher priority and which feature is in lower priority, and set the proper absolute value correspondingly. The specific value of feature priority reported by UE is more helpful for the network to decide the absolute value in an exact way. Therefore, we propose:

- UE Includes priorities information assigned for features.

The network may divide/partition the RA resource per feature or per feature combination, i.e., a preamble set is to be utilized for a feature or feature combination. For each configured preamble division, it has starting preamble index indicated by startPreambleForThisPartition-r17 and the total number of preambles can be used for this partition indicated by numberOfPreamblesForThisPartition-r17. Also it is possible for the network to change preamble division associated to RACH partition because the load on the RACH partition (feature combination) is changing over time. Considering the RA report is fetched afterwards, it is difficult for network to know the detailed RA resource allocated for RACH partition. If the network intends to optimize the RA resources allocated to this RACH partition, the RA report is preferred to log additional information correspondingly, i.e., starting preamble index and the total number of preambles, which helps network understands if the failed RA attempt is due to the RA load on certain feature is higher than the RA resource assigned for the feature therefore help optimizing the RA resource partition.

UE includes start preamble index and the number of preambles associated to used RACH partitioning in the RA report.

2.2 SDT

In RAN2#120 meeting it was agreed that UE includes RA and SDT information in the RA report if an SDT operation fails. what specific information regarding SDT operation to be provided by UE needs to be further studied. The first issue is that whether the network needs to distinguish whether the SDT operation fails. In our understanding, the network is interested in using RA report to identify any issue or potential issue deteriorating RA performance UE may encounter in a best effort way. Therefore, if SDT fails, the network needs to be aware of the failure. A separate indication could be used to indicate whether the SDT procedure is successful or not. Another possible solution is to use the intended/used feature combination together with a new value (SDT) of RA purpose. In our understanding, the intended/used feature combination and the new value of RA purpose only could denote that the RA procedure is initiated successfully for SDT or not, not reflecting whether the RA-SDT is completed successfully or not. So we recommend the separate indication solution.

A separate indication is introduced to indicate whether RA-SDT procedure is successful or not.

Another important aspect of the SDT configuration is the threshold for data volume to be transmitted, depending on which UE decide to use SDT or comes to connected via legacy RA process (SDT is intended if the pending data volume is smaller than or equal to the threshold; Otherwise the legacy RA procedure is intended). Some companies think of the legacy RA procedure as SDT operation failure, because the pending data is greater than the SDT data volume threshold. We believe that the normal legacy RA procedure for UL transmission should not and never be considered to be SDT failure, since the SDT operation is not intended in the first place. Therefore, we propose:

RAN2 clarify that the normal legacy RA procedure for UL transmission should not and never be considered to be SDT failure.

2.3 SN RA report forwarding in inter-MN handover case

We agreed that UE report a list of unique PSCell IDs outside NR RA report container to support the forwarding of SN NR RA reports. The LTE MN fetching the SN RA reports could use the PSCell ID to identify the corresponding NR SN node where the SN RA report is collected. However, Further study about whether the PSCell ID is sufficient to enable the forwarding in inter-MN handover case is required.

The inter-MN handover case attract a lot of attention due to that the direct interface for forwarding of SN RA report may not exist between the target MN fetching the SN RA reports and the correct SN. We opposed to the NG/S1 forwarding solution of RA report, which has been ruled against in the earlier Release for SON topic. Unlike the RLF report, handover report and SHR report via the CN, RA report forwarding via CN would occur much more frequently and consequently cause high signalling load at the CN.

Observation 1: Unlike the RLF report, handover report and SHR report via the CN, RA report forwarding via CN would occur much more frequently and consequently cause high signalling load at the CN.

Instead of CN forwarding of RA report, it is proposed to user the previous MN associated to the correct SN as intermediate node to forward the RA report. It is a certain thing that the previous MN has a direct interface with correct SN. Some companies have the concern that the target MN has no direct interface with previous MN in case of multi-hop inter-MN handover, for instance MN3 has no X2 interface with MN1 in inter-MN handover of MN1->MN2->MN3. In our understanding, once the UE access to the new E-UTRAN MN, since only one LTE RA report is allowed to be stored by UE and is overwritten to record a new RA report, the new MN needs to retrieve the RA report timely to avoid the loss of RA report, since there is a large support on requesting NR SN RA report simultaneously via the existing LTE RA report request in RAN2. So the worst case, i.e., MN2 does not retrieve SN RA report timely and MN3 has no direct interface with MN1, is a corner case.

Observation 2: The E-UTRAN MN always retrieve the RA report including LTE RA report and NR RA report timely since only one LTE RA report is available at the UE, and the LTE RA report and NR RA reports are most likely to be retrieved simultaneously.

To enable the previous MN as intermediate node of forwarding in alternative 1, the PCell ID associated to the PSCell ID should be included outside the NR container for the target MN to identify the previous MN.  Since the duplicated PCell IDs and PSCell IDs are only recorded once, the maximum overhead via Uu Interface is 16*(size of CGIs), i.e, for at most 8 PSCells and at most 8 PCells.

The unique PCell IDs corresponding to the PSCell ID, reported by UE, is recommended to enable the SN RA report forwarding in inter-MN handover case.

3. Conclusion

Based on the discussion in this contribution, we have the following observations and proposals:

- Observation 1: Unlike the RLF report, handover report and SHR report via the CN, RA report forwarding via CN would occur much more frequently and consequently cause high signalling load at the CN.

Observation 2: The E-UTRAN MN always retrieve the RA report including LTE RA report and NR RA report timely since only one LTE RA report is available at the UE, and the LTE RA report and NR RA reports are most likely to be retrieved simultaneously.

- UE Includes priorities information assigned for features.

- UE includes start preamble index and the number of preambles associated to used RACH partitioning in the RA report.

- A separate indication is introduced to indicate whether RA-SDT procedure is successful or not.

- RAN2 clarify that the normal legacy RA procedure for UL transmission should not and never be considered to be SDT failure.

- The unique PCell IDs corresponding to the PSCell ID, reported by UE, is recommended to enable the SN RA report forwarding in inter-MN handover case.

4. Reference