3GPP TSG-RAN WG2 Meeting #123	R2-2308628

Toulouse, France, 21 - 25 August 2023

Agenda Item:	7.13.8

Source:	Huawei, HiSilicon

Title:	Discussion on Fast MCG recovery and CPAC

Document for:	Discussion and decision

1. Introduction

Agreements on Fast MCG Recovery:

In RAN2#120 meeting, agreements regarding MRO for fast recovery have been achieved：

Fast MCG Recovery

Agreements

1	For fast MCG recovery MRO, prioritize NR-DC scenario. if time allows, study whether the same solution can be extended for others DC scenarios.

2	Consider at least below scenarios for fast MCG recovery MRO:

a.	T316 expiry

b.	SCG failure/deactivation during fast MCG recovery (i.e., running of T316). The “upon fast MCG recovery case” is FFS.

3	RLF report is enhanced to support fast MCG recovery MRO.

4	Fast MCG recovery failure cause shall be included for fast MCG recovery optimization. FFS details

RAN2#122 meeting has agreed to introduce the following cases:

Agreements:

1	RAN2 confirms scenario of near failure fast MCG recovery.

2	RAN2 confirms scenario f1, i.e., SCG fails or is deactivated before the UE sends the MCGFailureInformation. FFS RAN2 impact.

Agreements on CPAC:

In RAN2#120 meeting, agreements regarding MRO for CPAC have been achieved：

MRO for CPAC

Agreements:

1 RAN2 confirms the CPA/CPC scenarios agreed by RAN3 and discuss corresponding UE impacts.

2  SCGFailureInformation is enhanced to support CPAC MRO (i.e, no need to introduce new reports/message).

FFS: For CPAC MRO, information to differentiate CAPC from conventional SCG failure is needed (ffs by implicit or explicit indication).

Meanwhile in the RAN3#119 meeting, there is some progress about UE reporting for MRO for CPAC as follows:

Information available in the network nodes should not be included in the SCGFailureInformation.

For MRO for CPC and CPA, if there are multiple events configured for CPA/CPC, the UE reports the first triggered CPAC event, and the time duration between the two triggered CPAC events.

In this paper, for the agreed scenarios and the remaining FFS, we will give further consideration and potential solution to support this objective.

FFS: For CPAC MRO, information to differentiate CAPC from conventional SCG failure is needed (ffs by implicit or explicit indication).

For further discussion:

Proposal 12: For CPAC MRO, RAN2 discuss which of below time information is needed to be reported by UE (ffs reusing existing existing new IEs or introducing new IEs ):

a. The time elapsed between reception of CPAC configuration and the CPAC execution (4)

b. The time elapsed between the SCG failure in source SCG and the latest CPC configuration is received. (3)

c. The time elapsed between the CPAC execution towards the target PSCell and the corresponding latest CPAC configuration is received for the target PSCell. (2)

d. The time elapsed since the CPAC execution towards the target PSCell until the SCG failure. (1)

e. timeSinceCPACReconfig (1)

Proposal 14: For CPAC MRO, RAN2 discuss whether below type information is needed (ffs details):

a. Information to differentiate CPA and CPC

b. Information to differentiate SN or MN initiated CPAC procedure

c. None

Proposal 15: For CPAC MRO, RAN2 discuss which below CPAC configuration relevant information is needed:

a. The time elapsed between the two CPAC execution events e.g., A3 and A5 (4)

b. The first fulfilled event e.g., A3 or A5 (3)

c. The latest configured CPAC configuration including the CPAC execution condition(s) (2)

d. CPAC execution condition(s) fulfilled (2)

e. indication whether UE had stored intra-SN CPC configuration at the time of SCG failure. (1)

f. an indication regarding target cells that were previously reported to the network but not part of the received CPC configurations. (1)

g. None

Proposal 16: For CPAC MRO, RAN2 discuss which below measurements relevant information is needed:

a. CPAC candidate PSCells identity (3)

b. Candidate PSCell measurement results  (2)

c. the latest neighboring cell measurement results (3)

d. an indication on whether a measured neighbour cell was configured as a CPAC candidate or not (2)

e. Source PSCell info (cell ID and measurements) (1)

f. Target PSCell ID; (1)

g. Target PSCell  measurements (1)

h. None

Proposal 17: For CPAC MRO, RAN2 discuss which below information is needed:

a. the MCG timers such as t310 and t312 values if they are running

b. RLF retransmission counter value

c. one indication that before PCell handover, CPA/CPC configuration has received but not executed or the time difference between the CPA/CPC configuration and PCell handover

d. Success PSCell change/addition cause value (e.g., t304, t310, t312 cause, etc.)

e. None

In this paper, we shall provide our further considerations to the above issues.

2. Discussion

2.1 MRO for Fast MCG recovery

Further study for the optimization of the near failure scenario is required in the aspects including the definition of near failure (i.e. triggering condition), information reported for the UE, and the utilized report type.

In general, although the fast MCG recovery succeeds, there is underlying issues which might cause an actual fast MCG recovery failure. Besides, the consensus has been reached that the potential issue of fast MCG recovery failure could be lying both in the T316 setting and SCG RLF. Discussion on T316 related triggering condition has been going on for a while. On one hand, the too short T316 may lead to actual MCG recovery failure or near MCG recovery failure frequently, and on the other hand, the too long T316 may take UE wait too long before reestablishment upon detecting fast MCG recovery failure, which cause long service interruption. It is beneficial for the network to adjust the setting of T316. Except for T316 related triggering condition, the following potential issues regarding SCG failure detected by UE should be discussed:

- UE detect the radio link failure for the SCG due to T310 expiry

- UE detect the radio link failure for the SCG due to T312 expiry

- UE receives random access problem indication from SCG MAC, e.g., maximum number of preamble transmissions has been reached.

- UE receives indication form SCG RLC that the maximum number of retransmissions has been reached.

Therefore, in order to make proper adjustment in near failure case, the potential issues regarding SCG RLF need to be identified and solved. So we recommend the definition of near failure of fast MCG recovery should consider the triggering condition regarding SCG near failure as below:

- the ratio of SCG T310

- the ratio of SCG T312

- the number of preamble transmissions towards SCG has reached a threshold

- the number of SCG RLC retransmission has reached a threshold

- RAN2 to introduce the T316 related triggering threshold for the near failure scenario of fast MCG recovery.

- RAN2 to introduce the following triggering conditions regarding SCG near failure for the near failure scenario of fast MCG recovery：

      - the ratio of SCG T310

      - the ratio of SCG T312

      - the number of preamble transmissions towards SCG has reached a threshold

      - the number of SCG RLC retransmission has reached a threshold

Currently, the following cases for fast MCG recovery failure has been addressed both in RAN2 and RAN3:

- Case a: SCG fails when the UE is undergoing fast MCG recovery (i.e. SCG failure happens while T316 is running)

- Case f1: SCG fails or is deactivated yet before the UE sends the MCGFailureInformation

- Case b: the signalling delay is longer than the time the UE waits for the response (T316 expired)

- The difference between case a and f1 is that in case a, SCG fails during MCG RLF, and in case f1 MCG RLF occurs during SCG RLF or deactivation. So, whether the T316 starts(is) running distinguish case a from case f1, while the common thing between the two cases is during a short period both MCG failure and SCG failure/deactivation happen in a nearby location. When it is the case in which both the two incidents are failures, undoubtedly there is a coverage since the UE cannot get service at all. When it is the case in which one incident is failure and the other is SCG deactivation，the network could consider when & where to  deactivate the SCG is inappropriate, e.g. near the location of MCG failure. The time information between the two incidents among those 3 agreed failure cases is helpful for the network to enhance the mobility and reduce the improper SCG deactivation for fast MCG recovery optimization.

UE reports following time information for fast MCG link recovery optimization:

- Time between MCG failure (or transmitting MCGFailureInformation) and SCG failure for case a and f1

- Time between MCG failure (or transmitting MCGFailureInformation) and SCG deactivation for case f1

One thing we would like to address, when we discuss about near failure case, UE could only report the information when a triggering condition is met, e.g. T316 exceeds a threshold, or SCG T310 exceeds a threshold. The motive of defining a near case is to reduce the unnecessary reporting of a purely successful case. Therefore, we propose that either SHR or a SHR-like new report is utilized to cover the near failure case.

Either SHR or a SHR-like new report is utilized to cover the near failure case, including the trigger cause and SCG measurement results.

- Except for the already agreed cases, it should be noticed that there is a possibly that a HOF occurs after successful MCG recovery procedure, because the Ho decision is a failure handling decision based on outdated measurement result instead of a normal HO decision.

UE indicates which kind of RAR received from MN via SN, e.g., Hanover command or RRCRelease. If it is handover command, UE indicates the failed target Cell ID.

- When UE detects fast MCG recovery failure, UE immediately initiated RRC re-establishment procedure. A special handling during the re-establishment procedure, called CHO-based recovery, is UE executes HO towards the selected CHO candidate Cell, if UE has the stored CHO candidate configuration and select a CHO candidate Cell,. A HOF might occur due to improper CHO candidate configuration. Besides, the improper CHO configuration also takes responsibility on the MCG RLF due to too late CHO. The two failure events should be correlated and coordinated to adjust the CHO configuration properly.

UE indicates the failed CHO cell in CHO based recovery procedure for MRO analysis of correlating too late CHO with CHO based recovery.

2.2 MRO for CPAC

RAN2 has confirmed to use SCGFailureInformation for CPAC MRO, which is immediately reported by UE upon declaring SCG failure. Besides, some companies have concerns about increasing the size of SCGFailureInformation by adding information intended for MRO purpose. Besides, RAN3 also confirmed not to include information available at the network to SCGFailureInformation. We might as well take advantage of the immediate reporting of SCGFailureInformation. The very obvious benefit is that the UE context is always available at the network side (MN or last serving SN). we would rather think about the CPAC MRO enhancement in the perspective of minimizing the added information.

2.2.1 Time relevant

For further discussion:

Proposal 12: For CPAC MRO, RAN2 discuss which of below time information is needed to be reported by UE (ffs reusing existing existing new IEs or introducing new IEs ):

a. The time elapsed between reception of CPAC configuration and the CPAC execution (4)

b. The time elapsed between the SCG failure in source SCG and the latest CPC configuration is received. (3)

c. The time elapsed between the CPAC execution towards the target PSCell and the corresponding latest CPAC configuration is received for the target PSCell. (2)

d. The time elapsed since the CPAC execution towards the target PSCell until the SCG failure. (1)

e. timeSinceCPACReconfig (1)

First, we noticed that in [1] the definition of bullet e is to measure the elapsed time between the initiation of CPAC execution and the reception of CPAC config. Technically speaking, the time in bullet e is totally the same as the one in bullet a. So, we can merge bullet e to bullet a.

As we all know, in CHO, the UE decides the CHO execution without notification to the source MN. In Rel-17 CHO MRO, it is a shared understanding that it is important to know the CHO execution moment related time, which may be beneficial to retrieve the related CHO configuration for MRO purpose. The IE timeSinceCHO-Reconfig is introduced to provide the CHO execution moment related time information.

timeSinceCHO-Reconfig In case of handover failure, this field is used to indicate the time elapsed between the initiation of the last conditional reconfiguration execution towards the target cell and the reception of the latest conditional reconfiguration. In case of radio link failure, this field is used to indicate the time elapsed between the radio link failure and the reception of the latest conditional reconfiguration while connected to the source PCell. Actual value = field value * 100ms. The maximum value 1023 means 102.3s or longer.

Based on the similar consideration, some companies suggest introducing the time info in bullets a/e and c, which indicates the time elapsed between the reception of CPAC configuration and the CPAC execution.

To start the discussion, we can take the MN-initiated CPAC for example and the general flow is showed in Figure 2.

Figure 2: MN-initiated CPAC case

As illustrated in the above flow, it is worth noting that different from the CHO procedures, the UE will send the second RRCReconfigurationComplete message to the source MN if the CPAC is executed. In this message, the UE informs the source MN of the selected target candidate PSCell. Upon receiving the message, the MN can know the CPAC execution moment related information.

When detecting the SCG failure, the UE will immediately send the SCGFailureInformation message to the source MN. Correspondingly, the MN can be aware of the time when the SCG failure occurs. Based on the above information, it is believed that the MN can derive the time in bullets b and c, e.g., by computing time between transmission of RRCReconfig containing CPAC config and reception of SCGFailureInforamtion for bullet b and computing time between reception of RRCReconfigComplete corresponding to the selected PSCell and reception of SCGFailureInforamtion for bullet c.

Observation 1: The MN can derive the time in bullets a~e:

a. The time elapsed between reception of CPAC configuration and the CPAC execution

b. The time elapsed between the SCG failure in source SCG and the latest CPC configuration is received.

c. The time elapsed between the CPAC execution towards the target PSCell and the corresponding latest CPAC configuration is received for the target PSCell.

d. The time elapsed since the CPAC execution towards the target PSCell until the SCG failure.

e. timeSinceCPACReconfig

For CPAC MRO, the time information below is NOT needed to be reported by UE:

a. The time elapsed between reception of CPAC configuration and the CPAC execution

b. The time elapsed between the SCG failure in source SCG and the latest CPC configuration is received.

c. The time elapsed between the CPAC execution towards the target PSCell and the corresponding latest CPAC configuration is received for the target PSCell.

d. The time elapsed since the CPAC execution towards the target PSCell until the SCG failure.

e. timeSinceCPACReconfig

2.2.2 CPAC type relevant

For further discussion:

Proposal 14: For CPAC MRO, RAN2 discuss whether below type information is needed (ffs details):

a. Information to differentiate CPA and CPC

b. Information to differentiate SN or MN initiated CPAC procedure

c. None

We believe it is necessary to differentiate the CPA and CPC from classic PSCell addition/change at the MN side. Furthermore, for CPC, it is also reasonable to differentiate SN or MN initiated CPC procedure. This will be helpful for the MN to decide whether the SN is the initiating node and whether to forward the SCGFailureInformation to it. Currently, the UE will immediately report the SCGFailureInfomration upon SCG failure. It is believed that the MN is still maintaining the UE context and can retrieve the above information based on the UE context. Therefore, there is no need for the UE to report the above information explicitly.

Besides, the information for CPAC MRO to be enhanced into the SCGFailureInformation is still under discussion. If new CPAC information is introduced, explicit indication is not needed either.

For CPAC MRO, the information is NOT needed by explicit indication from UE:

a. Information to differentiate CPA and CPC

b. Information to differentiate SN or MN initiated CPAC procedure

2.2.3 CPAC configuration relevant

For further discussion:

Proposal 15: For CPAC MRO, RAN2 discuss which below CPAC configuration relevant information is needed:

a. The time elapsed between the two CPAC execution events e.g., A3 and A5 (4)

b. The first fulfilled event e.g., A3 or A5 (3)

c. The latest configured CPAC configuration including the CPAC execution condition(s) (2)

d. CPAC execution condition(s) fulfilled (2)

e. indication whether UE had stored intra-SN CPC configuration at the time of SCG failure. (1)

f. an indication regarding target cells that were previously reported to the network but not part of the received CPC configurations. (1)

g. None

For bullets a and b, RAN3#119 meeting has agreed the benefit of reporting bullet a and b, i.e, For MRO for CPC and CPA, if there are multiple events configured for CPA/CPC, the UE reports the first triggered CPAC event, and the time duration between the two triggered CPAC events.

With the information of bullets a and b, the network can fully know the triggering situation of the CPAC execution conditions. Therefore, bullet d is not needed. For bullets c and f, this can depend on network implementation to store the related CPAC configuration including the CPAC execution conditions. As for bullet f, if the UE reported the measurement results to the network, the network can know whether each reported cell is included into the CPAC configuration. There is no meaningful for the UE to redundantly report such information.

- Proposal 9a:  For CPAC MRO, the triggering event information is needed as indicated in RAN3 LS:

a. The time elapsed between the two CPAC execution events e.g., A3 and A5

b. The first fulfilled event e.g., A3 or A5

- Proposal 9b:  For CPAC MRO, the triggering event information is NOT needed:

c. The latest configured CPAC configuration including the CPAC execution condition(s)

d. CPAC execution condition(s) fulfilled

e. indication whether UE had stored intra-SN CPC configuration at the time of SCG failure

f. an indication regarding target cells that were previously reported to the network but not part of the received CPC configurations

g. None

2.2.4 Measurements relevant

For further discussion:

Proposal 16: For CPAC MRO, RAN2 discuss which below measurements relevant information is needed:

a. CPAC candidate PSCells identity (3)

b. Candidate PSCell measurement results (2)

c. the latest neighboring cell measurement results (3)

d. an indication on whether a measured neighbour cell was configured as a CPAC candidate or not (2)

e. Source PSCell info (cell ID and measurements) (1)

f. Target PSCell ID; (1)

g. Target PSCell measurements (1)

h. None

First of all, we agree on bullet c. the latest neighboring cell measurement results. This will make sense for the subsequent configuration optimization. If CPAC candidate PSCells can be measured, the results can be reported in the latest neighboring cell measurement results as bullet-b; otherwise, the UE doesn’t need to report the CPAC candidate PSCell ID in bullet-a additionally which is out of the measurement results.

Proposal 10a: For CPAC MRO, the following measurements relevant information is needed:

c. the latest neighboring cell measurement results

Besides, due to immediately reporting of the SCGFailureInformation, it is expected that the network can know the latest CPAC configuration. By comparing the reported measurement results and the stored CPAC configuration, the network can identify whether a measured neighbour cell was configured as a CPAC candidate or not. The UE is not needed to include bullet d related information as the measurement relevant information.

In Rel-17 classic PSCell change case, the intra-SN PSCell change is transparent to the MN. The UE need report the source and target PSCell information. In this way, the MN can decide the right SN node responsible for the SCG failure. However, in CPC, the MN is always aware of the occurrence of the CPC no matter whether it is intra- or inter-SN CPC. Correspondingly, when the MN receives the SCGFailureInformaiton, it will definitely know the initiating node for the SCG failure. Besides, as showed in the flow in the above section, it is known that the UE has reported the selected target candidate PSCell to the MN. Taking into consideration the previous discussion, bullets e, f and g are not needed.

Proposal 10b: For CPAC MRO, the below measurements relevant information is NOT needed:

a. CPAC candidate PSCells identity

b. Candidate PSCell measurement results

d. an indication on whether a measured neighbour cell was configured as a CPAC candidate or not

e. Source PSCell info (cell ID and measurements)

f. Target PSCell ID

g. Target PSCell measurements

h. None

2.2.5 Others

For further discussion:

Proposal 17: For CPAC MRO, RAN2 discuss which below information is needed:

a. the MCG timers such as t310 and t312 values if they are running

b. RLF retransmission counter value

c. one indication that before PCell handover, CPA/CPC configuration has received but not executed or the time difference between the CPA/CPC configuration and PCell handover

d. Success PSCell change/addition cause value (e.g., t304, t310, t312 cause, etc.)

e. None

In our understanding, all the information is for the case which mixes MCG failure/handover or successful CPAC case. We prefer to down-select the discussion on the above information.

Proposal 11: For CPAC MRO, RAN2 deprioritizes the discussion of the below information:

a. the MCG timers such as t310 and t312 values if they are running

b. RLF retransmission counter value

c. one indication that before PCell handover, CPA/CPC configuration has received but not executed or the time difference between the CPA/CPC configuration and PCell handover

d. Success PSCell change/addition cause value (e.g., t304, t310, t312 cause, etc.)

e. None

3. Conclusion

Based on the discussion in this contribution, we have the following proposals:

- RAN2 to introduce the T316 related triggering threshold for the near failure scenario of fast MCG recovery.

- RAN2 to introduce the following triggering conditions regarding SCG near failure for the near failure scenario of fast MCG recovery：

      - the ratio of SCG T310

      - the ratio of SCG T312

      - the number of preamble transmissions towards SCG has reached a threshold

      - the number of SCG RLC retransmission has reached a threshold

UE reports following time information for fast MCG link recovery optimization:

- Time between MCG failure (or transmitting MCGFailureInformation) and SCG failure for case a and f1

- Time between MCG failure (or transmitting MCGFailureInformation) and SCG deactivation for case f1

Either SHR or a SHR-like new report is utilized to cover the near failure case, including the trigger cause and SCG measurement results.

UE indicates which kind of RAR received from MN via SN, e.g., Hanover command or RRCRelease. If it is handover command, UE indicates the failed target Cell ID.

UE indicates the failed CHO cell in CHO based recovery procedure for MRO analysis of correlating too late CHO with CHO based recovery.

Observation 1: The MN can derive the time in bullets a~e:

a. The time elapsed between reception of CPAC configuration and the CPAC execution

b. The time elapsed between the SCG failure in source SCG and the latest CPC configuration is received.

c. The time elapsed between the CPAC execution towards the target PSCell and the corresponding latest CPAC configuration is received for the target PSCell.

d. The time elapsed since the CPAC execution towards the target PSCell until the SCG failure.

e. timeSinceCPACReconfig

For CPAC MRO, the time information below is NOT needed to be reported by UE:

a. The time elapsed between reception of CPAC configuration and the CPAC execution

b. The time elapsed between the SCG failure in source SCG and the latest CPC configuration is received.

c. The time elapsed between the CPAC execution towards the target PSCell and the corresponding latest CPAC configuration is received for the target PSCell.

d. The time elapsed since the CPAC execution towards the target PSCell until the SCG failure.

e. timeSinceCPACReconfig

For CPAC MRO, the information is NOT needed by explicit indication from UE:

a. Information to differentiate CPA and CPC

b. Information to differentiate SN or MN initiated CPAC procedure

- Proposal 9a:  For CPAC MRO, the triggering event information is needed as indicated in RAN3 LS:

a. The time elapsed between the two CPAC execution events e.g., A3 and A5

b. The first fulfilled event e.g., A3 or A5

- Proposal 9b:  For CPAC MRO, the triggering event information is NOT needed:

c. The latest configured CPAC configuration including the CPAC execution condition(s)

d. CPAC execution condition(s) fulfilled

e. indication whether UE had stored intra-SN CPC configuration at the time of SCG failure

f. an indication regarding target cells that were previously reported to the network but not part of the received CPC configurations

g. None

Proposal 10a: For CPAC MRO, the following measurements relevant information is needed:

c. the latest neighboring cell measurement results

Proposal 10b: For CPAC MRO, the below measurements relevant information is NOT needed:

a. CPAC candidate PSCells identity

b. Candidate PSCell measurement results

d. an indication on whether a measured neighbour cell was configured as a CPAC candidate or not

e. Source PSCell info (cell ID and measurements)

f. Target PSCell ID

g. Target PSCell measurements

h. None

Proposal 11: For CPAC MRO, RAN2 deprioritizes the discussion of the below information:

a. the MCG timers such as t310 and t312 values if they are running

b. RLF retransmission counter value

c. one indication that before PCell handover, CPA/CPC configuration has received but not executed or the time difference between the CPA/CPC configuration and PCell handover

d. Success PSCell change/addition cause value (e.g., t304, t310, t312 cause, etc.)

e. None

4. Reference

- R2-2213200, Summary on 8.13.8 others, ZTE